A1200 Hardware Frequently Asked Questions

Version 1.0

-----------------------------------------

Copyright � 1995 by Warren Block.

Distribution

------------

This document is FreeWare. It has been copyrighted to assure its availability

to everyone. Fred Fish and the Aminet are explicitly allowed to include this

document in CD-ROM or floppy compilations; others should ask first.

Disclaimer

----------

The information contained herein is a crock. It's full of dangerous errors,

omissions, and outright lies, and is not guaranteed to do anything other than

take up disk space. Not for internal use. Keep out of reach of demo coders.

Introduction

------------

After putting together a much fancier document for the A4000 (the Amiga 4000

Hardware Reference Guide), it was suggested that a something similar was

needed for the A1200. Additionally, I kept seeing the same questions asked

over and over in comp.sys.amiga.hardware, and it seemed like a FAQ might help

both those asking and answering the questions. Maybe it'll just change all

the answers to say "Read the FAQ, you moron." Nah. Amiga people are more

polite than that. They'd say "PLEASE read the FAQ, you moron."

Introductory Questions

----------------------

Q. I have a problem with my A1200. Can I email or call you for help?

A. I'd rather you ask any questions that aren't answered here in a public

forum in comp.sys.amiga. That way, you can draw on more experience than

just my own (which is admittedly somewhat limited with the A1200), and the

answers can benefit others with similar problems.

Q. Hey! You left out many details on some of these topics.

A. That was intentional; some of these answers have been simplified, both to

keep this document relatively short, and to avoid confusing the issues.

Please remember that this is an introductory document, and not a technical

reference guide, and is intended merely to answer the most common

questions. Nyah.

Q. Hey, FAQ boy, you screwed up, and some of this information is wrong, or

you just didn't include something that was completely obvious! Can I send

you email with the correct information?

A. Yes, please do. I am:

Warren Block

602 St. James

Rapid City  SD  57701

(605) 342-1632 Voice

wblock@silver.sdsmt.edu

Thank you for your interest, and good luck with your A1200!

------------------------------------------------------------------------------

Power Supply

------------

Q. I want to build a "super power supply" for my A1200. What are the pinouts

of the power supply connector?

A. Look on the power supply itself, where the pinouts are specified.

Q. Can I use an Amiga 500 power supply with my A1200?

A. Yes. The connectors and voltages match perfectly.

Q. What about the higher watt rating of the A500 supply? Won't all that extra

power melt and/or blow up my A1200?

A. No. The watt rating of a power supply only describes the absolute maximum

amount of power it can supply. The A1200 will only draw as much of this as

it needs to operate.

Floppy Drives

-------------

Q. Can I use an IBM high density drive on an A1200?

A. Not easily, simply, or reliably.  There are some hacks that allow this,

but, as of this writing, they all still have problems, and can be

considered experimental. If you're determined to try, there are several

plans in the hard/hack section on Aminet.

Q. Can I read and write Amiga diskettes on a PC clone?

A. No. The floppy controller hardware on the clones is incapable of reading

or writing the Amiga disk format. To transfer files on disk between a PC

clone and an A1200, use 3.5-inch low-density floppies formatted in the

720K MS-DOS format. The A1200 can read and write these using CrossDOS

(included with 3.0).

Q. Can I use high-density floppies with the A1200's normal disk drive?

A. Not reliably! Writing high-density disks with a low-density drive will

result in erratic operation of those disks. They may work fine for hours,

days, or weeks, only to fail when you need them most. The low-density

drives just can't write reliably to high-density disks. Use low-density

disks with Amiga low-density (880K) drives, like the one built into the

A1200.

Hard Drives

-----------

Q. Why doesn't my A1200 boot off the hard drive when I first turn it on?

A. Many hard drives don't spin up fast enough to be ready when the A1200

is ready to boot. If you can reset the A1200 after it comes up with the

purple "insert floppy" screen and it will then boot from the hard drive,

this is the case. The easiest solution is to just live with it by

resetting the machine when you first turn it on. If this really bothers

you, you could replace the drive with one that spins up faster.

Q. How hard is it to install a hard drive inside my A1200?

A. It's not hard. You need a few things: a 2.5-inch IDE hard drive (commonly

used for notebook computers), a short section of special 44-pin ribbon

cable to fit the drive (this cable is spaced more tightly than normal

ribbon), and, preferably, the Commodore Install disk. First, you install

the drive, then use the Install disk to partition it and place the system

software on it. That's it. The toughest part is obtaining the Install disk

and the special cable.

Q. When should I low-level format the hard drive?

A. On both IDE and SCSI drives, an initial low-level format has been

performed at the factory. You should never need to low-level format them

again. After partitioning, you *do* need to "high-level" format a hard

drive with the AmigaDOS Format command; when used with hard drives, you

can specifiy the QUICK keyword with the Format command to speed up this

high-level formatting immensely.

Q. Can I use a 3.5-inch hard drive inside my A1200?

A. Not without hacking it in there. Physically, it can be made to fit;

but problems can arise from the extra power consumption of the 3.5-inch

drives, coupled with the anemic stock power supply of the A1200. Obtaining

a cable to adapt the 44-pin high-density IDE cable to the 40-pin

normal-density IDE connector can be difficult, too.

Q. Can I use an "EIDE" or "Fast ATA" hard drive with the A1200?

A. Yes. These drives are merely refined IDE drives.

Q. Will large-capacity (500M and up) drives work with the A1200?

A. Yes. The 500M capacity "limitation" originates from the PC clones, and is

only a problem with them. For the Amiga, you can safely go up to a 2G

(~2000M) hard drive, provided you can find (and afford) one that large in

a 2.5-inch IDE or EIDE format.

Q. What is MaxTransfer?

A. MaxTransfer is a value (set in HDToolBox) that limits how much data can be

transferred to or from the hard drive in one "gulp."

Q. Why is MaxTransfer needed? Shouldn't the drive move as much data at one

time as possible?

A. Yes, it should, but some hard drives have arbitrary limits on how much

data they can move at one time. On the Amiga, if you have problems

transferring files of 100K or larger to or from the hard drive, this is a

likely cause of the problem.

Q. I have have no idea what the MaxTransfer value should be for my drive.

What value should I use?

A. For unknown drives, the maximum "safe" value is 0x0001FE00.

Q. What drives are known to need the "safe" MaxTransfer values?

A. As of this writing, some Maxtor drives need the maximum safe MaxTransfer

value of 0x0001FE00.

Q. What is Mask?

A. Mask is a value (set in HDToolBox) that determines what type of memory

(Chip or Fast) can be used to buffer data transfers to or from the hard

drive. Unless you have a good reason to change it, leave it at the default

setting in HDToolBox.

Q. What 2.5-inch IDE hard drives are known to work with the A1200?

A. Most do, although you may have to adjust the MaxTransfer value (see

above). I personally dislike the Seagate drives included with many A1200s

(I consider them the Kraco of hard drives), but they work fine for many

people. My own Toshiba works fine, although, like many drives, it has the

"slow spin-up" problem.

CD-ROM Drives

-------------

Q. What SCSI CD-ROM drives work with the A1200?

A. First you need a SCSI interface, like the Dataflyer SCSI+, the SCSI

interface built into certain accelerators, or a PCMCIA SCSI card. Some

SCSI controllers have trouble with certain drives, but in general, the

Apple CD300 (which is a Sony drive), Toshiba, and NEC drives have been

reported to work well with the Amiga. Personally, I can verify that the

NEC 3Xe works fine with the SCSI+ interface.

Q. Can an IDE CD-ROM drive be used with the built-in IDE of the A1200?

A. It's doubtful. Most non-SCSI CD-ROM drives are not IDE, but a "sort-of"

IDE. Newer drives are EIDE, but reports seem to indicate that they don't

function properly on the A1200 without additional software or hardware.

My opinion: SCSI gives you much more for your money, because you can use

the SCSI bus for other things in addition to CD-ROM drives. Some

accelerator boards have SCSI ports, offering an all-in-one solution.

Q. I need more information on CD-ROM disks, drives, and interfaces,

especially as they relate to the Amiga. Where can I look?

A. There is an excellent overview of the subject by Joachim E. Deu�en on

Aminet. Look at the file docs/help/CDROM20.lha.

Processors

----------

Q. What is the 68EC020 processor that comes inside the A1200?

A. It is an inexpensive version of the 68020, in a Plastic Leaded Chip

Carrier (PLCC) package. The only electronic difference between this

processor and the 68020 is that the EC version only brings out signals

for 24-bit addressing.

Q. What's the difference between the 68EC030 and 68030 processors that come

with accelerator boards?

A. The two are physically the same, with the difference being that a 68030

has a functional Memory Management Unit built into it, which the EC

version lacks. The MMU is useful for programmers (for debugging purposes),

allows running Unix systems like NetBSD, and also can be used to remap

Kickstart code, allowing the system to run faster (in some cases--this

depends on the individual system, accelerator board, and memory).

Q. Are there any 68040 or 68060 accelerators for the A1200?

A. Not at this time. It is probably impractical to build a 68040 accelerator

for the A1200, due to the amount of heat the 68040 generates and the

limited space inside the A1200. The 68060 uses less power and generates

less heat, but no 68060 accelerators are available for the A1200 at

present.

Q. Can I replace the 68EC020 that comes with the A1200 with a full 68020?

A. Not directly, no. The 68EC020 is soldered onto the A1200 motherboard, and

even if it weren't, the 68020 differs physically from it. The best way to

perform this upgrade is with one of the many inexpensive accelerator

boards.

Q. Can I replace a 68EC030 with a 68030 with an MMU?

A. Yes, as long as they're physically the same. Some 68030s are Pin Grid

Array (PGA), and some are Plastic Leaded Chip Carrier (PLCC). With the PGA

type, you can carefully and gently pry the old chip out of the socket and

then replace it with the new one. Some PLCC chips may be soldered down,

some may be in sockets. If it's in a socket, yes, you can replace it, but

you should get the proper removal tool to avoid breaking the socket.

Memory

------

Q. What do I need to expand the memory of my A1200?

A. You'll need either a RAM board or an accelerator board, either of which

will fit into the trap-door expansion port on the bottom of the A1200, and

will have sockets for memory expansion. Please note that some expansion

boards made by GVP use custom SIMM modules, which are hard to find and

much more expensive than standard 72-pin SIMMs.

Q. My accelerator or RAM board has one or more 72-pin sockets for memory

SIMMs. Can I use 72-pin SIMMs meant for PC clones?

A. Yes. Read on for more details.

Q. Some 72-pin SIMMs are 36-bit and some are 32-bit. Is this related to

"parity," and which do I need to use with the A1200?

A. PC clones use extra "parity" bits for double-checking memory. On the

A1200, these extra bits will simply be ignored, causing no problems. The

extra four bits on the 36-bit wide SIMMs are these parity bits. The 32-bit

wide SIMMs are usually meant for use on the Macintosh, which, like the

Amiga, don't use parity. Summing up: 36- or 32-bit wide SIMMs--which is

just another way of saying "with or without parity bits"--will work for

the accelerator or RAM expansion boards with 72-pin SIMM sockets.

Q. I've seen SIMMs with speed ratings of 60, 70, and 80 ns. What does this

mean, and which do I need?

A. These numbers rate the speed at which the SIMM can be reliably operated. A

60 ns SIMM is the fastest currently available at reasonable prices. To

keep a processor running at its fastest rate, the memory needs to be able

to keep up with it; the speed of your processor determines the minimum

memory speed needed. For an A1200 with just a RAM expansion board, 80 ns

SIMMs are fast enough. With an accelerator, it depends on how fast the

processor is going, so you'll need to check the manual. (Note: many boards

let you use slower SIMMs with them by enabling a "wait state," which

forces the processor to slow down to the SIMM's speed, and allows you to

use cheaper SIMMs. For instance, an accelator may say that it requires 70

ns SIMMs to run at "zero wait states," but will operate with an 80 ns SIMM

(slower) if you set a jumper to enable a wait state.)

Q. If I get faster SIMMs than I need, will it make the computer run faster?

A. No. This is a common misconception. The processor determines how fast the

memory needs to be, and will not run any faster even if the memory is

capable of it. For instance, if your accelerator needs a 70 ns SIMM, and

you use a 60 ns SIMM, it will not run any faster than it would with a 70

ns SIMM. (It may be a good idea to buy faster RAM than you need, though,

since you could then use it with a faster processor in the future.)

Monitors

--------

Q. I have a VGA or SVGA monitor from a PC clone. Can I use it with the A1200?

A. Probably. The "double" video modes of the A1200's AGA chipset provide

video frequencies of 23 kHz and up, and will work with many VGA monitors.

However, true PC clone-type VGA operates at a frequency of 31.5 kHz, and

some monitors won't operate at frequencies much lower than that. The "VGA

Only" monitor driver may be used to raise the A1200 output frequencies,

making them high enough to be recognized by some monitors. You should

certainly try any VGA monitor before you buy it, though, just to be sure.

(Note: many games and some system software operate only at 15.75 kHz. For

instance, the configuration screen you get by holding down the mouse

buttons during a reset only displays at 15.75 kHz, and will not be visible

with a VGA-type monitor.)

Q. Where can I get an adapter to go from the A1200's DB23 video port to the

goofy high-density HDD15 standard VGA connector that looks like a DB9?

A. The adaptor that comes with an A4000 can be used, or you can get a wiring

diagram from Aminet: hard/hack/ami2vga.lha. Alternatively, you could have

Redmond Cable build one for you (call them at 206-882-2009).

Q. Can I use a CGA monitor with an A1200?

A. Yes, but it won't work particularly well. CGA is digital RGB, so it can

only generate 16 predefined colors. If it's the only monitor you have, you

can probably live with it, but I would personally recommend even a

composite video monitor instead (the old 1702 would work well for this).

Q. What Commodore monitor should I buy for use with the A1200?

A. My opinion: Don't buy any Commodore monitor. There are several monitors

like the Microvitec 1438 and 1440 that are marketed specifically use with

the Amiga, and there are general-purpose VGA-type monitors that also work

well. In most cases, these monitors are less expensive, better made, and

more reliable than those that were sold by Commodore (which were OEMed

from monitor manufacturers anyway). If you're looking for something

inexpensive that will work with all the frequencies and output modes of

the A1200, see if you can locate a used Mitsubishi DiamonScan (AUM 1381A).

These monitors are quite cheap now, and besides handling all the RGB

output of the A1200, also have a composite video input. If you're buying

something new, I recommend a monitor with digital presets, which makes

screen adjustments much easier and eliminates the need to fiddle with

knobs every time you change screen modes (the Microvitec 1440 mentioned

above is reported to have digital presets).

Other Questions

---------------

Q. I don't like to shut off the A1200 to connect and disconnect joysticks,

modems, sound samplers, and printers. Can I damage the A1200 by changing

these connections with the power on?

A. Yes. Remember that the A1200 design uses chips that are almost all surface-

mount, soldered directly to the circuit board. This makes it very

reliable, but when you expose it to unsafe behavior (changing connections

with power applied), you risk damaging chips that are very difficult to

replace and extremely difficult to obtain. Before you switch cables, make

sure that power is off to the entire system, including the peripheral you

are connecting or disconnecting.

Q. Is there a way to get a battery-backed clock into the A1200 without buying

an expensive expansion board?

A. Yes. There is a header on the A1200 motherboard specifically for adding

one of these inexpensive clock boards, which usually cost less than $30.

However, a RAM expansion board with a clock is usually only about double

that, and gives you the ability to add 32-bit expansion memory later,

which will approximately double the operating speed of the A1200.

Q. Can I modify the A1200 to have a remote keyboard?

A. Yes, if you're really motivated. There is a file on doing this in the

hard/hack section of Aminet. It is non-trivial.

Q. Is there an adapter box that lets me use Zorro boards on the A1200?

A. There are a few adapters; details are scarce at present. As far as I know,

all these adapters are for Zorro II only, and will not work with Zorro III

boards like the DKB 4091 or FastLane hard drive controllers. Experience

with adapters of this type for earlier Amigas suggests that they may only

work with certain boards, or have difficulties with certain operations.

Q. Can I use a Video Toaster with the A1200?

A. No. The Toaster requires a video slot.

Q. Is there an Emplant Mac emulator for the A1200?

A. No. One was announced long ago by Utilities Unlimited, but has never

surfaced. You may be able to run the Zorro II Emplant board in one of the

Zorro II adapters mentioned above.

Conclusion

----------

Will Paula meet the blitter of her dreams? Will Akiko ever get together with

Fat Agnus? And what is the mysterious PCMCIA Corporation planning in their

corporate headquarters at 68030 DMA Drive? Tune in next week...