This is my hardware DOC file. It bears no relation to the stuff in any of

the Amiga manuals to my knowledge, but draws from the Amiga System Pro-

grammer's Guide by Abacus for much important material. Also, I include some

example pieces of code of my own, routines plus data structures, illustra-

ting how to manipulate the hardware directly.

List of Topics:

Hardware:Preamble

Hardware:The CIA Chips

Hardware:DMA

Hardware:Interrupts

Hardware:The Copper

Hardware:Bitplanes & Bitplane Control

Hardware:Sprite Management

Hardware:The Blitter

Hardware:Sound Management

Hardware:Disc Management

Hardware:Interfaces

Hardware:Mouse, Keyboard, Joysticks

Hardware:Some Notes

Hardware:Logic Tutorial (for mathematically skilled readers & masochists)

Hardware:Preamble

This DOC file is somewhat oddly organised. This is because several different

sections contain material that cross-references across topic headings. Where

a cross-reference exists, I'll signpost it using the # character followed

by a number. If you use Devpac to view this file, you can skip across the

cross-references using the search facility.

The main base addresses for the chips are $BFD000 and $BFE001 for

the CIAs, and $DFF000 for the gee-whizz custom chips. The custom chips are

organised as a linear block of word addresses from the base address onwards,

and the CIA register addresses occur at 256-byte intervals from the base add-

ress onwards. Other than that, there will occur extra pieces of information

relating the way that several of the chips are connected to various hardware

interfaces such as the keyboard and mouse. They are the main reason for the

implementation of the cross-referencing scheme in this file, because in the

case of the CIAs, they are connected to several different actual hardware

interfaces.

I find it best to use instructions of the form

MOVE.W	#DATA,register(A0)

when accessing the chips, because there's less time overhead than with those

instructions using absolute addresses. In the above case, A0 contains the

base address of the chip set being accessed.

One more point. The 68000 CLR instruction issues a read access and

then a write access singal across its bus. Other 680x0 chips from the 68020

onwwards perform a single write access only. Because of this, using something

like CLR COPJMP1(A0) is not recommended, even though it's shorter than using

MOVE.W #0,COPJMP1(A0), when accessing strobe registers (see later for an ex-

planation of what these are). Those blessed with assemblers capable of gen-

erating 68020/68030/68040 code and inline coprocessor code for the 68881 and

68882 maths coprocessors should NOT use these facilities when accessing the

custom chips in particular if they're lucky enough to have a 680x0 acceler-

ator board fitted. Save these facilities for accelerator-board specific code

only. Also be warned about invalid cache entry on these processors if the DMA

from the custom chips scribbles all over memory when the accelerator board

processor is attempting an access at that address range!

Given the propensity of games writers in particular to tell AmigaDos

to take a hike, I issue the following caveats:using MOVE to SR to kill off

iterrupts doesn't work properly-the interrupts are handled by the 4703 custom

chip and generated anyway. All that happens is that the 68000 ignores them,

and the relevant bits in the 4703 control registers don't get handled proper-

ly. This can cause problems. Also, supervisor mode doesn't provide any real

advantages over user mode (although I'm guilty of abusing it sometimes). If

you're handling your own exceptions directly, take account of differences in

exception stack frames especially if you've got a 680x0 accelerator board! I

recently found out that a gee-whizz 68040 board is now available with 2 megs

of memory from a company in Pasadena. 2000 and it's yours. Also, if you are

the sort of programmer who pees around with the ROM, watch out!

One final point. To save typing out addresses of the form $DFFxxx in

full for custom chip register addresses, I'll refer to them via offsets from

the base address of $DFF000. It's a VERY GOOD idea to do this in programs as

well! Load, say, A5 with $DFF000 and access the register of your choice using

something like

MOVE.W	#DATA,REGISTER_NAME(A5)

instead of littering code with absolute addresses (you'll also save 4 cycles

of processor time per access!). Any persons having encountered the dread set

of files called 'my_hardware.i' etc., which I have created for my own use &

recently distributed to others will find all of the custom chip registers in

name form defined as offset equates from the base address of $DFF000, and any

programs using these include files accessing the custom chips in the manner

above.

Hardware:The CIA Chips

There are two of these beasties on the Amiga. They're referred to as CIA-A &

CIA-B respectively. CIA-A lives at $BFE001 onwards, CIA-B at $BFD000 on. I

now present the register list. The formula for the address of each register

is register number*256+CIA base address, and all CIA registers are a

single byte wide.

Register		Name

--------		----

0		PRA (Port A Data Register) R/W

1		PRB (Port B Data Register)	R/W

2		DDRA (Port A Data Direction Register) R/W

3		DDRB (Port B Data Direction Register) R/W

4		TALO (Timer A lower 8 bits) R/W

5		TAHI (Timer A upper 8 bits) R/W

6		TBLO (Timer B lower 8 bits) R/W

7		TBHI (Timer B upper 8 bits) R/W

8		E.LSB (Event counter lower byte) R/W

9		E.MID (Event counter middle byte) R/W

A		E.MSB (Event counter high byte) R/W

B		not used

C		SP (Serial port data register) R/W

D		ICR (Interrupt control register) R/W

E		CRA (Control register A) R/W

F		CRB (Control register B) R/W

Note:the 'serial port' on the CIAs is NOT the same as the Amiga serial port

at the back! This is an internal serial port, and on the CIA-A it is connect-

ed to the 6500/1 processor handling the keyboard (#1). The main serial port

for external communications is handled via Paula (#2).

Data registers:the CIA data registers are connected in a seemingly odd manner

to various different pieces of hardware. They can be treated bitwise as input

or output registers. If the corresponding DDRx bit is 0, the same bit on the

PRx is an input, and an output if the DDRx bit is a 1. Writing to a data reg-

ister stores the value in it, and reading it reads the state of those port

lines to which it is connected.

There is a simple handshaking mechanism via two lines, called PC and

FLAG. PC goes low for 1 clock cycle on each access to PRB. The FLAG input re-

sponds to such transitions. Every time the state of FLAG changes from 1 to 0,

the FLAG bit in the ICR is set. These two lines allow a simple form of hand-

shaking in which the PC and FLAG lines of the CIAs are cross-connected. The

sender need only write its data to the port register & then wait for a FLAG

signal before sending each additional byte. Since FLAG can generate an int-

errupt if wanted, the sender can perform other tasks while waiting for the

FLAG signal, and continue the sending via an interrupt routine. The same also

applies to the receiver, except that it reads the data from the port instead

of writing to it. CIA-A has its PRB connected to the parallel port (more cor-

rectly, the Centronics port data lines), and CIA-B has a whole series of awk-

ward connections for PRB as follows (Cross-reference #6):

Bit	Connection

---	----------

7	/MTR motor signal to disc drive

6	/SEL3 drive 3 select

5	/SEL2 drive 2 select

4	/SEL1 drive 1 select

3	/SEL0 drive 0 select

2	/SIDE drive side select

1	DIR data direction signal to disc drive

0	/STEP step signal to drive head stepper motor

Both CIAs have awkward connections for PRA. They are (CIA-A first, then

CIA-B):

Bit	Connection

---	----------

7	Game port 1 pin 6 (fire button)

6	Game port 0 pin 6 (fire button)

5	/RDY disc ready signal from disc drive

4	/TK0 disc track 00 signal from disc drive

3	/WRPO write protect signal from disc drive

2	/CHNG disc change signal from disc drive

1	LED power LED (0=on, 1=off)

0	OVL memory overlay bit (DANGER!!!!)

7	/DTR DTR signal from serial interface

6	/RTS RTS signal from serial interface

5	/CD  CD signal from serial interface

4	/CTS CTS signal from serial interface

3	/DSR DSR signal from serial interface

2	SEL select signal for Centronics interface

1	POUT paper out signal from Centronics interface

0	BUSY busy signal from Centronics interface

When I said the connections were awkward, I wasn't kidding.

Control registers:the two control registers determine the mode of operation

of many of the other registers. CRA bit allocations are:

Bit	Function

---	--------

7	Not used

6	SPMOD serial port register mode 0=input, 1=output

5	INMODE 0=clock, 1=CNT (see later)

4	LOAD 1=force load (strobe) for timer A

3	RUNMODE 0=continuous, 1=one-shot for timer A

2	OUTMODE 0=pulse, 1=toggle

1	PBON 0=PB6 off, 1=PB6 on

0	START 0=off, 1=on

CRB bit allocations are:

Bit	Function

---	--------

7	0=TOD (time of day-see later), 1=ALARM

6,5	INMODE

00 = clock, 01=CNT

10 = timer A input,

11 = timer A + CNT

4	LOAD 1=force load (strobe) for timer A

3	RUNMODE 0=continuous, 1=one-shot for timer A

2	OUTMODE 0=pulse, 1=toggle

1	PBON 0=PB7 off, 1=PB7 on

0	START 0=off, 1=on

Timers:this is one of the complicated things. These timers can count down

from a preset value to zero. When reading, they are treated as TALO/TAHI

and TBLO/TBHI. When writing, they are treated as latches PALO/PAHI and

PBLO/PBHI. The latches should be loaded, low byte first, as a write access

to the high register causes the timer to be stopped and reloaded with the

latch value unless the LOAD bit in the control register is set, in which case

the latch value is transferred to the timers regardless of the timer state.

The LOAD bit in the CRA/CRB registers is a strobe bit. Writing simply causes

the given action to be performed-the bit value is not stored as such.

When the timer hits zero, the latch values are automatically put

back into the timer.

The latch is also known as the prescaler. The number of timeouts in

continuous mode is equal to the clock frequency divided by the value in the

prescaler/latch. Timer A is connected to the processor E clock, the frequency

being 716KHz.

Reading the timer can cause problems, because it has to be performed

in two separate read operations. One problem is that the timer could change

its state between the two reads. For example, if the timer bytes combined are

$0100 in value, and the high byte is read, the timer could decrement to $00FF

before the low byte is read, resulting in a final read value of $01FF, which

is incorrect (it should be $00FF). Stopping the timer from decrementing

during the read is one method of preventing this, but this is not the most

elegant way. Instead, read the high byte, then the low byte, then read the

high byte again in a separate register. If the two high byte reads are the

same, take this as the value. If not, repeat the process. It should only need

one repeat read to obtain the correct result, but in any case the following

code should obtain the correct value (code is for CIA-A):

LEA	$BFE001,A0	;CIA-A base address

get_timer	MOVE.B	$400(A0),D0	;get TALO

MOVE.B	$500(A0),D1	;get TAHI

MOVE.B	$400(A0),D2	;get TALO again

CMP.B	D0,D2		;timer values match?

BNE.S	get_timer	;repeat read if not

NOP			;here, got correct timer!

Bits 5 and 6 of the control registers control the timer functions. For timer

A, there are two possibilities. Either CRA INMODE=0, in which case timer A is

decremented during each clock cycle on the processor's E clock line, or else

CRA INMODE=1, in which case each high pulse on the CNT input line decrements

the timer.

Timer B has four possible operating modes, governed by the INMODE

bits of CRB. First there is INMODE mode 00, in which timer B decrements each

time a pulse occurs on the E clock line (connected to the f2 pin of the CIA)

just as for timer A INMODE=0. INMODE=01 decrements timer B every time there

is a pulse on the CNT line. INMODE=10 allows timers A and B to be combined to

form a 32-bit timer, and timer B decrements every time a timeout signal from

timer A is received. In this mode, timer A forms the low word of the 32-bit

timer, and timer B the high word. Finally, INMODE=11 allows the length of a

pulse on the CNT line to be measured. In this mode, timer A issues a timeout

when the CNT line is high. The timeout signals are in the interrupt control

register or ICR, to be dealt with later.

Two output modes for the timers can be selected with the OUTMODE bit

of the control registers. OUTMODE=0 causes timeout signals to appear as a

positive pulse one clock period long on the corresponding port line. When the

OUTMODE=1 (toggle mode), each timeout causes the corresponding port line to

change value from low to high or high to low. Each time the timer is started

in this mode, the output starts as a high signal.

The RUNMODE bit determines whether the timer operates in one-shot

mode (RUNMODE=1) or operates continuously (RUNMODE=0). In one-shot mode, the

timer stops after each timeout and sets the START bit to 0. In the contin-

uous mode the timer restarts after each timeout automatically.

Timer A of CIA-A is used by the operating system for communication

with the keyboard, and timer B by the operating system for some other tasks.

Timer A of CIA-B is used for serial data transfers, otherwise it is free, and

timer B is used to synchronise the blitter with the screen by the OS, other-

wise it is free.

Interrupt Control:this is actually implemented as two registers, one being a

read register and one being a write register. The bit allocations are given

in the table below, read register first. The read register is the ICR data

register, the write register the ICR enable mask register.

Bit		Function

---		--------

7		IR - Interrupt received/signalled

6		not used, always 0

5		not used, always 0

4		FLAG - PRB port handshaking

3		SP - serial port needs attention

2		Alarm signal

1		TB - Timeout for Timer B

0		TA - Timeout for Timer A

7		Set/Clr bit (explained below)

6		not used, send a 0

5		not used, send a 0

4		FLAG input enable

3		SP - serial port interrupt enable

2		Alarm input enable

1		Timer B timeout enable

0		Timer A timeout enable

The set/clr bit is used as follows. Let us assume that the byte to be written

to the ICR is %00010011. Bit 7 (the set/clr bit) is 0. This tells the input

latch for the ICR to clear all bits in the ICR write register corresponding

to the 1 bits in the byte just written to it, and to leave all other bits un-

changed. This particular write byte has bits 4,1 and 0 set, so the FLAG, TB

and TA interrupt inputs are disabled. If the byte was %10010011 instead, the

set bit 7 (corresponding to set/clr on the ICR) would tell the input latch

for the ICR to set all bits corresponding to the other 1 bits in the byte

just written, and leave all of the others unchanged. This write byte would

enable the modes disabled by the previous write byte example.

Note that this set/clr mechanism occurs throughout the Amiga chip

set, in particular for DMA control (#3), main system interrupt control (#4)

and other functions.

Having written a control byte to enable various interrupt sources

(CIA-local interrupt sources, NOT main system ones!), we can now read the

same address and the chip will return the value of the ICR data register,

which contains bitwise information about which interrupt source triggered

the CIA's internal interrupt mechanism. Note that any interrupt handled via

the CIAs is further passed onto the system as a whole, to allow 68000 auto-

vectored interrupt code to handle the interrupt. More about this later.

If this value is needed for multiple interrupt source tests, then it

must be saved, because reading the ICR data register causes the CIA to clear

it after the read. The IR bit is the interrupt request bit, and if set, indi-

cates that a valud CIA internal interrupt was triggered. But this and all of

the other ICR data bits are cleared upon reading, so if you fail to save the

result of the read (this code saves the read value);

LEA	$BFE001,A0		;CIA-A base addr

MOVE.B	$D00(A0),D0		;get ICR data reg value

MOVE.B	D0,last_icra_read(A6)	;save in my variable block!

and then obliterate the unprocessed bits without saving as above, then you

have lost forever those unprocessed bits. You have been warned!

The CIA signals an interrupt as follows. Whenever one of the various

interrupt sources sets its corresponding bit in the ICR data register, the

CIA checks to see if the corresponding ICR mask enable bit is also set. If

this is so, the CIA pulls the IRQ line low to signal the interrupt in hard-

ware, and then sets the IR bit (bit 7) in the ICR data register to signal the

interrupt in software also. How the main system treats this signal is handled

later. The IRQ line does not return to the high state (its normal state) un-

til the ICR data register is read (and hence the IR bit, and all others, are

cleared by the CIA).

Event Counter:this event counter differs from the 6526 Time-Of-Day counter

found, for example, in the Commodore 64 (yeuk!). This is almost the only

difference between the 6526 and the 8520 CIA. Those unfortunate enough to

have encountered the 6526 in past programming will breathe a sigh of relief.

There is one slight problem with the documentation that Commodore supply-

they insist on referring to TOD (time of day) in the literature, even though

this term is meaningless here.

Instead of the real-time clock which counts hours, minutes and sec-

onds on the 6526, the 8520 has a simple 24-bit event counter. It takes its

input signal on the TOD line (jeez, Commodore!) of the chip. The event coun-

ter starts at zero (or some other predefined state written to it) and counts

UPWARDS to $FFFFFF, before returning to zero. The event counter consists of

the counter proper and a latch register. When the high byte is read, the ac-

tual counter state is transferred completely to the 24-bit latch, and the

high byte of the latch returned. The counter continues counting undisturbed

while the remaining latch registers are read, mid-byte next, then low-byte.

ALWAYS read in the order high, mid, low or else this won't work!

Writing a value to the event counter causes the CIA to stop the

counter until the entire value is written, PROVIDED that it is written to

in the same order as it is read from above, namely high, mid, low. Once

the low byte is written, the timer starts up again, with the written value

as the start value. When writing the event counter value in normal mode, be

sure that the TOD/ALARM bit of the CRB control register is cleared!

An alarm function exists also. SET the Alarm bit in CRB as opposed

to clearing it as for normal event counter writing, and write a value into

the event counter. The chip will take this value as an alarm value, and if

the event counter value ever matches this alarm value, the alarm bit of the

interrupt control register is set. The value of the alarm setting CANNOT be

read, only written - reading the event counter always returns the current

counter latch value, regardless of the state of the TOD/Alarm bit in CRB. So

if you want to provide known alarm values, save them somewhere safe before

using them!

Serial Port:the serial port consists of the serial data register (which is

readable and writeable) and the shift register (not directly accessible).

Setting SPMODE=0 in the control register sets the serial port to input mode

and SPMODE=1 to output mode.

In input mode, the serial data on the SP line are shifted into the

shift register after each rising edge on the CNT line. After 8 CNT pulses the

shift register is full, and the data is transferred to the serial data reg-

ister. At the same time, the SP bit in the ICR data register is set. If more

CNT pulses are received, the data continues to shift into the shift register

until it is full again. If the user has read the serial data register before

this, the data is transferred across again, else it is lost. When using this

register for input, respond to it reasonably promptly!

In output mode, timer A is used to determine the send frequency. The

timeout rate of timer A (which must be operated in CONTINUOUS mode) controls

the baud rate of the transfer. The data are shifted out of the shift register

at half the timeout rate of timer A, whereby the maximum output rate is 1/4

of the clock frequency of the 8520.

The transfer begins after the first byte is written to the serial

data register. The CIA transfers the byte into the shift register. The

individual data bits now appear at half the timeout rate of timer A on the

SP line and the clock signal from timer A appears on the CNT line (it changes

value on each timeout so that the next bit appears on the SP line on each

negative transition [high to low] ). The transfer begins with the most sig-

nificant bit of the data byte. Once all 8 bits have been output, the CNT line

remains high and the SP line retains the value of the last bit sent. In add-

ition, the SP bit of the ICR data register is set to show that the serial

data register can be supplied with new data. If the next data byte was loaded

into the data register before the output of the last bit, the data output

will continue without interruption.

To keep the transfer continuous, the serial data register must be

re-supplied with fresh data at the proper time.

The SP and CNT lines are described as 'open-collector' outputs. This

allows the outputs of multiple CIAs to be connected together. See any good

text on electronic interfacing techniques for an explanation (especially the

mass of material on the IEEE-488 bus) of open-collector outputs.

On the Amiga, the SP line of CIA-A is connected to the KDAT line of

the keyboard 6500/1 processor, and the CNT line connected to the KCLK line of

the keyboard processor. So hardware keyboard reads can be performed by enab-

ling serial port interrupts in CIA-A (which AmigaDos does for its own use),

and writing an autovector interrupt routine to intercept the ICR SP signal

and read the serial data register.

The SP of CIA-B is connected to the Centronics BUSY signal (as is

PA0 of the CIA parallel port A) and the CNT line to the Centronics PAPER

OUT signal (as is PA1 of the CIA parallel port A).

Other Information:other connections for CIA-A are:

PC	/DRDY - Centronics handshake, data ready

FLAG	/ACK  - Centronics handshake, data acknowledge

IRQ	/INT2 input from Paula (#2)

RES	/RES reset line

and connections for CIA-B are:

PC	not used

FLAG	/INDEX - index signal from disc drive

IRQ	/INT6 input from Paula (#2)

RES	/RES reset line

One final point. For those with access to FAST RAM, it is possible to examine

the Kickstart ROM on the Amiga 500. Simply clear the Memory Overlay Bit (OVL,

bit 0, CIAAPRA) from a program in FAST RAM, and the CHIP RAM memory address

range becomes mapped onto the underlying Kickstart ROM. This can be read at

will. DO NOT TRY THIS FROM A PROGRAM WITHIN CHIP RAM - THE PROGRAM WILL BE

LEFT HANGING AS THE 68000 PC REFERENCES THAT PART OF THE KICKSTART ROM WHICH

HAS THE SAME ADDRESS AS THE NEXT INSTRUCTION IN YOUR OVL SWITCHING CODE! I

HEAR THE GURU KNOCKING ALREADY...

Hardware:DMA

DMA (Direct Memory Access) is the technique used by most of the Amiga custom

chips to perform their functions. The system is organised quite neatly, in

that there are two 'halves' to the processor busses. One half is connected

to all of those components accessible solely to the 68000 (such as FAST RAM

if it is present) and the CIAs. The other half contains the CHIP RAM and the

custom chips. The two halves are separated by a buffer, which disconnects

the custom chip half from the processor whenever the processor makes an acc-

ess to the FAST RAM or the CIAs. The custom chips then have total access to

the CHIP RAM.

If the processor accesses the CHIP RAM or the custom chip registers,

then the buffer re-establishes the connection. In this case, there exists a

risk of bus contention, where two bus controllers try to take over the busses

simultaneously with obviously disastrous results. Bus accesses are nested, so

that this problem is largely avoided. Also, the processor can wait until the

bus is free if the blitter has absolute priority (this can be set under the

control of software). Also, there exist odd and even bus cycles, and DMA acc-

ess is restricted to the odd bus cycles, the even bus cycles being granted

under normal circumstances to the processor.

Right, we've got that out of the way. Now for the details. The DMA

system on the Amiga consists of DMA channels, each channel assigned to one

of the hardware functions. The full list of DMA channels is:

Bitplane DMA (6 channels): These channels are used by that part

of the hardware which takes the bit

plane data & converts it for output to

the screen. If you select fewer bit

planes, then some bitplane DMA chan-

nels remain unused.

Sprite DMA (8 channels)	: These channels are used by the sprite

processor. Once a given sprite DMA

channel has been used for generating a

sprite, it can be re-used for another

(with some restrictions). See later.

Disc DMA (1 channel)	: Data transfer from disc to RAM or

vice versa.

Audio DMA (4 channels)	: These channels are used for processing

of audio data in RAM & passing them to

the sound chip. Incidentally, for any

one interested in music, conversion of

Amiga sound data to Fairlight synthe-

sizer data and vice versa is possible!

Paula's sound system possesses a limi-

ted Fairlight compatibility.

Copper DMA (1 channel)	: This channel is used for data transfer

of command words to the Copper. If the

Copperlist tells the Copper to change

the values of registers bound to other

DMA channels, those channels are used

for those purposes selected within the

Copperlist-the Copper DMA channel is

solely for Copperlist transfer to the

Copper.

Blitter DMA (4 channels): These channels are used for data trans-

fer to and from the blitter.

Now, to make life interesting in some ways (and simpler in others) the Amiga

designers related the DMA channel priorities and usages to the construction

of the video picture, and the timings in bus cycles are related to the video

picture mainly in order to make construction of the fabulous Amiga graphics

displays easier.

What is a bus cycle? Simply put, a time span of 280 nanoseconds. This

is the time taken for a single memory access across the bus by a device using

a DMA channel. Sad to say, the 68000 cannot match this speed and needs 560

nanoseconds per memory access (two DMA bus cycles). The system is designed so

that during these two bus cycles, access to the bus is split between the DMA

channels and the 68000 as mentioned above, into odd and even cycles. Note: I

haven't distinguished between read and write memory accesses for one simple

reason, namely that both take the same time within this system.

Now, if we number the cycles from zero upwards, cycle zero is given

over initially to the processor. If the processor wants access to the bus, it

gets it initially during cycle zero. Once cycle zero has finished, cycle one

begins, which is reserved (as are all odd cycles) for the DMA controller. The

DMA controller gets access to the bus during cycle one. Once cycle one has

elapsed, cycle two begins and so on, access to the bus alternating between

the 68000 and the DMA controller. This assumes that the 68000 wants to access

the bus continuously during these cycles. If the 68000 is performing internal

processing upon register data, or accessing true FAST RAM only, then that

half of the bus connected to the DMA controller is available to the DMA con-

troller during the even cycles as well, and should this be the case, the

buffer isolates the CHIP RAM section from the 68000, and the DMA controller

can access CHIP RAM during even bus cycles as well. So, in an ideal world,

with a 10 meg FAST RAM expansion, your Amiga runs like the proverbial bat out

of hell. Should the 68000 want to access CHIP RAM, however, then the fun be-

gins, because the DMA controller is stuck with the odd cycles, except under

certain conditions.

Generally, the audio, disc and sprite DMA only use up the odd bus

cycles. Thus audio, disc and sprite accesses do not slow up the processor. If

large amounts of bitplane activity is required, or the blitter is activated,

then some of the processor's even cycles are 'stolen' for these purposes. So

when this happens, the 68000 runs more slowly.

Ok, I said that bus cycles are related to the video picture. Well,

they are. Now I'll explain how. A raster line on the screen takes 63.5 micro-

seconds to produce. This is equal to 227.5 bus cycles per raster line. Each

of the bus cycles that occur during this time are allocated for some purpose

or other. The odd cycles are generally reserved for disc, audio and sprite

DMA first, then bitplane DMA. The Copper and the blitter both use even bus

cycles (tut, tut, that was naughty, Commodore!) and thus chew into the time

available for the processor.

Anyone possessing a copy of the Amiga System Programmer's Guide will

no doubt have seen the little chart of DMA bus cycles for one raster line. If

so, it will be noticed that the little boxes representing DMA bus cycles do

not add up according to the values given along the top of each chart section.

So my attempt to list in full how the DMA bus cycles are allocated here has

run into trouble. Should anyone come up with a neat system of explaining this

that doesn't require inclusion of an IFF picture with the DOC file as an aid

please get in touch.

Now for the start of the really useful information. The DMA control

register, called DMACON, consists of two parts. There is DMACON (at offset

$96) and DMACONR (at offset $02). DMACON is a write-only register, and

DMACONR is a read-only register. Generally, if the register name ends in the

letter 'R', it's a read-only register (nice sensible convention this). Both

registers are word sized, and each bit is allocated as follows (#3,#4):

Bit		Function

---		--------

15		SETIT (set or clear bits)

14		BBUSY (blitter busy-read only)

13		BZERO (blitter zero-read only)

12		Not used

11		Not used

10		BLTPRI (Blitter has absolute priority

over the 68000)

9		DMAEN (Master DMA enable)

8		BPLEN (Bitplane DMA enable)

7		COPEN (Copper DMA enable)

6		BLTEN (Blitter DMA enable)

5		SPREN (Sprite DMA enable)

4		DSKEN (Disc DMA enable)

3		AUD3EN (Enable Audio channel 3)

2		AUD2EN (Enable Audio channel 2)

1		AUD1EN (Enable Audio channel 1)

0		AUD0EN (Enable Audio channel 0)

For bits 10 down to 0, if the bit is set, the corresponding function is

enabled, else it is disabled. So to enable the blitter DMA, set bit 6 of the

DMACON register.

The DMACON register takes command words formed in a slightly odd way

if you don't understand the rationale behind it. Basically, to avoid the need

to read the current status, make a mask, exclusive-OR in your choice of bits

to change and write back the result, in order to ensure that ONLY your choice

of bits is changed, bit 15 is used to decide whether you want to set or clear

the appropriate bits (hence the name, SETIT), and the other 14 bits are used

to signal if each of the given bits is the bit of your choice (set if this is

so, clear if not). This has the welcome effect of ensuring that DMA channels

not under consideration are left alone during your write to DMACON.

Example:to enable the bitplane DMA, bit 8 must be set. The command

word thus has bit 8 set (select the bitplane DMA bit), bit 15 SET (to signal

that bit 8 is to be set) and all other bits zero (leave the other DMA chan-

nels unaffected). If I wished to disable the bitplane DMA, the command word

I would write would have bit 8 set, bit 15 CLEAR, all others clear.

Cross-reference #3 : the CIA chip documentation above refers to the

set/clear mechanism in connection with its control registers. The above para-

graphs give a complete explanation of this mechanism for anyone who has used

the cross-referencing scheme to find out more. This mechanism also is used by

the INTENA register (#4) and ADKCON (#5).

As an extra safety feature, DMA channels only become truly enabled

when the master DMA enable bit (bit 9) is set. So it is possible to select

DMA channels one at a time, then suddenly enable the lot in one go by enab-

ling the master DMA enable bit. Clearing the master DMA enable bit disables

ALL DMA channels. Of course, multiple channels can be enabled, e.g.,

MOVE.W	#$8380,DMACON(A5)

enables the master DMA control, the bitplane DMA, and the Copper DMA in one

go (see the Preamble for notes on my choice of addressing mode and why).

Note that the current DMA enable status can be obtained via the

instruction

MOVE.W	DMACONR(A5),D0

or something similar. A set bit indicates a DMA channel enabled.

Incidentally, should you want to kill off AmigaDos and Exec totally

(as games writers like to) then kill off all DMA using the instruction

MOVE.W	#$7FFF,DMACON(A5)

in your code. This alone won't do it-you'll need to kill off the interrupts

as well-but it goes a long way toward doing precisely that. The full method

is :

1) Kill off all DMA as above;

2) Kill off all interrupts (see below for how to do it);

3) Point all 68000 interrupt vectors to your own custom interrupt

handlers, or RTE if you don't want to handle a given IPLx int-

errupt level;

4) Get into supervisor mode & change the 68000 IPLx level to make

the 68000 acknowledge only those interrupts you want it to (the

4703 actually generates them-see below for how to make the 4703

generate only those interrupts that you want) using MOVE SR

(VERY NAUGHTY! SPANKING TIME FOR ALL YOU FETISHISTS OUT THERE!);

5) Set up your own DMA system but DON'T ENABLE YET;

6) Enable 4703 interrupts (again see below);

7) NOW ENABLE YOUR DMA!

At this point Exec etc., is dead and beyond resurrection other than

via a hard reset. If that's what you want, so be it.

Hardware:Interrupts

Ok, I've already mentioned a little about interrupts above in the DMA section

but not the whole story. Now is the time to correct the omissions.

I've already mentioned the existence of the 4703 interrupt control-

ler, which actually generates the interrupts. All that the 68000 does within

the Amiga is respond to these externally generated interrupts if the value of

the IPLx bits allows it to. The standard value of SR on the Amiga is $0100 in

user mode, $2100 in supervisor mode, and all interrupts from level 1 onwards

are responded to-there's a hell of a lot of interrupts on this computer!

The 4703 interrupt controller is programmed via two sets of custom

chip registers. Again, each set has a read-only and a write-only component.

These registers are:

INTREQ  (offset $09C) : write only

INTREQR (offset $01E) : read only

INTENA  (offset $09A) : write only

INTENAR (offset $01C) : read only

INTENA is the interrupt enable register, and INTREQ is the interrupt request

register. Again, the read-only ones end in 'R'. The structure of all four of

these registers is identical-they are all split into individual bits, which

are assigned as follows (the numbers in square brackets in the right-hand

column correspoond to the 68000 IPLx priority of the said source-note that

the NMI interrupt of priority 7 is never used):

Bit	Function

---	--------

15	SETIT (just like the DMACON register above)

14	INTEN (master interrupt enable)			[6] *

13	EXTER (int. from CIA-B or expansion port)	[6]

12	DSKSYN (disc sync value recognised)		[5]

11	RBF (serial receive buffer full)		[5]

10	AUD3 (output audio data channel 3)		[4]

9	AUD2 (output audio data channel 2)		[4]

8	AUD1 (output audio data channel 1)		[4]

7	AUD0 (output audio data channel 0)		[4]

6	BLIT (blitter ready)				[3]

5	VERTB (vertical blank interrupt)		[3]

4	COPER (Copper interrupt)			[3] *

3	PORTS (CIA-A or exapnsion port)			[2]

2	SOFT (reserved for software interrupts)		[2] *

1	DSKBLK (Disc DMA transfer done)			[1]

0	TBE (serial transmit buffer empty)		[1]

Once again, if the corresponding bit from bit 13 down is set in the INTENA

register, that interrupt source is enabled. If one of those bits is set by

an interrupt source in the INTREQ register, the corresponding interrupt is

generated & sent to the 68000. Setting or clearing the bits in INTENA/INTREQ

is done in an indentical fashion to the DMACON register above. Again, the

state of the master interrupt enable bit (bit 14) determines whether the 4703

can generate any interrupts at all. If bit 14 is clear, NO interrupts will be

generated by the 4703. Needless to say, having decided which interrupts to

enable, one can enable them singly or in one go by choosing the appropriate

value to write to INTENA. So, if the INTEN bit is set, and the given bit for

the interrupt source is set, that interrupt CAN be generated, even if there

is no guarantee that it WILL be generated.

Ok, we can decide which interrupts to respond to. How does the 4703

generate them? Simple. Any interrupt source setting a bit in the INTREQ reg-

ister causes the 4703 to generate the appropriate interrupt, at which point

the 68000 gets to know about it. This can either happen in hardware e.g., the

blitter finishes its job & posts its interrupt signal) or can be performed in

software, e.g., by setting the SOFT bit in your code directly-this will gene-

rate a software interrupt signal as used by Exec for its softints, but unless

Exec is alive and well, DON'T expect Exec to handle it like a softint! There

is a special case, the Copper interrupt. The Copper can be made to set its

own reserved Copper interrupt bit directly within a Copperlist (see later),

as a means of forcing a Copper interrupt other than the vertical blank which

is handled in hardware, and so is totally under your control. Those bits of

INTREQ capable of being set in software by your programs at will are marked

in the above list with a '*'. The others CAN be set, but care must be exer-

cised if you are to do this in your code (Usually done if you are writing a

set of interrupt handlers to handle the functions on a continuous basis, and

HAVE to set the appropriate INTREQ bit to start the sequence off).

So, the 68000 hears about an interrupt if:

1) INTEN (master interrupt enable) is set in INTENA;

2) The corresponding interrupt source bit is set in INTENA;

3) The corresponding interrupt source bit is also set in

INTREQ.

All three conditions need to be fulfilled, else the 68000 will think that no

interrupts are being generated by the interrupt source concerned. Further in-

formation is available in the file 'typed_interrupts.doc' in this series.

Cross-reference #4:for a full explanation of the SETIT bit, see the

cross-reference #3 above.

What do we do now? Well, you need an interrupt handler terminated by

an RTE & the appropriate 68000 interrupt vector changed to point to it. If

that interrupt is enabled, and its appropriate bit in INTREQ is set, then the

handler will be called. Your handler should do the following:

1) Read INTREQR to find out which interrupt request was made. Some

of the 68000 interrupt handlers will have to handle more than

one interrupt source, and need to know which caused the inter-

rupt exception.

2) If the bit corresponding to the interrupt source that you are

interested in is set in INTERQR , then CLEAR it in INTREQ to

signal that you've acknowledged it. The 4703 can now handle

another interrupt of the same type. Incidentally, things will

get interesting if the interrupt source posts its interrupts

faster than you can respond to them!

3) Process your interrupt as you wish from this point on.

This, of course, is over and above the usual things that interrupt handlers

are supposed to do, such as save scratch registers on the stack & other junk

that you should already be aware of. And for crying out loud, please use the

MOVEM instruction to do it! Note that unlike the CIAs above, the INTREQ bits

are NOT cleared when INTREQR is read! You have to clear them the hard way! I

also would like to point out that setting the SETIT bit of INTREQ will cause

a level 6 autovector interrupt to occur, so if you want, you can use this to

create your own level 6 soft interrupts-nothing else (other than the Copper

should you want it to) will set this bit but your code. Gee, isn't that nice?

A word of warning:on my machine at least (this may vary), something

quaint happens if one tries performing the instruction

CLR.W	INTREQ(A5)

or similar, due to the read/write access of the 68000 CLR instruction (see

the Preamble). The machine seems to 'hiccup' before carrying on as normal. I

would not suggest doing this too often, it might blow something expensive to

mend. In any case, the above instruction does bugger all to the status of the

interrupt system for reasons made obvious upon analysis (see #3 above), so I

don't think that there's much point in doing it-I only did it by accident. I

cannot stress too much, however, that the Preamble caveat about CLR should be

adhered to (I only found out after extensive use of CLR in my code & had to

rip it all apart again to make it work properly. Some unfortunates have some

copies of my old code instead of the CLR-free versions & wonder what the hell

is going on when they run it...).

Hardware:The Copper

This is one of the harder chapters to write, and for a very good reason. The

Copper is a very powerful little piece of hardware, but with that power comes

complexity.

The Copper is a coprocessor capable of writing values to the custom

chip registers independently of the 68000, and of performing actions based on

the position of the video beam. All in all, a highly useful little fellow. As

befits what is a processor in its own right, it has its own machine language

and it is programs written in this special Copper machine language that are

the famous Copperlists of Amiga parlance. At this point, those thinking to

themselves 'oh, no, not another machine language to learn!' should be reas-

sured by the knowledge that there are only three basic instruction types in

Copper machine language. These three basic instruction types are versatile,

however, and much can be done with them.

Sad to say, Copperlists have to be created by hand as far as I am

aware, at least if you want to take advantage of ALL Copperlist features.

I have been told that the Argasm assembler can take Copperlists in the form

of Copper assembly language & assemble them, but I've yet to check this. And

yes, there is a Copper assembly language to make life a little easier when

creating Copperlists.

So, to the Copper Assembly Language (CAL for short). The three basic

CAL instructions are:

MOVE	: Write an immediate data value into a custom chip

register (like the 68000 MOVE #nnnn,xxxx);

WAIT	: Wait until the electron beam generating the video

picture has reached a certain position;

SKIP	: If the electron beam has reached the specified

position, skip the next CAL instruction, else

execute sequentially as normal.

Doesn't seem much, does it? Well, you can do a hell of a lot with this to

hand. I shall deal with the instructions in turn.

The MOVE instruction:this instruction allows the Copper to write an

immediate value into a custom-chip register. The register is specified in the

instruction as an offset from the base address $DFF000 (now see why I prefer

using offset(An) in 68000 for addressing custom chip registers. The Copper

does it in hardware! Makes for consistency in programming) and the value to

write is always a WORD value. The CAL syntax is

MOVE	#value,register

for anyone blessed with a CAL assembler (or Argasm if it does this for you).

The actual way that the instruction is coded as machine words in memory is:

%0000000rrrrrrrr0,$XXXX

Here, rrrrrrrr represents the register address. Since all register offsets

from $DFF000 are even, bit 0 of the first word of a MOVE instruction is al-

ways zero. The second word contains the 16-bit value to write to the chosen

custom-chip register. For example, to write the value corresponding to the

colour light green into palette register 3, one would use the CAL syntax

MOVE	#$03C3,COLOR03

which would become (since COLOR03 is at offset $186) the machine words

$0186,$03C3

This seems simple enough. Now, the fun starts. There is a restriction upon

the registers that can be written to by the Copper. Under normal circumstan-

ces, the Copper cannot write to registers from offsets $000 to $07E (most of

these are read-only anyway). There exists a special custom chip register, the

COPCON register, consisting of one bit (bit 0). If this bit, which is called

the Copper Danger Bit or CDANG bit, is set, then the Copper can access the

custom chip registers from offsets $040 to $07E, which just happen to be the

blitter control registers. Access to the registers from offsets $000 to $03E

is NEVER allowed. COPCON itself is at $02E (write-only) and inaccessible to

the Copper itself (the 68000 must write to this register).

So, bearing this restriction in mind, the Copper can write to most

of the custom chip registers, and if allowed to by the 68000, can write to

the blitter control registers and influence the blitter. This alone gives the

Copper considerable power within the system. In particular, the Copper can

change the DMA enable status, the interrupt enable status, the sprite control

registers, the palette, the bitplane control, the sound chip, and to a limit-

ed extent, the disc controller! Needless to say, it's only safe to let the

Copper do all this when you know how it's all done. See each section in turn

for the requisite information.

The WAIT instruction:this instruction causes the Copper to do just

that, wait. The Copper waits for a specific position to be reached by the

electron beam generating the video picture before continuing execution of

the remaining instructions in the Copperlist. This is how various tricks are

achieved such as changing background colours at specified screen positions

to create 'sunset' effects etc. The Copperlist contains a sequence of WAITs

interspersed with MOVEs to the background colour palette register, COLOR00 at

offset $180. Should the electron beam have already passed the given position,

normal sequential execution is resumed. The CAL syntax for WAIT is

WAIT	(x1,y1) MASK (x2,y2) BFD

(Anyone used to the CMOVE/CWAIT macros on Devpac here forget those-they never

use the full power of the WAIT instruction. This is MY defined CAL syntax for

the WAIT instruction, that tells you everything).

In this syntax, x1 and y1 are the beam position to wait for. In this

syntax, the MASK and BFD entries are optional and can be omitted, but if they

are included have a profound effect. More of this below. If omitted, the WAIT

instruction makes the Copper wait until the beam reaches position (x1,y1) and

then resume normal sequential execution.

If the MASK specifier is included, the fun starts. Instead of using

(x1,y1) directly, the comparison of the beam position is made with the values

formed by logically ANDing together x1 and x2 for the horizontal position, &

ANDing together y1 and y2 for the vertical position. Omitting the MASK speci-

fier is to be regarded as having the same effect as having a MASK value of

(-1,-1), or all 1's in binary (in which case x1 AND x2 becomes x1, etc.).

This opens up many possibilities. For example, in the instruction

WAIT	(0,$0F) MASK (-1,$0F)

the WAIT condition will be fulfilled every 16 lines, i.e., whenever the lower

four bits of y1 are all 1. Note the -1 mask value for the horizontal position

(i.e., all 1's binary). Since in this example I am not interested in the hor-

izontal position at all, I could have had a horizontal mask of 0, but if the

horizontal position is important, choose the mask accordingly. The mask bits

affect BOTH the specified position in the instruction AND the actual beam

position coordinates before the position comparison is performed.

The machine words for the WAIT instruction take the form:

%vvvvvvvvhhhhhhhh1,%bvvvvvvvhhhhhhh0

In the first word, the vvv bits specify the vertical beam position, and the

hhh bits the horizontal beam position. Note that bit 0 is equal to 1. This

distinguishes WAIT (and SKIP below) from MOVE. WAIT is distinguished from

SKIP by having bit 0 of the second machine word set to zero (for SKIP, this

is set to 1).

In the second word, the b bit is the Blitter Finish Disable bit, or

BFD bit. In my CAL syntax, including BFD in the instruction specification has

the meaning 'set the BFD bit in the second word'. This bit is used when the

Copper is used to start a blitter operation. The Copper in general must know

when the blitter has finished whatever blitter operation was started at some

time past, whether started by the Copper or the 68000. If the BFD bit is zero

(omitting BFD in the CAL syntax means 'clear the BFD bit') then the Copper

will WAIT until the blitter finishes, and THEN check the wait condition. If

the BFD bit is one, the blitter status is ignored. If COPCON=0, and the

Copper does not affect the blitter in any way, BFD should be set to 1.

In the second word, the vvv bits are the vertical mask, and the hhh

bits the horizontal mask. If no mask is specified, these are all set to 1.

Note that bit 7 of the vertical position cannot be masked. If a vertical mask

is specified, then bit 7 of the vertical position is always treated as though

the mask bit for that position was 1. Note that since there are 313 lines of

display, and the vertical position is only 8 bits wide (and the vertical mask

7 bits wide), that WAITs for vertical positions greater than 255 have to be

performed as

WAIT	(0,255)

WAIT	(0,y)

NOTE : Horizontal positions are specified in steps of 4 low-resolution pixels

and NOT in pixel coordinates directly!

The SKIP instruction:the only difference between this instruction &

the WAIT instruction in terms of machine words is that bit 0 of the second

word is 1 instead of 0. The bits are otherwise identical in format to those

for the WAIT instruction.

The SKIP instruction allows conditional branches to be set up within

a Copperlist. The mechanism is slightly quirky, however. Basically, the beam

position is compared with the (x1,y1) arguments as for the WAIT instruction,

with MASK data also applying identically. If the beam position is greater

than or equal to the (x1,y1) argument (MASK notwithstanding) then the Copper

skips the next Copperlist instruction, and moves on directly to the instruc-

tion following it. Otherwise, instruction execution continues in the normal

sequential fashion. Full information on conditional branches using this in-

struction is given below. As may be expected, my CAL syntax for the SKIP in-

struction is

SKIP	(x1,y1) MASK (x2,y2) BFD

just as for the WAIT instruction. Comments regarding the optional MASK and

BFD arguments applying to WAIT apply identically to SKIP.

Now, we have the three Copper instructions. A Copperlist is simply

a sequential list of these instructions, in machine word format, in memory.

As may be expected, the Copperlist MUST be in CHIP RAM.

Having read this far, one may wonder about how a Copperlist is ter-

minated. A trick is used here. The final instruction in a Copperlist is a

WAIT instruction for an impossible beam position, such as WAIT (0,$FE). This

condition will never be fulfilled because a horizontal beam position greater

than $E4 isn't possible. I personally use the machine words

DC.W	$FFFF,$FFFE

as the end of my Copperlist. When the beam reaches the end of the video pic-

ture, the Copper is automatically restarted at the start of the Copperlist

(unless you arrange otherwise!).

Ok. We have a Copperlist, complete with the impossible WAIT at the

end. How do we tell the Copper to execute our Copperlist? The Copper has a

set of registers for this. These are:

COP1LCH		offset $080

COP1LCL		offset $082

COP2LCH		offset $084

COP2LCL		offset $086

COPJMP1		offset $088

COPJMP2		offset $08A

The two register pairs, COP1LCH/L and COP2LCH/L, are loaded with the 18-bit

CHIP RAM address of the start of your Copperlist (or some other address-see

later!). For simple Copperlists, COP1LCH/L alone is used. Having done this,

and turned on the Copper DMA, writing any value to COPJMP1 starts the Copper

executing your Copperlist. When the Copper reaches the impossible WAIT ins-

truction at the end of your Copperlist, it will WAIT until the vertical blank

occurs, at which point the Copper will restart at the address loaded into the

COP1LCH/L pair. More correctly, COPJMP1 causes the data in the COP1LCH/L pair

to be transferred to the Copper's internal program counter, and execution in-

itiated. If you have an address in COP2LCH/L, writing to COPJMP2 will cause

that value to be loaded instead. For simple Copperlists (no interlace or any

conditional branches) the COP1LCH/L and COPJMP1 set are the defaults used, &

these will be used by the Copper for restarting the Copperlist execution at

the vertical blank interval. NOTE:the Copper needs its 'programs' aligned on

a word boundary just like the 68000. In fact, word alignment holds for all

custom chip operations - unless I discover any exceptions and document them

in this file, treat word alignment as mandatory.

Note that the COPxLCH/L register pairs are NOT program counters! The

Copper's program counter is NOT directly accessible, and the values stored in

the COPxLCH/L register pairs are INITIAL program counter values, needing to

be loaded only once under normal circumstances. The values in COPxLCH/L never

change once loaded, unless 1) they are changed by the 68000 at some future

time (under your control!); 2) the Copperlist contains instructions such as

MOVE	#value_hi,COP1LCH	;high word of 18-bit value

MOVE	#value_lo,COP1LCL	;low word of same

which causes the Copper to alter them itself.

Conditional Branches:now it may already have become obvious to the

astute reader how to form a conditional branch in CAL. For those who haven't

yet worked out all of the details, here they are.

First, one needs to know the absolute address in memory of the point

to which to branch. BEFORE the branch is to be executed, reload COP1LCH/L

with this new address-the Copper can be made to do it using instructions such

as the example above. Then immediately after a SKIP instruction, place the

Copper instruction

MOVE	#0,COPJMP1

and the rest of the Copperlist following this. If the beam position when the

Copper reaches the SKIP instruction is greater than the SKIP instruction's

position argument, the MOVE above will be skipped, and the remaining inst-

ructions of the Copperlist following the MOVE executed. If the position is

less than the argument, the MOVE above will be executed, and the Copper will

load its internal program counter with the new value of COP1LCH/L that you

forced it to earlier on. Hey presto! Conditional branch! Of course, if you

wish to leave COP1LCH/L alone, you can use COP2LCH/L instead, and use a MOVE

to COPJMP2 to cause the branch instead.

WARNING:If you change COP1LCH/L in order to force a conditional br-

anch using the above mechanism, REMEMBER TO RESET IT TO POINT TO THE START OF

YOUR COPPERLIST AGAIN AFTER THE BRANCH! You have to do this in two places in

your Copperlist, once after the MOVE to COPJMP1 (to take account of branch

not performed) and once immediately after the branch point (to take account

of branch performed). Failure to do this will result in the Copper failing

to execute portions of your Copperlist after the first execution!

Interlaced playfields:you need two Copper lists for this, one for

the long frame and one for the short frame. The long frame Copperlist (the

first one) should initialise bitplane pointers to point to the FIRST line of

the bitplanes, and the short frame Copperlist should initialise the bitplane

pointers to point to the SECOND line of the bitplanes. At the end of the long

frame Copperlist, before the impossible WAIT, insert two instructions to set

the COP1LCH/L pair to point to the short frame Copperlist. Similarly, at the

end of the short frame Copperlist, place instructions to point COP1LCH/L at

the start of the long frame Copperlist. The Copper will then alternate back

and forth between the two Copperlists. In addition, the bitplane control (see

below) needs to have the LACE bit set, and various other instructions need to

be executed to ensure proper system synchronisation, and ensure that your in-

terlaced playfields are displayed properly. More details in the section below

on bitplane control.

Note:incorrect setting of the Copper registers can lead to the so-

called FIREWORKS_MODE of the Amiga occurring. This occurs when the Copper is

pointed to an invalid area of memory, and the Copper tries to execute what it

thinks is a Copperlist there. The Copper isn't intelligent (like some COBOL

programmers I know) and thinks that anywhere it's pointed to is a Copperlist

to execute, and happily runs it. This usually has weird and wonderful effects

such as screwing up the screen completely. This phenomenon (the runaway Cop-

per syndrome) is the ONE event that can crash the Amiga completely, beyond

even a Guru recovery. It's sometimes pretty to watch, but can result in your

Amiga containing a fried Agnus or something else fatal if you don't switch

off the moment it happens. YOU HAVE BEEN WARNED.

Copper interrupt:In the section on interrupt control, it was stated

that the Copper has its own interrupt control bit in INTENA/INTREQ. To signal

a Copper interrupt, place the instruction

MOVE	#$8010,INTREQ

into your Copperlist at the desired point, and the Copper will force the int-

errupt system to generate the Copper interrupt. Any interrupt bit can be set

this way, but the above instruction sets bit 4 of INTREQ, specially provided

for the Copper. By placing this instruction after suitable WAIT instructions

one can tell the 68000 that a given screen position has been reached, and is

the recommended method of setting up Raster Interrupts. Amiga Raster Inter-

rupts are completely controllable, and can be made to occur not only at a

given raster line position, but at a given screen column as well! Using the

MASK option in WAIT allows all manner of wonderful tricks to be performed.

The only limit is your imagination at this point.

Hardware:Bitplanes & Bitplane Control

Having discovered how to generate a Copperlist, the next logical step is to

learn how to control the bitplane usage. All bitplane control registers are

accessible to the Copper (except for those below offset $040), and thus one

can set up a Copperlist to set these registers to given values. This is of

particular value for interlaced playfields, already mentioned in the Copper

section above, but can be performed for any type of playfield if wanted.

Bitplane Control Registers:the full list of bitplane control regis-

ters is:

VPOSR	(offset $004)	Read MSB of vertical beam position

VHPOSR	(offset $006)	Read vertical/horizontal beam position

VPOSW	(offset $02A)	Write MSB of vertical beam position

VHPOSW	(offset $02C)	Write vertical/horizontal beam pos

DIWSTART	(offset $08E)	set top left corner of display window

DIWSTOP	(offset $090)	set bottom right corner of same

DDFSTART	(offset $902)	horiz. pos start of bitplane DMA fetch

DDFSTOP	(offset $094)	horiz. pos end of bitplane DMA fetch

BPL1PTH	(offset $0E0)	bitplane pointers, high/low, for

BPL1PTL	(offset $0E2)	up to 6 bitplanes as wanted

BPL2PTH	(offset $0E4)

BPL2PTL	(offset $0E6)

BPL3PTH	(offset $0E8)

BPL3PTL	(offset $0EA)

BPL4PTH	(offset $0EC)

BPL4PTL	(offset $0EE)

BPL5PTH	(offset $0F0)

BPL5PTL	(offset $0F2)

BPL6PTH	(offset $0F4)

BPL6PTL	(offset $0F6)

BPLCON0	(offset $100)	main bitplane control register

BPLCON1	(offset $102)	scroll values for outsize playfields

BPLCON2	(offset $104)	sprite/playfield & DUALPF control

BPL1MOD	(offset $108)	bitplane modulo for odd planes

BPL2MOD	(offset $10A)	bitplane modulo for even planes

The main bitplane control register BPLCON0 is organised as follows (bit set

equals function enabled,  bit clear equals function disabled):

Bit	Function

---	--------

15	HIRES		Turn on high-resolution mode

14	BPU2		These three bits contain the

13	BPU1		number of bitplanes used

12	BPU0

11	HOMOD		Hold & Modify mode on

10	DBPLF		Dual Playfield mode on

9	COLOR		Video output colour (always set this!)

8	GAUD		Genlock Audio on

7-4	Unused

3	LPEN		Lightpen input active

2	LACE		Interlace mode on

1	ERSY		External synchronisation on

0	Unused

Some restrictions exist. HOMOD and DBPLF cannot both be set simultaneously,

one or the other only can be set. Both bits can be cleared, however, and if

all six bitplanes are enabled, the hardware automatically selects the EXTRA-

HALFBRITE mode. Also, GAUD and ERSY are only useful with a Genlock interface

(DO NOT SET THEM UNLESS YOU'RE USING ONE!). The legal value range for the

BPUx bits is 0 to 6, 7 is not allowed. Don't ask me why 0 is allowed...

Having decided which screen mode is desired, one then needs to set

the bitplane sizes. The registers DIWSTART (Display Window Start) and DIWSTOP

(Display Window Stop) are used for this. Bits 15-8 contain the vertical pos-

ition, and bits 7-0 the horizontal position.

DIWSTART is assumed to rest in the top left quadrant of the screen.

This is a fairly sensible assumption, after all. Because the vertical pos-

ition can be from 0 to 313, which needs 9 bits, the top bit (not specified)

is assumed to be zero, giving vertical positions from 0 to 255. Similarly,

the missing 8th bit of the horizontal position is assumed to be 0, giving a

horizontal position from 0 to 255.

DIWSTOP is a little more complicated. It is assumed to lie in the

lower right quadrant of the screen (sensible again) and hence the 9th bit

of the horizontal position is assumed to be 1, giving horizontal positions

from 256 to 448. Because vertical end positions both greater than and less

than 255 should be possible, a trick is used. Bit 15 (the 7th bit of the

vertical position) of DIWSTOP is inverted to provide the 8th bit, making an

end position of 128 to 312 possible. For end positions from 256 to 312, make

this bit zero (thus making the hidden 8th bit equal to 1), and for end posi-

tions from 128 to 255, make this bit 1 (thus making the hidden 8th bit 0).

Also, DIWSTOP should have the horizontal and vertical values PLUS ONE set

into it to work properly.

Typical PAL values for the screen are TLC (top left corner) coord-

inates (129,41), and BRC (bottom right corner) coordinates (448,296). This

corresponds to DIWSTART = $2981, and since DIWSTOP should contain the values

(449,297) instead of (448,296), DIWSTOP = $29C1 is used. This produces a PAL

320 x 256 display area centred in the middle of the monitor display.

Limitations exist on these values. Firstly, monitor tube distortions

limit the values (the corners will be cut off if the entire monitor screen

area is used), and the blanking gaps need to be taken into account. Vertical

blanking gaps occupy lines 0 to 25, making the earliest TLC vertical position

26 lines from the VBL start, and the latest BRC vertical position is 312. The

horizontal situation is more complex. The horizontal blanking gap (HBL) lies

between colums 30 and 106. Horizontal positions from 107 are possible.

We have set the screen mode, and the bitplane size. Now we need to

set up the bitplane DMA. The DMA data fetch must start in synchronisation

with the start & stop values, to ensure that the pixels appear in the right

places on screen. Vertically, this is no problem. Screen DMA starts and ends

in synchronisation with the DIWSTART/DIWSTOP vertical positions automatically

and no register control of this is provided. Horizontally, this is a problem

however. To display a pixel on the screen, the current word needs to be read

from each bitplane. for 6 bitplanes, low-resolution, 8 bus cycles are needed.

In addition, the hardware needs a half bus cycle before the data can appear

on the screen. The bitplane DMA must therefore start exactly 8.5 cycles or

17 pixels before the start of the screen window for low-resolution screens,

and 4.5 cycles or 9 pixels before the start of the screen window for high-

resolution screens.

The registers controlling this data fetch are DDFSTART and DDFSTOP

(Display Data Fetch Start, and Display Data fetch Stop). Only bits 7 to 2

are writeable, the others are "don't care" bits and should be set to 0. Bit

2, the lowest writeable bit, should always be 0 for low-resolution screens

since the bitplanes are read once every 8 bus cycles, and the values of

both DDFSTART and DDFSTOP must be an exact multiple of 8. Regardless of the

resolution, the difference between DDFSTART and DDFSTOP (the Amiga System

Programmer's Guide is littered with misprints about here!) must always be

divisible by 8, since the hardware always divides the lines into sections

of 8 bus cycles each. Even in high-res mode, the bitplane DMA is performed

for 8 bus cycles beyond DDFSTOP, so that 32 points are always read.

Let H equal the horizontal start of the screen, as set in DIWSTART.

Also, let P equal the number of pixels per line. The values for DDFSTART and

DDFSTOP are thus computed as:

Low Resolution	: DDFSTART = (H/2 - 8.5) AND $FFF8

DDFSTOP  = DDFSTART + P/2 - 8

High resolution	: DDFSTART = (H/2 - 4.5) AND $FFF8

DDFSTOP  = DDFSTART + P/4 - 8

For our standard PAL window of 320x256 centred as above, we have H=129, P=320

and the values are thus

DDFSTART : (129/2 - 8.5) AND $FFF8 = $38

DDFSTOP  : $38 + 320/2 - 8 = $D0

For a high-resolution screen of 640x256 centred as above, we now have H=129,

P=640, and the values are thus

DDFSTART : (129/2 - 4.5) AND $FFF8 = $3C

DDFSTOP  : $3C + 640/4 - 8 = $D4

DDFSTART cannot be less than $18. This is because the first $18 bus cycles

are reserved for the memory refresh, disc and audio DMA, and the DMA channel

for sprite 0 (used as the mouse pointer) which cannot be turned off. DDFSTOP

is limited to a maximum of $D8 (horizontal blank occurs beyond this!).

Bitplane Pointers:the list above of registers includes the bitplane

pointers BPLxPTH/L. Each word-sized 'L' register combines with its 'H' coun-

terpart to form a pointer into the CHIP RAM. By setting these pointers to the

address of the start of each portion of bitplane memory, and then setting the

BPLCON0 register for the appropriate screen mode, bitplane control is almost

complete.

Note that in this case, the bitplane pointer contents are CHANGED by

the operation of the system, as opposed to the COPxLCH/L registers above. As

a result, the bitplane pointers need to be reset after each use, either by an

appropriate Copperlist, or by the 68000 during the VBL. There exist six other

registers called BPLxDAT which are accessible only by the DMA system. When a

BPLxPTH/L register pair is accessed to obtain a bitplane address, it is inc-

remented by two after the requisite data word is accessed and passed on to

the BPLxDAT register. Once the full complement of BPLxDAT registers for the

given screen are loaded, their data is passed to the display electronics, and

the process repeated. Either at the VBL or within the Copperlist, each of the

BPLxPTH/L registers need to be reset as a result. The actual reading of the

bitplane data occurs during the interval between the occurrence of DDFSTART &

DDFSTOP. After DDFSTOP has been reached, the bitplane pointers are changed by

the values contained in the BPLxMOD registers, and under normal circumstances

these registers are set to zero. The BPLxMOD registers will be covered more

fully later on in this section.

So far, this information assumes that the playfields are designed to

be the same size as the area displayed. It is perfectly possible to design a

collection of outsize playfields larger than the display area, and display a

portion of each. The playfield can be extra-tall, extra-wide or both, making

screen scrolling almost ridiculously easy in comparison with other computers

such as the Atari ST.

To manage extra-tall playfields is simplicity itself. Simply alter

the values of BPLxPTH/L used as the start point for vertical scrolling. If

this cannot be done in a Copperlist, use the 68000 during the VBL. One way

of making the Copperlist handle it is to write to the Copperlist directly,

using the Copper interrupt to signal that the Copper has executed beyond the

point at which you wish to write the new addresses into the Copperlist, and

performing the write operation during the Copper interrupt. Alternatively one

can use the 68000 during the VBL interrupt, storing the true base pointers

and the scrolled values somewhere safe beforehand, and updating the scrolled

values each time a scroll is performed.

Note that I mention using the Copper interrupt to signal that it is

safe to write into the Copperlist. If this is not done, it is possible to

write into the Copperlist at the same point being accessed by the Copper, and

thus ensuring that the Copper gets the wrong data. Use of the Copper inter-

rupt ensures that the Copper has genuinely finished with the portion of the

Copperlist being rewritten.

Managing extra-wide playfields is a little more complicated, but the

hardware makes for almost unbelievable speed in pixel-boundary horizontal

scrolling. BPLCON2 is used to control the pixel offset from 0-15 (remember,

the DMA system accesses bitplanes in 16-bit words, corresponding to 16 pixels

of bitplane data). Bits 15-8 of BPLCON2 are unused. Bits 7-4 are used to con-

trol the pixel offset for the odd planes, and bits 3-0 are used to control

the pixel offset for the even planes. BPLCON2 determines the number of pixels

to the LEFT that the screen is scrolled, so for scrolling to the RIGHT, one

must use 16-X (where X is the left scroll value) and add 2 to each of the

bitplane pointers.

Also, to ensure that the extra-wide playfield is properly displayed,

there exist modulo registers. Modulo registers are used extensively within

the Amiga hardware, particularly by the blitter. A modulo register contains a

value to be added on to a pointer register pair value in order to ensure that

the pointer points to the correct data word after a series of operations. An

example will illustrate.

Let us create a 640-pixel low-resolution display. This is twice as

wide as the standard display of 320 pixels. After reading the first 20 words

of the display (40 bytes), the bitplane pointers are pointing to word 21. In

a normal display, the BPLxMOD registers contain zero, and this is added on to

the BPLxPTH/L values to reference the next line. For our double-width play-

field, this is not correct. We want to skip another 20 words (40 bytes) to

reference the second line correctly. This is done by setting BPLxMOD to 40.

This is then added on to the BPLxPTH/L pairs by the system and the second

line of our double-width playfield is thus referenced correctly by the DMA

system.

Of course, both can be combined to make a huge display area, the

sole limitations being available CHIP memory and your imagination. This can

then be scrolled around at will.

Note that for smooth scrolling, the scroll values MUST be changed

outside the time used for displaying the actual bitplanes. This again is

possible using the Copper or the VBL interrupt as above.

Basically, scrolling smoothly is achieved by keeping a pixel scroll

value saved somewhere as well as the bitplane pointers. To scroll left, take

the pixel scroll value, add 1, AND with $0F and save back. If the result is

zero, add 2 to all bitplane pointers. Then write these values into the vari-

ous BPLCON2/BPLxPTH/L registers. For smooth right scrolling, take the pixel

scroll value, subtract 1, AND with $0F, save back. If the result equals $0F

then subtract 2 from all bitplane pointers. Write all of the resulting data

into the requisite registers. So now you know.

Double-Buffering:at this point, any reader having digested both of

the sections on Copperlists and Bitplane Control will have all the informa-

tion to hand to perform double-buffering in hardware. Set up a Copperlist for

the desired screen, complete with bitplane pointer initialisation. I find it

useful to refer to the screen within which rendering is performed as the

logical screen, and the screen currently being displayed as the physical

screen. The double-buffering technique keeps these screens separate. Set up

the Copperlist initially to point to one of the screens, which will become

the physical screen. Perform all rendering in the other screen, which will

become the logical screen, and use the Copper interrupt to determine when it

is safe to change the bitplane pointer initialisation section to restart the

Copperlist after the VBL with the identities of the two screens changed. The

previous logical screen, within which one has rendered all graphic data, will

become the new physical screen, and the current physical screen will then be-

come the new logical screen. Again, perform all rendering in the logical scr-

een. Provided that all rendering can be performed within the time taken to

display one frame (1/50th of a second), the resulting motions of graphic en-

tities within your program will be completely smooth and flicker-free. This

technique requires two sets of screen memory and is thus memory-hungry, but

it is the basic technique for most games requiring smooth object motions. I

shall add at this point that it may be possible to achieve smooth motion by

this means even if it takes up to three frames to perform all rendering, as

the movement of objects within 'Strike Force Harrier' on the ST is reason-

ably smooth (I once worked for the author of that game) even though frame

swapping only occurs at 16 frames per second. With the Amiga's far superior

hardware is should be possible to perform similar rendering at 24 frames per

second or even faster (Strike Force Harrier has up to 32 bob-type objects on

screen at once, hence the time taken for rendering!), and 50 frames per sec-

ond animation is perfectly possible with fewer objects to move, unless they

are truly huge (but see 'Menace'-some of that program's bobs are of a vast

size). With dual playfield mode and oversize playfields, it's even possible

to perform fast rendering using the blitter (see later) and perform parallax

scrolling or even two-direction scrolling!

Hardware:Sprite Management

This section has not been thoroughly tested by me for all of the possibili-

ties, because I haven't had occasion to use hardware sprites yet. However,

a mini-preamble will serve to open up ideas.

Denise, the chip responsible for sprite management, is a high-speed

sprite processor using its own DMA channels (8 in all). The existence of 8

DMA channels for sprite processing does NOT limit the programmer to 8 sprites

as on some lesser systems, and with clever programming it is possible to have

up to 72 sprites moving about at once! But before explaining sprite DMA chan-

nel reuse, the technique allowing this, the fundamentals should be covered.

First, some limitations. A hardware sprite has a maximum width of

16 pixels. It can be any size vertically up to the size of the entire screen

if wanted, though usually programmers work with 16x16 sprites or similar. A

sprite can be displayed anywhere on the screen, and appears in front of the

playfields. The Intuition mouse pointer is a hardware sprite, in actual fact

sprite 0, and the DMA allocation within a raster line never allows the time

allocated to sprite 0 to be stolen by bitplane DMA. If no other sprites are

used, the remaining sprite DMA slots CAN be stolen by bitplane DMA for really

wide displays, but it is a good idea not to do this during first experimenta-

tion with sprite management.

Also, a sprite is normally a 3-colour entity. It is possible to have

a 15-colour sprite by combining two sprites together. The restriction here is

that the sprites MUST be combined as follows:sprite 0 with sprite 1, sprite 2

with sprite 3, sprite 4 with sprite 5, and sprite 6 with sprite 7. No other

order is allowed.

Sprite colours are allocated differently for 3-colour sprites and

15-colour sprites. For 3-colour sprites, the allocations are:

Sprite No	Colour Registers

---------	----------------

0,1		17,18,19 (16 not used)

2,3		21,22,23 (20 not used)

4,5		25,26,27 (24 not used)

6,7		29,30,31 (28 not used)

The unused colours are treated as 'transparent', i.e., the playfield data

shows through where 'colour 0' pixels appear in the sprite, the 'colour 0'

for each sprite being thought of as corresponding to the unused colour reg-

isters. So, if sprite 0 has pixel colours 0,1,2,3, the actual colours used

are transparent,17,18,19. Needless to say, colours will only clash with the

other graphic objects on 5-bitplane screens, extra-halfbrite or HAM screens

in single-playfield mode. If your screen is 4 bitplanes or less, sprite col-

ours are independent of the main screen colours.

To put sprites on screen, almost all that is required is that the

programmer constructs a sprite data list in CHIP RAM, and passes a pointer

to the start of the sprite data list to Denise's sprite control registers.

Once that has been done, the DMA system handles the sprite all by itself.

A sprite data list consists of two control words, followed by the sprite

data itself, and then two more control words, which for a standard sprite

are zero to tell Denise that no more sprite processing is to be performed

using this DMA channel.

The initial two control words tell Denise where the sprite is to

be displayed, and also if two 3-colour sprites are combined to form one 15-

colour sprite.

Now the bad news. Allocation of bits in the sprite control words is

awkward to say the least. It runs as follows:

Control Word 1	: EEEEEEEEHHHHHHHH

Control Word 2	: LLLLLLLLA0000ELH

The E bits represent the first line of the sprite (called VSTART). Control

word 1 contains bits E7-E0 reading left to right, and control word 2 contains

E8. Bits E8-E0 make up the VSTART parameter.

The H bits represent the horizontal position of the sprite. Control

word 1 contains H8-H1 reading from left to right, and control word 2 contains

H0. Bits H8-H0 make up the horizontal position parameter, called HSTART.

The L bits represent the last line of the sprite plus one. Control

word 2 contains L7-L0 in the high byte, reading from left to right, and L8 at

bit 1 of the low byte. I told you it was bloody awkward! This value is known

as VSTOP.

The A bit in the second control word is the ATTACH bit. It tells the

sprite DMA system that this sprite is attached to another sprite, and is set

if this is the case (BUT ONLY IN THE SPRITE DATA LISTS FOR SPRITES 1,3,5,7!).

The comment in the Amiga System Programmer's Guide that these bits are divid-

ed somewhat impractically between these two control words is a masterpiece of

understatement!

Sprite resolution is one low-resolution pixel horizontally, and one

raster line vertically, and these values are constant since sprite DMA is in-

dependent of the playfield modes.

Now, the sprite data list is formed as follows for a single sprite:

Control Word 1, Control Word 2

Data Word 1 of L1, Data Word 2 of L1

Data Word 1 of L2, Data Word 2 of L2

Data Word 1 of L3, Data Word 2 of L3

Data Word 1 of L4, Data Word 2 of L4

...	...

Data Word 1 of LN, Data Word 2 of LN

Zero Word, Zero Word.

The sprite data is treated as being like mini bit planes, the data word 1

corresponding to 'bitplane 1' of the sprite, and data word 2 corresponding to

'bitplane 2' of the sprite. If a given bit in both words is 0, that pixel of

the sprite is transparent, else the colour allocation is as given in the 3-

colour sprite allocation table above. This preamble goes a long way toward

explaining why I haven't bothered with them up to now!

Now, if the ATTACH bit is set in sprite 1, this tells Denise that

sprite 1 is attached to sprite 0 to make a 15-colour sprite. In this case,

the sprite is treated as a '4-bitplane' entity, and the allocations are:

Data word 1, sprite 0 : 'bitplane 1'

Data word 2, sprite 0 : 'bitplane 2'

Data word 1, sprite 1 : 'bitplane 3'

Data word 2, sprite 1 : 'bitplane 4'

The same applies to the other sprites in combination in ascending numerical

order (PHEW!).

At this point, I warn the reader that if the sprite positions of

attached sprites do not match, Denise treats them as two separate sprites

anyway. POSITIONS OF ATTACHED SPRITES MUST BE IDENTICAL FOR THEM TO BE TREA-

TED AS ATTACHED SPRITES!

The colour allocations for a 15-colour sprite are transparent, then

all colour registers from 17 to 31 upwards, according to the value extracted

from a given bit position in each sprite data word. If, for example, bit 4 of

each sprite data word is 1,0,1,1 in the order above, this corresponds to a

colour value of %1101 or 13, and colour register 29 provides the colour value

for this pixel of the sprite. The colour register is 16+pixel value, unless

the pixel is %0000, in which case it's transparent. Again, PHEW!

Sprite DMA channel reuse:I mentioned earlier that it was possible to

display many sprites using the phrase 'sprite DMA channel reuse'. This means

that the two end control words of the sprite are not zero. To reuse a sprite

DMA channel, append the entire sprite data list of a second sprite onto the

first, replacing the zero control words of the first sprite with the starting

control words of the second. Again, if no more sprites are to be displayed,

the final control words of the entire list are zero, else the procedure of

appending a sprite data list continues for as many sprites as required, bear-

ing in mind an important limitation:there must be at least one raster line

between sprites thus appended into a reuse list, to give the DMA time to

read in the new control words.

Ok, what if you don't want to use all 8 sprites? well, turning on

sprite DMA activates all 8 sprite DMA channels, and so the unused ones must

be passed a pair of zero control words to render them inactive. One can use

the existing zero control words at the end of some genuine sprites for this

purpose.

Now for the important part. You have created your sprite data lists

and want to see them activated. Write the address of the start of each of

your sprite lists to the SPRxPTH/L register pairs. The offsets for each of

these registers are:

SPR0PTH	: offset $120

SPR0PTL	: offset $122

SPR1PTH	: offset $124

SPR1PTL	: offset $126

SPR2PTH	: offset $128

SPR2PTL	: offset $12A

SPR3PTH	: offset $12C

SPR3PTL	: offset $12E

SPR4PTH	: offset $130

SPR4PTL	: offset $132

SPR5PTH	: offset $134

SPR5PTL	: offset $136

SPR6PTH	: offset $138

SPR6PTL	: offset $13A

SPR7PTH	: offset $13C

SPR7PTL	: offset $13E

This can be done using the Copperlist as might be expected, or the hard way

using the 68000. In any case, initialisation of all of these pointers MUST

be performed in the vertical blank interval if sprite DMA is enabled even if

the registers are pointed at zero control words to disable them. Furthermore

the values stored in these registers change during sprite DMA usage, and so

every time the vertical blank interval occurs, the SPRxPTH/L registers must

be re-initialised, either by the 68000 or using a Copperlist.

Moving the sprites by changing the position data in the initial con-

trol words must also be performed during the vertical blank interval to en-

sure that Denise receives the correct data, else your sprites could jump all

over the screen in a weird and wonderful fashion! You can use the Copper int-

errupt to signal that it's safe to change them if using a Copperlist to init-

ialise the SPRxPTH/L values, by ensuring that the change only occurs AFTER

the Copper has initialised the registers. Upon initialisation, the control

words are read immediately & held for processing until the correct beam pos-

ition has been reached for displaying them, and once read, the values in the

sprite data lists can be changed safely.

Sprite and Playfield Priority:Having introduced the reader to the

hellish delights of the Sprite management system's basic control registers, I

now wish to make life even more complex by introducing sprite/playfield pri-

ority allocation.

First of all, the lower the sprite number, the higher the priority.

This means that sprite 0 hs higher priority than sprite 1, etc., and that the

sprites are displayed as though they were on separate planes, the plane for

sprite 0 being in front of the other sprite planes. In actuality, the sprite

priorities are grouped into the same pairs as for sprite attachment, particu-

larly when the playfields are brought into the whole picture.

So, considering the sprites as paired for priority purposes, in the

same manner as for sprite attachment, a playfield can be arranged in order

of priority according to the table below. In this table, P represents the

playfield, and the digit pairs 01, 23, etc., represent the sprite pairs. All

possible combinations are given below, the element to the left of the pri-

ority arrangement entry list being that with the highest priority.

Playfield Pos		Priority Arrangement

-------------		--------------------

0			P   01  23  45  67

1			01   P  23  45  67

2			01  23  P   45  67

3			01  23  45  P   67

4			01  23  45  67  P

Now, if only one playfield is selected, then this table holds for that play-

field. If dual playfield mode is selected, then this table holds for each of

the playfields INDEPENDENTLY (with a few limitations). The selection of the

priorities is controlled by BPLCON2 (offset $104), whose bits are allocated

as follows:

Bit		Function

---		--------

15-7		Unused

6		Playfield Relative Priority

5-3		Priority of Playfield 2 rel. to sprites

2-0		Priority of Playfield 1 rel. to sprites

If bit 6 of this word is set in dual-playfield mode, playfield 2 is deemed

to have higher priority than playfield 1, else playfield 1 has priority over

playfield 2 (the usual state of affairs). Bits 5-3 determine the priority of

playfield 2 relative to the sprites, and the 3-bit value to insert here is

the value in the sprite/playfield priority table above labelled 'Playfield

Pos', corresponding to the given sprite/playfield priority in the table. In

the same way, the 3-bit value for bits 2-0 determining the priority of play-

field 1 relative to the sprites is chosen from the above table.

Now, since the two playfields have a relative priority, and each of

the playfields has its own independent priority relative to the sprites, it

is a fair question to ask whether the playfields' priority relative to each

other has precedence over their priority relative to the sprites. The answer

is YES. In the Amiga System Programmer's Guide, an example is given for the

value BPLCON2 = $0003. Here, bit 6 is zero, so playfield 1 should be in front

of playfield 2. Bits 5-3 are zero, so playfield 2 should appear in front of

all sprites from 0-7. Bits 2-0 have the value 3, meaning that playfield 1 is

in front of sprites 6 & 7, and behind all of the others. A quick glance at

this description shows something amiss. Playfield 2 cannot be in front of all

sprites and at the same time behind playfield 1 (which is behind sprites 0 to

5). When one of the sprites 0-5 is BETWEEN playfields 1 and 2, it appears in

front of playfield 1, according to its priority. Since this is in front of

playfield 2, the sprite is visible at this point, although it must actually

be behind playfield 2. If only playfield 2 and the sprite are at a given pos-

ition, playfield 2 covers the sprite because of its priority.

In single playfield mode, the bit 6-3 have no function, and should

be set to zero. Bits 2-0 still control sprite priorities, and the position of

the single playfield relative to the sprites.

Sprite Collision:those readers wishing that they had never bothered

with sprite handling after reaching this point due to the complexity of the

sprite management system are about to burst into tears over sprite collision.

Firstly, let us overview the basic principles of object collision.

The fundamental principle of graphic element collision is this:when

two graphic elements overlap at a screen position, and both objects have a

set pixel at the same screen position, this is treated as a collision bet-

ween the two graphic elements. More sophisticated collision algorithms for

certain purposes do exist, but these will be ignored here, as they are not

implemented in the Amiga hardware, as will the coordinate comparison algo-

rithm (which is simpler still in some respects and very quick if speed is of

the essence).

When a collision between graphic elements occurs on the Amiga, it

is signalled by setting a bit in the CLXDAT register (offset $00E), which is

a read-only register from the 68000's point of view (only the sprite manage-

ment and blitter DMA can write to this register). The bit allocations of

CLXDAT are as follows:

Bit		Function

---		--------

15		Unused

14		Sprite 4/5 collides with sprite 6/7

13		Sprite 2/3 collides with sprite 6/7

12		Sprite 2/3 collides with sprite 4/5

11		Sprite 0/1 collides with sprite 6/7

10		Sprite 0/1 collides with sprite 4/5

9		Sprite 0/1 collides with sprite 2/3

8		Playfield 2 collides with sprite 6/7

7		Playfield 2 collides with sprite 4/5

6		Playfield 2 collides with sprite 2/3

5		Playfield 2 collides with sprite 0/1

4		Playfield 1 collides with sprite 6/7

3		Playfield 1 collides with sprite 4/5

2		Playfield 1 collides with sprite 2/3

1		Playfield 1 collides with sprite 0/1

0		Playfield 1 collides with playfield 2

The rules for collision detection are that any non-transparent sprite pixel

can cause a collision to be registered. However, it is possible to decide by

appropriate programming to choose which playfield bitplanes are used in the

determination of collision detection. Also, it is possible to include or ex-

clude any odd-numbered sprite from collision detection. Which graphic ele-

ments are used for collision detection purposes is decided by programming the

CLXCON register (offset $098), which is a write-only register from the point

of view of the 68000. The bit allocations for CLXCON are as follows:

Bit		Function

---		--------

15		Enable collision detection, sprite 7

14		Enable collision detection, sprite 5

13		Enable collision detection, sprite 3

12		Enable collision detection, sprite 1

11		Use bitplane 6 for collision detection

10		Use bitplane 5 for collision detection

9		Use bitplane 4 for collision detection

8		Use bitplane 3 for collision detection

7		Use bitplane 2 for collision detection

6		Use bitplane 1 for collision detection

5		Bitplane 6 collision match bit

4		Bitplane 5 collision match bit

3		Bitplane 4 collision match bit

2		Bitplane 3 collision match bit

1		Bitplane 2 collision match bit

0		Bitplane 1 collision mask bit

The first problem the programmer encounters is that collisions between adja-

cent numbered sprites used for sprite attachment cannot be performed. Thus

a collision between sprites 0 and 1, for example, will not be registered in

the CLXDAT register. Collisions between sprite 0 and any sprite from 2 to 7,

or between sprite 1 and sprites 2 to 7, will be registered if the appropriate

control bits are set. If the bit to enable sprite 1 collision detection is

cleared, only sprite 0 collisions between other sprites and/or the playfields

will be reported. If the bit is set, then BOTH sprite 0 AND sprite 1 collis-

ions will be reported, and furthermore reported in the same CLXDAT bit! Thus

choice of sprites needs to be handled carefully if distinctions between the

sprites are important for collision purposes, because in the example I have

just cited, sprites 0 & 1 use the same bit of CLXDAT for collision detection

and thus telling them apart is impossible by merely scanning CLXDAT. If two

sprites have been combined into a single 15-colour sprite using the ATTACH

bit, the corresponding bits for odd sprite collision detection must be set

in CLXCON in order for collision detection to be performed correctly.

For the playfields, the level of control is much more complete. If a

given bit is set in the CLXCON register above from bits 11 to 6, the relevant

bitplanes will be used in collision detection. Bits 5 to 0 are called Match

Bit Plane Value bits, and are used to determine what values to use for the

comparison before reporting collision detection.

Let us assume that we are using 6 bitplanes, and that the BPLCON0

value has enabled all 6 bitplane DMA channels (see the Bitplane Control sec-

tion above). If one of the bits 11 to 6 (called the Enable Bitplane bits)

is set, that bitplane is used. The corresponding match bit (bits 5 to 0) in

the CLXCON register is then used to compare with the given pixel. Let us ass-

ume that the Enable bit for bitplane 3 (bit 8 of CLXCON) is set. If the value

of the pixel data on bitplane 3 at the collision detection point MATCHES the

value of the match bit for bitplane 3 in CLXCON (bit 2) then a collision is

reported. So if the pixel bit is 0, and the match bit is 0 also, the collis-

ion is reported, as in the case when the pixel bit is 1 and the match bit in

CLXCON is 1. If we don't care about a particular bitplane (say for example we

wish to ignore bitplane 1 altogether for collision detection), clear the En-

able bitplane bit for bitplane 1 (bit 6). Now, the value of bit 0 of CLXCON

doesn't matter-the collision will be reported regardless of the state of

the pixels on bitplane 1.

The table given in the Amiga Systems Programmer's Guide is repro-

duced here for those who want it. It correlates directly with the above des-

cription of bitplane selection & collision detection control. The Enable Bit-

plane bits are referred to as ENBPx, the Match Bitplane Value bits as MVBPx.

The 'xx' bits in the table below are "don't care" bits-they can be either 0

or 1.

ENBPx	MVBPx	Collision possible with bit pattern

-----	-----	-----------------------------------

111111	111111	111111 only

111111	111000	111000 only

111100	1111xx	111100, 111101, 111110, 111111 only

011111	x00000	000000, 100000 only

000000	xxxxxx	Any bit pattern!!!

Take note, that if fewer than 6 bitplanes are used, the ENBPx bits for the

unused bitplanes MUST be set to zero!

Needless to say, if the colours are chosen suitably, various colli-

sion strategies based upon colour can be constructed, as well as strategies

based directly upon bitplane management. It is possible, for example, to set

sprite collision to register with only red and green pixels of a playfield,

or collision with the transparent points of playfield 1 to register only if

the underlying pixels of playfield 2 are black.

Spurious Sprite Video Data:occasionally software that directly con-

trols the hardware suffers from the appearance of a line down the screen at

some point. Analysis usually (but not always) shows that this line corres-

ponds in position to the position of the CLI sprite pointer prior to swit-

ching off sprite DMA. If this is done before the sprite management system has

finished displaying the CLI sprite pointer, then when sprite DMA is turned

off, the sprite management system cannot read the end control words, and thus

continues displaying the sprite data onscreen. Even when sprite DMA is re-

enabled, the contents of the sprite data pointers may not point to zero con-

trol words, in which case spurious sprite video data may continue to appear

until the sprite data pointers are changed.

There are two ways to prevent this. The first technique is to point

the sprite data pointer registers at zero control words. The second technique

(thanks to Count Zero) is to wait for the electron beam to reach a position

beyond the maximum possible display position of the CLI sprite pointer, and

then turn off the sprite DMA (use a raster line value of 300 for PAL Amigas).

Other Sprite Registers:there exist other sprite data registers, that

are normally accessed by the sprite management system DMA alone. These regis-

ters can be accessed in software by the 68000 also, and these registers are:

SPR0POS		(offset $140)

SPR0CTL		(offset $142)

SPR0DATA		(offset $144)

SPR0DATB		(offset $146)

...		    ...

SPR7POS		(offset $178)

SPR7CTL		(offset $17A)

SPR7DATA		(offset $17C)

SPR7DATB		(offset $17E)

The registers for SPRxPOS onwards occupy the entire range of offsets from

$140 to $17E, in ascending sprite number order.

When the programmer assigns sprite management control to the stan-

dard sprite management DMA channels, the sequence of events is:

1) DMA system loads two control words into SPRxPOS (control

word 1) and SPRxCTL (control word 2).

2) DMA system turns off sprite output.

3) DMA controller waits for the electron beam to reach the

value in the VSTART portion of the sprite control words.

4) Once this position is reached, data words are written

into SPRxDATA and SPRDATB.

5) DMA controller turns on sprite output again, and the

values in SPRxDATA and SPRxDATB are used for the

current raster line. These are positioned according to

the HSTART value.

6) DMA controller continues reading data into SPRxDATA/B

and displaying it until the VSTOP value is reached.

7) The DMA controller reads the two control words at the

end of the sprite data list. If these are non-zero, the

sprite data channel is being re-used, and the sequence

of events begins again at 1).

8) If the DMA system encounters the two zero control words

at the end of the sprite data list, the DMA controller

turns off sprite data output on this DMA channel until

the vertical blank interval occurs, at which point the

sprite DMA begins its display sequence at 1) again.

If the programmer uses the 68000 to access these registers, normally used

solely by the DMA controller (but accessible to the 68000), then there are

a few changes to take note of. First, the sprite data pointer registers need

to be initialised as for sprite management via DMA-this remains the same.

The sprite data list contents change somewhat, however. If the 68000 is used

to load the SPRxPOS/SPRxCTL registers, then only the HSTART value in the

sprite data list control words, plus the value of the ATTACH bit, need to be

valid. VSTART and VSTOP are used only by the DMA controller.

Sprite data output can then being by writing data to the SPRxDATA/B

registers. Write to SPRxDATB first, as writing to SPRxDATA causes the data

from both registers to be output to the screen. Note that if the DMA control-

ler is bypassed in this way, that fresh data needs to be supplied for each

line of the sprite to the SPRxDATA/B registers if needed (normally this is

performed by the DMA controller) unless all that is wanted is a solid column

of identical pixel data.

To turn off the sprite again, simply write some value to SPRxPOS. I

suggest writing the value zero.

Hardware:The Blitter

The blitter is THE chip that makes the Amiga so special, and it will come as

no surprise to realise that this section will probably be the largest section

in this file. It has the capacity to move data at a peak speed of 16 million

pixels per second, perform logical operations upon its data sources before

generating its output, and is used for three principal functions:

1) Transferring rectangular graphic data blocks to screen

bitplane memory (with logical operations to change the

plotting method)

2) Drawing lines between any two points on screen

3) Filling bounded areas (taking account of some restrictions)

to create filled polygon shapes

This is not its entire repertoire, however. The blitter contains all of the

on-chip logic necessary to perform vector-arcitecture mathematics, but is

prevented from doing so directly by its hard-wired design. However, it is

possible to make the blitter perform high-speed computational functions on

large blocks of data in one go, provided that one is familiar with Boolean

Algebra and has at least a first-year university level grounding in formal

logic up to the level of alternational normal schemata (I recommend as THE

definitive text on formal logic for those interested to be 'Methods In Logic'

by Willard Van Ormand Quine - ask for the Library of Congress record number

in preference to the ISBN number as it's an American publication).

Having whetted the appetite, now comes a small amount of bad news.

The blitter is powerful, but its power has associated with it a certain am-

ount of complexity. In particular, although it has a well-defined set of

registers for function control, the bit allocations of the BLTCONx control

registers (see register list below) change dramatically with each function.

There are also several rigid conventions to follow, otherwise the blitter

may just scribble at high speed all over critical program memory, as once

started up, it cannot be stopped halfway.

Blitter Register List:the list of registers associated with the

blitter is:

Register	Offset		Function

--------	------		--------

BLTDDAT	  000		Blitter Data D (read-only!)

BLTCON0	  040		Blitter Control Register 0

BLTCON1	  042		Blitter Control Register 1

BLTAFWM	  044		Blitter A first word mask

BLTALWM	  046		Blitter A last word mask

BLTCPTH	  048		Source C data pointer high word

BLTCPTL	  04A		Source C data pointer low word

BLTBPTH	  04C		Source B data pointer high word

BLTBPTL	  04E		Source B data pointer low word

BLTAPTH	  050		Source A data pointer high word

BLTAPTL	  052		Source A data pointer low word

BLTDPTH	  054		Destination D data pointer high word

BLTDPTL	  056		Destination D data pointer low word

BLTSIZE	  058		Controls data size/starts blitter

BLTCMOD	  060		Source C modulo register

BLTBMOD	  062		Source B modulo register

BLTAMOD	  064		Source A modulo register

BLTDMOD	  066		Destination D modulo register

BLTCDAT	  070		Blitter Source C Data

BLTBDAT	  072		Blitter Source B Data

BLTADAT	  074		Blitter Source A Data

Some of these registers are not accessed during standard blitter usage. They

do come into play for some of the less well-publicised functions for which

the blitter is used. More of this later.

Rectangular Data Block Moving:this is the first blitter function to

be dealt with in this section, and a preamble will help introduce certain key

concepts.

The blitter, when operating in data copy mode, takes data from up to

three different source areas of memory, combines them using a logical opera-

tion, and then writes the data out to the destination memory area. The main

use of this function is for copying large blocks of graphic data to bitplane

memory for display, or copying portions of bitplane memory to safe off-screen

areas for background saving.

In general, the source memory is linearly organised, and the graphic

data occurs in sequential words in memory. At this point I stress that the

blitter is a WORD-based device, and that all of the blitter's activities are

based upon word-aligned memory, just like the 68000's program access. Return-

ing to the graphic data, this linear organisation means that the data can be

read sequentially without any problems.

However, to illustrate the blitter methodology, let there exist a

low-resolution screen of one bitplane, and some graphic data that is to be

written to the screen. The low-resolution bitplane is 40 bytes across, or 20

words, and the graphic data has a maximum width of 48 pixels (3 words). The

pointer to the word on the screen bitplane where the data is to be put is

written to the BLTDPTH/L registers, and the pointer to the graphic data is

written to the BLTAPTH/L registers. For now let us ignore other sources and

the actual details of the logical operation used. The blitter functions by

using the address pointers to fetch a data word from each source (here we are

using one source only, source A), and after processing it internally, uses

the pointer to the destination to write the data to the screen. It then inc-

rements the pointers by 2 to access the next word. So for each blitter oper-

ation, the pointers MUST be re-initialised (unless they happen to end up

pointing to memory areas to be referenced by another blitter operation, in

which case the blitter can simply be started up again).

For the graphic data, this poses no problem, since it is sequential-

ly organised in memory. But after writing 3 words of data, the blitter must

have a correction added to the destination pointer to point to the next ras-

ter line to write to. This correction is called the modulo, and is stored in

the modulo register for the appropriate source and destination. In this exam-

ple, the blitter needs to have a correction of 17 words added to the destina-

tion pointer to reference the screen memory correctly, or 34 bytes. Thus the

modulo for the destination, BLTDMOD, is set to 34. The modulo for source A,

BLTAMOD, is set to zero. If other sources are used, the appropriate modulo

must be selected. If source B is used as a graphic data mask organised in the

same way as the actual pixel data, its modulo is again zero. If the source C

is used as a reference to the screen for background masking-in, its modulo

must be the same as the destination modulo, i.e., 34 in this example.

So, the organisation of source and destination memory needs to be

analysed before setting up both the BLTxPTH/L registers, and the BLTxMOD reg-

isters. This information should be sufficient to cover initialisation of the

registers just mentioned.

Now we need to decide how much data to transfer. The blitter accepts

this information as a single word, coded as follows:

HHHHHHHHHHWWWWWW

The H bits represent the height of the graphic data. This can take any value

from 0 to 1023 lines. Zero is taken to mean the maximum size of 1024 lines,

as a value of zero lines is otherwise silly. The W bits represent the width

of the graphic data in memory words, in our example this is 3. So if our data

is 40 raster lines deep, the value of the H bits is 40, and the value of the

W bits is 3. This gives a final value of

(40 * 64) + 3 = $0A03

This value is written to the BLTSIZE register in the table above. The method

I use for computing BLTSIZE is something of the order of

move.w	rows(a0),d0	;no of raster lines

and.w	#$3FF,d0		;ensure value is 0-1023

lsl.w	#6,d0		;shift

move.w	cols(a6),d1	;no of WORDS across!

and.w	#$3F,d1		;ensure value is 0-63

add.w	d1,d0		;add it in

move.w	d0,BLTSIZE(a5)	;here a5 contains $DFF000...

However, writing to the BLTSIZE register starts the blitter! So this

must be the LAST operation performed upon the blitter registers for a given

blitter operation.

Now we need to consider logical operations. It is of great assist-

ance if the programmer has a thorough grounding in Boolean Algebra at this

point, since the method used by the blitter relies heavily upon this. The

terminology used for each logical operation selected is 'minterm', which is

short and sweet. The formal technique for deriving minterms labours under the

unfortunate name of 'developed alternation normal schema formation', a fair

mouthful for anyone to handle.

The simple way of thinking about this is to remember that the blit-

ter has 8 possible logical operations hard-wired into it. These operations

are:

ABC	:	A AND B AND C

ABc	:	A AND B AND (NOT C)

AbC	:	A AND (NOT B) AND C

Abc	:	A AND (NOT B) AND (NOT C)

aBC	:	(NOT A) AND B AND C

aBc	:	(NOT A) AND B AND (NOT C)

abC	:	(NOT A) AND (NOT B) AND C

abc	:	(NOT A) AND (NOT B) AND (NOT C)

The next table shows which of the operations produce a 1 (true) bit output

dependent upon the values of the input bits:

Operation	A B C	LFx Bit No.

---------	- - -	----------

ABC		1 1 1	     7

ABc		1 1 0	     6

AbC		1 0 1	     5

Abc		1 0 0	     4

aBC		0 1 1	     3

aBc		0 1 0	     2

abC		0 0 1	     1

abc		0 0 0	     0

When the blitter processes its data from each source, A,B,C, it feeds the

data into circuits whose outputs are each of the 8 logical operations above.

These are then ORed together to produce the final result sent to the dest-

ination memory. Which ones are selected to OR together are under the control

of the programmer. The LFx bit numbers in the table above are used to select

them, and a byte containing the approriate bits set selects the given logical

operation. So, to select ABC+ABc+AbC+Abc, one uses the select byte $F0 (or,

%11110000 in binary).

But how do we select them? In our example, we want the destination

to match the source input. In other words, D = A. But since B and C are al-

ways present in the blitter, how do we introduce them? Well, we don't care

about the state of B, so we want operations containing both AB and Ab. In a

like manner, we don't care about C, so we want operations containing AC and

Ac. This is the same as performing the Boolean Algebra operations (here, + is

taken to mean OR, and AB is taken to mean A AND B):

A(B+b) = AB + Ab

A(C+c) = AC + Ac

But we want terms of the form ABC etc. Well, take the first expression

A(B+b) = AB + Ab

and append the (C+c) used in the second:

A(B+b)(C+c) = AB(C+c) + Ab(C+c)

= ABC +ABc + AbC + Abc

This just happens to be the example encoded as the select byte $F0 above. The

principle is the same throughout. Some more examples are:

Invert Graphic Data : D = a

D = a

= a(B+b)(C+c)

= aB(C+c) + ab(C+c)

= aBC + aBc + abC + abc

Which is encoded as the select byte $0F.

OR in a graphic into the bitplane : D = A + C

D = A + C

= A(B+b)(C+c) + C(A+a)(B+b)

= AB(C+c) + Ab(C+c) + CA(B+b) + Ca(B+b)

= ABC + ABc + AbC + Abc + CAB + CAb + CaB + Cab

= ABC + ABc + AbC + Abc + ABC + AbC + aBC + abC

= ABC + ABc + AbC + Abc + aBC + abC

Which is encoded as the select byte $FA. Note that where identical terms

appear in the expression above, surplus ones are simply deleted from the

expression.

The last example I shall give is the so-called 'cookie-cut' opera-

tion. This name originates from the way cookie biscuits are cut from the

biscuit mixture when making chocolate chip cookies, familiar to any American

especially one who has encountered 'Girl Ranger' cookies. This operation is

one where the data A is masked first. If the data A is 1 at this point, we

want the masked data to be written. If the data A is 0, we want it to be re-

garded as transparent, and hence the background to show through. This allows

the mask to create opaque 0 pixels within the data, and any 0 pixels in the

A data to be regarded as transparent by having the corrsponding mask pixel

set to 1. The operation becomes:

Cookie cut : D = AB + aC

D = AB(C+c) + a(B+b)C

= ABC + ABc + aBC + abC

which encodes as the select byte $CA. In this case, the mask will be a nega-

tive image (photographically speaking) of the graphic data except where any

transparent pixels are required.

So, we now know how to select which memory areas to transfer, how to

set modulo values for trasnfer to bitplanes, how to determine the data size

(and also start the blitter), and select the appropriate logical operation. I

now wish to intoduce the blitter control registers. These affect the manner

in which the blitter works. The two blitter control registers, BLTCON0 and

BLTCON1, have control bits allocated according to the following tables:

Bit	BLTCON0 Function

---	----------------

15-12	ASH3-0 : contain the source A shift distance (see later)

11-8	USEA-D : select which sources/destination are used

7-0	LFx    : logical function selection bits mentioned above

Bit	BLTCON1 Function

---	----------------

15-12	BSH3-0 : contain the source B shift distance (see later)

11-5	Unused

4	EFE    : Exclusive Fill Enable

3	IFE    : Inclusive Fill Enable

2	FCI    : Fill Carry In

1	DESC   : Descending mode

0	LINE   : Turn on Line Drawing Mode

In the case of BLTCON0, the LFx bits that select the minterms have already

been covered. This leaves the USEA-D bits and the ASH3-0 bits.

The USEx bits determine which sources/destination are used. If a

given USEx bit is set, the DMA control system for that source (or destin-

ation) is fully enabled, in which case the blitter operation proceeds norm-

ally as described earlier. If a source is not being used at all, clear the

appropriate USEx bit. This has the effect of disabling the full effects of

the DMA channel, but NOT of stopping data transfer altogether. Because of

this, minterms have to be chosen to effectively ignore the given source, as

well as clearing the given USEx bit. Instead, the same word contained in

the BLTxDAT registers for the given source is read continuously and not up-

dated. This can be used to fill memory with a given value, as follows:

move.w	#value,BLTADAT(a5)	;value to fill

move.w	#$01F0,BLTCON0(a5) ;minterms D=A, USED only

move.w	#0,BLTCON1(a5)	 ;mode = copy data

lea	memory_to_fill(pc),a0 ;start of memory area

move.l	a0,BLTDPTH(a5)	;point Blitter at it

move.w	#size,BLTSIZE(a5) ;how much to fill & startup!

If copying graphic data, pick the USEx bits carefully. Ensure that if you ARE

using a given source, set it's USEx bit. And whatever you do, don't forget to

set USED to enable the destination output (the number of programmers who have

forgotten to set USED at some time doesn't bear thinking about) or else the

blitter won't be able to generate the desired output!

The ASH3-0 bits are used to 'fine-tune' the blitter data positioning

for graphic output. Normally, the blitter can only output its data to a word

boundary, being a word-oriented device. To enable pixel-boundary data plot-

ting, the blitter has the ability to shift input data before outputting it.

The ASH3-0 bits contain the number of bit positions to shift the data from

source A to the right before outputting it. The bits BSH3-0 in BLTCON1 have

an almost identical function, this time affecting source B. Hence source A

is usually chosen to be the graphic data, and source B any mask data used. If

background data is used for transparency generation, source C is generally

the preferred choice. Both graphic and mask are shifted before output.

Moving on to BLTCON1, for data copying, set all bits other than the

BSH3-0 bits to zero. These bits are used for line drawing mode and boundary-

filling mode, covered later.

To complete the register list, there are two mask registers for the

data source A. These are BLTAFWM (Blitter source A first word mask) and

BLTALWM (Blitter source A last word mask). If these registers are both zero,

the first and last word of each raster line of copied graphic data will be

zeroed out-they are used as filter masks for the left and right edges of a

graphic data block. If the graphic is one data word wide, the two registers

operate upon the same source A data word, and only those source A bits that

have the corresponding bits set in BOTH mask registers are allowed through

unchanged (otherwise they are treated as zero). For wider graphic data blocks

the mask registers mask the end words of the data. As yet, I have not had

time to establish conclusively whether source A masking is performed BEFORE

or AFTER source A data shifting under BLTCON0 control, and as the result is

critically dependent upon this, I suggest experimentation before assuming one

way or the other.

As a final note concerning the blitter in data copy mode, I said

above that the remaining BLTCON1 bits other than the BSH3-0 bits should be

cleared. Normally this is the case (using the blitter for graphic data manip-

ulation), but if the blitter is used to move memory blocks, particularly if

the data blocks are overlapping, then it is time to consider setting the

DESC bit.

The DESC bit of BLTCON1 controls whether the address incrementing of

the BLTxPTH/L registers is positive (ascending mode, DESC=0) or negative (de-

scending mode, DESC=1). An example illustrates the point. Let us copy a block

of memory of size N bytes (N will have to be even for blitter copy), starting

at address X, to a new location at address Y. If X lies lower in memory than

Y, but the difference between X and Y is less than N bytes, then the initial

copy operation will erase some of the data at the end of the block to be cop-

ied. To prevent this, it is possible to copy backwards by setting DESC=1. If

this is done, however, the pointer registers should be set to addresses X+N

and Y+N instead of addresses X and Y, because copying will start at the END

of the blocks.

NOTE:until recently, I assumed that the choice of Source A for the

graphic data, Source B for the mask, and Source C for the background when a

masked blit was performed was OK. This has transpired to be incorrect. Source

channel selection SHOULD be:

Source A = MASK

Source B = DATA

Source C = BACKGROUND

If this order of selection is made, then the standard minterms mentioned in

the Amiga Hardware Reference Manual hold true, e.g., $CA is the 'cookie-cut'

minterm. See later for an example.

Line Drawing:the blitter has inbuilt line-drawing logic which was

added by its designer, Jay Miner, after discovering that the other functions

left space on the silicon for the line-drawing hardware (wow-history lesson

too!) and decided that this would be a welcome feature to include.

The problem with the blitter's line-drawing logic is that it uses a

method unfamiliar to anyone having no experience of hardware geometry engines

(a generic term used to describe high-speed graphics-specific processors). A

line drawn with the blitter has to be described in a manner conforming to the

method used. I shall try to make this simple, but if it seems hard going, I

implore you to persevere-this imformation will apply to far more sophistica-

ted geometry engines as well and hence has a wider application.

Lines are represented under normal circumstances using the start and

end coordinates. If the end points of the line being drawn are P (x1,y1) and

Q (x2,y2), this is usually sufficient information. The alternative informa-

tion used by the blitter and other geometry engines is 1) the address of the

memory word in the bitplane where the start point P lies; 2) the number of

points that the line will occupy once drawn; 3) the angular orientation of

the line represented in terms of which 45 degree compass segment (or octant)

that the said angle lies in (measured anticlockwise from zero degrees from

the X-axis:in this system of measurement 90 degrees is due North, 180 degrees

is due West, 270 degrees is due South etc). These compass segments, or oct-

ants, are defined using octant numbers, allocated according to the table:

Angle Range	Octant No.

-----------	---------

0-45 degrees	    0

45-90 degrees	    1

90-135 degrees	    2

135-180 degrees	    3

180-225 degrees	    4

225-270 degrees	    5

270-315 degrees	    6

315-360 degrees	    7

Note that this table assumes that the Y-axis is drawn to be positive

when pointing DOWNWARDS, thus setting up the usual screen coordinate system

where (0,0) is the TOP LEFT CORNER of the screen.

The octant information is insufficient in itself, however. Before I

explain the relationship between the octant numbers and the data actually

sent to the blitter, I shall give the equations describing the line that are

used by the blitter. These are :

dX = X2 - X1	These are used to determine

dY = Y2 - Y1	which octant the line lies in

DX = ABS (dX)	DX = Delta X

DY = ABS (dY)	DY = Delta Y

DS = MIN(DX,DY)	DS = Delta S (smaller Delta)

DL = MAX(DX,DY)	DL = Delta L (larger Delta)

These expressions form the basis of geometry engine line drawing (including

that of the blitter, even if it cannot be regarded as a true geometry engine

in the same manner as, for example, the Weitek 1164/1165 series used in the

1167 accelerator board for the Compaq DeskPro 386/20 PC, or the TMS34020).

Now for the hard part. Delta X, Delta Y and Delta S/L are used to

determine which octant should be selected for the line. This is done by the

conversion of the octant number into a 3-bit number, which corresponds to

three bits in the BLTCON1 register as used for line drawing control. How the

bits in BLTCON1 are allocated changes when line drawing is performed. The

bit allocations are:

Bit No		Name		Function

------		----		--------

15-12		TEXTURE3-0	Value for mask shifting (see

below)

11-7		Unused		Always set to zero

6		SIGN		Changes line drawing direction

5		Unused		Always set to zero

4		SUL		Sometimes Up or Left

3		SUD		Sometimes Up or Down

2		AUL		Always Up or Left

1		SING		Singular bit (see below)

0		LINE		Always set to 1 for line

drawing

The SUD/SUL/AUL bits are set or cleared according to the following values:

Octant No.	SUD	SUL	AUL

---------	---	---	---

0		 1	 0	 0

1		 0	 0	 0

2		 1	 1	 0

3		 0	 0	 1

4		 1	 0	 1

5		 0	 1	 0

6		 1	 1	 1

7		 0	 1	 1

and the SIGN bit is set if the computed value of (2 * DS) - DL is less than

zero (to change the direction in which the line is rendered for lines with a

negative slope).

Under normal circumstances, the USEA-D bits in BLTCON0 should be set

to the values:

USEA = 1

USEB = 0

USEC = 1

USED = 1

and the minterms set to $CA. The value of the ASH3-0 bits is set to the value

x1 MOD 16

to determine which bit in the start word is the start bit of the line, and in

most literature on blitter line drawing these bits change their name to the

START3-0 bits.

The line can be drawn using a mask, to provide dotted lines accord-

ing to a line dot pattern. This mask is written to BLTBDAT (see register list

above), and for solid lines the value to use is $FFFF. A value of $AAAA or

$5555 produces a finely dotted line, a value of $CCCC a more coarsely dotted

line.

So, to give the blitter register initialisation values. These are as

follows (noting the values DX, DY, DS and DL above):

BLTCPTH/L, BLTDPTH/L	: Put the start address of the first

point of the line in these registers.

BLTCMOD, BLTDMOD		: Number of bytes making up one raster

line of the bitplane within which the

line is to be rendered. For a low-res

bitplane this is 40. See example code

below.

BLTBMOD			: Set to 2 * DS.

BLTAMOD			: Set to (2 * DS) - (2 * DL).

BLTAPTL			: Set to (2 * DS) - DL.

BLTADAT			: Set to $8000

BLTBDAT			: Set to your chosen pattern mask

BLTAFWM			: Always set to $FFFF

BLTSIZE			: Last register initialised (it sets the

blitter going). Set width bits equal to

2 always. Set height bits equal to DL.

Hence value to use is (DL * 64) + 2.

One final point. If the blitter is used to draw the closed border of

a polygon, which is then to be filled by the blitter in boundary-fill mode as

documented later, then the line should be drawn with only one pixel set per

raster line. The blitter provides a line-drawing mode specially for this. To

draw lines normally, set the SING bit in BLTCON1 to zero, and to draw lines

in this special mode, set the SING bit to 1.

The following code can be etched out of this file, and used at will.

It is a drawline() routine complete with documentation which illustrates all

of the above concepts in action. Note, in order to use this code, it is best

to either kill off the operating system altogether or use Forbid() to ensure

that your task is the only one running within the system. My test code using

this kills off Exec, but I haven't included this in case anyone wishes to use

this code and keep the operating system alive.

* Blitter Line Drawing Code : Data Structures and Routines

* Assumes that all rendering is done in a low resolution

* screen of 4 bitplanes depth.

* new line structure definition V2.0. Can be defined statically using

* DC.W or generated by your program as required. Note that this code

* allows multiple lines to be drawn one after the other, and even allows

* mixing of SING and normal lines if wanted as well as lines of different

* colours in sequence!

rsreset

line_screen	rs.l	1		;ptr to 1st screen bitplane

;(assumes continuity)

line_ssize	rs.w	1		;size of 1 bitplane in bytes

line_smod	rs.w	1		;screen modulo

line_coords	rs.l	1		;ptr to line coord list

;workspace entries for drawline() routine

line_deltax	rs.w	1

line_deltay	rs.w	1

line_2S_L	rs.w	1

line_oct_bits	rs.b	1

line_pad		rs.b	1

line_sizeof	rs.w	0

* line coord list structure

rsreset

lc_next		rs.l	1	;=ptr to next coord list entry,

;0 if last in list

lc_x1		rs.w	1

lc_y1		rs.w	1	;coords of start point

lc_x2		rs.w	1

lc_y2		rs.w	1	;coords of end point

lc_pattern	rs.w	1	;should you want a dotted line...

lc_bits		rs.b	1	;bitplanes & SING bit if wanted

lc_pad		rs.b	1	;padding byte for alignment

lc_sizeof	rs.w	0

* lc_bits : 0-3 = colour (bitplanes in which drawn)

*	 : 4 = SING bit for line draw

;drawline(a4,a5) a4 = ptr to line structure definition V2.0

;a5 = ptr to custom chip registers

;draws line(s) according to the contents of the line definition

;structure(s). Note : line definition structure is a header structure

;containing workspace used by drawline() in this version. Actual

;coordinate and bitplane information etc., contained in separate list

;pointed to by an entry in the line definition structure.

;Core algorithm from Amiga System Programmer's Guide. Several

;addenda of my own for multiple bitplane handling, SING mode

;drawing etc. (SING mode needed for polygon drawing prior to

;polygon fill-see elsewhere for more info).

;d0-d7/a0-a3 corrupted

drawline		move.l	line_coords(a4),a3

drawline_l0	moveq	#0,d1		;clear line octant selector

move.w	lc_x2(a3),d0

sub.w	lc_x1(a3),d0	;compute deltax = x2-x1

roxl.w	#1,d1		;condition octant selector

tst.w	d0		;>0 or <0?

bge.s	drawline_b1	;>=0 so skip

neg.w	d0		;else absolute value

drawline_b1	move.w	d0,line_deltax(a4)

move.w	lc_y2(a3),d0

sub.w	lc_y1(a3),d0	;compute deltay = y2-y1

roxl.w	#1,d1		;condition octant selector

tst.w	d0

bge.s	drawline_b2

neg.w	d0		;absolute value again

drawline_b2	move.w	d0,line_deltay(a4)

move.w	line_deltax(a4),d2

move.w	d2,d3

sub.w	d0,d3		;want largest of Dx,Dy

roxl.w	#1,d1		;condition octant selector

tst.w	d3

bge.s	drawline_b3

exg	d0,d2		;ensure smallest of the

;two in d0

;From here on, DS = Delta S, DL = Delta L, as in the book. DS = smallest

;of Dx, Dy, and DL = largest of Dx, Dy. I reuse the line_deltax(a4)

;entries in the structure for these, simply changing the order in

;which they appear if needed instead of having separate line_deltas()

;and line_deltal() entries. Trivial really.

;Some stuff is pre-calculated, and then saved in workspace entries

;provided in the line definition structure for this purpose.

drawline_b3	movem.w	d0/d2,line_deltax(a4)	Dx = DS, Dy = DL

lea	octants(pc),a0	;ptr to octant selector table

add.w	d1,a0

clr.w	d1

move.b	(a0),d1		;get octant code

move.b	lc_bits(a3),d0

and.b	#$10,d0		;get SING bit

lsr.b	#3,d0

or.b	d0,d1		;put in eventual blitter

;control bits

asl.w	line_deltax(a4)		;2*DS

move.w	line_deltax(a4),d0

sub.w	line_deltay(a4),d0	;2*DS - DL

bge.s	drawline_b4

or.b	#$40,d1			;set SIGN bit if needed

drawline_b4	move.b	d1,line_oct_bits(a4)	;save BLTCON1 bits

;for later

move.w	d0,line_2S_L(a4)		;save 2*DS-DL

move.l	line_screen(a4),a0	;screen pointer

move.w	d0,d1

sub.w	line_deltay(a4),d1	;2*DS - 2*DL

move.w	lc_y1(a3),d2

mulu	line_smod(a4),d2		;y1 * bitplane size

move.w	lc_x1(a3),d3

asr.w	#4,d3

add.w	d3,d3			;2*int(x1/16)

ext.l	d3

add.l	d3,d2			;bitplane offset

move.w	line_deltax(a4),d3	;2*DS

moveq	#0,d4

move.w	lc_x1(a3),d4

and.w	#$F,d4			;frac(x1/16)

ror.w	#4,d4			;create STARTx bits

move.w	d4,d5

swap	d4

move.w	d5,d4			;copy to TEXTUREx bits

or.b	line_oct_bits(a4),d4	;create BLTCON1 bits

swap	d4

or.w	#$BCA,d4			;create BLTCON0 bits

swap	d4

move.w	line_deltay(a4),d5	;get DL

moveq	#3,d7			;no of bitplanes - 1

;NB : trick here. Upper word of d7=0 after moveq #3,d7. Use this as

;the bitplane bit number, doing an addq.w #1,d7 each time, and using

;swap d7 to alternate between bitplane number counter & bitplane bit

;position counter.

drawline_l1	swap	d7		;get bitplane bit number

move.w	d7,d6		;ready for test

addq.w	#1,d7		;next bitplane number

swap	d7		;back to loop counter

btst	d6,lc_bits(a3)	;bitplane flag set?

beq.s	drawline_a1	;no-get next

drawline_b5	btst	#6,DMACONR(a5)	;blitter ready?

bne.s	drawline_b5

move.l	a0,a1		;bitplane pointer

add.l	d2,a1		;offset to 1st word of line

move.w	d0,BLTAPTL(a5)	;2*DS-DL

move.w	d1,BLTAMOD(a5)	;2*DS - 2*DL

move.l	a1,BLTCPTH(a5)

move.l	a1,BLTDPTH(a5)	;bitplane pointers proper

move.w	d3,BLTBMOD(a5)	;2*DS

move.l	#-1,BLTAFWM(a5)		;set masks

move.w	#$8000,BLTADAT(a5)	;1 bit must be set

move.w	line_smod(a4),BLTCMOD(a5)

move.w	line_smod(a4),BLTDMOD(a5)	;bitplane moduli!

move.l	d4,BLTCON0(a5)	;set blitter control regs!

move.w	lc_pattern(a3),BLTBDAT(a5)	;line pattern

move.w	d5,d6

lsl.w	#6,d6

addq.w	#2,d6		;BLTSIZE = 64*DL+2

move.w	d6,BLTSIZE(a5)	;draw line

drawline_a1	add.w	line_ssize(a4),a0		;next bitplane pointer

dbra	d7,drawline_l1

move.l	lc_next(a3),d0	;check if more lines to do

beq.s	drawline_b6	;none so exit

move.l	d0,a3		;else set pointer

bra	drawline_l0	;and do it

drawline_b6	rts

octants		dc.b	4*4+1

dc.b	0*4+1

dc.b	6*4+1

dc.b	1*4+1

dc.b	5*4+1

dc.b	2*4+1

dc.b	7*4+1

dc.b	3*4+1

even

Before documenting the boundary-fill mode of the blitter, some ideas for fut-

ure experimentation include : using different minterms as described in the

Amiga System Programmers' Guide, and tinkering directly with the SING bit to

examine its effects. Also, the SIGN bit's effects can be examined, and the

effect of using values other than $8000 in BLTADAT. I have not tried all of

these, so exercise care. Some of the result could be very interesting assum-

ing that the Amiga doesn't conk out under the strain...

Boundary-filling:the blitter has a boundary-fill mode which makes

the construction of filled polygons quite simple, once the vagaries of which

registers have which values are dealt with.

The blitter's boundary-fill operation is very simple-minded. When it

fills a boundary, it recognises the boundary by virtue of the existence of a

single pixel marking the boundary. Once it has found that single pixel, the

blitter fills all blank pixels until it encounters another single pixel mark-

ing the end of the boundary. More correctly, it uses the value of the FCI bit

(the Fill Carry In bit) of BLTCON1 to determine what the value of filled pix-

els should be.

The algorithm is as follows:while a pixel equals zero, that pixel is

replaced by the value of the FCI bit. Initially for normal fills, this is set

to zero, ensuring that the initial fill leaves blank space around the bound-

ary. Once a set pixel is encountered, it inverts the FCI bit, and then uses

the ECE/ICE bits to determine what happens next. If the ICE bit (inclusive

carry enable) bit is set, the pixel is set to the new value of FCI AFTER the

inversion. If the ECE (exclusive carry enable) bit is set, the pixel is set

to the value of the FCI bit BEFORE the inversion. With ECE set, it is possi-

ble to obtain filled polygons with single-pixel corners, whereas corners of

filled polygons using ICE will always have at least two pixels at any corner

formed by the intersection of two boundary lines forming a peak (such as the

apex of a triangle). This makes most sense when I mention that the blitter

fills horizontally from right to left, a word at a time, until it has exhaus-

ted the row, and then moves on to the next row. Note also that if during the

fill of one raster line of pixels, it fails to encounter a boundary pixel,

the blitter will continue the fill on the next raster line until encounter-

ing a boundary pixel, resulting in weird and wonderful (but not always desi-

rable!) effects.

Furthermore, the algorithm works only in DESCENDING mode, so that

the blitter's DESC bit must be set.

So, to fill a boundary, the procedure is as follows:draw the poly-

gon boundary using the SING mode of the blitter's line-drawing function, so

that any lines with a slope of less than 45 degrees have only one pixel per

horizontal raster line. Having drawn the closed boundary in a suitable mem-

ory buffer, point the BLTxPTH/L registers at the END of the memory buffer,

and activate the blitter fill. This is fast-16 million pixels per second peak

speed.

To illustrate the procedure best of all, I present a piece of code

that performs this function. This code can be freely ripped out of this DOC

file and mutilated ad lib to suit the programmer's personal prejudices (note

the use of alliteration! I passed my English Language O-Level! HAH! So what

I hear you say...)

This code uses the line drawing code above to draw the boundary in

a memory buffer, then fills the buffer before transferring the contents of

the buffer to the screen. It uses a data structure, which I also include in

this section, to manage the polygon. Note that I refer to something called

the Laurence trick for managing the blitter. This refers to a trick devised

by a colleague of mine, a Belgian programmer called Laurence Vanhelsuwe who

first used the trick of setting the modulo to -2 for blitter data block move-

ments. I refer to it frequently in my blitter routines.

* polygon definition structure. See draw_polygon() routine

* for more info.

rsreset

poly_screen	rs.l	1	;1st bitplane to draw polygon on

poly_ssize	rs.w	1	;bitplane size

poly_smod	rs.w	1	;bitplane width in bytes

poly_buffer	rs.l	1	;where to draw polygon before rendering

;on the real screen...

poly_wide	rs.w	1	;width of buffer in words

poly_tall	rs.w	1	;height in raster lines

poly_border	rs.l	1	;pointer to line definition structure

poly_xpos	rs.w	1	;position to plot finished polygon

poly_ypos	rs.w	1	;on the screen

poly_flag	rs.b	1	;colours

poly_pad		rs.b	1

poly_sizeof	rs.w	0

* poly_flag	: bits 0-3 = colour

* poly_border : points to line definition structure. This in turn has

* a pointer to a line coord list, each entry of which should have the

* SING bit set in lc_bits.

;draw_polygon(a4,a5)

;a4 = ptr to polygon structure definition block

;a5 = ptr to custom chip registers

;creates polygon using blitter line draw in SING mode into a buffer

;followed by blitter fill. Then transfers the whole lot to the screen

;at the desired polygon coordinates. I.e. pre-plots in buffer. Only

;pre-plots in one bitplane, for max efficiency, then moves the entire

;lot over to the actual screen, copying only to those bitplanes

;required. Obviously, if plotting in colour 9 over a background with

;pixel data in colours 2 & 6, these will show up as colour 11 & 15

;pixel data (for a 4-bitplane screen).

;note:for optimum efficiency of memory usage, define your polygon

;with one corner at (0,0) or as close as possible to it.

;d0-d7/a0-a3 corrupt

draw_polygon	move.l	poly_border(a4),a3

move.l	line_coords(a3),d0	;get coord list

beq	draw_poly_done		;doesn't exist-BYE!

move.l	d0,a3

moveq	#0,d0		;potential x & y coord

moveq	#0,d1		;maxima

draw_poly_l1	move.w	lc_x1(a3),d2

move.w	lc_y1(a3),d3

cmp.w	d2,d0

bge.s	draw_poly_b1

move.w	d2,d0		;new x maximum

draw_poly_b1	cmp.w	d3,d1

bge.s	draw_poly_b2

move.w	d3,d1		;new y maximum

draw_poly_b2	move.w	lc_x2(a3),d2

move.w	lc_y2(a3),d3

cmp.w	d2,d0

bge.s	draw_poly_b3

move.w	d2,d0		;new x maximum

draw_poly_b3	cmp.w	d3,d1

bge.s	draw_poly_b4

move.w	d3,d1		;new y maximum

draw_poly_b4	move.l	lc_next(a3),a3	;find next set of coords

move.l	a3,d2		;check if they exist

bne.s	draw_poly_l1	;yes, back for more testing

;this for debug only

move.w	d0,debug

move.w	d1,debug+2

;Use max x&y coord values to determine size of buffer to clear

;using the blitter.

move.w	d0,d2

lsr.w	#4,d0		;int(max_x/16) = word count

and.w	#%1111,d2	;check fraction

beq.s	draw_poly_b5

addq.w	#1,d0		;1 word more

draw_poly_b5	move.w	d0,poly_wide(a4)	;WORD count!

move.w	d1,poly_tall(a4)	;height in raster lines

;prepare to clear buffer in which polygon is to be drawn.

;Clear using blitter - it's most efficient!

move.w	d1,d6

lsl.w	#6,d6

add.w	d0,d6		;this is BLTSIZE

move.w	d6,debug+4

clr.w	debug+6

move.w	#$01F0,BLTCON0(a5)	;USED only

;minterms $F0

clr.w	BLTCON1(a5)		;normal mode, no shift

move.l	poly_buffer(a4),BLTDPTH(a5)	;ptr to buffer

;to clear

clr.w	BLTDMOD(a5)	;D modulo zero

clr.w	BLTADAT(a5)	;A data zero for clear

moveq	#-1,d2

move.l	d2,BLTAFWM(a5)	;ensure masks allow data passage

move.w	d6,BLTSIZE(a5)	;and clear buffer!

move.l	poly_border(a4),a3	;ptr to coord

;struct for border

add.w	d0,d0		;WORD count to BYTE count

move.w	d0,line_smod(a3)

mulu	d0,d1		;size of buffer in bytes

move.w	d1,line_ssize(a3)	;won't be a long really!

move.l	poly_buffer(a4),d2

move.l	d2,line_screen(a3)

move.l	a4,-(sp)

move.l	a3,a4		;point to line structure

bsr	drawline		;& draw the lines

move.l	(sp)+,a4		;recover pointer

;now activate blitter fill. Note : that works only if descending mode

;selected. This code does that. This is an exclusive fill enable type

;fill, with FCI initially zero (non-inverting fill).

draw_poly_b6	btst	#6,DMACONR(a5)	;wait till blitter done

bne.s	draw_poly_b6	;busy wait (sigh)

move.l	poly_buffer(a4),a0	;where border is

move.w	poly_wide(a4),d1		;width in words

move.w	d1,d2

move.w	poly_tall(a4),d3

mulu	d3,d1			;area size in words

add.l	d1,d1			;area size in bytes

add.l	d1,a0			;descending mode-

;adjust pointer

subq.l	#2,a0			;point to last word

;of data proper!

move.l	a0,BLTDPTH(a5)

move.l	a0,BLTAPTH(a5)		;two pointers

clr.w	BLTAMOD(a5)

clr.w	BLTDMOD(a5)		;both moduli zero!

moveq	#-1,d0

move.l	d0,BLTAFWM(a5)		;ensure masks OK

move.w	#$09f0,BLTCON0(a5)	;USEA/D, minterms $F0

;		move.w	#$0012,BLTCON1(a5)	;EFE on, FCI=0, DESC=1

move.w	#$000A,BLTCON1(a5)	;IFE on, FCI=0, DESC=1

move.w	d3,d0			;height

lsl.w	#6,d0			;*64

add.w	d2,d0			;add on width in words

move.w	d0,BLTSIZE(a5)		;start blitter

;NB : when computing pointers for data blocks in above section,

;use subq.l #2,a0 to point to last words proper, instead of beyond

;the data blocks, otherwise the fill gets confused! Weird things

;happen if you don't do this!

move.l	poly_screen(a4),a0	;screen pointer

move.l	poly_buffer(a4),a2	;ptr to polygon buffer

moveq	#0,d0

move.w	#BP_BYTES,d5	;no of bytes per screen bitplane

move.w	poly_ypos(a4),d0

mulu	poly_smod(a4),d0	;y coordinate * screen modulo

add.l	d0,a0

move.w	poly_xpos(a4),d0

move.w	d0,d4			;save x coordinate

lsr.w	#4,d0			;2*int(x/16)

add.w	d0,d0

add.w	d0,a0			;now this is initial pointer

move.w	poly_smod(a4),d2	;screen modulo

move.w	poly_wide(a4),d3

addq.w	#1,d3		;word cols + 1:Laurence trick

add.w	d3,d3		;Part 2

sub.w	d3,d2

move.w	d2,d0		;C,D mods in d0

swap	d0

move.w	#-2,d0		;A,B mods also

moveq	#-1,d1		;blitter mask values:Laurence

clr.w	d1		;Trick Part 3

move.w	d4,d2		;get x coordinate

and.w	#%1111,d2	;frac(x/16)

ror.w	#4,d2		;put in top 4 bits for BLTCONx

move.w	d2,d3

or.w	#$0FCA,d2	;USEA/B/C/D, minterms $CA

swap	d2

move.w	d3,d2		;create BLTCONx bits

moveq	#0,d3

move.w	poly_tall(a4),d3

and.w	#$3FF,d3

lsl.w	#6,d3

move.w	poly_wide(a4),d6

addq.w	#1,d6

and.w	#$3f,d6

add.w	d6,d3		;this is BLTSIZE!!

moveq	#3,d7		;no of bitplanes

;here transfer polygon to screen bitplanes according to colour

;specifier. a0 = ptr to screen location, d0 = moduli (C,D high

;word, A,B low word), d1 = blitter mask values, d2 = BLTCON0

;and BLTCON1 control words, d5 = size of 1 screen bitplane in

;bytes, d3 = BLTSIZE value pre-calculated (will stay the same

;size throughout the operation) and a2 = ptr to polygon buffer.

;So leave d0-d5/a0/a2 alone while within the loop!. Leave a4 alone

;anyway or the routine will crash! Freely alter d6/a1/a3. This

;info in case you have any flash refinements to make. Note one of my

;favourite tricks-using SWAP & sticking counters/other data in both

;words of 1 reg.

;Note that the choice of sources A and B dosen't matter here because

;they're both the same! If you use different sources, remember to

;make Source A the MASK, Source B the DATA, Source C the BACKGROUND!

draw_poly_l2	swap	d7			;bitplane bit no

move.w	d7,d6			;copy

addq.w	#1,d7			;next bitplane no

swap	d7			;back to counter

btst	d6,poly_flag(a4)	;this bitplane?

beq	draw_poly_b8

draw_poly_b7	btst	#6,DMACONR(a5)	;wait till blitter done

bne.s	draw_poly_b7	;busy wait (sigh)

move.l	a0,BLTDPTH(a5)	;ptr to screen area to plot to

move.l	a0,BLTCPTH(a5)

move.l	a2,BLTAPTH(a5)	;ptr to polygon buffer

move.l	a2,BLTBPTH(a5)

move.w	d0,BLTAMOD(a5)	;A,B  moduli -2:Laurence trick

move.w	d0,BLTBMOD(a5)	;Part 1

swap	d0		;get C,D moduli

move.w	d0,BLTDMOD(a5)	;C,D moduli

move.w	d0,BLTCMOD(a5)

swap	d0		;recover A,B moduli again

move.l	d1,BLTAFWM(a5)	;blitter masks (see above)

move.l	d2,BLTCON0(a5)

move.w	d3,BLTSIZE(a5)	;start blitter

draw_poly_b8	add.w	d5,a0		;next bitplane

dbra	d7,draw_poly_l2	;continue

draw_poly_done	rts

Hardware:Sound Management

For those who like the Jean-Michel Jarre sound effects that the Amiga is able

to reproduce (guess who's bought all of his albums!) and wish to reproduce a

similar gee-whizz set of sound effects, this is the section for you.

However, I warn anyone who hasn't studied maths to a decent level of

the horrors to come. Sound synthesis using the additive method of the Paula

chip is best understood by those who know something about Fourier analysis! I

shall try to make this as painless as possible.

All sound waves, plotted graphically, are formed from lots of sine

(and/or cosine) curves added together. The fun part about adding sine and co-

sine functions together is that you get a more complex waveform as a result.

This is known by the lofty title of the principle of superposition of wave-

forms. So any waveform can be broken down into components of the form

A * sin ((n * x) + p))

where A is the amplitude of the component (which equates approximately to the

volume), n is a constant multiplier from 1 onwards, and p is the phase angle.

The phase angle describes how far along the x-axis the curve is shifted. As

it happens to be true that cos(x) = sin(x+w) where w=90 degrees or pi/2 rad-

ians, all components of a sound wave can be represented as above.

As an exercise, to see this in action, try plotting a graph of the

function

sin(x) - (1/3)sin(3x) + (1/5)sin(5x) - ...

for as many terms as you can bother to calculate. You'll find the waveform is

interesting.

This leads directly on to some musical definitions, and their rela-

ted mathematical definitions. The amplitude of the curve, namely the distance

between the highest peak and the lowest trough of the curve, defines the vol-

ume of the sound. The pitch is related to the frequency, which in turn is in

direct relation to the constant n in the first expression above. Where one of

the components of the curve has a large constant multiplier in front of it,

i.e., the value of A is large, that component is the dominant one and defines

the pitch of the note. This is the principal note. The lesser components are

known as the harmonics, especially when they are in some numeric relation to

the principal note. In the second expression above, the second and third com-

ponents define harmonics for the note, the principal is the term sin(x), and

the amplitude of the whole is the distance between the peaks & troughs, which

for this waveform should be 2*(1-1/3+1/5-...). Anyone now worrying about the

amplitude of this being infinite, because of it being an infinite series, be

reassured. The series may be infinite, but for good mathematical reasons it

ends up as a finite value! Trust me-I did this at university!

Anyone wishing to construct a waveform using this idea, be warned.

The ultimate result of embarking on this course is to immerse oneself in the

varagries of Fourier analysis, so called because Joseph Fourier, the French

mahtematician, first published a treatise on the subject (which he became in-

terested in precisely because he wished to analyse music mathematically). If

you carry this through, you'll find yourself immersed in masses of trigono-

metrical integration and orthogonality computations, and if you don't know

what that means, you're best avoiding this method!

Fortunately, there are other ways of creating your waveform. One can

simply draw a nice-looking graph of what looks like a waveform, and instead

of working out the x,y values the hard way as above, simply read them off the

graph. It is these values that Paula uses!

Now, it is time to discuss other effects. The frequency of the wave

need not be constant. Varying the frequency rapidly about the principal note

by a small amount creates vibrato. Slowing the rate of variation gives rise

to tremolo. The shape of the curve above defines a quality called timbre, and

this is the reason that so many musical instruments sound differently even if

they all play the same note. Their sounds, when taken via a microphone, sent

through an amplifier and displayed on an oscilloscope screen, give rise to a

whole variety of different curve shapes. It was these shapes that led Fourier

to perform his mathematics which describes them. And yes, Paula can reproduce

all of these! By way of example, a square wave has the equation

y = sin(x) + (1/3)sin(3x) + (1/5)sin(5x) + ...

which is slightly different to the first example. A sawtooth wave has the

equation

y = sin(x) + (1/2)sin(2x) + (1/3)sin(3x) + ...

and for your education, anyone with a Yamaha DX7 synthesiser will know that

this instrument builds its sounds in precisely this way (which is why it is

so hard to play one!).

Noise, as opposed to music, is defined as randomly superposed fre-

quencies. White noise, as it is called, is a mathematical idealisation that

is impossible to achieve in practice, and consists of all possible frequen-

cies superposed one upon another. Pink noise is a more limited selection of

such frequencies played simultaneously, and good pink noise generators are

capable of generating all audible frequencies simultaneously, thus provid-

ing a good approximation to white noise. Noise curves, when plotted, look

like the paths described by spiders walking after immersion in vodka. Anyone

who has seen sound traces on oscilloscopes will know what I mean.

Having finished the preamble, it is now time to consider how to im-

plement this on Paula. First, let us look at the register list for Paula:

Offset	Name	Function

------	----	--------

ADKCON	09E	Audio & Disc Controller

(Cross-reference #5)

AUD0LCH	0A0	High word, audio data address, channel 0

AUD0LCL	0A2	Low word, audio data address, channel 0

AUD0LEN	0A4	Data length, channel 0

AUD0PER	0A6	Period duration, channel 0

AUD0VOL	0A8	Volume, channel 0

AUD0DAT	0AA	Audio data, channel 0 (to D/A converter)

AUD1LCH	0B0	High word, audio data address, channel 0

AUD1LCL	0B2	Low word, audio data address, channel 0

AUD1LEN	0B4	Data length, channel 0

AUD1PER	0B6	Period duration, channel 0

AUD1VOL	0B8	Volume, channel 0

AUD1DAT	0BA	Audio data, channel 0 (to D/A converter)

AUD2LCH	0C0	High word, audio data address, channel 0

AUD2LCL	0C2	Low word, audio data address, channel 0

AUD2LEN	0C4	Data length, channel 0

AUD2PER	0C6	Period duration, channel 0

AUD2VOL	0C8	Volume, channel 0

AUD2DAT	0CA	Audio data, channel 0 (to D/A converter)

AUD3LCH	0D0	High word, audio data address, channel 0

AUD3LCL	0D2	Low word, audio data address, channel 0

AUD3LEN	0D4	Data length, channel 0

AUD3PER	0D6	Period duration, channel 0

AUD3VOL	0D8	Volume, channel 0

AUD3DAT	0DA	Audio data, channel 0 (to D/A converter)

Complications arise because ADKCON controls the disc and UART as well as the

sound output. However, ADKCON works on the SETIT principle in an identical

fashion to DMACON and other like control registers, so that the programmer

can choose to set only those bits pertinent to the sound system. Only the low

8 bits are used for sound, and bit 15 acts as the SETIT bit (#5) as for DMA-

CON. The bit assignments for sound are:

Bit	Name	Function

---	----	--------

15	SETIT	see DMACON

7	USE3PN	Audio channel 3 modulates nothing

6	USE2P3	Channel 2 modulates period of channel 3

5	USE1P2	Channel 1 modulates period of channel 2

4	USE0P1	Channel 0 modulates period of channel 1

3	USE3VN	Channel 3 modulates nothing

2	USE2V3	Channel 2 modulates volume of channel 3

1	USE1V2	Channel 1 modulates volume of channel 2

0	USE0V1	Channel 0 modulates volume of channel 1

These usages will be explained later.

There exist two possible ways of creating a sound. First, one could

draw a graph of one wave of the waveform, digitise it (i.e., read off the y

values at equally spaced x intervals, and rescale these y values to that they

lie within the range -127 to +127) and put the data into CHIP RAM. Then, set

Paula up to play the waveform, and write a piece of interrupt code to respond

to an audio interrupt by replaying the waveform repeatedly. This limits the

note to one waveform, but the pitch can be varied within the interrupt code.

The second method is to digitise an ENTIRE sound into CHIP RAM, and

set Paula up to play the whole lot in one go. This is obviously a task that

is not suited to hand-calculation, and to assist with this, special digitiser

hardware exists that can be interfaced to the Amiga to perform this function,

this hardware being known as a SAMPLER. By using a sampler, an entire single

record can, in theory, be digitised and played on the Amiga (for an example

of what is capable, listen to the Xenon 2 Megablast soundtrack by Bomb The

Bass!).

Problems occur because of the use of 8-bit data resolution within

the sound system. A certain quality can be achieved, but for comparison, a

compact disc player uses 16 bits for hi-fi quality. So anyone toying with the

idea of reproducing studio quality classical music on the Amiga directly will

possibly be disappointed-there may be a noticeable change in quality. Because

all analogue to digital conversion involves sampling errors, due to the roun-

ding effect of A/D conversion, a quantity called the quantization error res-

ults. For an 8-bit system, the maximum possible quantization error is 1/256

of the digitised value. The magnitude of the quantization error is directly

proportional to a noise value called the quantization noise, which may have a

deleterious effect upon sound quality (hence the use of 16 bits by CD players

which reduces the quantization error to 1/65536 of the digitised value).

Now, the first quantity that requires calculation is the sampling

rate. This is the rate at which the sampler 'snapshots' the analogue sound

data and converts it into digital data. If the data is not sent to Paula at

the same rate, then the sound will be shifted in frequency with the possi-

bility of distortion occurring (but of course, there is no reason why this

cannot be done deliberately for special effects!).

If using one waveform, the number of samples per waveform is equal

to the number of y values you have decided to use. To make sine curves con-

form more closely to the ideal, increase the number of samples.

A simple example illustrates this. Let us reproduce a pure tone, as

represented by a pure sine wave. One complete wave consists of the curve from

0 degrees to 360 degrees. Let us split this up into 16 samples. So we require

the values of sin(x) from 0 to 360 degrees in 16 steps, scaled so that the

end results lie within the range -127 to +127. Because sin(x) varies from -1

to +1, we therefore want the values of 127*sin(x) over this range. The values

thus required are:

0

48.6007

89.8025

117.3327

127

117.3327

89.8025

48.6007

0

-48.6007

-89.8025

-117.3327

-127

-117.3327

-89.8025

-48.6007

Rounding these to the nearest whole number (which introduces the quantization

error unfortunately, and cannot be avoided because Paula can't handle float-

ing point numbers!) and putting this list of bytes into CHIP RAM, yields the

desired sample data. Since Paula uses standard twos-complement bytes, and it

is usual for assemblers to generate these directly from negative numbers, we

can insert these values directly into a DC.B list, e.g.;

sine_wave	dc.b	0,49,90,117

dc.b	127,117,90,49

dc.b	0,-49,-90,-117

dc.b	-127,-117,-90,-49

Now, we point Paula's data registers at the start of the list. This

is usually done using something like

LEA	$DFF000,A5

LEA	sine_wave(PC),A0

MOVE.L	A0,AUD0LCH(A5)

Now we tell Paula how long the sample list is in WORDS. There are 16 actual

data bytes. The value to write to AUD0LEN is thus 16/2 = 8. For lists with

an odd number of bytes, e.g., 17 instead of 16, use 17/2 rounded DOWN to the

nearest integer, i.e., 8 again. Use the instruction:

MOVE.W	#8,AUD0LEN(A5)

to set the list length. For longer lists, use two labels, one at the start of

the list, one at the end of the list, and use something like

MOVE.W	#(end-start)/2,AUD0LEN(A5)

(as in the Amiga Systems Programmer's Guide).

Now we select the output volume. Since this is going to be a cons-

tant value, and we can choose any value from 0 to 64, let us choose half of

full volume:

MOVE.W	#32,AUD0VOL(A5)

Now we need to choose our sampling rate. In this case, the sampling rate will

affect the frequency of the sound. So we can use the relation

F = S/N

where F is the frequency, S is the sampling rate, and N is the number of sam-

ples per cycle. In this case, N=16. Now we cannot specify the frequency as a

specific number of Hertz, but need to relate it to the number of bus cycles.

A bus cycle is 279.365 nanoseconds. We need to compute the sample period as

given by the relation

P = 1/(S * 2.79365 * 10E-7)

where I use 10E-7 to represent 10 to the power of -7. To play our sine wave

at International A, or 440 Hz, we need to transform our frequency relation,

and yield

S = F * N

Using F=440, N=16, we have S = 7040 Hz. Now this will give P as the value

1/(7040 * 2.79365 * 10E-7)

or 508.4583. Rounding, this yields 508. We therefore set up Paula using

MOVE.W	#508,AUD0PER(A5)

to generate the appropriate International A note. The resulting note will in

actuality deviate by 0.4 Hz from the true value, but only a listener posses-

sing perfect pitch (particularly a classical musician) will notice the diff-

erence.

There exists some limit upon the AUDxPER values. Each audio channel

has one DMA slot per raster line, and hence one word, or two samples, can be

read in one raster line. Thus the smallest possible value for the sampling

period is 114 (NOT 124 as given in the Amiga Systems Programmer's Gude!). The

value is obtained as follows:one raster line equals 63.5 microseconds. So one

second contains 1/(63.5 * 10E-6) raster lines, which rounded up equals 15748

raster lines. Two samples can be read per raster line, or 31496 samples when

read at maximum speed. This yields P = 113.65, or rounded up, 114. Selecting

a value lower than 114 will result in some data words being output twice, as

the DMA system cannot fetch the next data word on time. This gives a sampling

rate of 1/(114 * 2.79356 * 10E-7) or 31399 Hz. Obviously, one can sample at a

slower rate, and even sample using P=65535 for weird effects!

Having set up the registers, we need to activate the audio DMA. The

instruction

MOVE.W	#$8201,DMACON(A5)

performs this (it sets AUD0EN and DMAEN just to be sure).

At this point, Paula will play the sound. The example above will be

heard as a single note playing continuously, and eventually you will become

so sick of hearing it you'll turn the volume off on your monitor! This exam-

ple sounds like the horrible 'beep' accompanying the BBC test card...

Now for some details. Paula has internal registers for data fetching

unlike the blitter (why the hell couldn't the blitter have them?), and when

Paula is started up, the AUDxLCH/L register pair contents are copied to these

internal registers before data fetching. Thus the initial values of AUDxLCH/L

are preserved. The same is true for AUDxLEN, AUDxVOL and AUDxPER (otherwise

you wouldn't hear the same 'beep' all the time when playing the example sound

above!). Only when the audio DMA is turned off will the sound stop!

Now since the AUDxCLH/L values, etc, are copied to internal regis-

ters while playing, the processor can supply new values to change the note on

a continual basis. Uninterrupted sound generation can thus continue.

Paula generates an interrupt at the end of each complete sound out-

put cycle. By enabling the audio interrupt bits, and linking in an interrupt

handler to the audio interrupt vector (IPL4) it is possible to change which

note is played under interrupt control. Be warned that for high frequencies

the interrupt occurs VERY OFTEN and if your interrupt code cannot respond to

it fast enough, the supervisor stack will overflow with unsatisfied inter-

rupt requests and crash the machine!

As for all interrupt code, the interrupt handler must clear the

INTREQ bit for the appropriate channel after checking which channel caused

the interrupt to occur, and then perform whatever function the user desires,

be it changing the frequency of a note such as played in the above example,

or performing more complex changes such as switching between waveforms.

Modulation:the term modulation is used to describe the process by

which one waveform can be used to change the output from another. Normally

two quantities are subject to modulation, volume and frequency.

When a waveform A is used to alter the volume at which waveform B is

output, waveform A is described as a VOLUME ENVELOPE. When the waveform A is

used to alter the pitch of the notes played, waveform A is then described as

a TONE ENVELOPE.

Volume envelopes are the simplest to describe. With a volume envel-

ope, it is possible to generate full ADSR synthesised sound. ADSR stands for

Attack, Decay, Sustain, Release, and describes four steps in the generation

of a sound. Attack describes how a sound builds up in volume from zero to a

maximum. Sustain describes the section where the volume is maintained at a

constant volume for a specific time period. Decay and Release both describe

how sound volume decreases, the Decay phase(s) following Attack/Sustain pha-

ses, and the Release phase occurring at the end of the note.

When using a volume envelope, the data for the volume envelope is

entered as for normal sample data. A second area of memory is used for the

sample data itself. For example, let channel 0 modulate channel 1. For this

process, AUD0VOL is turned off (channel 0 is not used for actual sound gene-

ration), AUD0LCH/L is pointed to the envelope data, and AUD0PER can be set

either to the same rate as AUD1PER for the actual sound data, or to a diff-

erent value, according to the effect desired. AUD1LCH/L is pointed to the

sample data, AUD1VOL is set to the maximum value (64), and AUD1PER to the

sampling rate pertinent for the sample data. Lastly, set USE0V1 in ADKCON and

set Paula running.

A frequency envelope is defined in a different way. Instead of defi-

ning it as a list of sample data bytes, it is defined as a list of sampling

rate words as written to the AUDxPER registers. If the base note of the sound

has a principal frequency of 440 Hz as in the example above, and thus has a

sampling period value of 508, the values in the frequency envelope can vary

around this value to create vibrato/tremolo effects, or alternatively, vary

in one direction to produce ascending/descending pitch. The larger the value

used, the lower the sampling rate used and the lower the frequency resulting

when the sample is played. Depending upon the rate of sampling used for the

frequency envelope, either vibrato or tremolo can be produced.

Note that it may be possible to set both USE0V1 and USE0P1 bits. If

this is done, the data for channel 0 will change BOTH the volume and the fre-

quency of the output from channel 1, but since there is a fundamental differ-

ence between amplitude and volume envelope data, the effects cannot readily

be predicted beforehand.

Sound generation problems:in the discourse above, it was mentioned

that the maximum sampling rate possible was 31399 Hz (ignore the value used

in the Amiga System Programmer's Guide-it is WRONG). For a digitised sine

wave of 16 sample data points, the maximum frequency possible for this sine

wave is 1962.4 Hz. To generate higher frequencies, the number of samples must

be reduced, so that, for example, a sine curve with 8 samples will have a

maximum possible reproduction frequency of 3924.8 Hz. But as the number of

samples decreases, the quality of the sine wave also decreases, until in the

limiting case, the result is an annoying buzzing noise.

One way of circumventing the problem is to digitise multiple waves

in the waveform instead of just one. This allows higher-frequency waveforms

to be generated without the problems of waveform degeneration. But a second

problem occurs with the higher frequencies. There exists a phenomenon assoc-

iated with high-frequency sound generation called aliasing distortion. This

comes in two forms, one as the sum of the sampling rate and the desired fre-

quency, and the second as the difference. So, for example, if the sound is of

3KHz frequency, and the sampling rate is 12KHz, the aliasing distortion will

occur at 9KHz and 15KHz.

To eliminate this aliasing distortion, Paula contains a device known

as a low-pass filter. This has been placed between the output of the D/A con-

verters and the audio connectors in Paula, and its effect is to allow all of

the frequencies below 4KHz to pass undisturbed. Frequencies between 4KHz and

7KHz are diminished in amplitude, and frequencies above 7KHz are not passed

at all.

In the example cited above, the 3KHz main signal is allowed to pass

to the speaker undisturbed, but both of the aliasing distortion frequencies

lie above the 7KHz cutoff point and are eliminated. Should the sampling rate

be reduced to 9KHz, however, the aliasing distortions now occur at 12KHz and

6KHz, and the 6KHz aliasing distortion, although diminished, is still allowed

past to the speaker.

So, when determining sampling rates, to ensure destruction of the

aliasing distortion and passage of the desired sound signal, the sampling

rate must be chosen so that it is greater than the frequency of the highest

frequency component of the sound PLUS the 7KHz cutoff point. So for a sound

with a maximum frequency component of 4KHz, the sampling rate must be greater

than 4KHz+7KHz, or 11KHz. Needless to say, given the upper limit of 31KHz on

the sampling rate as determined earlier, the highest possible frequency com-

ponent of ANY sound can be no higher than 24KHz. Many people lack the ability

to hear sounds above 16KHz, but some exceptional persons can hear up to 24KHz

before the sound exceeds their audible range, including one or two classical

musicians of note. Most music does not contain notes above 3KHz, so very high

sound frequencies are generally for specialist applications.

Fundamental notes are not the only consideration, however. To main-

tain the quality of the music, the timbre must be maintained. This is direct-

ly related to the shape of the curve of the waveform, which in turn depends

upon maintaining the existence of higher frequency harmonics. Harmonics are

often separated by octaves, and an increase of one octave results in the fre-

quency being doubled (octaves and frequencies are related logarithmically).

So a 3KHz note with several harmonics may have most or all of its harmonics

destroyed by the low-pass filter. In the extreme case, a square wave which

consists theoretically of an infinite number of harmonics may be reduced to

a sine wave by the low-pass filter.

Of course, digitising whole sections of music as opposed to single

notes solves this problem, but at the expense of memory. If the sampling rate

used by a hardware sampler is 20KHz, and one second of music is sampled, the

data takes up 20K of memory. This limits the maximum amount of sound that can

be digitised in one go at that sampling rate to 25.6 seconds (assuming that

you can fill all of CHIP RAM with the data!). For lower sampling rates more

data can be digitised, but the memory problem still remains. One solution is

to digitise sections at a time, compress the resulting data using a suitable

data compression a;lgorithm, then decompress them into CHIP RAM and play them

in sequence. Needless to say, the decompression algorithm needs to be fast,

and synchronised to the sound playing. The data must not be decompressed into

an area of memory already being used for sound reproduction, hence the decom-

pression of data MUST NOT catch up with the sound generation! Grabbing the

data from disc is another possible way around the problem-directly accessing

the disc tracks yields a data transfer rate of 62500 bytes per second peak,

and can hence be used to lob in data piecemeal, provided that there is suffi-

cient disc space. Again, compression/decompression may be needed.

Hardware:Disc Management

Disc drives on the Amiga are controlled from two sources, the CIAs and Paula.

The CIAs govern such features as motor control, drive selection (#6) and head

movement. The FLAG pin of CIA-B is connected to generate the /INDEX signal of

the disc drive.

The ADKCON register mentioned above in the sound management section

is also used to control the disc controller. The bits 14-8 of this register

are used for disc control, and the bit assignments are:

Bit	Name	Function

---	----	--------

15	SETIT	see DMACON

14,13	PRECOMP	Sets precompensation

12	MFMPREC	0=GCR encoding, 1=MFM encoding

11	UARTBRK	used for the UART, not used here

10	WORDSYNC	1=enable synchronisation

9	MSBSYNC	1=enable GCR synchronisation

8	FAST	0=4 microseconds/bit(GCR)

1=2 microseconds/bit(MFM)

By appropriate programming of this register, it is possible to enable Amiga

disc drives to read either MFM or GCR encoded discs. For explanation of the

terms MFM and GCR, see a comprehensive data source on disc drive operation,

as I have temporarily forgotten the relevant information.

Both encoding mechanisms require appropriate software to generate

the encoded data before writing to disc, and to decode the data read from the

disc. The appropriate routines for data encoding/decoding for MFM form part

of the trackdisk.device - the Amiga uses MFM encoding for its discs as the

default choice.

The need for such encoding is explained simply - raw data cannot be

written as is to disc. Because of limitations imposed by, among other things,

the laws of electromagnetism, the data has to be encoded in a manner allowing

the data to be securely stored in the form of magnetic flux changes on the

disc surface. I suggest searching out a concise reference work on the subject

before attempting one's own MFM/GCR encoding/decoding software!

As to the existence of these two systems, they have their own hist-

ory. However, GCR coding is used for Apple Macintosh discs, and there exists

a Macintosh emulator for the Amiga called A-MAX II using Paula's GCR encoding

ability to read Mac discs directly.

Precompensation is a little bit more difficult to explain. When the

disc system writes data to the disc, the data is written as a series of flux

changes onto the magnetic medium. The time for each flux change to occur is

called the half-zero-bit length, and the gap synthetically introduced to in-

crease data security for high-speed transfers is the precompensation. Normal-

ly the faster the data transfer, the higher the precompensation needed to av-

oid data read/write errors. Four possible settings are provided for by Paula.

Now we have the ability to set up the disc controller to provide the

encoding system, precompensation and disc controller clock rate. Now we need

to tell Paula where the disc transfer buffer lies. This involves the regis-

ters DKSPTH (offset $020) and DKSPTL (offset $022). These two registers pro-

vide a pointer into a buffer in CHIP RAM. This buffer can be used for either

read or write operations.

Having informed Paula of the disc transfer buffer location, we need

to inform Paula of 1) the length of data to transfer, and 2) the data direc-

tion (read/write). The register DSKLEN (offset $024) performs these functions

in one go. The bit assignments of DSKLEN are:

Bit	Name	Function

---	----	--------

15	DMAEN	Enable Disc DMA

14	WRITE	0=read data from disc, 1=write data to disc

13-0	LENGTH	14-bit number : no of WORDS to transfer

When DMAEN is set to 1, the data transfer is theoretically enabled. The use

of the word 'theoretically' is deliberate, because Paula contains a mechanism

to prevent accidental disc writes. Firstly, the DSKEN bit in DMACON must be

set, and even if it is set, the DMAEN bit of DSKLEN (CARE! confusion may ar-

ise in the names here!) must be set TWICE before the operation is executed.

In addition, the WRITE bit must only be set for a genuine write operation! If

you try changing the value of this bit during the second setting of WRITE, a

weird and wonderful sequence of events may just occur, leading among other

things to Paula shuffling off her mortal coil...

The orderly sequence for DSKLEN is as follows:

MOVE.W	#0,DKSLEN(A5)		;turn off disc

MOVE.W	#$8210,DMACON(A5)		;enable disc DMA

;just in case

LEA	disbuf(PC),A0

MOVE.L	A0,DSKPTH(A5)		;set up disc buffer

CLR.W	D0

BSET	#15,D0			;set DMAEN

BSET	#14,D0			;set WRITE if wanted

MOVE.W	#LENGTH,D1		;amount of data

;to transfer

ADD.W	D0,D1

MOVE.W	D1,DSKLEN(A5)		;set up disc

MOVE.W	D1,DSKLEN(A5)		;now execute!

...			;here wait until the

;disc DMA is finished...

MOVE.W	#0,DSKLEN(A5)	;and shut off when done.

The DSKBLK interrupt is provided in the INTREQ/INTENA registers so that the

processor can discover when the disc controller has finished. When the number

of words specified in DSKLEN has been transferred in whichever direction has

been chosen, the DSKBLK interrupt is signalled. It is generated when the last

word of data is transferred.

To examine the current status of the disc controller, there exists

the DSKBYTR (offset $01A) register, which is assigned as follows:

Bit	Name		Function

---	----		--------

15	BYTEREADY	Signals that byte in lower 8 bits is

valid

14	DMAON		Indicates if disc DMA is active. DMAON

set to 1 when DMAEN of DSKLEN is 1 AND

DSKEN of DMACON is 1 (CARE WITH NAMES!)

13	DSKWRITE		Copy of WRITE in DSKLEN

12	WORDEQUAL	Disc data equals DSKSYNC

11-8	Unused

7-0	DATA		Current data byte from disc

Incidentally, it is possible to use this data register to read the data from

the disc using the 68000 intsead of using DMA (should you want to!). Whenever

a complete byte is received from the disc, the disc controller sets the BYTE-

READY bit. The processor then knows that the data in the lower 8 bits is a

valid data byte. After DSKBYTR is read, the BYTEREADY flag is automatically

reset. The DMA <system normally performs this without intervention from the

68000.

Sometimes, instead of reading an entire track of data at once, the

programmer may wish to read data starting at a specific position. The prog-

rammer uses the DSKSYNC register (offset $07E) to determine where the disc

data transfer will begin. The value is an offset, indicating which data word

the transfer is to begin at (for normal whole-track transfers, the value of

DSKSYNC is zero). The disc controller maintains a count of words transferred,

and when that count is less than the value in DSKSYNC, the data is read by

the DMA system but NOT transferred. When the internal count of words read is

greater than or equal to the value of DSKSYNC, the data being read is duly

transferred. Thus the disc controller can be programmed to wait for the syn-

chronisation mark at the start of a data block.

Two other registers exist. These are DSKDAT (offset $026) which is

used to contain the data written to the disc by the DMA controller, and the

DSKDATR register (offset $008) which contains data read from the disc. DSK-

DATR is an early-read register, NOT accessible by the 68000.

Hardware:Interfaces

There are three interfaces to handle here. These are the parallel interface,

the serial interface, and the analogue inputs to the gameports (which can be

used for other uses apart from game paddles).

The parallel interface:this is primarily controlled via the CIAs, &

the data lines are coupled to PB7-PB0 of CIA-A (i.e., CIAAPRB, whose address

is $BFE101). The PC output of CIA-A is connected to the DATA READY signal of

the handshake line, and the FLAG pin to the DATA ACKNOWLEDGE signal. Since

data register B of each CIA is equipped with handshaking built in, whereby an

access (read or write) causes PC to go low for one clock cycle, writing to

CIAAPRB automatically sends out a DATA READY signal. When the connected dev-

ice responds, it pulls the line connected to FLAG low to signal DATA ACKNOW-

LEDGE, and the FLAG bit in the ICR is set. Because of this, it is possible to

process data output to the Centronics interface via an interrupt routine, &

allow programs to continue with other processing while the interrupt routine

handles the output - ideal for printer spooling, for example.

The handshaking process can also be used for data INPUT from the

Centronics interface (assuming a bidirectional perpipheral is connected!).

PC and FLAG are handled automatically by the CIA, and handling the FLAG int-

errupt is virtually all that the programmer needs to do under normal circum-

stances.

CIA-B is used for the SELECT and BUSY signals, bit 2 of CIABPRA (at

address $BFD000) being SELECT, and bit 0 being BUSY. The BUSY signal is used

for communication with slow peripherals (e.g., printers), and the interrupt

routine can also wait for the BUSY signal to change before continuing output

to a printer.

The serial interface:the serial interface is controlled by a combin-

ation of CIA registers and Paula registers. The CIA connections for the ser-

ial interface are:

/DTR signal	:	CIA-B PA7

/RTS signal	:	CIA-B PA6

/CD  signal	:	CIA-B PA5

/CTS signal	:	CIA-B PA4

/DSR signal	:	CIA-B PA3

All of these signals are sent through inverter logic as part of the RS-232

driver hardware, and so the signals are active low. Setting the corresponding

CIA-B bit to 0 sets the corresponding RS-232 line to high. TAKE CAREFUL NOTE

OF THIS! Forgetting the inverse logic of this RS-232 interface is a common

source of interfacing problems.

When using RTS/CTS protocol, RTS should be made an output (set the

corresponding bit in CIABDDRA) and CTS an input (clear the appropriate DDRA

bit). I am not currently sure how to handle XON/XOFF, so until I have access

to the appropriate data, I shall leave XON/XOFF undocumented.

One feature of serial data transfer using RS-232 is that clock sig-

nals are not provided. This means that both sender and receiver must provide

their own timing, and that the times must match for secure data transfer. A

set of standard baud rates exist for RS-232, typical values being 300 baud,

1200, 2400, 4800 and 9600 baud. Some fast peripherals (e.g., the new modems

for mainframe communications using SYSTEM-X exchanges) can have a maximum

baud rate of 38400 baud (but since they're 5,000 each, few readers of this

DOC file will have one coupled to their Amigas...) and high baud rates are

also a feature of experimental computer-moderated radio transmissions using

short-wave radio.

The serial interface controller, or UART (this admonitive acronym

stands for 'Universal Asynchronous Receiver/Transmitter', which is a lot of

fun to try and say after several Bacardis) allows setting of the baud rate

in the SERPER register (custom chips, offset $032, write-only). This regis-

ter also controls the data length to some extent. Bit 15 (the LONG bit), if

set, makes the length of the receive data 9 bits instead of 8. The remain-

ing 15 bits determine the baud rate.

Baud rate determination is indirect. Again, the number used is the

number of bus cycles (just as for audio data sampling rates) taken to trans-

mit one byte of data. If it takes N bus cycles to transmit a byte, the number

N-1 must be written to SERPER (for some perverse reason). So the relationship

between baud rate and the value to write into SERPER (here designated as S)

is

S	=	(1 / (B * 2.79365 * 10E-7) ) - 1

where B is the baud rate, and 2.79365 * 10E-7 is the time taken for one bus

cycle (279.365 nanoseconds). So, for 4800 baud transmit/receive rate, the

value is

S = (1/4800 * 2.79365*10E-7) - 1

which is 744.738. Rounding to the nearest integer, we have 745. So to select

4800 baud we use

move.w	#745,SERPER(a5)

for 8-bit data transfers, and

move.w	#$8000+745,SERPER(a5)

for 9-bit receives.

Ok, we can now set the baud rate, and have access to the control

signals. The other registers we need are the Paula serial data registers

(Cross-reference #2):

Register		Offset	Function

--------		------	--------

SERDAT	  	030	Contains data to send (RS-232 output)

SERDATR	  	018	Contains data to read (RS-232 input)

The SERDATR register for data reading (RS-232 input mode) has several bits

allocated to various functions. The bit assignments for SERDATR are:

Bit	Name	Function

---	----	--------

15	OVRUN	Overrun of receiver shift register if set

14	RBF	Receive buffer full if set

13	TBE	Transmit buffer empty if set

12	TSRE	Transmit shift register empty if set

11	RXD	Matches level on RXD line

10	Unused

9	STP	Stop Bit Value

8	DB8	Depends on LONG in SERPER

7-0	DB7-0	Receive data buffer bits 7-0

SERDAT (offset $030) is used to contain the data to be transmitted from the

Amiga. Because of the time taken for serial data transfer using normal RS-232

protocols, there is no provision for a data buffer pointer and a DMA control

read/write system to automatically read in or write out a block of data of a

given size, as the need was felt not to exist by the designers. The maximum

possible data transfer rate corresponds to a SERPER value of zero, and equals

approximately 3,580,000 baud! Not a regularly selected baud rate...few appli-

cations need this kind of speed (usually confined to military systems, which

also possess inbuilt data encryption, reversible frequency modulated pertur-

bation of the data stream and other weird features not present on the Amiga)

and it is very unlikely that readers of this DOC file will ever need it.

Two interrupts are provided for handling serial transfers. The RBF

(Receive Buffer Full) interrupt handled via the IPL5 interrupt vector is the

interrupt used for handling RS-232 transfers from the outside world to the

Amiga, and the TBE (Transmit Buffer Empty) interrupt handled via the IPL1

interrupt vector is used to handle RS-232 transfers from the Amiga to the

outside world. Both interrupt vectors should be initialised appropriately and

the interrupt code should principally restrict itself to either sending or

receiving a byte of data. Configuring the baud rates and other allied func-

tions should be left to other routines called by the main program.

The procedures are as follows:

* Reading a byte of data from RS-232

* This is interrupt code, so put an RTE after it

* if using 'as is'. If using AmigaDos, there are

* other ways of doing it - see elsewhere.

* assumes ptr to custom chips in A5!!

move.w	INTREQR(a5),d0	;get interrupt request

bclr	#11,d0		;RBF interrupt?

beq.s	not_RBF		;no

move.w	SERDATR(a5),d0	;get received data (clear RBF)

move.w	d0,ser_word	;save it

move.w	d0,INTREQ(a5)	;and acknowledge interrupt

;(see note below)

not_RBF		...

The RBF (Receive Buffer Full) bit is set in SERDATR and INTREQ/R whenever a

data word is transferred from Paula's internal shift register to the SERDATR

register. At this point, SERDATR should be read, to clear space for the next

incoming data word. Reading the next data word clears RBF, and signals that

SERDATR is ready to receive the next data word being read into Paula's int-

ernal shift register.

If SERDATR is not read, and the shift register has received another

complete data word, OVRUN is set. This signals that no more data can be rec-

eived because both SERDATR and the shift register contain inputted data. When

SERDATR is read under these conditions, OVRUN is reset, and RBF also. RBF is

then immediately set again and the full contents of the internal shift regis-

ter are loaded into SERDATR again, allowing more data to enter the shift reg-

ister. Obviously, once RBF is set again, the data must be read from SERDATR

again.

The format of the data to be read is determined by SERDATR and SER-

PER. If the LONG bit of SERPER is set, the data is 9 bits, else 8 bits. If

the data is 8 bits, then bits 9 and 10 mark the stop bits, if present, and

are set if there is a stop bit there. If the data is 9 bits, bit 10 is the

stop bit if present, again set if a stop bit encountered. Note that all data

is sent with one start bit, which always has the value 0. This applies both

to reception and transmission, and the hardware detects the end of a data

word by noting the transmission from the 1 of a stop bit to the 0 of the next

start bit. Note that when the data transmitter has finished sending its data

to the Amiga, the RBF bit will never be set after the last word has been pro-

cessed, and the interrupt routine will never be called from this point on.

Usually, RS-232 systems signal this, either by sending a data byte

or bytes at the start indicating the size of the data block being transmit-

ted, or by sending a special 'end of transmission' character at the end of

transmission. Two typical choices are CTRL-D (known as the ASCII EOT char-

acter) or CTRL-Z (EOF, or end of file, on many systems). If the transmission

consists of binary data instead of ASCII characters, then either the start

of the transmission must contain the byte count of the block, or else another

means of signalling end of transmission is required, as any of the control

characters could be a valid data byte, unless an encoding scheme is used.

* Writing a byte of data to RS-232

* Again,interrupt code, pointer to the

* custom chips in A5.

move.w	INTREQR(a5),d0	;get interrupt request

bclr	#0,d0		;TBE interrupt?

beq.s	not_TBE		;no

move.w	ser_word,d0	;get data to send

move.w	d0,SERDAT(a5)	;send it (clear TBE)

move.w	d0,INTREQ(a5)	;acknowledge interrupt

not_TBE		...

The data to be output in this case is written to SERDAT. It is then immedia-

tely transferred to the output internal shift register. This is signalled by

the TBE bit, which is set to indicate that SERDAT is able to receive more

data. Once TBE is set, more data should be written to SERDAT to maintain the

data flow. Should this not occur, and the output shift register is emptied

before SERDAT is reloaded, then the TSRE (Transmit Shift Register Empty) bit

is set. This is cleared when SERDAT is loaded, as is TBE. But TBE is immedia-

tely set again as the contents of SERDAT are sent to the output shift regis-

ter, and TBE is cleared again, allowing more data to be written.

The format of the data to be sent is determined by SERDAT. The data

formats for different cases are:

8 bit data, 1 stop bit	: 00000001 dddddddd

8 bit data, 2 stop bits	: 00000011 dddddddd

9 bit data, 1 stop bit	: 0000001d dddddddd

To stop transmission of data, one bit of the ADKCON register (offset $09E) is

provided, the UARTBRK bit (bit 11). Setting this bit using the instruction

move.w	#$8800,ADKCON(a5)

(ADKCON uses the SETIT mechanism-see DMACON) stops serial data transfer and

clears TXD (the transmit data line) of the serial port.

Analogue inputs:the gameports possess two analogue inputs each, and

it is possible to connect game paddles to them or other analogue signal gene-

rating equipment. Game paddles usually use a sliding or twist knob to change

the resistance of a potentiometer (known as a 'pot' for short - hence the use

of POTxxxx register names for this interface!).

Analogue joysticks, with a potentiometer for the X and the Y direc-

tion, can also be connected. The values that these produce are read in the

POTxDAT registers (POT0DAT, offset $012, POT1DAT, offset $014). Bits 0-7 are

used for the X-value, and bits 8-15 for the Y-value.

Now, how does this work? Well, Paula contains a circuit to handle a

simple analogue-to-digital conversion. The requirement is that the maximum

resistance of the potentiometers should be 470 Kilohms (with a tolerance of

plus or minus ten percent). One side of the potentiometer is connected to the

5-volt power supply, and the other to one of the analogue inputs. These lead

internally to Paula and to a capacitor, one for each input, connected between

the input and ground.

The paddle outputs are placed briefly at ground, discharging the ca-

pacitors. Also, the counters in POTxDAT are cleared. For each raster line,

the counters are incremented by one while the capacitors are charged through

the resistors. When the voltage across the capacitor exceeds a preset value,

the corresponding counter is stopped. Thus the counter state is directly pro-

portional to the input resistance. Small values equal low resistances, large

values equal high resistances.

The POTGO register (Cross-reference #7) determines whether the ana-

logue pins are inputs or outputs. The bits are assigned as follows:

Bit	Name	Function

---	----	--------

15	OUTRY	1=gameport 1 POTY bit is output, 0=input

14	DATRY	Gameport 1 POTY data bit

13	OUTRX	1=gameport 1 POTX bit is output, 0=input

12	DATRX	Gameport 1 POTX data bit

11	OUTLY	1=gameport 0 POTY bit is output, 0=input

10	DATLY	Gameport 0 POTY data bit

9	OUTLX	1=gameport 0 POTX bit is output, 0=input

8	DATLX	Gameport 0 POTX data bit

7-1	unused

0	START	Discharge capacitors & begin analogue

measurement

A write access to POTGO (offset $034) clears both POTxDAT registers. POTGOR

also exists (offset $016) to allow the states to be read.

Normally, START is set to 1 at the start of the vertical blank int-

erval, and the valid potentiometer values can be read at the start of the

next VBL, immediately prior to setting START to 1 again.

If the corresponding OUTxx bit above is set to 1, the corresponding

line is treated as a digital output, and the corresponding DATxx bit is sent

out along it. If OUTxx=0, then DATxx in POTGOR yields the current state of

those lines as digial outputs.

Paddle buttons use the same bits as the joystick data registers (ho,

hum!). The assignments are:

Gameport 0		Gameport 1

----------		----------

Left Button	JOY0DAT bit 9		JOY1DAT bit 9

Right Button	JOY0DAT bit 1		JOY1DAT bit 1

For each of these, the bit is 1 if the button is pressed.

Hardware:Mouse, Keyboard, Joysticks

I assume that most of those readers requiring this file for hardware documen-

tation know what a keyboard, a mouse and a joystick are. Some may even have

dismantled several of these items (I have-much to the dismay of those people

whose equipment I have dissected!) and found out about the inner workings of

the basic hardware. However, there exist extra components within these pieces

of hardware whose function needs a little extra explanation.

Modern keyboards are no longer simply a matrix of switches. Most of

the keyboards on modern computers have internal controller chips of their own

with their own RAM and ROM, allowing customisation and configuration of the

keyboard. IBM PCs (ugh!) use an Intel 8047 controller, Sinclair QLs (yeuck!)

use an Intel 8049, Atari STs (duh...) use a 6301 processor chip (this was at

one time the heart of expensive desktop computers!) and the Amiga? Well, the

Amiga uses the MOS 6500/1 processor. So what? Well, a 6500/1 is really a 6502

processor, as found in the PET/VIC-20/Commodore 64, with on-chip RAM and ROM.

The ROM is mask-programmed with the control program for the Amiga keyboard.

Cross-reference #1:the CIAs are coupled via several links to the 6500/1 for

keyboard communication.

So, if it wasn't for the mask programmed ROM, in theory anyone with

a 6502 assembler and the motivation could write his own keyboard controller

program for the Amiga. Sad to say, the existence of a control program embed-

ded in ROM, plus the one-way data traffic (keyboard to Amiga) makes this im-

possible. Atari ST keyboard controllers CAN be reprogrammed, but I warn any-

one tempted to try, it is HARD.

The 6500/1 has a 2K ROM, 64 bytes of static RAM, 4 bidirectional 8-

bit ports, a 16-bit counter with its own control input, and a clock generator

of its own. This chip is interfaced to the Amiga via two precision 556 timer

chips. These provide a reset signal for the Amiga. The mechanism by which the

reset mechanism is provided is interesting - more later.

The 6500/1 reads the keyboard matrix via ports C and D (the 6500/1

has four I/O ports on-chip) to obtain details about which key has been either

pressed or released. This information is converted from the bitwise port in-

formation, into a raw key code passed out to the Amiga via port A. This data

is transmitted serially, and the requisite line from the 6500/1 is connected

to the SP input line of CIA-A. The CNT line of the same CIA provides the key-

board system with the clock signal for signal synchronisation.

Having scanned the matrix, the 6500/1 returns raw key codes whenever

there is a state change in the keyboard. If a key is pressed, the code for

that key is sent. If that key is released, the code for that key, with bit 7

set, is sent to signal 'key released'. If a different key is pressed before

the release of the original key, the new keypress is sent first. The keyboard

events are sent in the order of occurrence where possible.

Special keys are wired into a different part of the matrix, so that

keycode clashes do not occur with the special keys SHIFT, CONTROL, ALT & the

two AMIGA keys. Both left and right SHIFT, left and right ALT, and left and

right AMIGA are given their own separate keycodes, making for a massive num-

ber of keycoding possibilities. CAPS LOCK also has its own key code, and is

treated somewhat differently. The 6500/1 simulates a push-button with this

key, no release information being supplied. The CAPS LOCK key state is only

judged to have changed when pressed - releasing the CAPS LOCK key is ignored

by the 6500/1. When pressed the first time, the LED lights up, and a code is

sent corresponding to 'key pressed'. The second time CAPS LOCK is pressed, at

which point the LED becomes unlit, the code for 'key released' (bit 7 set) is

sent. This is the only key treated thus - all other keys have their pressed/

released events handled in the normal way described above.

Key code groups are :

$00 - $3F	: Normal ASCII letter keys in the main

keyboard section between the grey ENTER

and the CAPS LOCK key, plus the numeric

keypad keys except ENTER.

$40 - $4F	: Codes of standard special keys, such as

the SPACE BAR, RETURN, TAB, BACKSPACE,

DEL, ESC, numeric keypad ENTER.

$50 - $5F	: Function keys F1-F10, HELP.

$60 - $6F	: SHIFT, ALT, CONTROL, AMIGA and CAPS LOCK

keys.

I shall supply the codes for the special keys individually. To work out the

other keys, it is possible to write a piece of code to scan the keyboard and

read the raw key codes. More of this later. The special key codes are :

Key		Code

---		----

LEFT SHIFT	$60

RIGHT SHIFT	$61

CAPS LOCK	$62

CONTROL		$63

LEFT ALT		$64

RIGHT ALT	$65

LEFT AMIGA	$66

RIGHT AMIGA	$67

There are also special codes sent for certain special functions. These are:

Keycode		Function

-------		--------

$F9		Last key code sent was incorrect

$FA		Keyboard buffer of 6500/1 is full

$FC		Error in keyboard self test (AGH! REPAIR TIME!)

$FD		Start of keys held down on power up

$FE		End of keys held down on power up

The $F9 code is sent if there has been disruption of the keyboard linkage. On

the A500, this usually means time to get the Amiga mended, but on A100/A2000

machines this can result from unplugging a keyboard & then plugging in a new

keyboard while the Amiga is switched on. This allows the keyboard resynchron-

isation system to be activated, re-establishing secure communications.

The 6500/1 has an internal character buffer of 10 characters. If the

buffer becomes full (because software is not reading it quickly enough), the

6500/1 sends the $FA code to indicate a full keyboard buffer, and that sub-

sequent keypresses will be lost.

Keyboard data communications are always conducted from the keyboard

to the Amiga. The Amiga sends handshaking signals to the keyboard and also a

clock signal for data transfer synchronisation.

The actual order in which the bits are sent is not the usual order

of 76543210, but 65432107. So when the data byte is received, it needs to be

rotated one bit position to obtain the true keycode. The CIA shift register

of CIA-A contains the data once read, and sends a level-2 interrupt once the

data has been received. The level-2 interrupt code should then read that data

byte, output the handshake pulse and save the received code somewhere safe to

be processed later by a user program.

Keycodes furthermore are inverted, because the circuitry is designed

as ACTIVE LOW. This means that a low voltage corresponds to a 1, and a high

voltage corresponds to a 0. To obtain the keycode, the bits must be inverted

back to the normal form.

Basically, the 6500/1 puts the data bits on its data line (KDAT),

plus a 20-microsecond low pulse on the clock line (KCLK). Between each of the

data pulses, 40-microsecond pauses are placed. Hence data transfer rate is 1

bit every 60 microseconds, or 16667 bits per second (16666 baud). After the

last data bit has been sent, the 6500/1 waits for a handshake pulse. This is

performed by the Amiga pulling KCLK low for 75 microseconds the moment that

the last data bit is received.

To handle the keyboard interface, the kind of code I would use looks

like this, on occasions when the operating system is being bypassed:

* complete level 2 interrupt handler for CIA-A

* to initialise CIA-A to activate this routine properly

* initialise with CIAAICR = $10, CIAACRA = $40 in the

* main program. This sets SPMOD = input,

* INMODE = CNT, generate interrupt when SP full.

* Don't forget to point IPL2 vector to this code as well!

ciaint		move.w	#$2700,sr	;lock out higher ints

;just in case

move.l	d0,-(sp)		;keep this

move.w	INTREQR(a5),d0

btst	#3,d0		;CIA interrupt?

beq.s	ciaint_1		;nope!

move.w	#$0008,INTREQ(a5)	;system acknowledge IRQ

move.b	CIAAICR,d0	;check CIA-A interrupt reg

btst	#7,d0		;CIA interrupt proper?

beq.s	ciaint_1		;no, cock up somewhere so ignore

btst	#3,d0		;SP data full?

beq.s	ciaint_1		;no, cock up somewhere so ignore

addq.w	#1,ciacount	;cia interrupt counter

move.b	CIAASP,d0	;get key code from keyboard

or.b	#$40,CIAACRA	;set SPMODE=output (pulls

;KCLK low!)

not.b	d0

ror.b	#1,d0

move.b	d0,rawkey	;proper raw key code

moveq	#8,d0		;wait for 75 microsecs

ciaint_0		subq.w	#1,d0		;while pulling KCLK low

bne.s	ciaint_0

and.b	#$BF,CIAACRA	;SPMODE=input again

ciaint_1		move.l	(sp)+,d0

rte

ciacount		dc.w	0

rawkey		dc.b	0,0

Keyboard reset mechanism:this is managed by the 6500/1. Pressing the sequence

of keys CTRL-AMIGA-AMIGA causes a hard reset. The 6500/1 control program will

sense this sequence, and pull KCLK low for about 0.5 seconds. This tells the

reset circuit of the Amiga to generate a hard reset. After one or more of the

keys are released, the 6500/1 also undergoes a reset, rebooting its control

program from scratch, signalled by flashing of the CAPS LOCK LED. Since KCLK

is connected to the CNT pin of CIA-A, and the above interrupt routine shows a

way of pulling KCLK low for 75 microseconds, it needs little imagination to

see that increasing the delay will allow software generation of a hard reset!

Just set SPMODE = OUTPUT for CIA-A, and hang the processor. After 0.5 seconds

or more, the Amiga will reset!

Mouse handling:the mouse counters are part of Denise. There are two

registers, called JOY0DAT (offset $00A) and JOY1DAT (offset $00C) for each of

the gameports 0 and 1. Just to relieve the confusion, the back panel of the

A500 says 'Joystick port 1' and 'Joystick port 2'. Subtract 1 from the num-

bers to get the appropriate gameport counter. The high byte of each counter

counts the vertical pulse count from 0 to 255, the low byte the horizontal

pulse count from 0 to 255.

The mouse counters count 200 pulses per inch (about 79 pulses/cm).

This makes the counters overflow after the mouse has moved about 4 cm. To

counter this, word-wide counters should be set up in software, and the actual

gameport counters used to update these. This is normally done by the opera-

ting system during the vertical blank interrupt. The following code is an

extract from a vertical-blank interrupt routine that I have used to handle

mouse counters:

;read mouse during VBL interrupt. mousex/y = old value of

;counters, mouseh/v = horiz/vert movement

move.w	JOY0DAT(a5),d0	;get mouse counter

move.w	d0,newmouse	;save for later

and.w	#$FF,d0		;x counter

sub.w	mousex,d0	;difference old-new

cmp.w	#-127,d0	;underflow?

bge.s	vblmouse1	;no

neg.w	d0		;else -255-diff

sub.w	#255,d0

bra.s	vblmouseh	;store it

vblmouse1	cmp.w	#127,d0		;overflow?

ble.s	vblmouseh	;no

neg.w	d0		;else 255-diff

add.w	#255,d0

vblmouseh	move.w	d0,mouseh	;store horiz difference >0 = right

move.w	newmouse,d0	;get saved mouse counter

lsr.w	#8,d0		;get vertical count

sub.w	mousey,d0	;difference old-new

cmp.w	#-127,d0	;underflow?

bge.s	vblmouse2	;no

neg.w	d0		;else -255-diff

sub.w	#255,d0

bra.s	vblmousev	;store it

vblmouse2	cmp.w	#127,d0		;overflow?

ble.s	vblmousev	;no

neg.w	d0		;else 255-diff

add.w	#255,d0

vblmousev	move.w	d0,mousev	;store vert difference >0 = down

moveq	#0,d0

move.w	newmouse,d0	;get mouse counters

lsl.l	#8,d0		;split across 2 words

lsr.w	#8,d0		;isolate x

move.w	d0,mousex	;save

swap	d0		;get y

move.w	d0,mousey	;save

newmouse	dc.w	0

mousex		dc.w	0

mousey		dc.w	0

mouseh		dc.w	0

mousev		dc.w	0

The algorithm used is as follows:the assumption is made that the mouse coun-

ters are not changed by more than 127 pulses between reads. Both old and new

values are maintained, and new compared with old. The value

diff = old - new

is calculated. If 0 < diff < 127, the mouse movement was either right or down

without overflow. If -127 < diff < 0, the mouse movement was either left or

up, without overflow. If diff > 127, movement was right or down, with a coun-

ter overflow. If diff < -127, movement was left or up, with a counter under-

flow. For overflow, the actual mouse movement is computed as 255-diff, while

for an underflow, the actual mouse movement is computed as -255-diff.

To reset the mouse counters, use the JOYTEST register (offset $036).

This register is unusual. The register bit allocation is as follows:

Y Y Y Y Y Y y y X X X X X X x x

The Y bits are the upper 6 bits of the vertical counter, and the X bits are

the upper 6 bits of the horizontal counter. The yy and xx bits are connected

directly to the mouse input signals, and are not located anywhere in memory.

So these values cannot be changed at all in software. JOYTEST has the effect

of resetting both sets of mouse counters, making JOY0DAT and JOY1DAT equal in

value. Whatever value is sent to JOYTEST is sent to both of the JOYxDAT regi-

sters.

The mouse buttons are handled separately. If the mouse is attached

to port 0, the signals occur as follows:

LEFT BUTTON		: CIAAPRA, Bit 6

RIGHT BUTTON		: DATLY of POTGO (#7)

MIDDLE BUTTON(*)	: DATLX of POTGO (#7)

For game port 1, the signals are:

LEFT BUTTON		: CIAAPRA, Bit 7

RIGHT BUTTON		: DATRY of POTGO (#7)

MIDDLE BUTTON (*)	: DATRX of POTGO (#7)

For all of these, a zero bit value means that the button is PRESSED.

Joysticks are handled in a similar way-they use the same counters & JOYTEST.

However, to sense the direction in which the joystick is moved, the software

algorithm differs. Basically, the following table holds:

Joystick Right		: Bit 1 JOYxDAT = 1

Joystick Left		: Bit 9 JOYxDAT = 1

Joystick Back		: (Bit 1 EOR Bit 0) = 1

Joystick Forward	: (Bit 9 EOR Bit 1) = 1

The following piece of code can be used to generate direction indicators for

the joystick (here called DX for left/right, DY for up/down):

move.w	JOY0DAT(a5),d0	;get joystick value

move.w	d0,d1		;& copy it

lsr.w	#1,d1		;shift copy left

moveq	#0,d2		;clear DX, DY

btst	#1,d0		;bit 1 set (right)?

beq.s	notright		;no

move.w	#1,d2

notright	btst	#9,d0		;bit 9 set (left) ?

beq.s	notleft		;no

move.w	#-1,d2

notleft		swap	d2

eor.w	d0,d1

btst	#0,d1		;result 1 (back) ?

beq.s	notback

move.w	#-1,d2

notback		btst	#8,d1		;result 1 (forward) ?

beq.s	notfront

move.w	#1,d2

notfront	swap	d2

Now D2 contains the value DX in the low word, DY in the high word, and the

programmer can handle this value ad lib. This method allows diagonal joystick

values to be managed (the example in the Amiga System Programmer's Guide does

not allow this) in a way that is useful for such things as games.

Note that to ensure diagonal values are read properly, it

might be prudent to embed the code above as a subroutine, save the value in

D2 in D3 after the first call, and then call the subroutine a few times more,

each time ORing the result of D2 into D3. This allows transient diagonal joy-

stick values to be strobed in case the joystick response is poor, a standard

trick used by games writers on systems with known poor joystick responses.

The joystick fire buttons for each port correspond to the left mouse

buttons in each case-the states of each are read from the same bits of the

same register (see mouse above).

Hardware:Some Notes

This file does NOT contain information about the Enhanced Chip Set (from here

on known as the ECS). The ECS is able to access a CHIP RAM range of 1MB as

opposed to the 512K of the standard chip set, and I assume this is achieved

by expanding the various pointer high word registers to 4 bits wide, making

a 20-bit address whole. Mind you, I have discovered that making logical ass-

umptions such as this about Commodore hardware can lead a programmer right

up shit creek without a paddle.

The ECS is also supposed to possess some other functional enhance-

ments. The exact details are not known to me at the time or writing. Anyone

possessing ECS information are requested to supply the requisite information

to the following address to allow me to maintain precise updates:

Dave Edwards

232 Hale Road

WIDNES

Cheshire

WA8 8QA

The same applies to other DOC files in this series, which are also obtain-

able by sending a blank disc plus SAE for return postage to

Mark Meany

1 Cromwell Road

Polygon

SOUTHAMPTON

Hants

SO1 2JH

marking your envelope "CLUB DISC 4" and enclosing a covering letter explain-

ing your requirements. Other useful files and software are also available on

the various Club Discs from Mark above, and any back copies may be obtained

(assuming that archive copies still exist) by marking your envelope with the

legend "Club Disc N" where N is the disc number. Don't forget to include an

SAE or better still a jiffy bag for return postage, as neither he nor I are

rich enough to provide a freepost service!

Hardware:Logic Tutorial

For the mathematically minded, I present a logic tutorial allowing derivation

of minterms via the mechanism of developing alternational normal schemata to

create the individual minterm components.

Conventions:In this tutorial, AB is used to represent A AND B, A + B

is used to represent A OR B, and b is used to represent NOT B. In an expres-

sion such as

ABC + ABc + AbC

the AND operation takes precedence over the OR operation, and the NOT opera-

tion applies only to that letter typed in lower case. The expression (NOT B)

AND (NOT C) is represented by

bc

and this list of conventions is thus complete.

Rules:The following laws apply to all logical expressions:

1) AB is the same as BA, and A + B is the same as B + A.

This is the Commutative law.

2) A(BC) and (AB)C are equivalent. Similarly, A + (B + C)

is equivalent to (A + B) + C. This is the Associative

law.

3) The expression A(B+C) expands to AB + AC. This is the

Distributive law of conjunction over alternation.

4) The logical AND of any single term with an expression

of the form (B + b) has no effect - this is the identity

operation (equivalent to mutiplying numbers by 1). So

A is equivalent to A(B+b) and further equivalent to

A(B+b)(C+c).

These fundamental laws apply to many mathematical systems other than Boolean

Algebra, in particular they apply to the arithmetic of integers, rational

numbers and real numbers.

Alternational Normal Schemata:This long-winded term describes any

logical expression which contains terms of the form ABC combined using the

OR operator. So the expression

AB + ABc + ABCD + ABcD + Abcd

is an alternational normal schema, whereas

(A+BC)D + ABCD

is not. The reason for the name is this:alternation is another name for the

OR operator in mathematical literature, and a normal schema is one in which

the format of the component terms obeys a strict set of rules.

A DEVELOPED alternational normal schema is one where all of the com-

ponents contain the same number of component variables. The creation of a de-

veloped alternational normal schema involves scanning the expression for the

term with the maximum number of variables, and expand all terms deficient in

variables until all terms match in number of variables. An example:

AB + C + b

becomes

AB(C+c) + (A+a)(B+b)C + (A+a)b(C+c)

and then

ABC + ABc + ABC + AbC + aBC + abC

+ AbC + Abc + abC + abc

Collecting together identical terms into single terms, and eliminating the

surplus terms, this becomes

ABC + ABc + AbC + Abc + aBC + abC + abc

This is a devleoped alternational normal schema. All of the terms have the

same number of variables, and the expression is made up of terms of the form

ABC, all combined by alternation or the logical OR operation.

Now, the connection between developed alternational schemata and

minterms is simple - they're one and the same. This tutorial is simply a dem-

onstration of a more formal method of derivation for those with the necessary

background. The example I have used to demonstrate the technique in action

was chosen deliberately to illustrate this connection. Basically, all that a

programmer does when picking minterms is an operation of the above kind, even

if using a more intuitive and less formal method.