+----------------------------------------------------------------------------+

|                             8520 CIA Chips:                                |

|                         CIA Data Ports A and B                             |

|                                                                            |

|                     Location Range $BFE001-$BFE301                         |

+----------------------------------------------------------------------------+

The CIA Chip registers allow your Amiga to interface with the outside world.

They grant access to disk-drives motors, mouse/joystick buttons states,

parallel port, serial port and other important functions such as timing.

Bits of data written to the PRA and PRB registers ($BFE001 and $BFE101)

can be sent to external devices, while bits of data those devices send

can be read here.

A bit can be set up for only input or output, not both at once. (I.e.;

these registers are not bi-directional) To define the I/O status of a

bit, you use the Data Direction Registers.

Register DDRA ($BFE201) controls data port A, and register DDRB ($BFE301)

controls data port B. 1 = output mode, and 0 = input mode. Note that DDRA

is preset by the Amiga on power up.  Modifying the contents may cause the

computer to lock up.

----------------------------------------------------------------------------

$BFE001 PRA          Peripheral Data Register for Data Port A

Status: R/W	     CIA-A

0 OVL    Memory overlay bit (Dont change! Keep 0)

1 LED    Power LED/Cutoff filter (1 = LED dimmed and

cutoff filter inactive, 0 = LED/filter normal)

2 CHNG   Disk Change (1 = no change, 0 = disk has

been removed)

3 WPRO   1 = Disk Write Protected  0 = Unprotected

4 TK0    1 = head not on trk 0, 0 = head on track 0.

5 RDY    1 = currently selected drive is ready for

commands, 0 = not ready.

6 FIR0   Fire button on port 1, pin 6 (0 = pressed)

7 FIR1   Fire button on port 2, pin 6 (0 = pressed)

Notes: On bits 6-7, you may also output data  through pin 6 on either

game port.  Simply set bit 6 or 7 (depending on which port you wish to

output data o) in the DDRA register to 1 for output and then toggle the

same bit in the PRA register.

Setting the PRA bit to one outputs approximately 5 volts on that line.

Setting the PRA bit equal to zero pulls the line low to zero volts.

For compatibility purposes, especially if your using the OS, or intend to

re-enable the OS to set the data direction registers back to their original

value after using them.

----------------------------------------------------------------------------

$BFE101 PRB          Peripheral Data Register for Data Port B

Status: R/W          CIA-A

0 D0  Parallel port pin 2

1 D1  Parallel port pin 3

2 D2  Parallel port pin 4

3 D3  Parallel port pin 5

4 D4  Parallel port pin 6

5 D5  Parallel port pin 7

6 D6  Parallel port pin 8

7 D7  Parallel port pin 9

Notes: This register is used exclusively by the Amiga's parallel port.

It controls the port's data lines, and is therefore responsible for all

parallel port transmissions.

For example, whenever the Amiga sends a character to a printer that's

connected to the parallel port, it uses this register.  It just sets all

the bits in the DDRB equal to one and writes the byte of data to be

output here.

----------------------------------------------------------------------------

$BFE201 DDRA         Data Direction Register A

Status: R/W          CIA-A

0 Select bit 0 PRA I/O (Must be 1 = Output)

1            1                  1

2            2                  0 = Input

3            3                  0

4            4                  0

5            5                  0

6            6         (Normally 0)

7            7         (Normally 0)

----------------------------------------------------------------------------

$BFE301 DDRB         Data Direction Register B

Status: R/W          CIA-A

This sets which mode we are in, input or output mode. A bit set or clear

here makes the corresponding bit in the PRB set for output or input,

respectively.

----------------------------------------------------------------------------

+----------------------------------------------------------------------------+

|                             8520 CIA Chips:                                |

|                       CIA Timers A & B L/H Bytes                           |

|                                                                            |

|                    Location Range $BFE401 - $BFE701                        |

+----------------------------------------------------------------------------+

These four timer registers (two for each timer) have different functions

depending on whether you are reading from or writing to them.  When you read

from these registers, you get the present value of the Timer Counter (which

counts down from its initial value to zero).  When you write to these

registers, the value is stored in the Timer Latch, and from there it can be

used to load the Timer Counter using the LOAD bit of the CRA or CRB

registers (see $BFEE01 and $BFEF01).

These interval timers can hold a 16 bit value from 0 to 65535, in low byte -

high byte format.  Once the Timer Counter is set to an initial value and the

timer is started, the timer will decrement every 10 uP cycles.

Since the clock speed of an NTSC Amiga is 7.159090 MHz, using every count

takes approximatels a millionth of a second. (10 / 7159090 = 0.0000013968256)

The formula for calculating the amount of time it will take for the timer

to count down from its latch value to zero is:

Time = Latch Value / Count Speed

Where Latch Value is the value written to the low and high timer registers,

and Count Speed is 715909 for NTSC, and 709379 for PAL.

When Timer A or B reaches 0, it will set bit 0 or 1 in the Interrupt

Control Register at $BFED01.

If the timer interrupt has been enabled, an interrupt will take place, and

the high bit of the Interrupt Control Register will be set to one.

Alternately, if the PBON bit is set, the timer will wirte data to bit 6 or 7

on Port B.  After the timer gets to zero, it will reload the Timer Latch

value and either stop or count down again, depending on whether it is in

one-shot or continuous mode (determined by bit 3 of the Control Register).

Although usually a timer will be used to count every 10 uP cycles, Timer

A can also count external pulses on the CNT line, which is connected to the

Amiga's keyboard.

Timer B is even more versatile.  In addition to these two sources, Timer

B can count the number of times that Timer A goes to zero.  By setting Timer

A to count the uP clock, and Timer B to count the number of times Times A

reaches 0, you effectively link the two into one 32 bit timer that can count

for more than an hour before resetting.

In the Amiga, CIA-A Timer A is used to time the transfer of keyboard

data.  Timer B is used by Exec (the core of the Amiga's multitasking

operating system) to synchronize task switching; it also serves as the

timer.device's UNIT_MICROHZ timer.  As a result, neither Timer A or B is

available for general use.

----------------------------------------------------------------------------

$BFE401 TALO            Timer A Low Byte

$BFE501 TAHI            Timer A High Byte

$BFE601 TBLO            Timer B Low Byte

$BFE701 TBHI            Timer B High Byte

Status: R/W             CIA-A

----------------------------------------------------------------------------

CIA-A (TOD) Time Of Day Counters

Location Range $BFE801 - $BFEB01

In addition to two general purpose timers, the 8520 CIA has a special

purpose 24-bit counter that can count up to 16777215 ($FFFFFF).  Unlike the

chip's other two timers, the TOD clock counts up, and when the TOD clock

reaches its maximum value, starts over at zero and continues counting.  The

TOD clock can also be set to generate an interrupt whenever it reaches a

determined value.

The TOD clock increments its counter approximately 60 times per second

(50 for PAL) - the same frequency as the computer's power line tick.

The TOD registers can be used for two purposes, depending on whether you

are reading from or writing to them.  If you are reading from them, you will

always be reading the timer's current value.  There is a latching feature

associated with the high byte register that freezes the other two bytes

until you read the low byte.  This is to keep inaccurate readings from

occurring.  (Note that the timer will not continue to count until you read

the low byte!)

Writing to these registers sets either the counter or alarm time

depending on the setting of bit 7 of Control Register B (CRB, $BFEF01).  If

that bit is set to 1, writing to the TOD registers sets the alarm.  If the

bit is set to zero, writing to the TOD registers sets the clock.  In either

case, as with reading the registers, there is a latch function.  This

function stops the clock from updating when you write to the high byte

register.  The clock will not start again until you write to the low byte

register.

The CIA-A's TOD clock serves as the timer.device's UNIT_VBLANK timer.

Unlike the Timer A driven UNIT_MICROHZ clock (which is also used by Exec),

UNIT_VBLANK is not slowed down if the computer is running several programs

at once.

----------------------------------------------------------------------------

$BFE801 TODLO        TOD Counter Low Byte

$BFE901 TODMID       TOD Counter Mid Byte

$BFEA01 TODHI        TOD Counter High Byte

$BFEB01 TODHR        UNUSED - Not connected to the Amiga

Status: R/W          CIA-A

----------------------------------------------------------------------------

$BFEC01 SDR          Serial Data Register

Status: R/W          CIA-A

The CIA chip has a built in serial port, which allows you to send or

receive a byte of data one bit at a time, with the MSB (7) shifted out

first.  Control Register A at $BFEE01 allows you to choose input or output

modes.

In input mode, a bit of data is read from the chip's SP line whenever a

signal on the CNT line appears.  After 8 bits are received, and the data

has been placed in the SDR, an interrupt is generated to let you know that

the register should be read.

In output mode, you write data to the SDR, and it is sent out over the SP

line using Timer A for the baud rate generator.  Whenever a byte of data is

written to using this register, transmission will start as long as Timer A

is running in continuous mode.  Data is sent at half the Timer A rate, and

an output will appear on the CNT line whenever a bit is sent.  After all 8

bits have been sent, an interrupt is generated to indicate that it is time

to load the next byte to send into the Serial Data Register.

The Amiga usus this register to receive data from the keyboard's KDAT

line (via pin 2 of the keyboard port), and the CNT line is connected to the

keyboard's KCLK line (via pin 1 of the keyboard port).  Data is never sent

to the keyboard, so this register is always operating in input mode.

By reading this register directly, you can find the RAWKEY value of the

current key being pressed.  A rawkey value specified the physical location

of the key on the keyboard.  Normally, the Amiga's operating system reads

the value from the SDR and uses the computer's current keyboard map table

(such as USA1) to convert it to ASCII.  Keymap tables are important because

different countries use different keyboard layouts.  There are at least 15

different Amiga keyboard configurations in existance.

The number returned does not have the high bit (7) set during the key

press.  However, when the key is released, bit 7 is set.  Thus, a key's

release value is 128 greater than its depress value.

You should know the shift or control keys do not modify the rawkey

values.  Instead, the shift or control keys send their own rawkey codes.

There are six special codes that can be sent via the SDR:

$78 - Control/Amiga/Amiga

$F9 - Ignore last character: transmission error

$FA - 10 character keyboard buffer is full

$FC - Keyboard self test failed

$FD - Header: Any following codes were keys pressed on power up

$FE - Termination code after the keys pressed on power up.

(i.e.: $FD xx xx ... $FE)

Using the console.device's RawKeyConvert() functiom, you can convert a

rawkey code to normal ASCII.

----------------------------------------------------------------------------

$BFED01 ICR          Interrupt Control Register

Status: R/W          CIA-A

Notes: Bit layout is the same as the ICR at $BFDD00.  This register is

used to control the five interrupt sources on the 8520 CIA chip.  These

sources are Timer A, Timer B, the TOD clock, the SDR, and the chip's FLAG

line.  Timer's A & B cause an interrupt when they count down to zero.  The

TOD clock generates an alarm when it reaches the alarm time.  The SDR

generates an interrupt when it compiles 8 bits of input or output.  An

external signal pulling the CIA chip's FLAG line low will also cause an

interrupt (on CIA-A, the FLAG line is connected to the ACK line - pin 10 of

the parallel port).

Even if the conditions for an interrupt are satisfied, the interrupt must

still be enabled for the interrupt to occur.  This is done by writing to the

ICR.  What happens when you write to this register depends on the way you

set bit 7.  If it is set to 0, any bit you write to with a 1 is cleared.  If

it is set to a 1, any bit that you write to with a 1 is set, and the

corresponding interrupt is enabled.  In either case, any bits written to

with a zero will be unaffected.

For example, to disable all interrupts from machine language, you could

MOVE.B #$7F,$BFED01.  Don't try this with multitasking turned on, as it will

turn off Timer B, which handles task switching.

To turn on a Timer B interrupt, a program could MOVE.B #$82,$BFED01.  Bit

7 is set to 1, and so is bit 1, so the interrupt which corresponds with bit

1 (Timer B) is enabled.

When you read this register, you can tell if any of the conditions for a

CIA interrupt were satisfied because the corresponding bit will be set to a

one.  For example, if Timer A counter down to zero, you would read a #$01

from this register.  If in addition the enable bit that corresponds to that

interrupt source is set to 1 and an interrupt occurs, bit 7 will also be

set.  This allows a multi-interrupt system to read one bit and see if the

source of a particular interrupt is CIA-A.  You should note, however, that

reading this register clears it, so you should preserve its contents if you

want to test more than one bit.  In order for the Amiga to recognize a CIA-A

chip interrupt, bit 3 of the INTENA register must be set to 1.  See

registers INTENA and INTENAR ($DFF09A, $DFF01C) for more information.

----------------------------------------------------------------------------

$BFEE01 CRA          Control Register A

$BFEF01 CRB          Control Register B

Status: R/W          CIA-A

See $BFDE00 and $BFDF00 for information on the bit layout.

----------------------------------------------------------------------------

------------------------------------------------------------------------------

8520 CIA Chips:

Location Range $BFD000-$BFDF00

CIA-B

------------------------------------------------------------------------------

Locations $BFD000-$BFDF00 are used to address the Amiga's second Complex

Interface Adaptor (CIA-B).  Since the chip itself is identical to the CIA-A,

which is addressed at $BFE001, the discussion here will be limited to the

use the Amiga makes of this particular chip.

For more general information on the chip registers, see the corresponding

entries for CIA-A.  Unlike the custom chip registers, the CIA registers

are all one byte long and can be both read from and written to.

------------------------------------------------------------------------------

$BFD000 PRA          Peripheral Data Register for Data Port A

Status: R/W          CIA-B

0 BUSY   Centronics busy (parallel port pin 11)

1 POUT   Centronics paper out (pin 12)

2 SEL    Centronics select (pin 13)

3 DSR    RS232 data set ready (serial port pin 6)

4 CTS    RS232 clear to send (pin 5)

5 DCD    RS232 carrier detect (pin 8)

6 RTS    RS232 request to send (pin 4)

7 DTR    RS232 data terminal ready (pin 20)

-------------------------------------------------------------------------

$BFD100 PRB          Peripheral Data Register for Data Port B

Status: R/W          CIA-B

0 STEP   Move drive head by one track in direction

specified by the DIR bit (set to 1, 0,

then 1 again to move head.)

1 DIR    Direction to move drive head (1 = out to 0,

0 = in to track 79+)

2 SIDE   Select drive head (1 = bottom, 0 = top)

3 SEL0   0 = Select DF0:  1 = not selected

4 SEL1   0 = Select DF1:

5 SEL2   0 = Select DF2:

6 SEL3   0 = Select DF3:

7 MTR    Motor on/off status (1 = motor off,

0 = motor on)

Disk-Drive Control Notes:

-------------------------

Bit 0: After moving the drive's head, it is important that you wait 3

microseconds before performing any other disk activities.  Since software

timing loops will be shorter on accelerated Amigas, use the CIA timers or

count vertical blanks VHPOS ($DFF006).

Bit 1: You can check if the drive head of a selected drive is on track

0 by reading bit 4 of the CIA-A chip's PRA register located at $BFE001.

Bit 2: When you select the head, the value of this bit affects only

the currently selected drives.

Bits 3-6: You may select more than one drive at a time.  Thus, you can

move the heads on more than one drive simultaneously.

Bit 7: This bit turns the motors on and off.  The drive LED is directly

wired to the motor power.  This bit should be set before you select a drive.

If a drive is already selected and you wish to change the state of its motor,

deselect the drive, set this bit, and then reselect the desired drive.

When turning on the motor, you must wait for the motor to reach its full

rotation speed before performing any other disk activities. Bit 5 of the

CIA-A's PRA ($BFE001) is set equal to 0 whenever the drive has reached full

speed and is ready to receive another command.

Note that all of this assumes that you have disabled multitasking and

have taken full control of the computer.

----------------------------------------------------------------------------

$BFD200 DDRA            Data Direction Register A

Status: R/W             CIA-B

A bit set in this register causes the corresponding bit in PRA to be

defined as output.  A bit clear causes it to be defined as input.  Note that

bits 0-5 are normally clear, and bits 6-7 are set.

----------------------------------------------------------------------------

$BFD300 DDRB            Data Direction Register B

Status: R/W             CIA-B

This register controls the I/O of PRB.

All bits MUST be set to 1.

----------------------------------------------------------------------------

CIA-B Timers A & B L/H Bytes

Location Range: $BFD400 - $BFD700

According to some early Commodore-Amiga manuals, Timer A is reserved for

Commodore 8-bit serial bus communications - the bus system used by the C64

to talk to the 1541, 1571, 1581, as well as C64 compatible printers.  For

all practical purposes, the Amiga has no use for these timers.  Timer B is

also unused by the operating system.  Feel free to use them yourself.  For

more details, see the entry for the CIA-A Timer A & B registers at $BFE401

through $BFE701.

$BFD400 TALO   Timer A Low Byte.  R/W

$BFD500 TAHI   Timer A High Byte. R/W

$BFD600 TBLO   Timer B Low Byte.  R/W

$BFD700 TBHI   Timer B High Byte. R/W

----------------------------------------------------------------------------

CIA-B TOD Counters

Location Range: $BFD800-$BFDB00

This 24 bit timer serves as a horizontal blank counter for the

graphics.library , and is used to synchronize the output of graphics with

the position of the video beam.

The CIA-B chip's TICK line is connected to the Agnus chip's _HSYNC line,

so the TOD clock's counter is incremented once every horizontal blank, or

31500 times per second.

$BFD800 TODLO  TOD counter low byte    R/W   CIA-B (0-7)

$BFD900 TODMID TOD counter mid byte    R/W   CIA-B (8-15)

$BFDA00 TODHI  TOD counter high byte   R/W   CIA-B (16-23)

$BFDB00 TODHR  UNUSED - Not connected to the Amiga

-----------------------------------------------------------------------------

$BFDC00 SDR          Serial Data Register

Status: R/W          CIA-B

The CIA chip has an on-chip serial port, which allows you to send or

receive a byte of data one bit at a time, with the MSB (7) shifted out

first.

The CIA-B's SP line, which carries bits to the Serial Data Register, is

connected to bit 0 of this chip's data port A, and in turn, to pin 11 of the

parallel port.  The chip's CNT line, which carries the bits that are output

by the Serial Data Register, is connected to bit 1 of this chip's Data Port

A, and in turn, to pin 12 of the parallel port.

For more information about the use of this register, see the entry for

location $BFEC01.  The Amiga's operating system does not use this register.

----------------------------------------------------------------------------

$BFDD00 ICR          Interrupt Control Register

Status: R/W          CIA-B

0 R - did Timer A count down to zero? (1 = yes)

W - enable or disable Timer A IRQ (1 = enabled  0 = disabled)

1 R - did Timer B count down to zero? (1 = yes)

W - enable or disable Timer B IRQ (1 = enabled  0 = disabled)

2 R - did TOD clock reach alarm time? (1 = yes)

W - enable or disable TOD alarm IRQ (1 = en  0 = dis)

3 R - did the serial data register finish a byte? (1 = yes)

W - enable or disable serial data register IRQ (1/0 = en/dis)

4 R - was a signal sent on the FLAG line? (1 = yes)

W - enable/disable flag IRQ (1/0 = en/dis)

5     UNUSED - always returns a zero

6     UNUSED - always returns a zero

7 R - did any CIA-A source cause an interrupt? (1 = yes)

W - set or clear bits of this register.  (1 = bits written to

with a 1 will be set, 0 = bits written with a 1 will be

cleared.)

This register is used to control the 5 interrupt sources on the 8520 CIA

chip.  For details on its operation, see $BFED01.

The main difference between the two CIA chips pertaining to this register

is that on CIA-B, the FLAG line is connected to the disk controller's

DSKINDEX line.  And in order for the Amiga to recognize a CIA-B chip

interrupt, bit 13 or the INTENA register must be set to 1.  See registers

INTENA and INTENAR at $DFF09A and $DFF01C for more information on Amiga

interrupts.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

CIA Control Registers A & B

Location Range $BFDE00 - $BFDF00

NB: Refer to locations $BFEE01 and $BFEF01 for details.

----------------------------------------------------------------------------

$BFDE00 CRA          Control Register A

Status: R/W          CIA-B

0 START   Start Timer A (1 = start, 0 = stop)

1 PBON    Select Timer A output on Data Port B (1 = Timer B

output appears on bit 6 for Data Port B)

2 OUTMODE Data Port B output mode (1 = toggle bit 6, 0 = pulse

bit 6 every 10 microprocessor cycles)

3 RUNMODE Timer A run mode (1 = one shot, 0 = continuous)

4 LOAD    Force latched value to be loaded to Timer A counter

(1 = force load strobe)

5 INMODE  Timer A input mode (1 = count signals on CNT line at

pin 1 of keyboard port, 0 = count every 10 uP cycles)

6 SPMODE  Serial Port ($BFEC01) mode (1 = output, 0 = input)

7 UNUSED

----------------------------------------------------------------------------

$BFDF00 CRB          Control Register B

Status: R/W          CIA-B

0 START   Start Timer B (1 = start, 0 = stop)

1 PBON    Select Timer B output on Data Port B (1 = Timer B

output appears on bit 7 for Data Port B)

2 OUTMODE Data Port B output mode (1 = toggle bit 7, 0 = pulse

bit 7 every 10 microprocesor cycles(

3 RUNMODE Timer B run mode (1 = one shot, 0 = continuous)

4 LOAD    Force latched value to be loaded to Timer B counter

(1 = force load strobe)

5-6 INMODE  Timer B input mode

00 = count every 10 uP cycles

01 Count cycles on CNT line at pin 1 of keyboard port

10 Count each time that Timer A counts down to 0.

11 Count Timer A 0's when CNT pulses are present

7 ALARM   Select TOD write status (1 = writing to TOD registers

sets counter, 0 = writing to TOD registers sets alarm)

------------------------------------------------------------------------------

End.

Part B

**************************************

--------------->> Another account of the Ciaa`s follows <<----------------

Hardware:The CIA Chips

There are two of these beasties on the Amiga. They're referred to as CIA-A &

CIA-B respectively. CIA-A lives at $BFE001 onwards, CIA-B at $BFD000 on. I

now present the register list. The formula for the address of each register

is register number*256+CIA base address, and all CIA registers are a

single byte wide.

Register		Name

--------		----

0		PRA (Port A Data Register) R/W

1		PRB (Port B Data Register)	R/W

2		DDRA (Port A Data Direction Register) R/W

3		DDRB (Port B Data Direction Register) R/W

4		TALO (Timer A lower 8 bits) R/W

5		TAHI (Timer A upper 8 bits) R/W

6		TBLO (Timer B lower 8 bits) R/W

7		TBHI (Timer B upper 8 bits) R/W

8		E.LSB (Event counter lower byte) R/W

9		E.MID (Event counter middle byte) R/W

A		E.MSB (Event counter high byte) R/W

B		not used

C		SP (Serial port data register) R/W

D		ICR (Interrupt control register) R/W

E		CRA (Control register A) R/W

F		CRB (Control register B) R/W

Note:the 'serial port' on the CIAs is NOT the same as the Amiga serial port

at the back! This is an internal serial port, and on the CIA-A it is connect-

ed to the 6500/1 processor handling the keyboard (#1). The main serial port

for external communications is handled via Paula (#2).

Data registers:the CIA data registers are connected in a seemingly odd manner

to various different pieces of hardware. They can be treated bitwise as input

or output registers. If the corresponding DDRx bit is 0, the same bit on the

PRx is an input, and an output if the DDRx bit is a 1. Writing to a data reg-

ister stores the value in it, and reading it reads the state of those port

lines to which it is connected.

There is a simple handshaking mechanism via two lines, called PC and

FLAG. PC goes low for 1 clock cycle on each access to PRB. The FLAG input re-

sponds to such transitions. Every time the state of FLAG changes from 1 to 0,

the FLAG bit in the ICR is set. These two lines allow a simple form of hand-

shaking in which the PC and FLAG lines of the CIAs are cross-connected. The

sender need only write its data to the port register & then wait for a FLAG

signal before sending each additional byte. Since FLAG can generate an int-

errupt if wanted, the sender can perform other tasks while waiting for the

FLAG signal, and continue the sending via an interrupt routine. The same also

applies to the receiver, except that it reads the data from the port instead

of writing to it. CIA-A has its PRB connected to the parallel port (more cor-

rectly, the Centronics port data lines), and CIA-B has a whole series of awk-

ward connections for PRB as follows (Cross-reference #6):

Bit	Connection

---	----------

7	/MTR motor signal to disc drive

6	/SEL3 drive 3 select

5	/SEL2 drive 2 select

4	/SEL1 drive 1 select

3	/SEL0 drive 0 select

2	/SIDE drive side select

1	DIR data direction signal to disc drive

0	/STEP step signal to drive head stepper motor

Both CIAs have awkward connections for PRA. They are (CIA-A first, then

CIA-B):

Bit	Connection

---	----------

7	Game port 1 pin 6 (fire button)

6	Game port 0 pin 6 (fire button)

5	/RDY disc ready signal from disc drive

4	/TK0 disc track 00 signal from disc drive

3	/WRPO write protect signal from disc drive

2	/CHNG disc change signal from disc drive

1	LED power LED (0=on, 1=off)

0	OVL memory overlay bit (DANGER!!!!)

7	/DTR DTR signal from serial interface

6	/RTS RTS signal from serial interface

5	/CD  CD signal from serial interface

4	/CTS CTS signal from serial interface

3	/DSR DSR signal from serial interface

2	SEL select signal for Centronics interface

1	POUT paper out signal from Centronics interface

0	BUSY busy signal from Centronics interface

When I said the connections were awkward, I wasn't kidding.

Control registers:the two control registers determine the mode of operation

of many of the other registers. CRA bit allocations are:

Bit	Function

---	--------

7	Not used

6	SPMOD serial port register mode 0=input, 1=output

5	INMODE 0=clock, 1=CNT (see later)

4	LOAD 1=force load (strobe) for timer A

3	RUNMODE 0=continuous, 1=one-shot for timer A

2	OUTMODE 0=pulse, 1=toggle

1	PBON 0=PB6 off, 1=PB6 on

0	START 0=off, 1=on

CRB bit allocations are:

Bit	Function

---	--------

7	0=TOD (time of day-see later), 1=ALARM

6,5	INMODE

00 = clock, 01=CNT

10 = timer A input,

11 = timer A + CNT

4	LOAD 1=force load (strobe) for timer A

3	RUNMODE 0=continuous, 1=one-shot for timer A

2	OUTMODE 0=pulse, 1=toggle

1	PBON 0=PB7 off, 1=PB7 on

0	START 0=off, 1=on

Timers:this is one of the complicated things. These timers can count down

from a preset value to zero. When reading, they are treated as TALO/TAHI

and TBLO/TBHI. When writing, they are treated as latches PALO/PAHI and

PBLO/PBHI. The latches should be loaded, low byte first, as a write access

to the high register causes the timer to be stopped and reloaded with the

latch value unless the LOAD bit in the control register is set, in which case

the latch value is transferred to the timers regardless of the timer state.

The LOAD bit in the CRA/CRB registers is a strobe bit. Writing simply causes

the given action to be performed-the bit value is not stored as such.

When the timer hits zero, the latch values are automatically put

back into the timer.

The latch is also known as the prescaler. The number of timeouts in

continuous mode is equal to the clock frequency divided by the value in the

prescaler/latch. Timer A is connected to the processor E clock, the frequency

being 716KHz.

Reading the timer can cause problems, because it has to be performed

in two separate read operations. One problem is that the timer could change

its state between the two reads. For example, if the timer bytes combined are

$0100 in value, and the high byte is read, the timer could decrement to $00FF

before the low byte is read, resulting in a final read value of $01FF, which

is incorrect (it should be $00FF). Stopping the timer from decrementing

during the read is one method of preventing this, but this is not the most

elegant way. Instead, read the high byte, then the low byte, then read the

high byte again in a separate register. If the two high byte reads are the

same, take this as the value. If not, repeat the process. It should only need

one repeat read to obtain the correct result, but in any case the following

code should obtain the correct value (code is for CIA-A):

LEA	$BFE001,A0	;CIA-A base address

get_timer	MOVE.B	$400(A0),D0	;get TALO

MOVE.B	$500(A0),D1	;get TAHI

MOVE.B	$400(A0),D2	;get TALO again

CMP.B	D0,D2		;timer values match?

BNE.S	get_timer	;repeat read if not

NOP			;here, got correct timer!

Bits 5 and 6 of the control registers control the timer functions. For timer

A, there are two possibilities. Either CRA INMODE=0, in which case timer A is

decremented during each clock cycle on the processor's E clock line, or else

CRA INMODE=1, in which case each high pulse on the CNT input line decrements

the timer.

Timer B has four possible operating modes, governed by the INMODE

bits of CRB. First there is INMODE mode 00, in which timer B decrements each

time a pulse occurs on the E clock line (connected to the f2 pin of the CIA)

just as for timer A INMODE=0. INMODE=01 decrements timer B every time there

is a pulse on the CNT line. INMODE=10 allows timers A and B to be combined to

form a 32-bit timer, and timer B decrements every time a timeout signal from

timer A is received. In this mode, timer A forms the low word of the 32-bit

timer, and timer B the high word. Finally, INMODE=11 allows the length of a

pulse on the CNT line to be measured. In this mode, timer A issues a timeout

when the CNT line is high. The timeout signals are in the interrupt control

register or ICR, to be dealt with later.

Two output modes for the timers can be selected with the OUTMODE bit

of the control registers. OUTMODE=0 causes timeout signals to appear as a

positive pulse one clock period long on the corresponding port line. When the

OUTMODE=1 (toggle mode), each timeout causes the corresponding port line to

change value from low to high or high to low. Each time the timer is started

in this mode, the output starts as a high signal.

The RUNMODE bit determines whether the timer operates in one-shot

mode (RUNMODE=1) or operates continuously (RUNMODE=0). In one-shot mode, the

timer stops after each timeout and sets the START bit to 0. In the contin-

uous mode the timer restarts after each timeout automatically.

Timer A of CIA-A is used by the operating system for communication

with the keyboard, and timer B by the operating system for some other tasks.

Timer A of CIA-B is used for serial data transfers, otherwise it is free, and

timer B is used to synchronise the blitter with the screen by the OS, other-

wise it is free.

Interrupt Control:this is actually implemented as two registers, one being a

read register and one being a write register. The bit allocations are given

in the table below, read register first. The read register is the ICR data

register, the write register the ICR enable mask register.

Bit		Function

---		--------

7		IR - Interrupt received/signalled

6		not used, always 0

5		not used, always 0

4		FLAG - PRB port handshaking

3		SP - serial port needs attention

2		Alarm signal

1		TB - Timeout for Timer B

0		TA - Timeout for Timer A

7		Set/Clr bit (explained below)

6		not used, send a 0

5		not used, send a 0

4		FLAG input enable

3		SP - serial port interrupt enable

2		Alarm input enable

1		Timer B timeout enable

0		Timer A timeout enable

The set/clr bit is used as follows. Let us assume that the byte to be written

to the ICR is %00010011. Bit 7 (the set/clr bit) is 0. This tells the input

latch for the ICR to clear all bits in the ICR write register corresponding

to the 1 bits in the byte just written to it, and to leave all other bits un-

changed. This particular write byte has bits 4,1 and 0 set, so the FLAG, TB

and TA interrupt inputs are disabled. If the byte was %10010011 instead, the

set bit 7 (corresponding to set/clr on the ICR) would tell the input latch

for the ICR to set all bits corresponding to the other 1 bits in the byte

just written, and leave all of the others unchanged. This write byte would

enable the modes disabled by the previous write byte example.

Note that this set/clr mechanism occurs throughout the Amiga chip

set, in particular for DMA control (#3), main system interrupt control (#4)

and other functions.

Having written a control byte to enable various interrupt sources

(CIA-local interrupt sources, NOT main system ones!), we can now read the

same address and the chip will return the value of the ICR data register,

which contains bitwise information about which interrupt source triggered

the CIA's internal interrupt mechanism. Note that any interrupt handled via

the CIAs is further passed onto the system as a whole, to allow 68000 auto-

vectored interrupt code to handle the interrupt. More about this later.

If this value is needed for multiple interrupt source tests, then it

must be saved, because reading the ICR data register causes the CIA to clear

it after the read. The IR bit is the interrupt request bit, and if set, indi-

cates that a valud CIA internal interrupt was triggered. But this and all of

the other ICR data bits are cleared upon reading, so if you fail to save the

result of the read (this code saves the read value);

LEA	$BFE001,A0		;CIA-A base addr

MOVE.B	$D00(A0),D0		;get ICR data reg value

MOVE.B	D0,last_icra_read(A6)	;save in my variable block!

and then obliterate the unprocessed bits without saving as above, then you

have lost forever those unprocessed bits. You have been warned!

The CIA signals an interrupt as follows. Whenever one of the various

interrupt sources sets its corresponding bit in the ICR data register, the

CIA checks to see if the corresponding ICR mask enable bit is also set. If

this is so, the CIA pulls the IRQ line low to signal the interrupt in hard-

ware, and then sets the IR bit (bit 7) in the ICR data register to signal the

interrupt in software also. How the main system treats this signal is handled

later. The IRQ line does not return to the high state (its normal state) un-

til the ICR data register is read (and hence the IR bit, and all others, are

cleared by the CIA).

Event Counter:this event counter differs from the 6526 Time-Of-Day counter

found, for example, in the Commodore 64 (yeuk!). This is almost the only

difference between the 6526 and the 8520 CIA. Those unfortunate enough to

have encountered the 6526 in past programming will breathe a sigh of relief.

There is one slight problem with the documentation that Commodore supply-

they insist on referring to TOD (time of day) in the literature, even though

this term is meaningless here.

Instead of the real-time clock which counts hours, minutes and sec-

onds on the 6526, the 8520 has a simple 24-bit event counter. It takes its

input signal on the TOD line (jeez, Commodore!) of the chip. The event coun-

ter starts at zero (or some other predefined state written to it) and counts

UPWARDS to $FFFFFF, before returning to zero. The event counter consists of

the counter proper and a latch register. When the high byte is read, the ac-

tual counter state is transferred completely to the 24-bit latch, and the

high byte of the latch returned. The counter continues counting undisturbed

while the remaining latch registers are read, mid-byte next, then low-byte.

ALWAYS read in the order high, mid, low or else this won't work!

Writing a value to the event counter causes the CIA to stop the

counter until the entire value is written, PROVIDED that it is written to

in the same order as it is read from above, namely high, mid, low. Once

the low byte is written, the timer starts up again, with the written value

as the start value. When writing the event counter value in normal mode, be

sure that the TOD/ALARM bit of the CRB control register is cleared!

An alarm function exists also. SET the Alarm bit in CRB as opposed

to clearing it as for normal event counter writing, and write a value into

the event counter. The chip will take this value as an alarm value, and if

the event counter value ever matches this alarm value, the alarm bit of the

interrupt control register is set. The value of the alarm setting CANNOT be

read, only written - reading the event counter always returns the current

counter latch value, regardless of the state of the TOD/Alarm bit in CRB. So

if you want to provide known alarm values, save them somewhere safe before

using them!

Serial Port:the serial port consists of the serial data register (which is

readable and writeable) and the shift register (not directly accessible).

Setting SPMODE=0 in the control register sets the serial port to input mode

and SPMODE=1 to output mode.

In input mode, the serial data on the SP line are shifted into the

shift register after each rising edge on the CNT line. After 8 CNT pulses the

shift register is full, and the data is transferred to the serial data reg-

ister. At the same time, the SP bit in the ICR data register is set. If more

CNT pulses are received, the data continues to shift into the shift register

until it is full again. If the user has read the serial data register before

this, the data is transferred across again, else it is lost. When using this

register for input, respond to it reasonably promptly!

In output mode, timer A is used to determine the send frequency. The

timeout rate of timer A (which must be operated in CONTINUOUS mode) controls

the baud rate of the transfer. The data are shifted out of the shift register

at half the timeout rate of timer A, whereby the maximum output rate is 1/4

of the clock frequency of the 8520.

The transfer begins after the first byte is written to the serial

data register. The CIA transfers the byte into the shift register. The

individual data bits now appear at half the timeout rate of timer A on the

SP line and the clock signal from timer A appears on the CNT line (it changes

value on each timeout so that the next bit appears on the SP line on each

negative transition [high to low] ). The transfer begins with the most sig-

nificant bit of the data byte. Once all 8 bits have been output, the CNT line

remains high and the SP line retains the value of the last bit sent. In add-

ition, the SP bit of the ICR data register is set to show that the serial

data register can be supplied with new data. If the next data byte was loaded

into the data register before the output of the last bit, the data output

will continue without interruption.

To keep the transfer continuous, the serial data register must be

re-supplied with fresh data at the proper time.

The SP and CNT lines are described as 'open-collector' outputs. This

allows the outputs of multiple CIAs to be connected together. See any good

text on electronic interfacing techniques for an explanation (especially the

mass of material on the IEEE-488 bus) of open-collector outputs.

On the Amiga, the SP line of CIA-A is connected to the KDAT line of

the keyboard 6500/1 processor, and the CNT line connected to the KCLK line of

the keyboard processor. So hardware keyboard reads can be performed by enab-

ling serial port interrupts in CIA-A (which AmigaDos does for its own use),

and writing an autovector interrupt routine to intercept the ICR SP signal

and read the serial data register.

The SP of CIA-B is connected to the Centronics BUSY signal (as is

PA0 of the CIA parallel port A) and the CNT line to the Centronics PAPER

OUT signal (as is PA1 of the CIA parallel port A).

Other Information:other connections for CIA-A are:

PC	/DRDY - Centronics handshake, data ready

FLAG	/ACK  - Centronics handshake, data acknowledge

IRQ	/INT2 input from Paula (#2)

RES	/RES reset line

and connections for CIA-B are:

PC	not used

FLAG	/INDEX - index signal from disc drive

IRQ	/INT6 input from Paula (#2)

RES	/RES reset line

One final point. For those with FAST RAM, it is possible to examine the

Kickstart ROM on the Amiga. Simply clear the Memory Overlay Bit (OVL,

bit 0, CIAAPRA) from a program in FAST RAM, and the CHIP RAM memory address

range becomes mapped onto the underlying Kickstart ROM. This can be read at

will, hence relocating the kickstart addresses. Kickstart normally appears

at $FC0000 (256k ROMS) or $F80000 (512k ROMS)

WARNING: DO NOT TRY THIS FROM A PROGRAM WITHIN CHIP RAM - THE PROGRAM WILL BE

LEFT HANGING AS THE 68000 PC REFERENCES THAT PART OF THE KICKSTART ROM WHICH

HAS THE SAME ADDRESS AS THE NEXT INSTRUCTION IN YOUR OVL SWITCHING CODE! I

HEAR THE GURU KNOCKING ALREADY...

End.