T H E   M E C H A N I C S   O F   D I S K   A C C E S S

By Phil!93

If  you've  ever  tried  writing a routine to read the disk drives directly from

hardware  you'll  know that it can be a a right err, pig.  There are quite a few

areas  where  critical time delays must be observed and the involvement of those

Completely  Incomprehesable Adapters (CIAs) doesnt jolly along the process much!

The  hardware  reference  manual  has  only  the  briefest  of  coverage  to the

disk-interface so I hope to fill in the gaps a bit.

Once  you've  identified  the  registers  that  are required for disk access and

sorted  out the timing loops everything becomes clear - Tab clear.  The are only

2  cia  registers  involoved  in  the "mechanics" of disk access and first up is

$BFD100.  Here's the run down;

Bit 7-MTR - Motor On/Off line  *

6-SEL3-Select drive 3: df3 *

5-SEL2-Select drive 2: df2 *

4-SEL1-Select drive 1: df1 *

3-SEL0-Select drive 0: df0 *

2-SIDE-Disk side to access.*

1-DIR -Direction to move heads

0-STEP-Step those heads!   *

*  The  motor,select,side  and  step  bits  are "active low" which means that to

activate  the  "device"  using  that  bit,  you write a 0 into that bit location

instead of the usual 1.

Now more info about each of these bits in turn.

BIT 7, MRT - The motor line is a funny fish.  This is non-standard on the Amiga.

A  drive  will  latch  the value of this bit into its own flip/flop whenever its

select  bit  is  turned  on.  And drives remember the state of their motors when

they  arent selected.  So to switch on the motor in the internal drive you could

do this:

a.  bclr #7,$bfd100 - motor line on.

b.  bclr #3,$bfd100 - select df0

c.  bset #7,$bfd100 - motor line off

d.  bset #3,$bfd100 - deselect df0

The  motor  actually  switches  on  after  line  b has been executed by the CPU.

Notice  that it doesnt matter that the motor line is switched off (at c) cus the

drive  has  already  latched (stored) the on motor value.  Also it doesnt matter

that the drive is deselected because it will remember its motor latch value.  To

switch off the motor you could use:

a.  bset #7,$bfd100 - motor line off

b.  bclr #3,$bfd100 - select df0

c.  bset #3,$bfd100 - deselect df0

BITS  6-3, (SEL3-0) - These are the select lines for the drives.  Any drive that

has  a  0  in  its respective bit location will become "active" (even though its

motor  may  not  be on) - its possible to have more than one drive selected at a

time but you have to be careful that you are getting the status info (see below)

from the drive you want.

BIT  2, SIDE - Just tells the Drive which head it should use and therefore which

disk  side  to  read  from.  A 0 here means use the upper head and a 1 the lower

head.  Also see timing info.

BIT  1,  DIR - Tells the drive which direction it should step the heads when the

STEP  bit is pulsed (see below).  A 0 means step inwards (toward track 79) and a

1 therefore means step outwards towards track 0 (the one with the boot block!).

BIT  0,  STEP  - This bit actually causes the heads to be stepped up or down the

disk.  It is an active low line and it should always be sent a pulse, first a 0,

then  a  1.  A NOP between setting the line to 0 then 1 should be enough for the

drive  to  realise you want the heads stepped.  The head actually moves within 3

microseconds  of  receiving  the  1.   Remember  that  the  head is a mechanical

creature  and  therefore requires a time to actually move and settle after doing

so (see timing info).

So  that's  $bfd100  taken  care of, the second cia location that deals with the

drives is:  $BFE001.

BITS 7 & 6 are not used for disk access.

BIT 5 - RDY: Disk ready or ID stream.

BIT 4 - TK0: Active when head over track0

BIT 3 - WRPRO: Disk is write protected.

BIT 2 - CHNG: Disk removed/changed.

BITS 1 & 0 are not used for disk access.

All the above disk bits are active low The data in these bits is relevant to the

selected drive.  Now for an in depth look at each bit's meaning.

BIT  5, RDY - This actually has two uses.  If the motor in the selected drive is

on then a 0 here means that a disk is inserted and the motor is up to speed.  If

the  motor  is  off,  this bit acts as an identification stream to enable you so

find out what drives are connected to the Amiga (See example later).

BIT  4,  TK0.   A  Nice  simple  one.   If  the  heads on the selected drive are

positioned over track 0 then a zero is presented here.

BIT  3,  WRPRO  -  Simple  again.  If the disk inserted in the selected drive is

write protected then a zero appears here.

BIT  2,  CHNG.   If  you select a drive that has had a disk removed/changed this

becomes  zero.   THAT  drive  will  not  return  a 1 to this bit until a disk in

inserted  in  it and its heads are stepped.  This is why Amiga drives click when

there's no disks in 'em!

So  with  the  CIA  addresses  taken care of we must consider timing.  As I said

before  drives  are  mecahnical  devices and require time to settle/respond etc.

The timing requirements are as follows;

Head  stepping  delay  -  Drives  are  guarrenteed  to  step  the  head within 3

milliseconds (.003 seconds).

Head  settling  time  (before  disk  access):  15 milliseconds should be allowed

before  disk  access  after  the last step.  When reversing the direction of the

head 18 milli-seconds should be allowed after the last step pulse.

Disk side selection - This signal must be stable for at least 100 micro- seconds

before disk access.  And 1.3 milliseconds must elapse before switching sides.

Anyone who's thinking of using a simple counter loop for the timings- FORGET IT!

Instant  A1200  incompatibilty  is  probably all you'll achieve!  Not all Amigas

have a 68000 running at 7.14Mhz you know!

One  method  of timing that will work on any Amiga model is to wait for x raster

lines.   You  can find a conveniant raster line counter in $bfd800.  A better if

more  complex  method  is to use the CIA timers (which run at 715.379 Khz on PAL

Amigas  and  715.909 on NTSC Amigas -So long as you are a bit generous with your

timings  you  should be OK in the USA!).  Using these timers allows you to issue

an  interupt  at  the  end of your delay if you want.  Anyway, the timers are 16

bit,  ie:   they can count 65535 clock pulses before wrapping to 0.  There are 4

of  these  in  all,  2 in each cia (Bfd400/500 and Bfd600/700 and Bfe401/501 and

bfe601/701).   These  timers  can  be  set  up  so  that when you write to these

addresses the Cia uses your value as a figure to count down from to 0, then sets

an  interrupt  flag,  whether  this  actually causes an interrupt depends on the

settings  of  the  IRQ  mask.  Cia_a causes a lev 2 irq and cia_b causes a lev 6

irq.

So to do a simple busy wait delay using CIAb, timer B, the method is:-

move.l #$bfd000,a0    ;cia_b base

move.w #TimerValue,d0 ;countdown

move.b d0,$600(a0)    ;set lo byte

lsr.w #8,d0

move.b d0,$700(a0)    ;set hi byte

move.b #%00011001,$f00(a0) ;start

loop

btst #1,$d00(a0)      ;wait for time

beq.s loop            ;to elapse

OTHER CONSIDERATIONS

Obvious this, but theres no way of knowing where the head is apart from stepping

the  head to zero, once you know this you can set up a track marker variable and

just  step  the head to where you want it to go with a vector (+/- x tracks).  I

read  somewhere that swapping disks can shift the head on some drives so you may

want to reinitialize the head back to zero whenever a disk is changed (Amiga Dos

does this to find info on the dos type etc anyway).

The  Head  Direction  must be set up before the step pulse is issued.  Don't set

both  operations  into  one  instruction.   Stick  a  NOP  or  two  between  the

instructions as well if you like.

Dont  try  to  step the heads beyond track 0, too much of this could result in a

knackered drive.

After  starting  the  motor,  wait for the RDY to become active before accessing

this disk - This could be up to 500 milliseconds.

Don't  over  do  the  head step delay!  It won't cause any damage but its sounds

terrible!  Especially when moving the heads over more than a few tracks.

If  you're  tinkering about with the drive hardware make sure the system and any

unwanted interrupts are off!

IDENTIFYING AVAILABLE DRIVES

Its  amazing  how  many disk routines still ingnore the presence of any external

drives  - forcing you to swap disks even when there's no need.  The fact is that

its a piece of piss to check for external drives.

The method for identifying drives is:

a. Select the drive to check.

b. Switch its motor on then off.

c. (Re)select the drive to check.

d. Read the DISK_RDY bit and store in a 32 bit shift register.

e. Deselect drive & loop back to b 32 times.

f. Invert the shift register.

This will provide you with a I.D.  longword which will be either:

a. $00000000 - No drive present.

b. $FFFFFFFF - Amiga 3.5" Drive.

There are other IDs but these are the only ones we're interested in.

Normally  you only want to check for DF1:  so here's some source code to do just

that:

move.l #$bfd100,a0 ;cia we need.

move.b #$7f,(a0)

bclr #4,(a0)       ;df1: motor on.

- wait -

move.b #$ff,(a0)   ;df1: motor off

bclr #4,(a0)

Moveq #$1f,d7      ;loop 32 times.

moveq #$00,d0

nxt lsl.l #1,d0

bclr #4,(a0)       ;select df1:

btst #5,$f01(a0)   ;read RDY.

beq skp

ori.w #1,d0

skp bset #4,(a0)       ;deselect df1:

dbf d7,nxt

not.l d0

cmpi.l #$ffffffff,d0

beq DF1available

df1notavailable....

code.....

df1available

rest of code......

The wait should be at least .5 seconds

I know that was a very long winded way of coding it.  But I wanted to follow the

"method text" more or less to the letter.

Actually doing the DMA to read and write has been covered loads of times and the

MFM  layout  of  a disk was on GV15 so theres no need to repeat it here.  If you

want to write to me, see my ad in the Advert section.