MOTOROLA 68000 INSTRUCTION SET

==============================

This file is Public Domain. It contains the Motorola 68000 instruction set,

fully documented, in a manner conforming to the standards laid down in the

official Motorola documentation. The information contained here should be

sufficient not only to allow newcomers to 680x0 assembly language to famil-

iarise themselves with the instruction set, but for more experienced 68000

programmers to devise the basis of their own assembler, containing as it

does the actual machine words corresponding to each instruction in binary

format.

The file includes extra information:

1) Extra instructions found upon the 68010 processor, and

where possible, some extra instructions found upon the

68020 processor.

2) Extra addressing modes found upon the 68020 processor.

There are several conventions used here. The following convention is used to

illustrate condition code status:

X N Z V C

- 0 1 * U

-	Flag unchanged by operation

0	Flag set to 0 by operation

1	Flag set to 1 by operation

*	Flag changed functionally according

to operation

U	Flag undefined after operation

Where instruction words are given, they are given in binary, with various bit

fields picked out using various letter combinations. These bit fields in turn

have their own explicit documentation within each instruction. The characters

X, N, Z, V and C refer to those condition code bits throughout except where a

different convention from below specifies otherwise.

The following abbreviations are used in the documentation of the var-

ious 68000 instructions:

src	Source Operand

dst	Destination Operand

Dn	Any Data Register

An	Any Address Register

Rn	Any 680x0 Register, be it

Data or Address Register

Xn	Any 680x0 Register, be it

Data or Address Register,

in the context of an index

register

CCR	Condition Code Register

SR	Status Register

PC	Program Counter

<ea>	Any Effective Address using a legal

680x0 addressing mode.

#nnn	Any Immediate Operand

(10)	Specifies BCD Operand

op{N}	Specifies bit N of Operand for

68000/68010.

The main documentation will consist of two parts. The first part is

the individual instructions and their descriptions, the second is a full list

of possible addressing modes and condition code computations.

Furthermore, some instructions are PRIVILEGED. This means that they

can ONLY be executed when the 68000 is operating in SUPERVISOR mode (i.e.,

bit 13 of the status register is set). Attempting to execute these instruc-

tions with the 68000 in USER mode will cause the 68000 to enter exception

processing, and trap via the Privilege Violation exception vector.

NOTES : Some warnings for programmers inclined to use various tricks:

1) All arithmetic performed upon address registers uses the ENTIRE

32 bits of the address register! WORD size operands are SIGN

EXTENDED to 32 bits within the 68000 prior to the operation.

Source operands are NOT affected, however.

2) MOVEM.W from memory to registers SIGN EXTENDS TO 32 BITS. THIS

INCLUDES DATA REGISTERS! So, if using upper halves of data reg-

isters to save important data, best to MOVEM.L to and from memory

instead, otherwise the upper halves of the data registers will

be corrupted!

3) To ensure correct testing for multiple-precision extended-bit

arithmetic instructions (ADDX, SUBX), use something such as

MOVEQ #0,D0 to set the Z flag prior to the ADDX/SUBX sequence.

Then, testing for zero operands at the end of the operations

will work correctly (unless other instructions in your arith-

metic loop affect Z also!).

4) DIVS/DIVU take a 32-BIT SOURCE. The result is a 16-BIT DESTIN-

ATION, and a 16-BIT REMAINDER. If an overflow occurs, then the

destination is NOT AFFECTED. This holds for 68000/68010. For

68020 onwards, extended DIVS/DIVU instructions exist.

5) MULS/MULU take 16-BIT source and destination operands, but the

result is a 32-BIT destination. So the upper half of Dn will

be corrupted. If using upper halves of data registers to save

important data, save the data somehow first. This holds for

68000/68010. For 68020 onwards, extended MULS/MULU instructions

exist.

6) DBcc only affects low words of data registers. So two counters

can be kept in one register and SWAPped as appropriate (see

DBcc and SWAP below).

7) Since 68000 address pointers are 24-bit pointers, there are

always 8 bits spare at the uppermost byte of an address reg-

ister on the 68000. These can be used to mix flags data with

address pointers. DO NOT USE THIS TRICK IF CODE IS INTENDED

TO BE PORTABLE TO 68020/68030/68040 ENVIRONMENTS! These pro-

cessors use the full 32 bits of an address register, and have

a full 32-bit address space. Bus errors could result. See the

Motorola documentation for more information.

8) Programmers extending the instruction set via LINEF instructions

can do so with impunity on the 68000. On the 68020/68030/60840,

this is NOT possible. LINEF instructions are used heavily by the

68881/68882 mathematics coprocessors for the 68020/68030, and the

68040 is effectively a 68030/68882 combination on one chip. So for

higher 680x0 processors, treat LINEF with care. See Motorola docu-

mentation for more information.

9) LINK/UNLK make recursive code simpler to write than on other pro-

cessors. Exercise care with extraneous returns! If using so-called

"married" recursive functions, ensure that there are no 'secret

back-door exits' that bypass the stack cleaning routines!

68000 - The Instructions

========================

ABCD	: Add Decimal With Extend.

Operation : src(10) + dst(10) + X -> dst

Assembler :	ABCD	Dy,Dx

ABCD	-(Ay),-(Ax)

Attributes:size=Byte

Description	: Perform Binary Coded Decimal (BCD) addition of

the source and destination operands, together

with the Extend (X) bit of the CCR. The only

legal addressing modes are those given above.

This is a byte operation only.

Condition Codes	:	X N Z V C

* U * U *

N Undefined

V Undefined

Z Cleared if nonzero result.

UNCHANGED otherwise!

C Set if DECIMAL carry generated.

Cleared otherwise.

X Set identically to C.

Notes		: Normally, the Z bit is set via programming

(this is the PROGRAMMER'S responsibility)

before the start of an ABCD operation. This

allows successful tests upon completion of

multiple-precision ABCD operations.

Instruction Format:

1 1 0 0 X X X 1 0 0 0 0 M Y Y Y

Instruction Fields:

XXX : Register Rx field. Specifies which register

is used for the destination operand, D0-D7

for ABCD Dy,Dx or A0-A7 for ABCD -(Ay),-(Ax)

YYY : Register Ry field. Specifies which register

is used for the source operand, D0-D7

for ABCD Dy,Dx or A0-A7 for ABCD -(Ay),-(Ax)

M   : R/M field. Specifies addressing mode.

0 : mode is ABCD Dy,Dx

1 : mode is ABCD -(Ay),-(Ax)

ADD	: Add Binary.

Operation : src + dst -> dst

Assembler : ADD <ea>,Dn

ADD Dn,<ea>

Attributes:size=Byte,Word,Long

Description	: Add the source operand to the destination

operand, and store the result in the dest-

ination operand. The operation may be spec-

ified to be byte, word or long in size. The

mode of the instruction indicates which is

the source and which is the destination

operand.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

Instruction Format :

1 1 0 1 R R R M M M E E E A A A

Instruction Fields :

RRR : Register field. Specifies any data register

D0-D7.

MMM : Op-Mode field. Specifies operation type in

accordance with the table:

Byte	Word  Long	   Operation

----	----  ----	   ---------

000	001   010	  ADD <ea>,Dn

100	101   110	  ADD Dn,<ea>

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The legal modes are:

Source <ea> : All addressing modes.

Destination <ea> : The following are ILLEGAL addressing

modes:

An

offset(PC)

offset(PC,Xn)

#nnn

All others are legal.

Notes		: 1) If the destination is a data register,

then the ADD <ea>,Dn form of the instruc-

tion MUST be used.

2) ADDA is used for ADD <ea>,An. ADDI is

used for ADD #nnn,<ea> where <ea> is not

a data register. ADDQ is also used for

ADD #nnn,<ea> when possible. Most 68000

assemblers can automatically make this

distinction.

ADDA	: Add Address

Operation : src + dst -> dst

Assembler : ADDA  <ea>,An

Attributes:size=Word,Long

Description	: Add the source operand to the destination

address register, and store the result in

the destination address register. The size

of the operation may be specified to be a

word or long operation.

NOTE:All address arithmetic on address

registers is performed using the ENTIRE

32 bits of the address register. Word

operands are sign-extended to 32 bits

before adding.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 0 1 R R R M M M E E E A A A

Instruction Fields :

RRR : Specifies one of the address registers A0-A7.

This is ALWAYS the destination operand.

MMM : Op-Mode field. Specifies the size of the

operation.

011 : Word operation. See NOTE above.

111 : Long operation.

EEE and AAA fields. Defined in addressing mode section

below. All legal 68000 addressing modes are allowed for

the source operand.

ADDI	: Add Immediate Operand

Operation : immediate data + dst -> dst

Assembler : ADDI  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Add the immediate data contained in the

instruction to the destination operand,

and store the result in the destination

location.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Word : 2nd word of operand.

Long : 2nd and 3rd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

Instruction Format :

0 0 0 0 0 1 1 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

ADDQ	: Add Immediate Quick Operand

Operation : immediate data + dst -> dst

Assembler : ADDQ  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Add the immediate data contained in the

instruction to the destination operand,

and store the result in the destination

location. The range of values for the

immediate operand is 1 to 8, represented

in the instruction by 0 to 7 (0 repre-

senting a value of 8). The immediate data

is automatically coerced to the operation

size by the 68000.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

NOTE : Condition codes NOT AFFECTED if addition

destination is an address register!

Instruction Format :

0 1 0 1 D D D 0 S S E E E A A A

Instruction Fields :

DDD	: Data field. 3 bits of immediate data,

0 representing a value of 8, 1-7 repre-

senting those values repsectively.

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

An (Word and Long only)

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

ADDX	: Add With Extend

Operation : src + dst + X -> dst

Assembler : ADDX  Dy,Dx

ADDX -(Ay),-(Ax)

Attributes:size=Byte,Word,Long

Description	: Add the source operand to the destination

operand, along with the extend bit, and

store the result in the destination loca-

tion.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Cleared if result is non-zero.

Unchanged otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

NOTE : Normally the Z bit is set via programming

(e.g., using MOVEQ #0,D0) before the start

of the operation. This allows successful

tests for zero operands upon completion of

multiple-precision operations.

Instruction Format :

1 1 0 1 X X X 1 S S 0 0 R Y Y Y

Instruction Fields :

XXX	: Register Rx field. Specifies the destination

register, depending upon R field.

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

R	: R/M field, specifies operation addressing mode.

0 : operation is ADDX Dy,Dx, and XXX,YYY

fields specify data registers.

1 : Operation is ADDX -(Ay),-(Ax), and XXX,YYY

fields specify address registers.

YYY	: Register Ry field. Specifies the source

register, depending upon R field.

AND	: Logical AND operation

Operation : src AND dst -> dst

Assembler : AND	<ea>,Dn

AND	Dn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (bitwise) AND the source operand

with the destination operand, and store the

result in the destination location.

Condition Codes :	X N Z V C

- * * 0 0

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not affected.

Instruction Format :

1 1 0 0 R R R M M M E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers.

MMM	: Op-Mode field. Specifies operation

according to:

Byte	Word  Long	   Operation

----	----  ----	   ---------

000	001   010	  AND <ea>,Dn

100	101   110	  AND Dn,<ea>

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Source Operand:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

#nnnn

Destination Operand:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTES : 1) If the destination is a data register, then

it CANNOT be specified using destination <ea>

addressing mode, but MUST use the destination

Dn mode above instead.

2) ANDI is used when the source is immediate data.

Most assemblers automatically make this distinc-

tion.

ANDI	: Logical AND immediate

Operation : immediate data AND dst -> dst

Assembler : ANDI  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (Bitwise) AND the immediate data

contained in the instruction to the destin-

ation operand, and store the result in the

destination location.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Word : 2nd word of operand.

Long : 2nd and 3rd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

- * * 0 0

N Set if the most significant bit of the

result is set. Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 0 0 0 0 0 1 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

ANDI	: Logical AND immediate to CCR

Operation : immediate data AND CCR -> CCR

Assembler : ANDI  #nnnn,CCR

Attributes:size=Byte

Description	: Logically (Bitwise) AND the immediate data

contained in the instruction to the contents

of the Condition Code Register, and store the

result in the Condition Code Register (low 8

bits of Status Register).

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Cleared if Bit 3 of the operand is zero.

Unchanged otherwise.

Z Cleared if Bit 2 of the operand is zero.

Unchanged otherwise.

V Cleared if Bit 1 of the operand is zero.

Unchanged otherwise.

C Cleared if Bit 0 of the operand is zero.

Unchanged otherwise.

X Cleared if Bit 4 of the operand is zero.

Unchanged otherwise.

Instruction Format :

0 0 0 0 0 0 1 0 0 0 1 1 1 1 0 0

0 0 0 0 0 0 0 0 B B B B B B B B

Instruction Fields:

BBB...	:	Byte data instruction field. The

immediate data is contained in

this field.

ANDI	: Logical AND immediate to SR (PRIVILEGED INSTRUCTION)

Operation : If 68000 in supervisor state

THEN

immediate data AND SR -> SR

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : ANDI  #nnnn,SR

Attributes:size=Word

Description	: Logically (Bitwise) AND the immediate data

contained in the instruction to the contents

of the Status Register, and store the result

in the Status Register.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Word : 2nd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Cleared if Bit 3 of the operand is zero.

Unchanged otherwise.

Z Cleared if Bit 2 of the operand is zero.

Unchanged otherwise.

V Cleared if Bit 1 of the operand is zero.

Unchanged otherwise.

C Cleared if Bit 0 of the operand is zero.

Unchanged otherwise.

X Cleared if Bit 4 of the operand is zero.

Unchanged otherwise.

Instruction Format :

0 0 0 0 0 0 1 0 0 1 1 1 1 1 0 0

W W W W W W W W W W W W W W W W

Instruction Fields:

WWW...	:	Word data instruction field. The

immediate data is contained in

this field.

ASL,ASR	: Arithmetic Shift

Operation : dst (shifted by count) -> dst

Assembler : ASd	Dx,Dy

ASd #nnnn,Dy

ASd <ea>

Attributes:size=Byte,Word,Long

Description	: Arithmetically shift the bits of the destination

operand in the direction specified. The carry bit

receives the last bit shifted out of the operand.

The shift count for the operation can be specified

in two ways:

1) Immediate. The shift count value is 1-8.

2) Data Register. The value contained in the

specified data register, modulo 64, is

used.

If the destination operand is a memory operand,

then the shift count is ALWAYS 1, and the operand

size is ALWAYS word sized.

ASL : Operand is shifted left by the value of the

shift count. Bits shifted out of the high

order bit are copied to the C and X bits of

the CCR. V bit indicates sign changes during

the operation.

ASR : Operand is shifted right by the value of the

shift count. Bits shifted out of the low order

bit are copied to the C and X bit of the CCR.

The sign bit (high-order bit) is replicated

into the now vacant high-order bits.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if most significant bit is changed

at any time during the operation. Cleared

otherwise.

C Set according to the last bit shifted out

of the operation. Cleared for a shift count

of zero.

X Set according to the last bit shifted out

of the operand. UNAFFECTED for a shift

count of zero!

Instruction Format (Register Shifts) :

1 1 1 0 C C C D S S L 0 0 R R R

Instruction Fields :

CCC	: Count/Register field. If immediate count,

CCC = 1-7 for a shift count of 7, 0 for a

shift count of 8. If data register count,

CCC = data register number for register Dx.

The value contained in bits 0-5 of Dx is

used as the shift count (i.e., Dx modulo

64).

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

L	: L/R field. Specifies shift count type.

0 = immediate shift count in CCC field.

1 = data register count, register in CCC field.

RRR	: Register field. Specifies any of the 8 data

registers whose content is to be shifted.

Instruction Format (Memory Shifts) :

1 1 1 0 0 0 0 D 1 1 E E E A A A

Instruction Fields :

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

Bcc	: Branch Conditionally (Also Including BRA)

Operation : IF condition is TRUE

THEN

PC + d -> PC

ELSE

continue normal sequential operation

Assembler : Bcc <LABEL>

Attributes:size=Byte,Word

Description	: The conditional expression specified in the

instruction is evaluated and the result tested.

If the result is TRUE, the displacement is

sign-extended to 32 bits and added onto the

current value of the program counter, at which

point program execution continues. If the res-

ult is FALSE, program execution continues sequ-

entially as normal.

Possible Instructions (U = unsigned test, S=signed test) :

BT	Branch if True (Also BRA, Branch Always)

BF	Branch if False (DOES NOT EXIST:SEE BSR)

BHI	Branch if Higher		(U)

BLS	Branch if Lower or Same 	(U)

BCC	Branch on Carry Clear		(U)

BCS	Branch on Carry Set		(U)

BNE	Branch if Not Equal (to Zero)	(U and S)

BEQ	Branch if Equal (to Zero)	(U and S)

BVC	Branch if Overflow Clear

BVS	Branch if Overflow Set

BPL	Branch if Result Plus (N Clear)

BMI	Branch if Result Minus (N Set)

BGE	Branch if Greater Or Equal (to Zero?)	(S)

BLT	Branch if Less Than (Zero?)		(S)

BGT	Branch if Greater Than (Zero?)		(S)

BLE	Branch if Less Than or Equal (to Zero?)	(S)

Synonyms supported by some assemblers include:

BHS (Branch if Higher Than or Same) for BCC	(U)

BLO (Branch if Lower Than) for BCS		(U)

Also (Not Approved By Motorola) :

BBS (Branch on Bit Set) for BNE

BBC (Branch on Bit Clear) for BEQ

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 1 0 C C C C D D D D D D D D

16-bit 2s complement displacement extension

word follows if DDDD... field is zero.

Instruction Fields :

CCCC	: Condition field. Specifies the conditional

expression to compute prior to the branch

decision. Values are given in the condition-

al branch table at the end of this file.

DDDD..	: Displacement field. For short (Bcc.S) branch

instructions, this field contains the signed

(2s complement) 8-bit displacement to be added

onto the PC if the branch condition holds true.

For word-sized displacements, the DDDD... field

is zero, and the 16-bit signed (2s complement)

displacement follows the Bcc instruction word

as an extension word. This extension word ONLY

exists if the DDDD... field is zero.

NOTE : A short branch to the instruction immediately follow-

ing the Bcc instruction CANNOT exist. This is because

the resulting short branch would be zero, and the

68000 would treat this as a signal to access the next

program word as a displacement for a word-sized cond-

itional branch. This would possibly result in an add-

ress error exception caused by the next instruction

word being odd, and the resulting PC value upon the

branch execution being odd. Most assemblers issue a

warning about such branches.

BCHG	: Test Bit And Change

Operation : NOT ( dst{N} ) -> Z

NOT ( dst{N} ) -> dst{N}

Assembler : BCHG Dn,<ea>

BCHG #nnnn,<ea>

Attributes:size=Byte,Long

Description	: Copy a bit of the destination operand into 2

temporary locations within the 68000 ALU.

invert copy 1, and write the result to the Z

bit of the CCR. Then invert copy 2, and write

the result back into the bit position of the

destination whence it originated, thus changing

the value of the desired bit.

If a data register is the destination, then the

operation is a LONG operation, and the bit pos-

ition is treated as a modulo 32 value, allowing

access to all bits of the register.

If a memory operand is the destination, then the

operation is a BYTE operation, and the bit pos-

ition is treated as a modulo 8 value, allowing

access to all 8 bits of the BYTE addressed oper-

and.

Condition Codes :	X N Z V C

- - * - -

N Not Affected.

Z Set if the bit tested is ZERO.

Cleared otherwise.

V Not Affected.

C Not Affected.

X Not affected.

Instruction Format (Bit Number in Data Register) :

0 0 0 0 R R R 1 0 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers from which the bit

position is extracted.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

Instruction Format (Bit Number Immediate) :

0 0 0 0 1 0 0 0 0 1 E E E A A A

Extension Word, Bit Position as Immediate Data

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BCLR	: Test Bit And Clear

Operation : NOT ( dst{N} ) -> Z

0 -> dst{N}

Assembler : BCLR Dn,<ea>

BCLR #nnnn,<ea>

Attributes:size=Byte,Long

Description	: Copy a bit of the destination operand into 2

temporary locations within the 68000 ALU.

invert copy 1, and write the result to the Z

bit of the CCR. Then clear copy 2, and write

the result back into the bit position of the

destination whence it originated, thus clear-

ing the desired bit.

If a data register is the destination, then the

operation is a LONG operation, and the bit pos-

ition is treated as a modulo 32 value, allowing

access to all bits of the register.

If a memory operand is the destination, then the

operation is a BYTE operation, and the bit pos-

ition is treated as a modulo 8 value, allowing

access to all 8 bits of the BYTE addressed oper-

and.

Condition Codes :	X N Z V C

- - * - -

N Not Affected.

Z Set if the bit tested is ZERO.

Cleared otherwise.

V Not Affected.

C Not Affected.

X Not affected.

Instruction Format (Bit Number in Data Register) :

0 0 0 0 R R R 1 1 0 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers from which the bit

position is extracted.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

Instruction Format (Bit Number Immediate) :

0 0 0 0 1 0 0 0 1 0 E E E A A A

Extension Word, Bit Position as Immediate Data

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BKPT	: Breakpoint Instruction (68010)

Operation : Acknowledge breakpoint, then TRAP via

illegal instruction exception.

Assembler : BKPT #<data>

Attributes:Unsized

Description	: This instruction causes the 68010 to run a

breakpoint acknowledge cycle, with zeros on

all address lines. Following the termination

of the breakpoint acknowledge cycle, the

processor then TRAPS via the illegal instruc-

tion exception.

This instruction provides hardware support for

debug monitors and real-time software emulators.

The exact operation of this instruction will be

implementation dependent.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 0 0 0 0 1 0 0 1 V V V

Instruction Fields :

VVV	: Vector field. Specifies the breakpoint

number. This can be used by various im-

plementations to cause different funct-

ions to be executed for different break-

point types.

BRA	: Branch Always. See Bcc Above.

BSET	: Test Bit And Set

Operation : NOT ( dst{N} ) -> Z

1 -> dst{N}

Assembler : BSET Dn,<ea>

BSET #nnnn,<ea>

Attributes:size=Byte,Long

Description	: Copy a bit of the destination operand into 2

temporary locations within the 68000 ALU.

invert copy 1, and write the result to the Z

bit of the CCR. Then set copy 2, and write

the result back into the bit position of the

destination whence it originated, thus set-

ting the desired bit.

If a data register is the destination, then the

operation is a LONG operation, and the bit pos-

ition is treated as a modulo 32 value, allowing

access to all bits of the register.

If a memory operand is the destination, then the

operation is a BYTE operation, and the bit pos-

ition is treated as a modulo 8 value, allowing

access to all 8 bits of the BYTE addressed oper-

and.

Condition Codes :	X N Z V C

- - * - -

N Not Affected.

Z Set if the bit tested is ZERO.

Cleared otherwise.

V Not Affected.

C Not Affected.

X Not affected.

Instruction Format (Bit Number in Data Register) :

0 0 0 0 R R R 1 1 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers from which the bit

position is extracted.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

Instruction Format (Bit Number Immediate) :

0 0 0 0 1 0 0 0 1 1 E E E A A A

Extension Word, Bit Position as Immediate Data

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BSR	: Branch to Subroutine

Operation : PC -> -(SP) ; PC + d -> PC

Assembler : BSR <label>

Attributes:size=Byte,Word

Description	: The long word address of the instruction that

immediately follows the BSR instruction is

pushed onto the system stack. The 2s complement

displacement part of the instruction is then

sign-extended to 32 bits, and added onto the

program counter.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 1 0 0 0 0 1 D D D D D D D D

16-bit 2s complement displacement word follows

if DDDD... field is zero.

Instruction Fields :

DDDD...	: 8-bit displacement field for short branch

(BSR.S). For 16-bit word-sized displacements

contained in an extension word, the DDDD...

field is zero.

NOTE : A short branch to the instruction immediately following

the BSR instruction CANNOT exist. This is because the

displacement is zero. A zero short displacement would

instruct the 68000 to treat the next instruction as a

word displacement for the BSR instruction, and if the

instruction word thus fetched was odd, would result in

an address error exception immediately after execution.

BTST	: Test Bit

Operation : NOT ( dst{N} ) -> Z

Assembler : BTST Dn,<ea>

BTST #nnnn,<ea>

Attributes:size=Byte,Long

Description	: Copy a bit of the destination operand into a

temporary location within the 68000 ALU.

invert the copy, and write the result to the Z

bit of the CCR.

If a data register is the destination, then the

operation is a LONG operation, and the bit pos-

ition is treated as a modulo 32 value, allowing

access to all bits of the register.

If a memory operand is the destination, then the

operation is a BYTE operation, and the bit pos-

ition is treated as a modulo 8 value, allowing

access to all 8 bits of the BYTE addressed oper-

and.

Condition Codes :	X N Z V C

- - * - -

N Not Affected.

Z Set if the bit tested is ZERO.

Cleared otherwise.

V Not Affected.

C Not Affected.

X Not affected.

Instruction Format (Bit Number in Data Register) :

0 0 0 0 R R R 1 0 0 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers from which the bit

position is extracted.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

Instruction Format (Bit Number Immediate) :

0 0 0 0 1 0 0 0 0 0 E E E A A A

Extension Word, Bit Position as Immediate Data

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

CHK	: Check Register Against Bounds

Operation : IF Dn<0 or Dn > src

THEN TRAP

Assembler : CHK <ea>,Dn

Attributes:size=Word

Description	: The low order word of the specified data register

is examined and compared to the upper bound (which

is specified in the <ea> specification), treated

as a 2s complement integer. If the register value

is less than zero, or greater than the upper bound

contained in the source operand, then the 68000

takes the CHK exception. Otherwise, instruction

execution proceeds normally.

Condition Codes	:	X N Z V C

- * U U U

N Set if Dn < 0,

Cleared if Dn > src,

Undefined otherwise.

Z Undefined.

V Undefined.

C Undefined.

X Not Affected.

Instruction Format :

0 1 0 0 R R R 1 1 0 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the data register

whose contents are checked.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL addressing modes

for CHK are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

CLR	: Clear an operand

Operation : 0 -> dst

Assembler : CLR <ea>

Attributes:size=Byte,Word,Long

Description	: The destination is cleared to all zero bits.

The size of the operation may by byte, word

or long.

Condition Codes	:	X N Z V C

- 0 1 0 0

N Always Cleared.

Z Always Set.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 0 0 1 0 S S E E E A A A

Instruction Fields :

SS	: Size field. Determines the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes.The LEGAL addressing modes

for CLR are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

CMP	: Compare

Operation : dst - src -> CCR (set condition code bits only)

Assembler : CMP <ea>,Dn

Attributes:size=Byte,Word,Long

Description	: Subtract the source operand from the destination

operand, but set the condition codes only. The

destination operand is NOT changed. The size of

the operand may be byte, word or long.

Condition Codes	:	X N Z V C

- * * * *

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Set if an overflow is generated.

Cleared otherwise.

C Set if a borrow is generated.

Cleared otherwise.

X Not Affected.

Instruction Format :

1 0 1 1 R R R M M M E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register.

MMM	: Op-mode field. Defined according to:

Byte	Word  Long	Operation

----	----  ----	---------

000	001   010	Dn - <ea>

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands.

NOTE : CMP An,Dn can only be either Word or Long.

CMPA is used when the destination is an

address register. CMPI is used when the

source is an immediate operand. CMPM is

used for memory to memory comparisons.

Most assemblers automatically make this

distinction.

CMPA	: Compare Address.

Operation : dst - src -> CCR (set condition code bits only)

Assembler : CMPA <ea>,An

Attributes:size=Word,Long

Description	: Subtract the source operand from the destination

operand, but set the condition codes only. The

destination operand is NOT changed. The size of

the operand may be word or long.

NOTE : All Word sized operands are sign extended

to 32 bits prior to the comparison oper-

ation.

Condition Codes	:	X N Z V C

- * * * *

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Set if an overflow is generated.

Cleared otherwise.

C Set if a borrow is generated.

Cleared otherwise.

X Not Affected.

Instruction Format :

1 0 1 1 R R R M M M E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register.

MMM	: Op-mode field. Defined according to:

011 : Word operation. The source operand

is sign extended to 32 bits and the

operation performed upon the entire

32 bits of the address register.

111 : Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands.

CMPI	: Compare Immediate

Operation : dst - immediate data -> CCR

(set condition code bits only)

Assembler : CMPI #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Subtract the source operand from the destination

operand, but set the condition codes only. The

destination operand is NOT changed. The size of

the operand may be byte, word or long.

Condition Codes	:	X N Z V C

- * * * *

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Set if an overflow is generated.

Cleared otherwise.

C Set if a borrow is generated.

Cleared otherwise.

X Not Affected.

Instruction Format :

0 0 0 0 1 1 0 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies size of operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. LEGAL addressing modes

for the destination operand are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

CMPM	: Compare Memory

Operation : dst - src -> CCR (set condition code bits only)

Assembler : CMPM (Ay)+,(Ax)+

Attributes:size=Byte,Word,Long

Description	: Subtract the source operand from the destination

operand, but set the condition codes only. The

destination operand is NOT changed. The size of

the operand may be byte, word or long.

Condition Codes	:	X N Z V C

- * * * *

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Set if an overflow is generated.

Cleared otherwise.

C Set if a borrow is generated.

Cleared otherwise.

X Not Affected.

Instruction Format :

1 0 1 1 X X X 1 S S 0 0 1 Y Y Y

Instruction Fields :

XXX	: Register Rx field. ALWAYS specifies the

destination register. Specifies an address

register A0-A7.

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

YYY	: Register Ry field. ALWAYS specifies the

source register. Specifies an address

register A0-A7.

DBcc	: Test Condition, Decrement and Branch

Operation : IF condition=FALSE

THEN

Dn - 1 -> Dn

IF Dn <> -1

THEN

PC + d -> PC

ELSE

PC + 2 -> PC

END IF

ELSE

PC + 2 -> PC

END IF

Assembler : DBcc Dn, <label>

Attributes:size=Word

Description	: A complex loop control instruction. The sequence

of operations is:

1) Test condition. If condition is TRUE,

fall through to next instruction.

2) If condition is FALSE, decrement the low

word of the data register used as the

counter.

3) If the resulting value of the counter is -1,

fall through to next instruction.

4) If counter value is not -1, sign-extend the

displacement to 32 bits, and add it onto

the current value of the PC.

The instruction should be read as "Decrement &

Branch UNTIL condition (is true)", as this best

summarises the operating logic.

Possible Instructions (S=signed test, U=unsigned test) :

DBT	Until True

DBF	Until False (Also written DBRA)

DBHI	Until Higher			(U)

DBLS	Until Lower or Same 		(U)

DBCC	Until Carry Clear		(U)

DBCS	Until Carry Set			(U)

DBNE	Until Not Equal (to Zero)

DBEQ	Until Equal (to Zero)

DBVC	Until Overflow Clear

DBVS	Until Overflow Set

DBPL	Until Result Plus (N Clear)

DBMI	Until Result Minus (N Set)

DBGE	Until Greater Or Equal (to Zero?)	(S)

DBLT	Until Less Than (Zero?)			(S)

DBGT	Until Greater Than (Zero?)		(S)

DBLE	Until Less Than or Equal (to Zero?)	(S)

Synonyms supported by some assemblers include:

BHS (Until Higher Than or Same) for BCC	(U)

BLO (Until Lower Than) for BCS		(U)

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 1 C C C C 1 1 0 0 1 R R R

16-bit 2s complement displacement follows as

extension word.

Instruction Fields :

CCCC	: Condition specifier field. See table of

condition code specifiers at the end of

this file.

RRR	: Register field. Specifies which data register

D0-D7 to use.

NOTES : When no terminating condition is required for a loop,

and the counter decrementing to -1 is the sole exit

condition required (as in PASCAL 'FOR' loop, etc.)

then DBF is used. Most assemblers allow the synonym

DBRA to be used.

Two methods of loop entry are available. Either the

program can fall into the loop, in which case the

loop will be executed N+1 times if the value of Dn

upon entry is N, or the program can branch direct to

the DBcc instruction, in which case, for a value of

N in Dn, the loop will be executed N times. The latter

method is preferred, as it exits consistently for a

counter value of 0 analogously to a PASCAL 'FOR' loop,

unless there are good reasons for using the former

method.

DIVS	: Signed Divide

Operation : dst / src -> dst

Assembler : DIVS <ea>,Dn

Attributes:size=Word

Description	: 	Divide the 32-BIT destination operand by the

16-BIT source operand, using SIGNED arith-

metic, Generate a 32-bit result formed from

two 16-bit quantities:

Low Word  : Quotient resulting from the

division.

High Word : Remainder resulting from the

division.

and then store the result in the destination

location.

The sign of the remainder is always the same

as the sign of the dividend unless the rem-

ainder is zero. Two special conditions can

arise:

1) Division by zero initiates exception

processing. The 68000 takes the Division

By Zero exception.

2) Overflow may be detected and set before

the completion of the instruction. If an

overflow is detected, the condition is

flagged, and the operands are unaffected.

Condition Codes	:	X N Z V C

- * * * 0

N Set if the QUOTIENT is negative.

Cleared otherwise.

Z Set if the QUOTIENT is zero.

Cleared otherwise.

Undefined if overflow detected.

V Set if an overflow is generated.

Cleared otherwise.

C Always Cleared.

X Not Affected.

Instruction Format :

1 0 0 0 R R R 1 1 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register D0-D7. This field ALWAYS

specifies the destination operand.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands EXCEPT

address register direct (i.e., DIVS An,Dn is NOT

allowed).

NOTE : Overflow occurs if the QUOTIENT is larger

than a 16-bit signed integer. So, for

example, DIVS #10,D0 will generate an

overflow if D0 contains a value greater

than 655,359 decimal.

DIVU	: Unsigned Divide

Operation : dst / src -> dst

Assembler : DIVU <ea>,Dn

Attributes:size=Word

Description	: 	Divide the 32-BIT destination operand by the

16-BIT source operand, using UNSIGNED arith-

metic, Generate a 32-bit result formed from

two 16-bit quantities:

Low Word  : Quotient resulting from the

division.

High Word : Remainder resulting from the

division.

and then store the result in the destination

location.

Two special conditions can arise:

1) Division by zero initiates exception

processing. The 68000 takes the Division

By Zero exception.

2) Overflow may be detected and set before

the completion of the instruction. If an

overflow is detected, the condition is

flagged, and the operands are unaffected.

Condition Codes	:	X N Z V C

- * * * 0

N Set if the most significant bit of the

QUOTIENT is set. Cleared otherwise.

Z Set if the QUOTIENT is zero.

Cleared otherwise.

Undefined if overflow detected.

V Set if an overflow is generated.

Cleared otherwise.

C Always Cleared.

X Not Affected.

Instruction Format :

1 0 0 0 R R R 0 1 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register D0-D7. This field ALWAYS

specifies the destination operand.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands EXCEPT

address register direct (i.e., DIVU An,Dn is NOT

allowed).

NOTE : Overflow occurs if the QUOTIENT is larger

than a 16-bit unsigned integer. So, for

example, DIVS #10,D0 will generate an

overflow if D0 contains a value greater

than 655,359 decimal.

EOR	: Logical Exclusive-OR operation

Operation : src EOR dst -> dst

Assembler : EOR	Dn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (bitwise) Exclusive-OR the source

operand with the destination operand, and

store the result in the destination location.

THIS OPERATION IS RESTRICTED TO DATA REGISTERS

AS THE SOURCE OPERAND! The destination operand

is specified in the effective address field.

The size of the operation may be specified to

be byte, word or long.

Condition Codes :	X N Z V C

- * * 0 0

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not affected.

Instruction Format :

1 0 1 1 R R R M M M E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers.

MMM	: Op-Mode field. Specifies operation

according to:

Byte	Word  Long	   Operation

----	----  ----	   ---------

100	101   110	  EOR Dn,<ea>

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are (for

the destination operand only):

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTES : 1) Memory to data register operations are NOT

allowed with this binary form of the instruc-

tion. There are other binary forms, allowing

EOR of immediate data with data registers.

See NOTE 2 below and EORI instruction.

2) EORI is used when the source is immediate data.

Most assemblers automatically make this distinc-

tion.

EORI	: Logical Exclusive-OR immediate

Operation : immediate data EOR dst -> dst

Assembler : EORI #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (bitwise) Exclusive-OR the immediate

data with the destination operand, and store

the result in the destination location. The

size of the operand may be specified to be

byte, word or long. The immediate data matches

the operation size.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Word : 2nd word of operand.

Long : 2nd and 3rd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

- * * 0 0

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not affected.

Instruction Format :

0 0 0 0 1 0 1 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are (for

the destination operand only):

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

EORI	: Exclusive OR immediate to Condition Code Register

Operation : immediate data EOR CCR -> CCR

Assembler : EORI #nnnn,CCR

Attributes:size=Byte

Description	: Logically (bitwise) Exclusive-OR the immediate

data with the contents of the Condition Code

Register, and store the result in the Condition

Code Register (low order byte of the Status Reg-

ister).

Condition Codes :	X N Z V C

* * * * *

N Inverted if Bit 3 of immediate operand

is 1. Unchanged otherwise.

Z Inverted if Bit 2 of immediate operand

is 1. Unchanged otherwise.

V Inverted if Bit 2 of immediate operand

is 1. Unchanged otherwise.

C Inverted if Bit 0 of immediate operand

is 1. Unchanged otherwise.

X Inverted if Bit 4 of immediate operand

is 1. Unchanged otherwise.

Instruction Format :

0 0 0 0 1 0 1 0 0 0 1 1 1 1 0 0

0 0 0 0 0 0 0 0 B B B B B B B B

Instruction Fields :

BBB..	:	Byte immediate data field. This field

contains the 8-bit immediate data.

EORI	: Exclusive OR immediate to Status Register

(PRIVILEGED INSTRUCTION)

Operation : IF 68000 in supervisor state

THEN

immediate data EOR SR -> SR

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : EORI #nnnn,SR

Attributes:size=Byte

Description	: Logically (bitwise) Exclusive-OR the immediate

data with the contents of the Status Register,

and store the result in the Status Register.

Condition Codes :	X N Z V C

* * * * *

N Inverted if Bit 3 of immediate operand

is 1. Unchanged otherwise.

Z Inverted if Bit 2 of immediate operand

is 1. Unchanged otherwise.

V Inverted if Bit 2 of immediate operand

is 1. Unchanged otherwise.

C Inverted if Bit 0 of immediate operand

is 1. Unchanged otherwise.

X Inverted if Bit 4 of immediate operand

is 1. Unchanged otherwise.

Instruction Format :

0 0 0 0 1 0 1 0 0 1 1 1 1 1 0 0

W W W W W W W W W W W W W W W W

Instruction Fields :

WWW...	:	Word data instruction field. The

immediate data is contained in

this field.

EXG	: Exchange Registers

Operation : Rx <-> Ry

Assembler : EXG Ry,Rx

Attributes:size=Long

Description	: Exchange the contents of the two registers

specified. This exchange is ALWAYS a Long

operation (all 32 bits of the register are

exchanged). The exchange works in three

modes:

1) Data register to Data register.

2) Address register to Address register.

3) Data register to Address register.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 0 0 X X X 1 M M M M M Y Y Y

Instruction Fields :

XXX	: Register Rx field. Specifies a data

register or an address register de-

pending upon the mode. If the exchange

is between a data register and an

address register, this field ALWAYS

specifies the DATA register.

MMMMM	: Op-Mode field. Specifies whether exchanging:

01000	: Data Registers.

01001 : Address Registers.

10001 : Data Register and Address Register.

YYY	: Register Ry field. Specifies a data

register or an address register de-

pending upon the mode. If the exchange

is between a data register and an

address register, this field ALWAYS

specifies the ADDRESS register.

EXT	: Sign Extend

Operation : dst.B -> dst.W

or

dst.W -> dst.L

Assembler : EXT Dn

Attributes:size=Word,Long

Description	: Take the sign bit of the operand, and copy the

value of this bit into higher bits of the operand

according to the size/attribute specification of

the instruction. If EXT.W is specified, take bit

7 of the operand, and copy its value into bits 8

to 15 (word-sized sign extension). If EXT.L is

specified, take bit 15 of the operand, anc copy

its value into bits 16-31 (long word sized sign

extension).

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 1 0 0 M M M 0 0 0 R R R

Instruction Fields :

MMM	: Op-mode field. Defined according to:

010 : EXT.W (Sign extend byte to word)

011 : EXT.L (Sign extend word to longword)

RRR	: Register field. Specifies the destination

data register.

ILLEGAL	: Illegal instruction

Operation : PC -> -(SSP) ;

SR -> -(SSP) ;

(Illegal Instruction Vector) -> PC

Assembler : ILLEGAL

or

DC.W $4AFC

Attributes:none

Description	: This bit pattern will ALWAYS generate an illegal

instruction exception on Motorola 680x0 processors.

All other bit patterns that currently result in an

illegal instruction exception on current proces-

sors are reserved for future extension of the in-

struction set on next-generation members of the

680x0 family.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 0 1 0 1 1 1 1 1 1 0 0

JMP	: Jump

Operation : dst -> PC

Assembler : JMP <ea>

Attributes:Unsized

Description	: Take the value of the destination operand

and load it into the program counter. Pro-

gram execution continues from this new ad-

dress onwards.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values for the

EEE/AAA fields is listed in the section describing

the addressing modes. LEGAL addressing modes for

JMP are:

(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

JSR	: Jump to subroutine

Operation : PC -> -(SP)

dst -> PC

Assembler : JSR <ea>

Attributes:Unsized

Description	: The long word address of the instruction

immediatedly following the JSR is pushed onto

the system stack. Program execution then

continues at the address specified in the

instruction.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 1 0 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values for the

EEE/AAA fields is listed in the section describing

the addressing modes. LEGAL addressing modes for

JSR are:

(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

LEA	: Load Effective Address

Operation : dst -> An

Assembler : LEA <ea>,An

Attributes:size=Long

Description	: The effective address itself, NOT its contents,

are copied into the specified address register.

All 32 bits of the address register ar affected

by this operation.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0  R R R 1 1 1  E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

address register.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The possible LEGAL add-

ressing modes for LEA are:

(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

LINK	: Link And Allocate Workspace

Operation : An -> -(SP)

SP -> An

SP + d -> SP

Assembler : LINK An,#<displacement>

Attributes:Unsized

Description	: The current contents of the specified address

register is pushed onto the stack. After the

push, the updated stack pointer contents are

loaded into the specified address register.

Finally, the 16-bit signed displacement is

added onto the stack pointer. The contents of

the address register occupies two words on the

stack. A positive displacement is used to

access parameters pushed onto the stack (as in

code generated by C compilers), and a negative

displacement reserves workspace for local var-

iables.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 0 1 0 R R R

D D D D D D D D D D D D D D D D

Instruction Fields :

RRR	: Register field. Specifies the destination

address register.

DDD...	: Displacement field. Contains the 16-bit

value ot be sign-extended and added on to

the stack pointer.

LSL,LSR	: Logical Shift

Operation : dst (shifted by count) -> dst

Assembler : LSd	Dx,Dy

LSd #nnnn,Dy

LSd <ea>

Attributes:size=Byte,Word,Long

Description	: Logically shift the bits of the destination op-

erand in the direction specified. The carry bit

receives the last bit shifted out of the operand.

The shift count for the operation can be speci-

fied in two ways:

1) Immediate. The shift count value is 1-8.

2) Data Register. The value contained in the

specified data register, modulo 64, is

used.

If the destination operand is a memory operand,

then the shift count is ALWAYS 1, and the operand

size is ALWAYS word sized.

LSL : Operand is shifted left by the value of the

shift count. Bits shifted out of the high

order bit are copied to the C and X bits of

the CCR. Zero bits are shifted into the va-

cated low order bits.

LSR : Operand is shifted right by the value of the

shift count. Bits shifted out of the low order

bit are copied to the C and X bit of the CCR.

Zero bits are shifted into the vacated high

order bits.

Condition Codes :	X N Z V C

* * * 0 *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Set according to the last bit shifted out

of the operation. Cleared for a shift count

of zero.

X Set according to the last bit shifted out

of the operand. UNAFFECTED for a shift

count of zero!

Instruction Format (Register Shifts) :

1 1 1 0 C C C D S S L 0 1 R R R

Instruction Fields :

CCC	: Count/Register field. If immediate count,

CCC = 1-7 for a shift count of 7, 0 for a

shift count of 8. If data register count,

CCC = data regsiter number for register Dx.

The value contained in bits 0-5 of Dx is

used as the shift count (i.e., Dx modulo

64).

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

L	: L/R field. Specifies shift count type.

0 = immediate shift count in CCC field.

1 = data register count, register in CCC field.

RRR	: Register field. Specifies any of the 8 data

registers whose content is to be shifted.

Instruction Format (Memory Shifts) :

1 1 1 0 0 0 1 D 1 1 E E E A A A

Instruction Fields :

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

MOVE	: Move Data From Source To Destination.

Operation : src -> dst

Assembler : MOVE <ea>,<ea>

Attributes:size=Byte,Word,Long

Description	: Move the contents of the source effective

address to the location specified by the

destination effective address. The data is

examined as it is moved, and the condition

codes set accordingly. The operation may be

specified to be a byte, word or long word

operation.

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 0 S S X X X M M M N N N Y Y Y

Instruction Fields :

SS	: Size field. Specifies the size of operand

to be moved.

01 = Byte operation.

11 = Word operation.

10 = Long operation.

XXX, MMM : Destination effective address field.

See the Effective Address Specification

section below for values, where MMM=EEE,

XXX=AAA.

YYY, NNN : Source effective address field.

See the Effective Address Specification

section below for values, where NNN=EEE,

YYY=AAA

The LEGAL addressing modes for this instruction are:

Source Effective Address:All addressing modes (except

for byte sized operation where destination is An).

Destination Effective Address:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTE : MOVEA is used when the destination is an address

register. Most assemblers automatically make this

distinction.

MOVEQ can also be used for loading certain immediate

constant values into data registers.

MOVE	: Move Data From Condition Code Register to Destination

(68010 upwards - NOT 68000).

Operation : CCR -> dst

Assembler : MOVE CCR,<ea>

Attributes:size=Word

Description	: Move the contents of the status register to

the destination effective address. The source

operand is a word, but only the low order byte

contains the condition codes. The upper byte

is set to all zeros.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 0 0 1 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The possible LEGAL add-

ressing modes for LEA are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTE : MOVE from CCR is a word operation.

MOVE	: Move Data From Source To Condition Code Register

Operation : src -> CCR

Assembler : MOVE <ea>,CCR

Attributes:size=Word

Description	: Move the contents of the soure effective ad-

dress to the condition code register. The

source operand is a word, but only the low

order byte is used to update the condition

codes. The upper byte is ignored.

Condition Codes	: X N Z V C

* * * * *

N Set to the same value as bit 3 of the

source operand.

Z Set to the same value as bit 2 of the

source operand.

V Set to the same value as bit 1 of the

source operand.

C Set to the same value as bit 0 of the

source operand.

X Set to the same value as bit 4 of the

source operand.

Instruction Format :

0 1 0 0 0 1 0 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All addressing modes

except address register direct are legal (i.e.,

MOVE An,CCR is illegal).

NOTE : MOVE to CCR is a word operation. AND, OR

and EOR to CCR is a byte operation.

MOVE	: Move Data From Status Register to Destination

(68000 Version)

Operation : SR -> dst

Assembler : MOVE SR,<ea>

Attributes:size=Word

Description	: Move the contents of the status register to

the destination effective address. The source

operand is a word.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 0 0 0 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The possible LEGAL add-

ressing modes for this instruction are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTE : A memory destination is READ before it is

WRITTEN to.

MOVE	: Move Data From Status Register to Destination

(68010 Version - PRIVILEGED INSTRUCTION).

Operation : If 68000 in supervisor state

THEN

SR -> dst

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : MOVE SR,<ea>

Attributes:size=Word

Description	: Move the contents of the status register to

the destination effective address. The source

operand is a word.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 0 0 0 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The possible LEGAL add-

ressing modes for this instruction are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTE : On 68010/68020 etc., use MOVE from CCR to

access the condition codes.

MOVE	: Move Data From Source To Status Register

(PRIVILEGED INSTRUCTION)

Operation : IF 68000 in supervisor state

THEN

src -> SR

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : MOVE <ea>,SR

Attributes:size=Word

Description	: Move the contents of the soure effective ad-

dress to the status register. The operation

is word sized and all bits of the status

register are affected.

Condition Codes	: X N Z V C

* * * * *

N Set to the same value as bit 3 of the

source operand.

Z Set to the same value as bit 2 of the

source operand.

V Set to the same value as bit 1 of the

source operand.

C Set to the same value as bit 0 of the

source operand.

X Set to the same value as bit 4 of the

source operand.

Instruction Format :

0 1 0 0 0 1 1 0 1 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All addressing modes

except address register direct are legal (i.e.,

MOVE An,CCR is illegal).

MOVE	: Move User Stack Pointer

(PRIVILEGED INSTRUCTION)

Operation : IF 68000 in supervisor state

THEN

USP -> An

or

An -> USP

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : MOVE USP,An

MOVE An,USP

Attributes:size=Long

Description	: Transfer the contents of the User Stack Pointer

to or from the specified address register.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 0 D R R R

Instruction Fields :

D	: Direction field. Specifies the direction

of transfer.

0 = MOVE An,USP

1 = MOVE USP,An

RRR	: Register field. Specifies the address register

to move to/from.

MOVEA	: Move Address

Operation : src -> An

Assembler : MOVE <ea>,An

Attributes:size=Word,Long

Description	: Move the contents of the source effective address

to the speficied destination address register. If

a word sized operation is specified, the operand

is sign extended to a full 32-bit operand before

the transfer. All 32 bits of the destination ad-

dress register are affected by both word and long

sized operations.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 0 S S R R R 0 0 1 E E E A A A

Instruction Fields :

SS	: Size field. Specifies the size of operand

to be moved.

11 = Word operation. The source operand is

sign-extended to a long operand and all

32 bits are loaded into the address

register.

10 = Long operation.

RRR	: Destination Register field. Specifies the

destination address register to use.

EEE	: Source Effective Address field.

AAA	: Source Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All addressing modes are

legal.

MOVEC	: Move To/From Control Register (PRIVILEGED INSTRUCTION)

(68010 onwards-NOT 68000)

Operation : If 68000 in supervisor state

THEN

Rc -> Rn

or

Rn -> Rc

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : MOVEC Rc,Rn

MOVEC Rn,Rc

Attributes:size=Long

Description	: Copy the contents of the speficied control

register to the specified general register,

or copy the contents of the specified general

register to the specified control register.

This is ALWAYS a 32-bit transfer even though

the specified control register may be imple-

mented with fewer bits. Unimplemented bits are

read as zeros, and ignored for write operations.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 1 0 1 D

A R R R C C C C C C C C C C C C

Instruction Fields :

D	:	Direction field. Specifies the direction

of data transfer.

0 = MOVEC Rc,Rn

1 = MOVEC Rn,Rc

A	:	A/D field. Specifies the type of general

register.

0 = Data register.

1 = Address register.

CCC...	:	Control Register field. Specifies the control

register number. Currently defined control

registers for the 68010 are:

Binary	Hex	Register

------	---	--------

0000 0000 0000	000	SFC : Source

function code

register

0000 0000 0001	001	DFC : Destination

function code

register

1000 0000 0000	800	User Stack Pointer

(USP).

1000 0000 0001	801	VBR : Vector Base

register.

All other codes cause an illegal instruction

exception.

MOVEM	: Move Multiple Registers

Operation : src -> Registers

Registers -> dst

Assembler : MOVEM Reglist,<ea>

MOVEM <ea>,Reglist

Attributes:size=Word,Long

Description	: Selected registers are transferred to or from

consecutive memory locations, starting at the

location specified by the effective address. A

register is transferred if the corresponding bit

in the register mask field is set. Either the

entire 32 bits of the specified registers can

be transferred or the low 16 bits.

If the transfer is FROM memory TO the

registers, then word-sized transfers are sign-

extended to 32 bits prior to transfer. THIS IS

ALSO TRUE FOR DATA REGISTERS!

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 D 0 0 1 S E E E A A A

M M M M M M M M M M M M M M M M

Instruction Fields :

D	:	Direction field. Sprcifies the direction

of the transfer.

0 = register to memory.

1 = memory to register.

S	:	Size field. Specifies the size of operands

being transferred.

0 = word transfer. Word operands are sign-

extended to 32 bits and the entire

register loaded if a memory to register

transfer is being performed.

1 = long transfer.

EEE	: Effective Address field.

AAA	: Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. Legal addressing modes for

MOVEM are:

Register to memory:

(An)

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

Memory to register:

(An)

(An)+

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

MMM...	:	Register list mask field. Specifies which

registers are to be transferred. The mask

register for predecrement addressing mode

(-(An) mode) is:

D0 D1 D2 D3 D4 D5 D6 D7 A0 A1 A2 A3 A4 A5 A6 A7

while for all other modes the mask register

is:

A7 A6 A5 A4 A3 A2 A1 A0 D7 D6 D5 D4 D3 D2 D1 D0

NOTE : An extra read bus cycle occurs for memory operands.

This amounts to a memory word at one address higher

than expected being addressed during operation.

MOVEP	: Move Peripheral Data

Operation : src -> dst

Assembler : MOVEP Dx,d(Ay)

MOVEP d(Ay),Dx

Attributes:size=Word,Long

Description	: Data is transferred between a data register

and alternate bytes of memory, starting at the

location specified and incrementing by two. The

high order byte of the data register is trans-

ferred first, followed by the low order bytes in

descending order, the lowest order byte being

transferred last. If the effective address is

even, transfer is made on the high half of the

data bus:if the effective address is odd, then

transfer is made on the low half of the data bus.

Example:Long transfer to/from an EVEN address

Byte Organisation In Register

HI  MIDHI  MIDLO  LO

Byte Organisation In Memory

(Lowest Address At Top)

Even		Odd

HI		---	ADDR

MIDHI		---	ADDR+2

MIDLO		---	ADDR+4

LO		---	ADDR+6

Example:word transfer to/from an ODD address

Byte Organisation In Register

HI  LO

Byte Organisation In Memory

(Lowest Address At Top)

Even		Odd

---		HI	ADDR

---		LO	ADDR+2

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 0 0 0 R R R M M M 0 0 1 A A A

D D D D D D D D D D D D D D D D

Instruction Fields :

RRR	: Data Register field. Specifies the data

register Dx to use.

AAA	: Address register field. Specifies the

address register Ay to use.

MMM	: Op-Mode field. Specifies the direction and

the size of the operation.

100 : MOVEP.W d(Ay),Dx

101 : MOVEP.L d(Ay),Dx

110 : MOVEP.W	Dx,d(Ay)

111 : MOVEP.L Dx,d(Ay)

DDD...	: Displacement field. Specifies the displacement

d to use in d(Ay).

MOVEQ	: Move Quick

Operation : Immediate Data -> Dn

Assembler : MOVEQ #nnn,Dn

Attributes:size=Long

Description	: Move the contents of the immediate field to a

data register. The data is contained in an 8-bit

field in the instruction word itself. The data is

sign-extended to a long operand and all 32 bits

are transferred to the data register.

Condition Codes	: X N Z V C

- * * 0 0

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 1 1 R R R 0 D D D D D D D D

Instruction Fields :

RRR	:	Register field. Specifies which data

register Dn to use.

DDD...	:	Data field. These 8 bits are the data

constant to use, and are sign-extended

to a long operand before transfer.

MOVES	: Move To/From Address Space (PRIVILEGED INSTRUCTION)

(68010 onwards-NOT 68000)

Operation : If 68000 in supervisor state

THEN

src <SFC>  -> Rn

or

Rn -> dst <DFC>

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : MOVES <ea>,Rn

MOVES Rn,<ea>

Attributes:size=Byte,Word,Long

Description	: Move the byte, word or long word operand from the

specified general register to a location within

the address space specified by the destination

function code register (DFC). Or move the byte,

word or long word operand from a location within

the address space specified by the source function

code register (SFC) to the specified general regis-

ter.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 S S E E E A A A

G R R R D 0 0 0 0 0 0 0 0 0 0 0

Instruction Fields :

SS	:	Size field. Specifies the size of the

operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

G	:	General register field. Specifies the

type of general register to use.

0 = Data register.

1 = Address register.

RRR	:	Register number field. Specifies which data

or address register Rn to use.

D	:	Direction field. Specifies the direction of

the transfer.

0 = MOVES <ea>,Rn

1 = MOVES Rn,<ea>

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The legal modes are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

MULS	: Signed Multiply

Operation : dst * src -> dst

Assembler : MULS <ea>,Dn

Attributes:size=Word

Description	: 	Multiply the signed 16-bit source operand by

the signed 16-bit destination operand, and

store the 32-bit signed operand in the dest-

ination data register. If the source operand

is a data register, then the low 16 bits are

used:the ligh 16 bits are ignored.

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 0 0 R R R 1 1 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register D0-D7. This field ALWAYS

specifies the destination operand.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands EXCEPT

address register direct (i.e., MULS An,Dn is NOT

allowed).

MULU	: Unsigned Multiply

Operation : dst * src -> dst

Assembler : MULU <ea>,Dn

Attributes:size=Word

Description	: 	Multiply the signed 16-bit source operand by

the signed 16-bit destination operand, and

store the 32-bit signed operand in the dest-

ination data register. If the source operand

is a data register, then the low 16 bits are

used:the ligh 16 bits are ignored.

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the most significant bit of the

result is set. Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 0 0 R R R 0 1 1 E E E A A A

Instruction Fields :

RRR	: Register field. Specifies the destination

data register D0-D7. This field ALWAYS

specifies the destination operand.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. All possible 68000 add-

ressing modes are legal for source operands EXCEPT

address register direct (i.e., MULU An,Dn is NOT

allowed).

NBCD	: Negate Decimal With Extend.

Operation : 0 -  dst(10) - X -> dst

Assembler :	NBCD	<ea>

Attributes:size=Byte

Description	: The operand addressed at the destination and the

current value of the extend (X) bit of the CCR are

subtracted from zero. The operation is performed

using Binary Coded Decimal (BCD) arithmetic. The

result is saved at the destination location. This

instruction produces the 10s complement of the

destination if the extend bit is clear, the 9s

complement if the extend bit is set. This is a byte

operation only.

Condition Codes	:	X N Z V C

* U * U *

N Undefined

Z Cleared if nonzero result.

UNCHANGED otherwise!

V Undefined

C Set if DECIMAL borrow generated.

Cleared otherwise.

X Set identically to C.

Notes		: Normally, the Z bit is set via programming

(this is the PROGRAMMER'S responsibility)

before the start of an NBCD operation. This

allows successful tests upon completion of

multiple-precision NBCD operations.

Instruction Format:

0 1 0 0 1 0 0 0 0 0 E E E A A A

Instruction Fields:

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The legal modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NEG	: Negate an operand

Operation : 0 - dst -> dst

Assembler : NEG <ea>

Attributes:size=Byte,Word,Long

Description	: The destination is cleared to all zero bits.

The size of the operation may by byte, word

or long.

Condition Codes	:	X N Z V C

* * * * *

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Set if an overflow was generated.

Cleared otherwise.

C Cleared if the result is zero.

Set otherwise.

X Set identically to C.

Instruction Format :

0 1 0 0 0 1 0 0 S S E E E A A A

Instruction Fields :

SS	: Size field. Determines the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL addressing modes

for NEG are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NEGX	: Negate With Extend

Operation : 0 - dst - X -> dst

Assembler : NEGX <ea>

Attributes:size=Byte,Word,Long

Description	: The destination operand and the value of the

extend bit are subtracted from zero. The result

is stored at the destination effective address.

The size of the operation may be specified to be

byte, wor dor long.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Cleared if result is non-zero.

Unchanged otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary borrow is generated.

Cleared otherwise.

X Set identically to C.

NOTE : Normally the Z bit is set via programming

(e.g., using MOVEQ #0,D0) before the start

of the operation. This allows successful

tests for zero operands upon completion of

multiple-precision operations.

Instruction Format :

0 1 0 0 0 0 0 0 S S E E E A A A

Instruction Fields :

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL addressing modes

for NEG are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOP	: No Operation.

Operation : None

Assembler : NOP

Attributes:Unsized

Description	: No operation occurs. The processor state,

other than the program counter, is unchanged.

Execcution continues with the instruction

immediately following the NOP.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 0 1

NOT	: Logical complement.

Operation : ~dst -> dst

Assembler : NOT <ea>

Attributes:size=Byte,Word,Long

Description	: The ones complement of the destination operand

is computed, and the result stored at the dest-

ination effective address. The size of the oper-

ation may be specified to be byte, word or long.

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the result is negative.

Cleared otherwise.

Z Set if the result is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 0 1 1 0 S S E E E A A A

Instruction Fields :

SS	: Size field. Determines the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL addressing modes

for NEG are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

OR	: Logical OR operation

Operation : src OR dst -> dst

Assembler : OR	<ea>,Dn

OR	Dn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (bitwise) OR the source operand

with the destination operand, and store the

result in the destination location.

Condition Codes :	X N Z V C

- * * 0 0

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not affected.

Instruction Format :

1 0 0 0 R R R M M M E E E A A A

Instruction Fields :

RRR	: Register field. Specifies any of the

8 data registers.

MMM	: Op-Mode field. Specifies operation

according to:

Byte	Word  Long	   Operation

----	----  ----	   ---------

000	001   010	  OR <ea>,Dn

100	101   110	  OR Dn,<ea>

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Source Operand:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

#nnnn

Destination Operand:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

NOTES : 1) If the destination is a data register, then

it CANNOT be specified using destination <ea>

addressing mode, but MUST use the destination

Dn mode above instead.

2) ORI is used when the source is immediate data.

Most assemblers automatically make this distinc-

tion.

ORI	: Logical OR immediate

Operation : immediate data OR dst -> dst

Assembler : ORI  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Logically (Bitwise) OR the immediate data

contained in the instruction to the destin-

ation operand, and store the result in the

destination location.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Word : 2nd word of operand.

Long : 2nd and 3rd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

- * * 0 0

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 0 0 0 0 0 0 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

ORI	: Logical OR immediate to CCR

Operation : immediate data OR CCR -> CCR

Assembler : ORI  #nnnn,CCR

Attributes:size=Byte

Description	: Logically (Bitwise) OR the immediate data

contained in the instruction to the contents

of the Condition Code Register, and store the

result in the Condition Code Register (low 8

bits of Status Register).

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Set if Bit 3 of the operand is 1.

Unchanged otherwise.

Z Set if Bit 2 of the operand is 1.

Unchanged otherwise.

V Set if Bit 1 of the operand is 1.

Unchanged otherwise.

C Set if Bit 0 of the operand is 1.

Unchanged otherwise.

X Set if Bit 4 of the operand is 1.

Unchanged otherwise.

Instruction Format :

0 0 0 0 0 0 0 0 0 0 1 1 1 1 0 0

0 0 0 0 0 0 0 0 B B B B B B B B

Instruction Fields :

BBB...	:	Byte Data field. Contains the 8-bit data

with which the CCR is ORed.

ORI	: Logical OR immediate to SR (PRIVILEGED INSTRUCTION)

Operation : If 68000 in supervisor state

THEN

immediate data OR SR -> SR

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : ORI  #nnnn,SR

Attributes:size=Word

Description	: Logically (Bitwise) OR the immediate data

contained in the instruction to the contents

of the Status Register, and store the result

in the Status Register.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Word : 2nd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Set if Bit 3 of the operand is 1.

Unchanged otherwise.

Z Set if Bit 2 of the operand is 1.

Unchanged otherwise.

V Set if Bit 1 of the operand is 1.

Unchanged otherwise.

C Set if Bit 0 of the operand is 1.

Unchanged otherwise.

X Set if Bit 4 of the operand is 1.

Unchanged otherwise.

Instruction Format :

0 0 0 0 0 0 0 0 0 1 1 1 1 1 0 0

W W W W W W W W W W W W W W W W

Instruction Fields:

WWW...	:	Word data instruction field. The

immediate data is contained in

this field.

PEA	: Push Effective Address

Operation : dst -> -(SP)

Assembler : PEA <ea>

Attributes:size=Long

Description	: The effective address itself, NOT its contents,

are pushed onto the stack. A long word address

is pushed onto the stack.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 0 0 0 0 1 E E E A A A

Instruction Fields :

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The possible LEGAL add-

ressing modes for PEA are:

(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

d(PC)

d(PC,Xi)

RESET	: Reset External Devices (PRIVILEGED INSTRUCTION)

Operation : IF 68000 in supervisor state

THEN

Assert RESET line

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : RESET

Attributes:Unsized

Description	: The RESET line of the processor is asserted,

which causes all external devices to be reset.

The processor state, other than the program

counter, is unaffected and execution continues

with the next instruction.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 0 0

ROL,ROR	: Rotate Without Extend

Operation : dst (rotated by count) -> dst

Assembler : ROd	Dx,Dy

ROd #nnnn,Dy

ROd <ea>

Attributes:size=Byte,Word,Long

Description	: Rotate the bits of the destination operand in the

direction specified. The extend bit is not inclu-

ded in the rotation. The carry bit receives the

last bit shifted out of the operand. The shift

count for the operation can be specified in two

ways:

1) Immediate. The shift count value is 1-8.

2) Data Register. The value contained in the

specified data register, modulo 64, is

used.

If the destination operand is a memory operand,

then the shift count is ALWAYS 1, and the operand

size is ALWAYS word sized.

ROL : Operand is rotated left by the value of the

shift count. Bits shifted out of the high

order bit are copied to the C and X bits of

the CCR, and back into the low order bits.

Zero bits are shifted into the vacated low

order bits.

ROR : Operand is rotated right by the value of the

shift count. Bits shifted out of the low

order bit are copied to the C and X bit of

the CCR, and back into the high order bit.

Zero bits are shifted into the vacated high

order bits.

Condition Codes :	X N Z V C

- * * 0 *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Set according to the last bit shifted out

of the operation. Cleared for a shift count

of zero.

X Not Affected.

Instruction Format (Register Shifts) :

1 1 1 0 C C C D S S L 1 1 R R R

Instruction Fields :

CCC	: Count/Register field. If immediate count,

CCC = 1-7 for a shift count of 7, 0 for a

shift count of 8. If data register count,

CCC = data regsiter number for register Dx.

The value contained in bits 0-5 of Dx is

used as the shift count (i.e., Dx modulo

64).

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

L	: L/R field. Specifies shift count type.

0 = immediate shift count in CCC field.

1 = data register count, register in CCC field.

RRR	: Register field. Specifies any of the 8 data

registers whose content is to be shifted.

Instruction Format (Memory Shifts) :

1 1 1 0 0 1 1 D 1 1 E E E A A A

Instruction Fields :

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

ROXL,ROXR	: Rotate Without Extend

Operation : dst (rotated by count) -> dst

Assembler : ROd	Dx,Dy

ROd #nnnn,Dy

ROd <ea>

Attributes:size=Byte,Word,Long

Description	: Rotate the bits of the destination operand in the

direction specified. The extend bit IS included in

the rotation. The carry bit receives the last bit

shifted out of the operand. The shift count for the

operation can be specified in two ways:

1) Immediate. The shift count value is 1-8.

2) Data Register. The value contained in the

specified data register, modulo 64, is

used.

If the destination operand is a memory operand,

then the shift count is ALWAYS 1, and the operand

size is ALWAYS word sized.

ROXL : Operand is rotated left by the value of the

shift count. Bits shifted out of the high

order bit are copied to the C and X bits of

the CCR, and back into the low order bits.

The previous value of the X bit prior to

each single-position rotate is moved into

the vacated low-order bit.

ROXR : Operand is rotated right by the value of

the shift count. Bits shifted out of the low

order bit are copied to the C and X bit of

the CCR, and back into the high order bit.

The previous value of the X bit prior to

each single-position rotate is moved into

the vacated high-order bit.

Condition Codes :	X N Z V C

- * * 0 *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Always Cleared.

C Set according to the last bit shifted out

of the operation. UNAFFECTED for a shift

count of zero!

X Not Affected.

Instruction Format (Register Shifts) :

1 1 1 0 C C C D S S L 1 0 R R R

Instruction Fields :

CCC	: Count/Register field. If immediate count,

CCC = 1-7 for a shift count of 7, 0 for a

shift count of 8. If data register count,

CCC = data regsiter number for register Dx.

The value contained in bits 0-5 of Dx is

used as the shift count (i.e., Dx modulo

64).

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

SS	: Size field. Specifies the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

L	: L/R field. Specifies shift count type.

0 = immediate shift count in CCC field.

1 = data register count, register in CCC field.

RRR	: Register field. Specifies any of the 8 data

registers whose content is to be shifted.

Instruction Format (Memory Shifts) :

1 1 1 0 0 1 0 D 1 1 E E E A A A

Instruction Fields :

D	: Direction field. Specifies shift direction.

0 = right shift

1 = left shift

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

RTD	: Return and Deallocate Parameters

(68010 upwards - NOT 68000)

Operation : (SP)+ -> PC

SP + d -> SP

Assembler : RTD #<displacement>

Attributes:Unsized

Description	: The program counter is pulled from the stack.

The previous program counter value is lost.

After the program counter is read from the

stack, the displacement value is sign-extended

to 32 bits and added to the stack pointer.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 1 0 0

D D D D D D D D D D D D D D D D

Instruction Fields :

DDD...	:	Displacement field. The 16-bit displacement

is contained in this field.

RTE	: Return from Exception (PRIVILEGED INSTRUCTION)

(68000 version)

Operation : IF 68000 in supervisor state

THEN

(SP)+ -> SR

(SP)+ -> PC

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : RTE

Attributes:Unsized

Description	: The status register and program counter are

pulled from the system stack. The previous

status register and program counter are lost.

All bits in the status register are affected.

Condition Codes	: Set according to the content of the word

on the stack.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 1 1

RTE	: Return from Exception (PRIVILEGED INSTRUCTION)

(68010 upwards version)

Operation : IF 680x0 in supervisor state

THEN

(SP)+ -> SR

(SP)+ -> PC

IF (SP)+ = long format

THEN

full machine state restore

ELSE

do nothing

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : RTE

Attributes:Unsized

Description	: The status register and program counter are

pulled from the system stack. The previous

status register and program counter are lost.

All bits in the status register are affected.

The vector offset word is then pulled from

the stack, and the format field is examined

to determine the type of stack frame to pull

off the stack to restore the machine state.

Condition Codes	: Set according to the content of the word

on the stack.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 1 1

Vector Offset Word Format :

F F F F 0 0 V V V V V V V V V V

Vector Offset Word Format Fields :

FFFF	:	Format field. Specifies the amount of

information to pull from the stack to

restore the machine state.

0000 : Short frame. Four words are

pulled from the stack.

1000 : Long frame. 29 words are pulled

from the stack.

Any other value is an error on the

68010. The processor then takes the

format error exception.

RTR	: Return And Restore Condition Codes

Operation : (SP)+ -> CC

(SP)+ PC

Assembler : RTR

Attributes:Unsized

Description	: The condition codes and program counter are

pulled from the stack. The previous program

counter and condition codes are lost. The

supervisor portion of the status register

(the upper 8 bits) are unaffected.

Condition Codes	: Set according to the content of the word

on the stack.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 1 1 1

RTS	: Return From Subroutine

Operation : (SP)+ -> PC

Assembler : RTS

Attributes:Unsized

Description	: The program counter is pulled from the stack.

The previous value of the program counter is

lost.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 1 0 1

SBCD	: Subtract Decimal With Extend.

Operation : dst(10) - src(10) - X -> dst

Assembler :	SBCD	Dy,Dx

SBCD	-(Ay),-(Ax)

Attributes:size=Byte

Description	: Perform Binary Coded Decimal (BCD) subtraction

of the source and destination operands, together

with the Extend (X) bit of the CCR. The only

legal addressing modes are those given above.

This is a byte operation only.

Condition Codes	:	X N Z V C

* U * U *

N Undefined

Z Cleared if nonzero result.

UNCHANGED otherwise!

V Undefined

C Set if DECIMAL borrow generated.

Cleared otherwise.

X Set identically to C.

Notes		: Normally, the Z bit is set via programming

(this is the PROGRAMMER'S responsibility)

before the start of an SBCD operation. This

allows successful tests upon completion of

multiple-precision SBCD operations.

Instruction Format:

1 0 0 0 X X X 1 0 0 0 0 M Y Y Y

Instruction Fields:

XXX : Register Rx field. Specifies which register

is used for the destination operand, D0-D7

for ABCD Dy,Dx or A0-A7 for ABCD -(Ay),-(Ax)

YYY : Register Ry field. Specifies which register

is used for the source operand, D0-D7

for ABCD Dy,Dx or A0-A7 for ABCD -(Ay),-(Ax)

M   : R/M field. Specifies addressing mode.

0 : mode is ABCD Dy,Dx

1 : mode is ABCD -(Ay),-(Ax)

Scc	: Set according to condition codes

Operation : IF condition is TRUE

THEN

11111111 -> dst.B

ELSE

00000000 -> dst.B

Assembler : Scc <ea>

Attributes:size=Byte

Description	: The conditional expression specified in the

instruction is evaluated and the result tested.

If the result is TRUE, then the byte referred

to by the destination effective address is set

to the value 11111111 binary ($FF Hex). If the

result is FALSE, then the bye referred to by

the destination effective address is set to the

value 00000000 binary ($00 Hex). The values $FF

and $00 are taken to represent TRUE and FALSE

respectively.

Possible Instructions (U = unsigned test, S=signed test) :

ST	Set if True

SF	Set if False

SHI	Set if Higher			(U)

SLS	Set if Lower or Same 		(U)

SCC	Set on Carry Clear		(U)

SCS	Set on Carry Set		(U)

SNE	Set if Not Equal (to Zero)	(U and S)

SEQ	Set if Equal (to Zero)		(U and S)

SVC	Set if Overflow Clear

SVS	Set if Overflow Set

SPL	Set if Result Plus (N Clear)

SMI	Set if Result Minus (N Set)

SGE	Set if Greater Or Equal (to Zero?)	(S)

SLT	Set if Less Than (Zero?)		(S)

SGT	Set if Greater Than (Zero?)		(S)

SLE	Set if Less Than or Equal (to Zero?)	(S)

Synonyms supported by some assemblers include:

SHS (Set if Higher Than or Same) for SCC	(U)

SLO (Set if Lower Than) for SCS		(U)

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 1 C C C C 1 1 E E E A A A

Instruction Fields :

CCCC	: Condition field. Specifies the conditional

expression to compute prior to the branch

decision. Values are given in the condition-

al branch table at the end of this file.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

STOP	: Load Status Register and Stop Execution

(PRIVILEGED INSTRUCTION)

Operation : IF 680x0 in supervisor state

THEN

immediate data -> SR

HALT PROCESSOR

ELSE

TRAP EXCEPTION (PRIVILEGE VIOLATION)

Assembler : STOP #nnnn

Attributes:Unsized

Description	: The immediate data is moved into the entire

status register. The program counter is then

incremented to point to the next instruction

in the execution sequence immediately follow-

ing the STOP instruction, and then the pro-

cessor is placed in the HALT state, where the

fetching and execution of instructions ceases.

Execution resumes when a trace, interrupt or

reset exception occurs. A trace exception will

occur if the TRACE bit of the status register

is set prior to execution of the STOP instruc-

tion. If an interrupt request arrives whose

priority is greater than the current processor

IPLx priority, then an interrupt exception will

occur, otherwise the interrupt request has no

effect. If the bit of the immediate data cor-

responding to the S-bit of the status register

(the supervisor bit) is zero, execution of the

instruction will cause a privilege violation

exception even if the machine is currently in

the supervisor state. A reset exception (caused

by an external reset) will ALWAYS initiate the

processing of a reset exception.

Condition Codes	: Set according to the content of the immediate

data.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 1 0

D D D D D D D D D D D D D D D D

Instruction Fields :

DDD...	:	Immediate data field. The immediate data

to be loaded into the status register is

found in this instruction word.

SUB	: Subctract Binary.

Operation : dst - src -> dst

Assembler : SUB <ea>,Dn

SUB Dn,<ea>

Attributes:size=Byte,Word,Long

Description	: Subtract the source operand from the dest-

ination operand, and store the result in the

destination operand. The operation may be

specified to be byte, word or long in size.

The mode of the instruction indicates which

is the source and which is the destination

operand.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary borrow is generated.

Cleared otherwise.

X Set identically to C.

Instruction Format :

1 0 0 1 R R R M M M E E E A A A

Instruction Fields :

RRR : Register field. Specifies any data register

D0-D7.

MMM : Op-Mode field. Specifies operation type in

accordance with the table:

Byte	Word  Long	   Operation

----	----  ----	   ---------

000	001   010	  SUB <ea>,Dn

100	101   110	  SUB Dn,<ea>

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The legal modes are:

Source <ea> : All addressing modes.

Destination <ea> : The following are ILLEGAL addressing

modes:

Dn

An

offset(PC)

offset(PC,Xn)

#nnn

All others are legal.

Notes		: 1) If the destination is a data register,

then the SUB <ea>,Dn form of the instruc-

tion MUST be used.

2) SUBA is used for SUB <ea>,An. SUBI is

used for SUB #nnn,<ea> where <ea> is not

a data register. SUBQ is also used for

SUB #nnn,<ea> when possible. Most 68000

assemblers can automatically make this

distinction.

SUBA	: Subtract Address

Operation : dst - src -> An

Assembler : SUBA  <ea>,An

Attributes:size=Word,Long

Description	: Subtract the source operand from the dest-

ination address register, and store the result

in the destination address register. The size

of the operation may be specified to be a word

or long operation.

NOTE:All address arithmetic on address

registers is performed using the ENTIRE

32 bits of the address register. Word

operands are sign-extended to 32 bits

before adding.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 0 0 1 R R R M M M E E E A A A

Instruction Fields :

RRR : Specifies one of the address registers A0-A7.

This is ALWAYS the destination operand.

MMM : Op-Mode field. Specifies the size of the

operation.

011 : Word operation. See NOTE above.

111 : Long operation.

EEE and AAA fields. Defined in addressing mode section

below. All legal 68000 addressing modes are allowed for

the source operand.

SUBI	: Subtract Immediate Operand

Operation : dst - immediate data -> dst

Assembler : SUBI  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Subtract the immediate data contained in

the instruction from the destination operand,

and store the result in the destination

location.

NOTE:The contents of the immediate operand

part of the instruction is as follows:

Byte : 2nd word of operand, upper byte zero.

Word : 2nd word of operand.

Long : 2nd and 3rd word of operand.

Extension words for complex addressing modes

follows the immediate operand.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary borrow is generated.

Cleared otherwise.

X Set identically to C.

Instruction Format :

0 0 0 0 0 1 0 0 S S E E E A A A

W W W W W W W W B B B B B B B B

L L L L L L L L L L L L L L L L

Instruction Fields :

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

BBB...	: Byte immediate data field. The immediate

data is contained here. The WWW... field

is set to zero (00000000 binary) and the

LLL... field does not exist.

WWW...	: Word immediate data field. The immediate

data occupies both WWW... and BBB... fields.

The LLL... field does not exist.

LLL...	: Long immediate data field. The immediate

data occupies all BBB, WWW and LLL fields,

and the long word is divided as follows:

WWW...BBB...	HIGH word of data

LLL...		LOW word of data

SUBQ	: Subtract Immediate Quick Operand

Operation : dst - immediate data -> dst

Assembler : SUBQ  #nnnn,<ea>

Attributes:size=Byte,Word,Long

Description	: Subtract the immediate data contained in

the instruction from the destination op-

erand, and store the result in the dest-

ination location. The range of values for

the immediate operand is 1 to 8, represented

in the instruction by 0 to 7 (0 representing

a value of 8). The immediate data is automa-

tically coerced to the operation size by the

68000.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Set if result is zero. Cleared otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

NOTE : Condition codes NOT AFFECTED if the

destination for the subtraction is

an address register!

Instruction Format :

0 1 0 1 D D D 1 S S E E E A A A

Instruction Fields :

DDD	: Data field. 3 bits of immediate data,

0 representing a value of 8, 1-7 repre-

senting those values repsectively.

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes. The LEGAL modes are:

Dn

An (Word and Long only)

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

SUBX	: Subtract With Extend

Operation : dst - src - X -> dst

Assembler : SUBX  Dy,Dx

SUBX -(Ay),-(Ax)

Attributes:size=Byte,Word,Long

Description	: Subtract the source operand from the dest-

ination operand, along with the extend bit,

and store the result in the destination loca-

tion.

Condition Codes :	X N Z V C

* * * * *

N Set if result negative when treated as

2's complement. Cleared otherwise.

Z Cleared if result is non-zero.

Unchanged otherwise.

V Set if an overflow is generated. Cleared

otherwise.

C Set if a binary carry is generated.

Cleared otherwise.

X Set identically to C.

NOTE : Normally the Z bit is set via programming

(e.g., using MOVEQ #0,D0) before the start

of the operation. This allows successful

tests for zero operands upon completion of

multiple-precision operations.

Instruction Format :

1 0 0 1 X X X 1 S S 0 0 R Y Y Y

Instruction Fields :

XXX	: Register Rx field. Specifies the destination

register, depending upon R field.

SS	: Size field. Specifies operation size.

00 = byte operation

01 = word operation

10 = long operation

R	: R/M field, specifies operation addressing mode.

0 : operation is ADDX Dy,Dx, and XXX,YYY

fields specify data registers.

1 : Operation is ADDX -(Ay),-(Ax), and XXX,YYY

fields specify address registers.

YYY	: Register Ry field. Specifies the source

register, depending upon R field.

SWAP	: Swap Register Halves.

Operation : Register {31:16} <-> Register {15:0}

Assembler : SWAP Dn

Attributes:size=Word

Description	: Exchange the upper and lower 16-bit halves of

the specified data register.

Condition Codes :	X N Z V C

- * * 0 0

N Set if 32-bit result negative when

treated as 2's complement. Cleared

otherwise.

Z Set if the 32-bit result is zero.

Cleared otherwise.

V Always Cleared.

otherwise.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 1 0 0 0 0 1 0 0 0 R R R

Instruction Fields :

RRR : Register field. Specifies the data register

D0-D7 to swap.

TAS	: Test And Set

Operation : dst tested : result -> CCR

1 -> dst{7}

Assembler : TAS <ea>

Attributes:size=Byte

Description	: Test and set the byte operand addressed by the

effective address field. The current value of

the operand is tested and the N and Z flags of

the CCR are set accordingly. The high order bit

(bit 7) of the operand is set.

This operation is performed using an indivisible

read-modify-write memory cycle, and allows the

synchronsiation of several processors in a multi-

processor 680x0 environment.

Condition Codes :	X N Z V C

- * * 0 0

N Set if operand was negative.

Cleared otherwise.

Z Set if operand was zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 1 0 1 0 1 E E E A A A

Instruction Fields :

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes.

The following are ILLEGAL addressing modes:

Dn

An

offset(PC)

offset(PC,Xn)

#nnn

All others are legal.

Notes : Bus error retry is inhibited on the read portion

of the read-modify-write bus cycle to ensure

system integrity. The bus error exception is

always taken.

TRAP	: 680x0 TRAP Instruction

Operation : PC -> -(SSP)

SR -> -(SSP)

(Trap Vector) -> PC

Assembler : TRAP #nnnn

Attributes:Unsized

Description	: The processor initiates exception processing. The

vector number is used to reference the TRAP inst-

ruction vector specified by the low order four

bits of the instruction. Sixteen TRAP instruction

vectors are available.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 0 0 V V V V

Instruction Fields :

VVVV	:	Vector field. Specifies which of the 16

TRAP exception vectors is to be referenced

to obtain the new program counter.

TRAPV	: TRAP on overflow.

Operation : IF V bit of CCR set

THEN

TRAP (VIA TRAPV EXCEPTION)

Assembler : TRAPV

Attributes:Unsized

Description	: Test the overflow bit of the CCR. If V is

cleared, continue execution at the next in-

struction in sequence. If V is set, initiate

exception processing via the TRAPV exception

vector.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 1 1 0

TST	: Test an operand

Operation : dst - 0  : result -> CCR

Assembler : TST <ea>

Attributes:size=Byte,Word,Long

Description	: Compare the operand with zero. The result

is not saved directly. Instead, the various

bits of the CCR are conditioned according

to the result. The size of the operation may

be specified to be byte, word or long.

Condition Codes	:	X N Z V C

- * * 0 0

N Set if the operand is negative.

Cleared otherwise.

Z Set if the operand is zero.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

0 1 0 0 1 0 1 0 S S E E E A A A

Instruction Fields :

SS	: Size field. Determines the size of the operation.

00 = Byte operation.

01 = Word operation.

10 = Long operation.

EEE	: Destination Effective Address field.

AAA	: Destination (Address) Register field.

EEE and AAA specify the effective address of the

<ea> operand. The table of possible values of the

EEE and AAA fields appears in the section describ-

ing the addressing modes.The LEGAL addressing modes

for TST are:

Dn

(An)

(An)+

-(An)

d(An)

d(An,Xi)

Abs.W

Abs.L

UNLK	: Unlink Workspace.

Operation : An -> SP

(SP)+ -> An

Assembler : UNLK An

Attributes:Unsized

Description	: The stack pointer is loaded from the specified

address register. The address register is then

loaded from with the long word pulled from the

top of the stack.

Condition Codes	: NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 0 1 1 R R R

Instruction Fields :

RRR	: Register field. Specifies the destination

address register through which the unlinking

is done.

Effective Address Specification

-------------------------------

The possible values for the EEE and AAA fields for specifying effect-

ive addresses in standard 68000 instructions is defined by the table:

EEE	AAA	Addressing Mode

---	---	---------------

000	nnn	Data Register Dn, nnn = 0-7

001	nnn	Address register An, nnn = 0-7

010	nnn	(An), nnn = 0-7 to specify address register

011	nnn	(An)+, nnn = 0-7 to specify address register

100	nnn	-(An), nnn = 0-7 to specify address register

101	nnn	d(An), nnn = 0-7 to specify address register

110	nnn	d(An,Xi), nnn = 0-7 to specify address register.

Xi is specified by an extension word to the

instruction.

111	000	Absolute word operand

111	001	Absolute long operand

111	010	d(PC)

111	011	d(PC,Xi). Xi is specified by an extension word

to the instruction.

111	100	Immediate data.

Addressing Modes

----------------

The various addressing modes available on the 68000 are listed below. EA

refers to Effective Address, Generation refers to the mechanism for obtain-

ing the effective address,EEE Mode is the value of the EEE bits in the in-

struction word, and AAA Mode is the value of the AAA bits in the instruction

word.

1) Data Register Direct.

Generation	: EA = Dn (n = 0 to 7)

Assembler	: Dn

EEE Mode	: 000

AAA Mode	: nnn (0 to 7)

The operand is the specified data register.

2) Address Register Direct.

Generation	: EA = An (n = 0 to 7)

Assembler	: An

EEE Mode	: 001

AAA Mode	: nnn (0 to 7)

The operand is the specified address register.

3) Address Register Indirect.

Generation	: EA = (An) (n = 0 to 7)

Assembler	: (An)

EEE Mode	: 010

AAA Mode	: nnn (0 to 7)

The operand is the memory location whose address is contained

in the specified address register.

4) Address Register Indirect with Postincrement.

Generation	: EA = (An) (n = 0 to 7)

An = An + N

(N = operand size:1=byte, 2=word, 4=long)

Assembler	: (An)+

EEE Mode	: 011

AAA Mode	: nnn (0 to 7)

The operand is the memory location whose address is contained

in the specified address register, PRIOR to the addition of

the operand size to the address register contents.

5) Address Register Indirect with Predecrement.

Generation	: An = An - N

EA = (An) (n = 0 to 7)

(N = operand size:1=byte, 2=word, 4=long)

Assembler	: -(An)

EEE Mode	: 100

AAA Mode	: nnn (0 to 7)

The operand is the memory location whose address is contained

in the specified address register, AFTER the subtraction of

the operand size from the address register contents.

6) Address Register Indirect with Displacement.

Generation	: EA = (An) + d (n = 0 to 7)

Assembler	: Label(An)

EEE Mode	: 101

AAA Mode	: nnn (0 to 7) specifies the address

register An.

The effective address of the operand is obtained by computing

the sum of the contents of the specified address register with

the 16-bit displacement using standard 680x0 address arithmetic.

Thus, the 16-bit displacement is sign-extended to 32 bits prior

to the computation.

7) Address Register Indirect with Displacement And Index.

Generation	: EA = (An) + (Xn) + d (n = 0 to 7)

Assembler	: Label(An,Xn.W)

Label(An,Xn.L)

EEE Mode	: 110

AAA Mode	: nnn (0 to 7) specifies the address

register An. Xn is specified by an

extension word.

The effective address of the operand is obtained by computing

the sum of the contents of the specified address register with

the 8-bit displacement and the contents of the specified index

register using standard 680x0 address arithmetic. Thus, the 8-

bit displacement is sign-extended to 32 bits prior to the compu-

tation. The index register may be any data or address register

(hence Xn may be the same as An in the instruction) and the value

obtained from the index register may be either a word (which will

be sign-extended for the addition) or a long.

8) Absolute Short.

Generation	: EA given

Assembler	: Label.W

EEE Mode	: 111

AAA Mode	: 000

The effective address is the contents of the word following the

instruction word. This word is sign-extended to 32 bits prior to

use as an address reference.

9) Absolute Long.

Generation	: EA given

Assembler	: Label.L

EEE Mode	: 111

AAA Mode	: 001

The effective address is the contents of the long word following

the instruction word.

10) Program Counter Indirect with Displacement.

Generation	: EA = (PC) + d (n = 0 to 7)

Assembler	: Label(PC)

EEE Mode	: 111

AAA Mode	: 010

The effective address of the operand is obtained by computing

the sum of the contents of the program counter with the 16-bit

displacement using standard 680x0 address arithmetic. Thus, the

16-bit displacement is sign-extended to 32 bits prior to the

computation.

11) Program Counter Indirect with Displacement And Index.

Generation	: EA = (PC) + (Xn) + d (n = 0 to 7)

Assembler	: Label(PC,Xn.W)

Label(PC,Xn.L)

EEE Mode	: 111

AAA Mode	: 011

Xn is specified by an extension word.

The effective address of the operand is obtained by computing

the sum of the contents of the program counter with the 8-bit

displacement and the contents of the specified index register

using standard 680x0 address arithmetic. Thus, the 8-bit disp-

lacement is sign-extended to 32 bits prior to the computation.

The index register may be any data or address register (hence

Xn may be the same as An in the instruction) and the value ob-

tained from the index register may be either a word (which will

be sign-extended for the addition) or a long.

12) Immediate Data.

Generation	: Operand given

(EA = address of instruction word + 2)

Assembler	: #nnnn

EEE Mode	: 111

AAA Mode	: 100

The effective address of this operand is the address of the in-

struction word plus two. For byte and word operands, the word

at this address contains the immediate data. For long operands,

the long word at this address contains the immediate data.

Extension Words for Complex Addressing Modes

--------------------------------------------

The following extension words are valid for 68000 addressing modes:

d(An)	 : Extension Word is 16-bit signed value of d.

d(An,Xi) : Extension Word D R R R W 0 0 0 I I I I I I I I

D	: D/A field. Specifies index register type.

0 : Xi = Dn

1 : Xi = An

RRR	: Index register number 0-7. Either D0-D7 if D/A=0,

or A0-A7 if D/A = 1.

W	: Index size. Specifies size of index register.

0 : Word size. Low word of index register used,

sign-extended to 32 bits before effective

address computation.

1 : Long size. Entire 32 bits of index register

used in effective address computation.

III...	: Displacement. 8 bits of displacement data.

d(PC)	 : Extension Word is 16-bit signed value of d.

d(PC,Xi) : Extension Word D R R R W 0 0 0 I I I I I I I I

D	: D/A field. Specifies index register type.

0 : Xi = Dn

1 : Xi = An

RRR	: Index register number 0-7. Either D0-D7 if D/A=0,

or A0-A7 if D/A = 1.

W	: Index size. Specifies size of index register.

0 : Word size. Low word of index register used,

sign-extended to 32 bits before effective

address computation.

1 : Long size. Entire 32 bits of index register

used in effective address computation.

III...	: Displacement. 8 bits of displacement data.

Rules For Extension Words

-------------------------

In a complex instruction, such as:

MOVE.L	#$8000,8(A6,D1.L)

the order of extension words is:

Source Immediate Data Word(s)

Source Extension Word(s)

Destination Extension Word(s)

For the above example, the instruction words will be (in hexadecimal):

2DBC	(MOVE.L)

0000	#$8000.L

8000

1808	Extension word for 8(A6,D1.L)

For the instruction ADD.W $300(PC),D2, the instruction words will be

(Again in hexadecimal):

D47A	(ADD.W)

0300	$300 offset in $300(PC)

And MOVE.B #$0F,$4A000.L will appear in memory as:

1E7C	(MOVE.B)

000F	Immediate data

0004	Absolute long

A000	address data

This becomes completely revised when 68020 and higher processors are taken

into account, which have the extended Memory Indirect addressing modes, and

which may have as many as ten extension words following the instruction word!

Conditional Branch Table

------------------------

The list of possible values for conditional expression specifiers

in the CCCC fields of Bcc, DBcc etc., is defined by the table:

DBcc	CCCC	Condition		Expression

----	----	---------		----------

DBT	0000	Always True		1

DBF	0001	Always False		0

DBHI	0010	Higher Than		(~C) AND (~Z)

DBLS	0011	Lower Or Same		C OR Z

DBCC	0100	Carry Clear		~C

DBCS	0101	Carry Set		C

DBNE	0110	Not Equal To Zero	~Z

DBEQ	0111	Equal To Zero		Z

DBVC	1000	Overflow Clear		~V

DBVS	1001	Overflow Set		V

DBPL	1010	Plus (Positive)		~N

DBMI	1011	Minus (Negative)	N

DBGE	1100	Greater Or Equal	(N AND V) OR (~N AND ~V)

DBLT	1101	Less Than		(N AND ~V) OR (~N AND V)

DBGT	1110	Greater Than		(N AND V AND Z) OR

(~N AND ~V AND ~Z)

DBLE	1111	Less Than Or Equal	Z OR (N AND ~V) OR

(~N AND V)

In the case of Bcc, the instruction BF does not exist on current assemblers,

and BT is usually written BRA. Similarly, DBF is usually written DBRA (even

though the logic of the operation is somewhat different).