RADIO REMOTE CONTROL PROGRAM FOR OS/2

By Lydia Rosa

Copyright 1995,1997  by Lydia Rosa

RADIO REMOTE CONTROL PROGRAM FOR OS/2

CONTENTS

WHAT'S NEW IN THIS RELEASE ....................................... 1

DISCLAIMERS ...................................................... 3

INTRODUCTION ..................................................... 4

REQUIREMENTS ..................................................... 5

STARTUP .......................................................... 7

General Description .............................................. 7

THE DISPLAY ...................................................... 9

VFO Frequency.  .................................................. 9

Modulation.  ..................................................... 9

Channel #.  ...................................................... 9

VFO step value.  ................................................ 10

Priority Indicator .............................................. 10

Digit Scrolling ................................................. 11

Freq/Memory Scroll Buttons ...................................... 11

Signal Meter .................................................... 11

Attenuator ...................................................... 12

Scan Mode Buttons.  ............................................. 12

Manual Mode ................................................... 12

Scan Mode ..................................................... 12

Search Mode ................................................... 14

Load and Set Buttons ............................................ 15

Load ALL ...................................................... 15

Set  ALL ...................................................... 16

Mem->VFO ...................................................... 16

VFO->Mem ...................................................... 16

Operation Modification Buttons .................................. 17

COMPUTER/RADIO Button ......................................... 17

VFO Select Button ............................................. 17

Priority ...................................................... 18

Banks On ...................................................... 18

Resume ........................................................ 18

Ack ........................................................... 18

Logging ....................................................... 19

Radio Selection ............................................... 19

Speed Control ................................................. 19

MEMORY  CHANNEL DATA FIELDS ..................................... 19

CHANNEL number ................................................ 20

VALID (Vld) ................................................... 20

DELAY (Dly) ................................................... 20

ATTENUATION (Att) ............................................. 21

RESUME (Rsm) .................................................. 21

FREQUENCY ..................................................... 21

MODE .......................................................... 21

COUNT ......................................................... 22

TIME .......................................................... 22

SIGNAL ........................................................ 22

SERVICE ....................................................... 22

Contents  ii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

TIME STAMP .................................................... 22

SCAN BANK  DATA FIELDS .......................................... 23

CHANNEL number ................................................ 23

Valid ......................................................... 23

Start ......................................................... 23

STOP .......................................................... 24

SERVICE ....................................................... 24

SEARCH BANK DATA FIELDS ......................................... 25

CHANNEL number ................................................ 25

VALID ......................................................... 25

Start-Freq .................................................... 26

Stop-Freq ..................................................... 26

Incr-Freq ..................................................... 26

Mode .......................................................... 26

Service ....................................................... 27

Status line ..................................................... 27

SPECTRUM DISPLAY ................................................ 27

The Menu ........................................................ 28

File menu ....................................................... 28

Database menu ................................................... 28

Common Parms panel .............................................. 29

Priority ...................................................... 29

Delay ........................................................ 29

Resume ....................................................... 29

Spectrum Background .......................................... 30

Tone Decode Enable ........................................... 30

Radio  Parms panel .............................................. 30

Radio Name .................................................... 30

Radio Type .................................................... 30

TXADR ......................................................... 31

Max Channel ................................................... 31

Origin .......................................................... 31

Max Frequency ................................................... 31

Min Frequency ................................................... 31

Baud Rate ....................................................... 32

Com Port ........................................................ 32

Data Bits ....................................................... 32

Stop Bits ....................................................... 32

Flow Control .................................................... 32

Signal Detect ................................................... 33

Send Delay ...................................................... 33

SPECIAL KEYS .................................................... 34

ENTER ......................................................... 34

ESC ........................................................... 34

tab keys ...................................................... 34

PgDn/PgUp ..................................................... 34

ctl-PgDn/ctl-PgUp ............................................. 34

alt-PgDn/ctl-PgUp ............................................. 35

DATA FILE FORMATS ............................................... 36

LOGFILE format ................................................ 36

FREQLIST.DBF .................................................. 36

data file format .............................................. 37

Contents  iii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

RS232 INTERFACE ................................................. 46

PROGRAM LIMITS AND RANGES ....................................... 47

Contents  iv

RADIO REMOTE CONTROL PROGRAM FOR OS/2

WHAT'S NEW IN THIS RELEASE

Version 0.1

Beta Test Release

Version 0.2

Bug Fix release.

Fixed problem with entering SERVICE data  with signal present.

Fixed several hang problems.

Added VFO attenuation button

Added active HELP display

Added support for AOR8000 (untested)

Version 0.3

Fixed WARP problem with signal strength meter not showing.

Some radio parameters were not being saved in .INI file

Fixed various problems with the KENWOOD protocol.

Increased number of radios to 8.

Added Modem Status Reg (MSR) display.

Added scroll bars for FREQUENCY and MEMORY CHANNEL

Added frequency display sync selection

Fixed spectrum display problems.

Version 0.4

Fixed problem with certain errors not displaying.

Fixed problem with DBASE selection.

Added leading blank removal on Service string load.

Added save/recall of strength values.

enhanced error handling of file read.

added CHAIN command (in TEXT files) for file chaining.

added support for certain commands in TEXT files.

Added support for AOR2500 (untested)

Version 0.5

Added resume for individual memory channels.

Added time stamp for memory channel log.

Modified spectrum display routines.

Fixed sort bug that left displayed channel unsorted.

Version 0.6

Fixed bug -service not being read on SCAN bank.

Fixed bug -SCAN bank info not being saved correctly.

Fixed bug -comments not being rewritten to file.

Fixed bug -KENWOOD modulation not being correctly read.

Fixed bug -Two copies of Radioctl could not run as same time.

bank number now shows current bank.

Added buffer for service info. DBASE file now closed after read.

Added pop-up window on exit for data save.

Added command line option for loading a file.

Version 0.7

Removed UP/DN buttons. Replaced with direct mouse clicks.

Rearranged screen to make room for longer SERVICE strings.

WHAT'S NEW IN THIS RELEASE  1

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Added file logging.

Fixed problem that kept program from working with normal speed UART.

Added support for OptoScan protocol.

Added hardware signal detect for AOR radios.

Added additional PL parms in save file (for future enhancement)

Fixed a few AOR8000 bugs.

Version 0.8

added several pop-up status messages.

Fixed a few file read and write bugs.

Fixed search bug: existing frequency not found in database when

blank channels were present.

Fixed a problem that caused corrupted SERVICE data

when DBASE file had no service information on frequency record.

Version 0.9

Added VFO selection for KENWOOD radios.

Fixed a problem with some KENWOOD radios not being able to SET

channels.

Added support for KENWOOD radios that have signal strength logging.

Fixed memory channel scroll bar so that radio channel changes

when MEMORY tracking mode is selected.

Version 0.91

Added verification screen for SET ALL operation.

Tuned serial port routines for better reliability.

Fixed array bounds problem that cause intermitant failures.

Added VFO selection for ALL radios that support it.

Fixed bug - Hardware/software signal select not working correctly.

Improved AOR8000 routines.

Dropped support for AOR2500 due to difficulties in making this

radio conform to the RadioCtl program. The protocol selection

has been left in, but the routines will most likely not work

correctly.

Version 0.92

Fixed problem with KENWOOD TS870 not setting memory channels.

Version 0.93

Fixed more AOR8000 problems.

Fixed problem with display not being updated during search.

Fixed problem with NEW DBASE menu grayed out if DBASE file not

available at load time.

Version 0.94

Put some hooks to minimize hanging when a radio is not attached.

Fixed data file write that caused "(null)" written when no service.

Version 0.95

Fixed problem with AOR3000 not setting data correctly.

Fixed still MORE problems with the AOR8000 (tested with a REAL one).

Added update of VFO display when doing SETALL.

WHAT'S NEW IN THIS RELEASE  2

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of Icom incorporated.

KENWOOD is a trademark of Kenwood Corporation.

AOR     is a trademark of AOR inc.

DBASE III is a trademark of Borland International.

RADIO SHACK is a trademark of Tandy Corporation.

The author  takes no  responsibility for  any damages that  may

occur  to your radio or  computer due to  the operation  of  this

program.

This program is being distributed as SHAREWARE. This means that

voluntary  contributions  are  the  only  source of  remuneration

obtained for this work. A contribution of $25.00  gives  you  the

following:

1.  Fixes for any bugs.

2.  Schematics and instructions for the interfaces.

3.  A clear conscience.

Please mail  all contributions, problem reports  and  suggestions

to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

The author (and  the  latest versions of  RADIOCTL) can also be

contacted on the following Bulletin Board System:

Rosa's Folly BBS

(914) 236-7607

300/1200/2400/9600/14400 baud

Thank you for your support.

DISCLAIMERS  3

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM for  OS/2  will allow  you to

control selected radios from  any  computer  serial  port (RS232)

using  OS/2.  This program expands the capabilities of your radio

to 10000 channels and  100 programmable scan or search banks.  In

addition, scanning, searching, and logging  facilities  are  also

provided.  Some radios  will  need  an  additional  interface  to

utilize  the  signal  detection  facilities  of  this program.  A

circuit  diagram  and  build instructions  are  provided free  of

charge to all registered users.

The program allows you to select various baud rates, COM ports,

and other parameters  associated with individual  radios.  Up  to

eight different radio configurations  can  be  defined  for  easy

switching.  A  LOCAL mode is also  provided  to  run  the program

without the need for a radio connection.

Frequency information can be loaded  or saved from a text file.

An interface to a DBASE type  file is also provided for frequency

to service lookup. Most setup information is saved in a .INI file

for ease in restarting the program.

INTRODUCTION  4

RADIO REMOTE CONTROL PROGRAM FOR OS/2

REQUIREMENTS

In order to run this program you will need the following:

*    A 386,486 or Pentium based computer.

*    At least 4mb of memory (8mb is recommended).

*    about 1mb of free disk space (for the program and document).

*    OS/2 2.11, WARP, or higher level of OS/2.

*    one of the following models of radio:

-   ICOM R-7000*

-   KENWOOD R-5000*

-   AOR3000a

-   AOR8000

-   RadioShack  radio equipped with OPTOSCAN  interface (from

OptoElectronics).

*NOTE:Although not  tested,  this program  should work with other

models of this manufacturer equipped with a serial port.

For Kenwood and Icom Radios, a modified interface  is  required

for  full program functionally  (signal detection). For all other

radios, the  manufacturer's interface will work.  However, signal

detection can be speeded  up by use of the modified interface for

all of the radios. Plans for  this interface are  made  available

free of charge for all registered users.

It is also recommended  that the COM  driver included with OS/2

be replaced with  the  SIO driver by  Ray Guinn.  This driver has

proved to be more reliable than the default  COM.SYS driver. This

driver  package is  available  in shareware, and can be found  on

most bulletin boards.

If you are controlling the R7000, you will need either the ICOM

CI-V interface adapter, or the  modified RS232 interface.  If you

wish to  use  the signal  detection feature  of this program, you

will need to use the modified interface.

For  the  R-5000,  you  will  need  the  IC-10  option  kit.  A

modification must  be made to the R-5000 if you  wish to  use the

signal  detection feature of  this  program.  Some other  KENWOOD

radios provide  for software signal  detection. Although RADIOCTL

can  make  use of this,  the hardware detection will increase the

spee of scanning.

REQUIREMENTS  5

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The AOR3000  and  OPTO equipped  radios  already  have  a RS232

interface. No modification is needed for these radios,  as signal

detection  is done in software.  However,  use  of  the  modified

signal detection will allow an increase in the speed of scanning.

In addition,  the modification instructions include a  change  to

the  AOR3000 to allow 9600 baud  operation (from the default 4800

baud) .

For  the AOR8000, an  interface is also needed. The OPTOLINK from

Optoelectronics has been tested with this program. This interface

allows the use of hardware signal detection for  faster scanning.

This interface will  also work with ICOM radios as well. However,

most interfaces for the AOR8000 should work with this program.

It is highly recommended that  the modified  interface  be used

for ICOM and  KENWOOD  radios.  Most  of  the  features  of  this

program  make  use  of  the  detection  of  an  available  signal

(SQUELCH).  These radios do NOT currently  have squelch detection

as part  of the  standard  interface.  Although this program will

work fine without this  signal detection, operations such as SCAN

and SEARCH will have no usability.

The interface circuit is a  inexpensive, easy to build project.

All  the  parts  are  readily available at any electronic  store.

Estimated cost for each interface  is $15. Plans and instructions

for  these interfaces  are provided  to  you  when  you  become a

registered user.

REQUIREMENTS  6

RADIO REMOTE CONTROL PROGRAM FOR OS/2

STARTUP

It  is  recommended (but not required)  that  this  program  be

installed  in  it's  own  directory.  You  should  also  add  the

directory to your PATH and  DPATH statements  in CONFIG.SYS.  The

FREQLIST.DBF  file should be  placed in the same subdirectory  as

the RADIOCTL.EXE  file. Start the program by entering RADIOCTL on

the command line.  If you  wish, you can  also specify a  file to

load  on the command  line. This file MUST be a  RADIOCTL  format

file  (see  APPENDIX for the  format  of  this  file).  Once  the

program loads, the main control panel  will be displayed. You can

maximize  the window to see all of the  display,  or you can size

the window as you like.

GENERAL DESCRIPTION

RADIOCTL provides the user with a flexible way of controlling a

radio. It also provides a  uniform  interface for different types

of radios. This is a high-level block diagram of the program:

��������Ŀ

|search  |---�

|banks   |   |

����������   |

|

|

|      ������Ŀ

��������Ŀ   �----->|      |   serial  ��������Ŀ

|memory  |<---------|VFO   |---------->|  radio |

|channels|--------->|      |<----------|        |

����������          ��������  �------->|        |

^  ^                      | port   ����������

|  |                      |

|  |    ���������Ŀ       |

|  �----|channel #|-------�

|       �����������

������Ŀ

|scan  |

|banks |

��������

Memory  channels  store  information  on frequency, modulation,

squelch  count, duration, and  other  important  parameters.  The

program  provides  for  a  maximum  of  10000  memory   channels.

Information  can  be  directly  entered  from the control  panel,

loaded  from  a  text  file, loaded from  the  radio's memory, or

transferred from the program's VFO.

The VFO  represents  a  single  frequency and  modulation. This

setting can be sent to the radio, or stored in a  memory channel.

STARTUP  7

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The VFO  can be set from  the control  panel direct input, loaded

from a memory channel, loaded from a search bank, or set from the

radio.

Scan banks provide the ability to group blocks  of channels for

scanning.  Scan  banks  allow  the  user  to  define  up  to  100

different blocks of memory channels for selection.

Search banks provide groups of frequencies to search. Up to 100

different ranges of  frequency, modulation and step  value can be

defined.

The Channel # provides selection of the memory channels. It can

also be used to address the memory channels of the current radio.

Radio control  information  is  sent  to and received from  the

radio  by  way  of  the  serial  port.  The program  provides for

selection of up to eight different radios, each with a unique set

of  operation  characteristics.  The  program  can  also  provide

simulation  of a  radio.  The  actual commands  to the  radio are

defined  by  a  protocol  setting.  The  program current supports

protocols for KENWOOD, ICOM, AOR8000,  AOR3000, and OPTO equipped

radios. A LOCAL protocol is also provided which does not  require

a radio to be attached.

STARTUP  8

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE DISPLAY

The control panel shows you the status  of  your radio, as well

as the status  of the program.  There are  numerous controls, and

various status  fields. Controls are toggled  by moving the mouse

cursor over the control  and  clicking.  Entry  fields  require a

ENTER or TAB key to complete the entry.

VFO FREQUENCY.

This display shows  the current frequency of the program.  This

frequency may or may  not  reflect  the VFO state of the attached

radio.  Any frequency information  sent to the radio is displayed

in this field. The  value of this field can be changed using  the

scroll  buttons to the left of the display or by using  the mouse

buttons when the mouse pointer is over the digit  displayed.  The

left mouse button will  increase  a digit, and  the  right button

will  decrease.  A  frequency  value can also be  entered  in the

entry field under  the  display. If a  value  is entered, changed

with mouse  buttons, or scrolled with  the VFO  scroll bar, it is

also sent to the radio.

MODULATION.

This is  a  combination display and selection box. It shows the

current  modulation  used  by  the program. This may or  may  not

reflect  the state  of the radio. A  modulation  can  be  set  by

pressing the drop-down  control on  the  right  of  the box,  and

double-clicking  on  the desired  modulation. When selected,  the

radio's modulation will be set.

CHANNEL #.

This display shows the current channel number being used by the

program. Depending on the state of the COMPUTER/RADIO  switch, it

will represent the  internal memory  channels  in the program, or

the  radio's  memory channels. The memory channel can be  changed

using the  scroll buttons to the left of the display, using  left

(increment) or  right  (decrement) mouse  buttons,  or entering a

channel  number  in the  entry field  under  the display.  When a

memory channel  number is changed, and  the COMPUTER/RADIO switch

is set to  COMPUTER,  the corresponding computer  memory  channel

information  is  set  to  the  VFO,  and to  the  radio.  If  the

THE DISPLAY  9

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMPUTER/RADIO  switch  is in the  RADIO  position,  the  radio's

memory channel is changed.  If the program is in MANUAL mode, and

the MEM SYNC switch is selected, changing the channel number will

display the corresponding Memory Channel information.

NOTE:The AOR8000 is organized into 20 banks of 50 channels.  Each

bank is identified as a lower or upper case letter. RADIOCTL maps

these channels as follows:

��������������������������������������������������������Ŀ

� radio   �   computer       �    radio    �   computer  �

��������������������������������������������������������Ĵ

�A00-A49  � 000 - 049        �    a00-a49  � 050 - 099   �

��������������������������������������������������������Ĵ

�B00-B49  � 100 - 149        �    b00-b49  � 150 - 199   �

��������������������������������������������������������Ĵ

�C00-C49  � 200 - 249        �    c00-c49  � 250 - 299   �

��������������������������������������������������������Ĵ

�D00-D49  � 300 - 349        �    d00-d49  � 350 - 399   �

��������������������������������������������������������Ĵ

�E00-E49  � 400 - 449        �    e00-e49  � 450 - 499   �

��������������������������������������������������������Ĵ

�F00-F49  � 500 - 549        �    f00-f49  � 550 - 599   �

��������������������������������������������������������Ĵ

�G00-G49  � 600 - 649        �    g00-g49  � 650 - 699   �

��������������������������������������������������������Ĵ

�H00-H49  � 700 - 749        �    h00-h49  � 750 - 799   �

��������������������������������������������������������Ĵ

�I00-I49  � 800 - 849        �    i00-i49  � 850 - 899   �

��������������������������������������������������������Ĵ

�J00-J49  � 900 - 849        �    j00-j49  � 950 - 999   �

����������������������������������������������������������

VFO STEP VALUE.

This  display  shows  the current step  value that is used for

Search  mode. This value will be  added to or subtracted from the

VFO frequency to obtain the next VFO frequency. This value can be

set  using  the  left  (increment)  or  right  (decrement)  mouse

buttons, or direct entry in the field under the display. During a

bank search, this field will be loaded from the selected bank.

PRIORITY INDICATOR

When  the  unit  is  scanning,  and  the  PRIORITY  button  is

selected, a specific channel  is defined as the priority channel.

When  this channel  is  being  scanned, the Priority Indicator is

lit. This is a red "P" situated next to the Channel # display.

THE DISPLAY  10

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DIGIT SCROLLING

The VFO Step Value, VFO  frequency,  and Channel # displays can

be changed  using  the left and  right mouse buttons to increment

and decrement the  digit.  The mouse pointer selects the digit to

change, and the left button will increment, and the  right button

will decrement the digit.  This  allows quick changing of  any of

the information in the display.

FREQ/MEMORY SCROLL BUTTONS

Both frequency and memory channel number can  be incremented or

decremented  using  the  scroll buttons  next  the  the  affected

display.  For the Frequency,  the increment  or  decrement  value

will  be the  current  VFO  STEP  setting. For the memory channel

number, increment or decrement value is always 1.

SIGNAL METER

The signal is a 12 step indicator  corresponding to  the signal

strength being detected. For ICOM and KENWOOD radios, the  signal

indicated will always  be 9. This  is  due  to the fact  that the

signal detection is a binary (On or OFF)  line,  with no  way  to

read  the  actual  strength.  For all other  radios,  the  actual

strength detected  by the radio will be  displayed. For  a  LOCAL

radio, clicking  any of  the signal display buttons will turn  on

the signal  at that strength. For all radios, the OFF button will

temporarily turn off the signal.  If scanning or searching,  this

will  cause  the  radio to  resume  scan  or  search.  For  LOCAL

operation, the OFF button is the only way to  turn off the signal

detection.

When not running in LOCAL mode, the  signal buttons can be used

to define  a  minimum squelch trigger. Clicking on a button  will

cause the program  to ignore any signal at  or below that  signal

level. When the squelch minimum is active, the signal limit level

buttons are displayed in green.

NOTE:This feature is not useful for the ICOM and KENWOOD radios,

as these  radios  do not  have  the  ability  to determine signal

strength values.

THE DISPLAY  11

RADIO REMOTE CONTROL PROGRAM FOR OS/2

ATTENUATOR

The AOR3000 and AOR8000 have a built in attenuator. This can be

switched  on  to reduce overload from  a strong signal.  Pressing

this button will  toggle  the current attenuator  setting  in the

radio.  When  in SCAN  mode,  this button  will  be  set  by  the

currently scanning memory channel's attenuator setting.

SCAN MODE BUTTONS.

These buttons select the scanning mode. The program runs in one

of three scanning modes. MANUAL, SCAN and SEARCH  modes determine

the primary operation of the program. MANUAL mode is the default.

Manual Mode

Of  three scanning modes. MANUAL mode  is the default.  In this

mode, the program will continuously  monitor the signal  strength

from the radio. For the  ICOM, changes in the radio  frequency or

modulation will be logged in the VFO displays.

The program will  switch to manual mode if one of the following

conditions is met:

*   The MANUAL button is pressed.

*   SCAN mode detects no valid channels or banks.

*   SEARCH mode detects no valid  banks

Scan Mode

The  SCAN button  starts the  program scanning through selected

memory channels. Memory channels  that are flagged with the VALID

field will be scanned. Channels with  the  VALID flag off will be

bypassed. When  a signal is detected,  scanning  will  stop,  the

signal  strength will be  displayed  in  the  Signal  Meter,  and

information (duration, squelch count, etc) will  be logged in the

memory channel.  If  the MEM SYNC switch  is selected, the Memory

Channel display will change to the  Memory Channel number that is

being logged.  When the  signal  stops,  scanning  will continue.

Memory  channels are normally incremented. The direction  of  the

scan can be  changed by pressing the CHANNEL NUMBER scroll button

THE DISPLAY  12

RADIO REMOTE CONTROL PROGRAM FOR OS/2

to the side of the channel display.

Several  parameters  and  memory  channel  settings  affect the

operation of SCAN. Each channel  has a VALID flag associated with

it. If this flag  is set off,  the corresponding  memory  channel

will  NOT  be scanned. If  the  DELAY  field of the corresponding

memory  channel  is  set, the  program will delay for a number of

seconds after  the  signal stops before resuming scan.  The DELAY

value can be set from the PARMS panel.

The SPEED CONTROL setting determines how fast each channel will

be  scanned. In the event that a scan is too fast  to  respond to

certain  frequencies, it  can  be slowed  by adjusting  the SPEED

CONTROL.  Setting the SPEED CONTROL to a  lower value causes  the

computer to wait on each channel when there is no signal present.

This gives the radio time for the squelch to detect a signal.

The  RESUME  switch  determines  the  action  after the squelch

breaks. If RESUME  is off, scanning will  not continue until  the

signal ends.  If RESUME  is on,  scanning will  resume  after the

number  of seconds specified by the RESUME value.  This value can

be set from the PARMS panel.

NOTE:The  RESUME  switch overrides any  RESUME  setting  for  an

individual memory channels.

If a channel is entered  in  the PRIORITY  channel, it will  be

scanned  every interval specified  by  the priority value  if the

PRIORITY switch is  set.  If a signal is detected on the priority

channel, normal scanning stops,  and the  program switches to the

priority channel.  Normal scanning will resume when the signal is

no  longer present  on  the  PRIORITY  channel. The  interval for

priority checking is set on the PARMS panel.

Note:The  RESUME counter does not  function  when stopped on the

priority channel.  Scan or Search  will  resume  from a  priority

channel ONLY when the signal disappears.

If BANKS  ON  button is on, the  order  of channel scan will be

determined by the START and END of the  current bank. The current

bank is  scanned  from 0 to 99.  When  BANKS ON is selected, each

valid bank will determine starting  and ending channels for scan.

Channel numbers will be incremented for  a  selected  bank if the

START  channel  number  is less than  the  STOP  channel  number.

Otherwise, channel numbers will be decremented.  Failure  to find

a valid bank, or  a  valid channel  for  the specified banks will

force MANUAL mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels.  If  this button is selected for SCAN  or SEARCH

mode, the displayed Memory Channel information will change to any

channel  where  a  signal   is  detected.   This  allows  you  to

automatically  display  the   channel  information  whenever  the

squelch breaks.  If this button  is off,  then the memory channel

display will NOT change when a signal is detected.

The   COMPUTER/RADIO  switch  determines   the  source  of  the

frequency  information for  the radio.  If  COMPUTER is selected,

frequency and modulation  information  is sent to  the radio from

THE DISPLAY  13

RADIO REMOTE CONTROL PROGRAM FOR OS/2

the computer's memory channels. If RADIO is set, then the program

will  change  the  radio's memory  channel to  correspond  to the

channel  being  sent.  This  allows  use  of the  radio's  memory

instead of the information in the computer. If RADIO is selected,

only the channel  numbers supported by the selected radio will be

allowed. The  maximum  channel  number can  be  changed using the

PARMS panel.

NOTE.The OPTO  protocol does  not  provide  for  memory  channel

control. When using the OPTO protocol, this switch remains in the

COMPUTER position, and cannot be changed.

Search Mode

Search  Mode allows  you  to  scan  a range of  frequencies for

action.  Up to 100 different ranges can be programmed. Each range

consists of a starting  frequency, stopping  frequency, increment

value and  modulation. In addition, searching  can  start at  the

current  VFO frequency and be incremented or decremented by a set

amount.  For  non-bank  processing,  the  program  will  normally

increment the frequency.  Pressing VFO  STEP  UP/DN  buttons will

change the search direction.

When  a  range is  searched, the radio is  set  to the  initial

frequency and modulation. If a signal  is detected, the frequency

and  and modulation are recorded in the next free log record, and

the VALID  flag  is  set  for this record.  If this  frequency is

already recorded in the  log, then COUNT,  TIME and SIGNAL values

are updated. If the log entry has the  VALID flag off, the signal

is ignored.  If  there are no more free channels (frequency  = 0)

then  the message "SEARCH FULL" is displayed, and the information

is NOT saved.

Several  parameters  and  memory  channel  settings  affect the

operation of SEARCH.  Each  channel has a VALID  flag  associated

with it. If  this flag  is set  off, the corresponding  frequency

will NOT respond to a squelch break.  If the DELAY field of the a

memory channel is set, and the corresponding frequency breaks the

squelch, the program will delay for a number of seconds after the

signal stops before resuming  scan.  The DELAY value  can  be set

from the PARMS panel.

The  SPEED  CONTROL setting determines how fast each  frequency

will be monitored. In the event that  a  search  is  too  fast to

respond to certain frequencies, it can be slowed by adjusting the

SPEED CONTROL.  Setting the SPEED CONTROL to a lower value causes

the computer to wait on each  frequency when there  is  no signal

present. This gives the radio time  for the  squelch to detect  a

signal.

The  RESUME  switch  determines  the  action after  the squelch

breaks.  If RESUME is off, searching  will not continue until the

signal  ends. If RESUME is  on,  scanning will resume  after  the

number of  seconds specified by the RESUME value. This value  can

THE DISPLAY  14

RADIO REMOTE CONTROL PROGRAM FOR OS/2

be set from the PARMS panel.

NOTE:The  RESUME  switch  overrides  any RESUME setting  for  an

individual memory channels.

If BANKS ON  button is on, the order of  frequency scan will be

determined by the START and END of the current bank.  When  BANKS

ON  is  selected,  each  valid  bank will determine  starting and

ending  frequency  ,  increment  and  modulation.  If  the  START

frequency  is  less  than  the STOP frequency, then the frequency

will   be  incremented.   Otherwise,  the   frequency   will   be

decremented.  Failure  to  find  a  valid  bank will force MANUAL

mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this button is selected, the displayed Memory

Channel information will change to  any channel where a signal is

detected.  This  allows you to automatically display the  channel

information  whenever the squelch breaks.  If this button is off,

then the  memory channel display will NOT change when a signal is

detected.

The  PRIORITY  button does NOT have any effect on  Search Mode.

Priority  is  not  supported during a search operation. Likewise,

the COMPUTER/RADIO  switch is  also  ignored. Search  Mode always

uses the computer's VFO.

LOAD AND SET BUTTONS

Four buttons provide load and set operations.  These operations

transfer  information between the VFO, memory  channels  and  the

computer. When one of  these buttons is pressed, the current scan

mode  is suspended,  and the operation is  started.  Pressing the

button  a  second  time  before the  operation is  complete  will

terminate  the  operation  and  resume  the suspended  scan mode.

Selecting the  MANUAL scan button will also terminate any load or

set operation.

Load ALL

When  pressed, all of the  radio's memory  channels are  loaded

into the computer's memory channels. The load starts at the first

channel  in  the  radio (0  or  1) and  continues to the  maximum

channel. The maximum channel can be  set on  the PARMS panel. The

contents of computer memory channels are overwritten with the new

information. Any  memory  channel  with  a non-zero frequency  is

marked  as  valid. The count,  duration  and  strength fields are

reset to  0.  The  service string  is also cleared.  For  a LOCAL

radio, a set of random frequency and modulations is set for  each

channel.

THE DISPLAY  15

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Note:  The OPTO protocol  does not  provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

Set  ALL

When pressed, the contents of the  computer memory channels are

stored into the radio's memory channels. The  set  starts at  the

first  channel in the radio (0 or 1) and continues to the maximum

channel. The maximum channel can  be set on the PARMS panel.  The

contents of radio  memory  channels  are overwritten with the new

information. For  ICOM and KENWOOD  radios, a zero frequency will

cause  the memory channel to be cleared. For AOR3000 radios, zero

frequency channels will NOT  be set.  For LOCAL mode, each memory

channel will be transferred to the VFO in turn.

NOTE.The  OPTO  protocol  does  not  provide  for memory channel

control. When using the OPTO protocol, this switch is disabled.

Mem->VFO

This  button  will transfer the contents of the Memory  Channel

indicated  by  the  MEMORY  #  display,  to  the  VFO.   If   the

COMPUTER/RADIO switch is in the COMPUTER position, the Computer's

memory channel will  be  transferred.  If  the  RADIO position is

selected, the information will be transferred  from  the  radio's

memory. For a LOCAL radio, the input will  be  a random frequency

and modulation. This  operation is the same as a Load All for one

channel.

VFO->Mem

This button will transfer the contents of the VFO frequency and

modulation  to the  Memory  Channel  indicated by  the  MEMORY  #

display.  If  the  COMPUTER/RADIO  switch   is  in  the  COMPUTER

position, the  Computer's memory channel will be updated.  Count,

Strength,  and Duration fields  are reset. The  Service string is

also cleared.  If the RADIO position is selected, the information

will be  transferred to the radio's memory. For a LOCAL radio, no

data  is transferred.  This operation  is the same as a Save  All

for one channel.

THE DISPLAY  16

RADIO REMOTE CONTROL PROGRAM FOR OS/2

OPERATION MODIFICATION BUTTONS

Next to  the Channel  #  display  is  a column  of buttons that

affect various operations. These buttons determine what memory is

used,  priority, resume, bank selection,  acknowledgment  and the

radio being used. Each of these buttons is a  toggle switch. Move

the mouse cursor  over the button and right click to toggle on or

off.  The Priority  switch  has an  entry field next  to  it. The

others have a status field.

COMPUTER/RADIO Button

For  various  Scan modes and  operations, the program needs  to

determine  who's  memory is  the  source or  destination  of  the

operation.  When in  the COMPUTER position, the  program is using

the  internal  Memory Channels  for the operation.  If the  RADIO

position  is  selected,  the  operation  will  affect the  memory

channels in the selected radio.

Note:  The  OPTO protocol does  not provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

VFO Select Button

Above the Frequency display is a button used to select a VFO (A

or  B) to be  used  in  the selected radio. Some radios have  two

selectable frequency memories (VFO). This button determines which

VFO will be selected when setting the frequency on the radio.  If

the radio that is currently connected does  not support two VFOs,

then this button will not have any effect.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this  button is  selected for SCAN or  SEARCH

mode, the displayed Memory Channel information will change to any

channel  where  a  signal  is  detected.  This   allows  you   to

automatically   display  the  channel  information  whenever  the

squelch breaks.  If  the  program  is  in  MANUAL  mode, then the

memory  channel display  will  change in  sync  with  the  memory

channel number.

THE DISPLAY  17

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Priority

Priority scanning allows  a selected channel to be tested for a

signal once every interval.  The  channel  number to select as  a

Priority channel  is  entered  in  the entry field  next  to  the

Priority switch.  If the Priority switch is ON, then the priority

channel will be used.  The  priority checking interval can be set

using the PARMS panel. Priority only works during SCAN mode.

Banks On

When in SCAN or SEARCH mode,  the program can make use of banks

of scan  or search information. These banks allow the grouping of

channels or  frequencies  for the SCAN mode.  The Banks On switch

allows the  scan  mode to  use  the bank associated with the SCAN

mode.  When  selected,  the  indicator  next to the  switch  will

display what bank number (00-99) is currently being used.

Resume

The  Resume  switch  will  allow the Scan or Search to continue

when a signal is present. This  is useful  when a Scan or  Search

encounters a  constant  signal.  With Resume  on, the  signal  is

ignored after  a set amount  of  time. This time  interval is set

using  the  Parms panel. The indicator next  to the Resume switch

shows   approximately   how  many  seconds  are  left  until  the

SCAN/SEARCH resumes.

Note:This switch is a global setting. If on,  the  RESUME switch

for each memory channel is ignored.

Ack

Ack sets radio  acknowledgement  on. When  a command is sent to

the radio, the program will normally wait  for acknowledgement of

that command  to come  from  the  radio.  In  the  event that the

command is lost or garbled, the program will resend  the command.

Ack  forces the  program to wait  until  this  acknowledgement is

received before  continuing  to the next channel during  SCAN  or

SEARCH mode.  If Ack is OFF, then  the  program will not wait for

acknowledgment.  This  will speed up some Scan/Search operations,

but  may  cause incorrect data to  be exchanged between the radio

and program.

NOTE:  If  Ack is ON, the radio will retry a command up to three

times if acknowledgement is set  on. After the third failure, the

THE DISPLAY  18

RADIO REMOTE CONTROL PROGRAM FOR OS/2

program will continue to the next command.

Logging

When this switch is selected, all actions are  logged to a data

file. This includes mode changes and squelch breaks. A time stamp

is also  logged with each event. This file is cumulative, and  is

not erased by this program. The name of  the  logging file can be

changed with the FILE menu selection.

Radio Selection

Up  to  eight different  radios  can be defined to the program.

These radios can have different BAUD rates, Com ports, protocols,

etc. Each radio is assigned a name. You  can change the name (and

other parameters) for a given radio  from  the Parms panel. Radio

Selection is  a  drop-down selection  field. Click  on the  right

button to  display  the radio  list.  Double click  on a radio to

select it. The Baud  and Com port used will be displayed next  to

the Radio Selection control.  NOTE: When a new radio is selected,

Manual mode is forced.

Speed Control

For SCAN or SEARCH mode, the speed control determines the time

between each channel or frequency.  When  the control is  all the

way to the left, the time between each setting is minimal. If the

slider  is  moved  to  the right,  the program  will  delay for a

certain  time  before  changing the setting  on the radio. During

this  time, the program will monitor the radio for  a signal. The

SPEED control allows you to reduce  the SCAN or  SEARCH speed  to

give the radio more time to respond to a signal.

MEMORY  CHANNEL DATA FIELDS

Program  Memory  channels  contain  information for  a specific

radio  setting.  Each  channel contains information on Frequency,

Modulation and attenuation to  be sent to (or  received from) the

radio.  In addition,  information on signal  strength,  number of

squelch breaks and the frequency assignment are also displayed.

THE DISPLAY  19

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Each program channel displayed  has  several  fields associated

with it.  Some of these fields are entry fields that  information

can be  entered.  Other  fields are toggle  switches  that can be

turned  on and off.  Note  that entry field  information  is  NOT

accepted until ENTER is pressed,  a TAB key  is  pressed,  a  new

entry field is selected with the mouse, or the channel display is

changed with the scroll bars.

CHANNEL number

This is the first  field  on the display. It  determines  which

memory  channel  is  being  displayed  or  changed.  The   memory

channel can be changed by  direct  entry in the channel field, or

using  the  scroll  bars to the left  of the channel number.  The

PgUp and PgDn  keys  will  also scroll  the memory channel up and

down.  If entering the channel number, keep in mind that the data

will NOT be entered unless you press ENTER or a TAB key.

VALID (Vld)

The VALID field  determines whether or not SCAN and SEARCH will

use  the frequency currently entered into that channel.  For SCAN

mode, the program will not scan the memory channel if VLD is OFF.

For the SEARCH operation, a frequency entered in  a memory record

that has the VALID flag off will NOT cause the search to stop and

log.

DELAY (Dly)

The DELAY  field is use  during the SCAN and SEARCH operations.

If delay is set ON,  scanning  will  continue DELAY seconds after

the signal ends  on  that  channel.  If  delay  is  set OFF, then

scanning  will continue immediately after the  signal  ends.  The

DELAY time value can be set on the PARMS panel.

THE DISPLAY  20

RADIO REMOTE CONTROL PROGRAM FOR OS/2

ATTENUATION (Att)

The ATTENUATION field selects  the  attenuator  of  the  radio.

This is a circuit that reduces  the  signal strength so that high

level  signals do not overload the radio. The AOR3000 and AOR8000

radios  support  this  feature.  This  option  has no effect  for

KENWOOD and ICOM radios.

RESUME (Rsm)

The RESUME field selects the RESUME function for the individual

memory  channel. If this switch is on,  and a  signal is detected

for this memory channel  in SEARCH  or  SCAN mode, SEARCH or SCAN

will continue after  a set number of second if the signal has not

gone away. This works the same as the global RESUME switch except

that it only affects the single channel.

FREQUENCY

The FREQUENCY field is used to record or set the current memory

channel  frequency.  Frequency  information  is  entered  in Mhz.

Note that INS and DEL keys do not work when editing the FREQUENCY

field.

MODE

The  MODE  field  represents  the modulation  being used  for a

specific frequency. This field is a drop down selection  box.  To

select a modulation, click on the button to the right of the box.

Double click  on  the  desired  modulation.  Note  that  not  all

modulations will be accepted by your particular radio.

THE DISPLAY  21

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COUNT

This  field displays the number of times  a signal was detected

on a specific  channel. This  field  cannot be  edited.  However,

left-clicking on  this field with the mouse cursor will reset the

field to 0.

TIME

This field displays the average number of seconds that a signal

was detected on a specific channel. This field cannot  be edited.

However, left-clicking on this field  with  the mouse cursor will

reset the field to 0.

SIGNAL

The signal strength  of  the last  recorded signal  is reported

here. This  field  cannot be  edited.  However,  left-clicking on

this field with the mouse cursor will reset the field to 0.

SERVICE

This  field  is  used  to record  the  service  for  a specific

channel.  This  is a  scratch pad location to record  information

about a  specific  frequency.  If  the  squelch  breaks  on  this

channel during  SCAN  or SEARCH mode, the program will attempt to

lookup the service for the frequency entered. If the frequency is

found in the current frequency database, and the SERVICE field is

blank, then it will be  filled in with  the service  found in the

database.

TIME STAMP

When  ever  a squelch  break  occurs for this channel, the time

stamp is  updated with the time and date of the occurrence.  This

field  cannot be edited.  However,  left-clicking  on this  field

with the mouse cursor will reset the field to 0.

THE DISPLAY  22

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SCAN BANK  DATA FIELDS

Scan Bank entry  allows you  to  define banks of  channels  for

scanning.  RADIOCTL allows you to assign any range of channels to

various banks. Up to 100 banks (0-99) can be assigned. Banks  can

overlap, and can be turned on  or off selectively. In  order  for

SCAN mode to recognize banks, the Banks On button must be active.

The bank displayed has several fields associated with it.

CHANNEL number

This  is  the first  field on the display. It  determines which

scan bank  is  being displayed  or  changed.  The range for  this

channel  number is 0 to 99.  The Channel number can be changed by

direct entry in the channel field,  or  using the scroll  bars to

the left of  the  channel number.  The  shift-PgUp and shift-PgDn

keys  will  also  scroll  the  memory  channel  up and  down.  If

entering the channel number,  keep in mind that the data will NOT

be entered unless you press ENTER or a TAB key.

Valid

The VALID  field  determines whether or not SCAN  will use this

bank.  Scanning banks without  the  VALID flag will be ignored at

all times.

Start

START identifies the first  channel to  start scanning with for

the given bank. When this bank is active, START will be the first

channel scanned  if  the  channel's  VALID  flag  is set. If  the

channel's  VALID flag is NOT set,  then the first  channel to  be

scanned will be the next sequential channel with a VALID flag.

THE DISPLAY  23

RADIO REMOTE CONTROL PROGRAM FOR OS/2

STOP

STOP  identifies  the  last channel to scan when  the  bank  is

active.  After  this channel  is scanned, scanning  will continue

with the START channel of the next VALID scan bank.

If the STOP channel is set lower  than the  START  channel, the

SCAN  will  decrement  channel numbers. If  STOP is greater  than

START,  channel  numbers  will  increment. If  STOP and START are

equal,  the  single channel  number  in  START  and  END will  be

scanned.

SERVICE

This field is provided to enter  comments  about a given  bank.

It is not used or modified by any program function.

THE DISPLAY  24

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SEARCH BANK DATA FIELDS

The  search  bank  panel allows you to define banks of channels

for  searching.  RADIOCTL  allows you  to  assign  any  range  of

frequencies,  along  with  a   modulation  and  step   value  for

searching.  Up  to 100  search  banks (0-99) can be  defined.  In

order for  SEARCH mode to recognize  banks,  the  Banks On button

must be active.  The bank displayed has several fields associated

with it.

When searching, the program  will start  with  the first search

channel marked as valid,  set the radio VFO to  the  frequency in

the  START field and the modulation in the MODE  field, and check

for a  signal.  If  no  signal  is  detected,  the  program  will

increment or decrement the  frequency  by  the amount in the  INC

field, and repeat  the  process.  When  the frequency in the STOP

field is reached, the  program will select the next  valid search

bank and repeat the process. When all valid search channels  have

been processed, the program returns to the first search bank. For

more details on  the SEARCH operation, see the description of the

SEARCH mode.

CHANNEL number

This is the first  field on the  display.  It  determines which

search bank  is being  displayed or changed.  The  range for this

channel number is 0 to 99.  The Channel number can be  changed by

direct entry  in the channel field, or  using the scroll  bars to

the left  of the channel  number.  The alt-PgUp and alt-PgDn keys

will also scroll the memory channel up and down.  If entering the

channel number, keep in mind that  the data  will NOT be accepted

unless you press ENTER or a TAB key.

VALID

The  VALID field determines whether or not SEARCH will use this

bank.  Search banks without the VALID flag will be ignored at all

times.

SEARCH BANK DATA FIELDS  25

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Start-Freq

Start-Freq  identifies the  first  frequency  to  set  for  the

search.  Please note that the program does NOT check the range of

this value.  If it is outside the  range of  the radio, the radio

may  ignore the setting.

Stop-Freq

Stop-freq  identifies the last frequency to set for the search.

The search bank ends when the vfo frequency reaches the Stop-freq

value.  If the Stop-freq value is less than the Start-freq value,

the  frequency is decremented. If the  Stop-freq value is greater

than the Start-freq  value, the frequency is incremented.  Please

note that the program does NOT check the range of  this value. If

it  is  outside the range of the radio, the radio may  ignore the

setting.

Incr-Freq

Inc-Freq determines  the  steps for the given search  bank. The

Inc-Freq value is added to (or subtracted from) the VFO frequency

for each step. If an Inc-Freq  value of 0 is entered, the  SEARCH

mode will automatically reset the VLD flag for that bank.

Mode

The MODE field represents  the  modulation  being  used for the

search range. The  radio  will be set to this modulation  at  the

beginning  of  the  search range.  This  field  is  a  drop  down

selection box. To select a modulation, click on the button to the

right of  the box. Double  click on the desired modulation.  Note

that not  all  modulations  will  be accepted by your  particular

radio.

SEARCH BANK DATA FIELDS  26

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Service

This  field  is provided to enter comments  about a given bank.

It is not used or modified by any program function.

STATUS LINE

The status line displays messages from the program. This may be

error  messages, or normal operation messages.  The time and date

are included with all messages as an indication of when the error

occurred.

SPECTRUM DISPLAY

The  spectrum display  shows  a graph  of  frequency vs. signal

strength.  An active channel will be shown as a red  line (if the

graph  is  in  the  range of the  active frequency).  Frequencies

representing past activity will be displayed in green.

In  the lower left hand corner, the starting frequency will  be

displayed. The ending frequency of the spectrum will be displayed

in the lower  right  hand  corner  of  the  display.  The  center

frequency  of the spectrum is  changed by  moving  the horizontal

scroll  bar. The minimum start frequency is 0 mhz and the maximum

stop  frequency  is   9999.9mhz.  The  horz   step  increment  is

determined by the spectrum resolution.  The resolution is changed

by  moving  the vertical scroll bar. Resolution goes  from 100khz

per pixel to 1khz per pixel in 1khz steps.

Tic marks are  displayed for reference. Tic marks are displayed

every 10 mhz when the  resolution is above 10khz/pixel.  When the

resolution is  10khz or below,  the tic marks are displayed every

1mhz.

SEARCH BANK DATA FIELDS  27

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE MENU

The menu on the top of the panel provides additional functions.

these functions provide the  ability to load and save information

to disk,  customize your  radio  setup, and manipulate the memory

channel information. Menu  items can be  selected by  placing the

mouse cursor  over the item and pressing  the right mouse button.

indicated by an underline  under  the character  in  the specific

menu  item.  For the  primary menu,  the  ALT key  must  also  be

pressed.  For  drop-down menus,  only the key  itself is need  to

select an item.

FILE MENU

The File menu provides the ability to load or save the  current

memory channel, scan  banks,  and search banks to  or from a text

file.  The file  format for this  file is  described in  the DATA

FORMAT section.  The scan information can  be  saved to or loaded

from  many  different files.  When a file is  loaded, only  those

memory channels, scan bank channels, or search  channels that are

included  in  the file are  changed.  For  a  save,  only  memory

channels with a frequency greater than zero , and scan and search

banks  that  are  marked as valid are  written.  If  the  program

detects a file format problem when loading, the type of error and

the line number causing the error will be displayed, and the load

will stop.

A new DBASE file can  also  be selected  from this  panel. This

file  is  used  for service lookup operations. The format of this

file is also discussed in the DATA FORMAT section.

The name of the logging  file is also set from this menu.  This

is the file that is written to when the LOGGING switch is on.

The File menu also  contains the Quit for the program. Function

key 3 will also terminate the program.

DATABASE MENU

The Database  menu allows you to manipulate  various aspects of

the memory channels. A sort of memory channel by frequency can be

done from  this panel.  With a sorted frequency  list,  the radio

being controlled requires less settling time  for  each frequency

change, allowing for faster signal detection.

The  Service  search  will update  all  Service fields for  any

memory channel with a non-zero frequency. The service information

The Menu  28

RADIO REMOTE CONTROL PROGRAM FOR OS/2

is obtained from the current DBASE file.

The  Clear option allows you  to clear all memory channels, all

service fields from all memory channels, or all  count, time  and

signal logs from all memory channels.

COMMON PARMS PANEL

Various parameters determine the operation of the program.  For

the  most part, these parameters are  set  once  when  you  first

install the program. The first page of the  Parms Notebook allows

you to set program operation parameters.

Priority

This common  panel  parm  sets  the  time  (in seconds) between

priority checks.

Delay

The number of seconds that the program will wait  on a  channel

after a signal  disappears.  This value  only has  affect  if the

DELAY flag is set on the current memory channel.

Resume

If  RESUME is selected, this is the number of seconds that SCAN

or SEARCH will continue after a signal is detected.

The Menu  29

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Spectrum Background

This is  the  color of the SPECTRUM DISPLAY  window background.

The default is BLACK. However, it can be changed to suit personal

preference.

Tone Decode Enable

This  selection  is  for a future enhancement. It is  currently

disabled, and cannot be selected.

RADIO  PARMS PANEL

The  program  provides settings for up to  eight  radios.  Each

radio can be setup with different operation characteristics.  For

the most part,  these parameters  are  set once  when  you  first

install  the program. However,  you  may  need  to  update  these

parameters if you connect different radios.  Except for Baud rate

and  Com port, the defaults for the particular radio types should

only be changed if a non-standard radio is being supported.

Radio Name

This  is  the  name  that  you give the  radio.  There  are  no

restrictions  on  the  name,  other  than  is  is  limited  to 30

alpha-numeric  characters.   This  name  is  used  in  the  Radio

Selection control on the main panel.

Radio Type

This  field  defines the type of radio  (protocol).  There  are

currently six types of radios  supported. ICOM, KENWOOD, AOR3000,

AOR8000,  OPTO  and  LOCAL.  Other radios may  be  added  in  the

future.

The Menu  30

RADIO REMOTE CONTROL PROGRAM FOR OS/2

TXADR

ICOM and OPTO radios require  an address.  For  the R-7000, the

address is normally  8. If you connect  a  different ICOM or OPTO

radio, or you have more than one  ICOM  radio connected, you  can

change this address to match.  This parameter  has no  affect for

other radio types.

Max Channel

This  field defines the maximum  memory  channel  in the radio.

For  the  ICOM R-7000, and KENWOOD R-5000, this would normally be

99. For the AOR3000 this value should be 399. Changing this field

allows the program to support  other radios  with more  (or less)

memory.  This parm  has no effect  for the OPTO  protocol, as the

OPTO protocol does not support memory channels.

ORIGIN

Origin is  the  first  memory  channel. ICOM radios start  with

memory  channel  1.  The  AOR3000 and KENWOOD  start with  memory

channel 0.  This parm has no effect for the OPTO protocol, as the

OPTO protocol does not support memory channels.

MAX FREQUENCY

This is the maximum frequency that the radio will accept.

MIN FREQUENCY

This is the minimum frequency that the radio will accept.

The Menu  31

RADIO REMOTE CONTROL PROGRAM FOR OS/2

BAUD RATE

The baud  setting for this radio. Most radios can be configured

to accept baud rates from 1200  to 9600 baud. The default setting

for  ICOM and  AOR3000 is 9600. For the  KENWOOD,  the default is

4800 baud. Adjust this setting  to  match the baud rate that your

radio is set for.

COM PORT

This  is  the  com  port  that  your radio is attached  to. The

program allows  you to select com  1 thru com 4 or LOCAL. A LOCAL

selection will bypass all serial port output.

DATA BITS

Most of the radios supported work with an 8 bit data format. If

you have a radio that requires 7 bits, this can be changed.

STOP BITS

Most of the radios supported use  2  stop bits.  If needed, you

can change this to 1 stop bit.

FLOW CONTROL

Flow   Control   provides  the   ability  to  stop  and   start

communications  based on the ability of the radio and computer to

accept data.  Flow control requires the use of additional control

lines (RTS/DTS).  These  lines are used as signals to indicate to

the radio and the computer to start and stop sending information.

The ICOM interface that this program is  designed  to  work  with

does NOT provide  for RTS/CTS control lines (flow  control).  The

KENWOOD and AOR3000 interfaces  will use RTS/CTS flow control. It

is recommended  that flow control be left on for these radios, as

they may not work properly with it off.

The Menu  32

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SIGNAL DETECT

There are three ways to detect  a  squelch break. The first  is

using a spare signal on the RS-232 port. This signal detection is

part  of  the  modified  interface, and also  provided with  OPTO

equipped radios. This is referred to as HARDWARE detection.

The  second detection method  involves using  a  command to the

radio to  detect squelch break.  This is  referred to as SOFTWARE

detection.  This  method  also  uses  a  command to the  radio to

determine  the  signal  strength.  SOFTWARE   detection  is   NOT

available  for the ICOM protocol.  Some KENWOOD radios provide  a

software signal detection.  The KENWOOD  R5000 does NOT have this

feature.

The  third method  is a  combination of HARDWARE and  SOFTWARE.

With  this  technique,  the signal line  is checked  for  squelch

break, and a signal  strength  command  is  issued  ONLY  if  the

squelch break signal  has been detected. This method is  the most

efficient for scanning speed. However, it cannot be used for ICOM

radios, as there are no signal strength commands available.

Please note that to make use of  the HARDWARE, or HARDWARE  and

SOFTWARE  detect  methods,  either  the modified interface or the

OPTO interface must be present.

SEND DELAY

Some radios require  a delay between  each  character sent.  If

this value is set greater than 0, the computer will pause for the

number of milliseconds entered between each character sent to the

radio.  Although  this  delay  is  not  needed  on  the currently

supported radios, this  option is provided for future support  of

other  radios. The maximum value  that the program will allow for

this value is 50. If a  greater  value is set, it will be reduced

when  the  PARMS panel is  exited.  Be  aware  that  setting this

parameter  will  slow  down  the SCAN and  SEARCH  rates  of  the

program.

NOTE:  The  delay time generated is approximate. This  is due to

the   unpredictability    of   the    multi-tasking   dispatching

environment. If this option is required, some experimentation may

be needed to determine the correct value.

The Menu  33

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SPECIAL KEYS

ENTER

The ENTER key finalizes  input  to the  program. For data entry

fields, the information is stored into the computer's memory, and

the cursor advances to the next field.

ESC

Pressing ESC terminates the current operation. It is equivalent

to pressing the MANUAL button.

tab keys

The  tab keys will move the cursor to the next field. A forward

or reverse tab key will complete data entry of the current field.

PgDn/PgUp

These  keys increment or decrement the  current  memory channel

being displayed.

ctl-PgDn/ctl-PgUp

These keys increment or decrement the current scan bank channel

being displayed.

SPECIAL KEYS  34

RADIO REMOTE CONTROL PROGRAM FOR OS/2

alt-PgDn/ctl-PgUp

These  keys  increment or  decrement  the current  search  bank

channel being displayed.

SPECIAL KEYS  35

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DATA FILE FORMATS

There are several  files  that this program makes use of. These

files are used to store and retrieve data used by the program.

LOGFILE format

When LOGGING is enabled, certain transactions are logged to  a

file.  The  name  of this  file is set by the  user.  The initial

setting for this filename is RADIOCTL.LOG.

For a given transaction, the following information is logged:

*   Timestamp- The Date and Time of the transaction.

*   The scanning mode (SCAN/SEARCH/MANUAL).

*   The VFO frequency.

*   The VFO modulation.

*   The current memory channel.

*   The signal strength at the time of transaction.

*   The length of the transaction.

*   The service information for the current memory channel.

FREQLIST.DBF

This  is  a  standard  DBASE III+ type  data  file. It contains

frequencies and  allocations  for the  radio spectrum. There  are

three fields that are used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If  the  ACTION  field is not present, the program will use the

DATA FILE FORMATS  36

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DESCRIPT  field  to  determine  ACTION.  A blank  DESCRIPT  field

signals an inactive channel.

The length of the DESCRIPT field can be changed, if so desired.

This program only uses the first 80 chars of this field.

Any other fields can be added or deleted from this file without

affecting the operation of  the program.  However,  if either the

FREQUENCY or DESCRIPT field  is not present in the database,  the

DBASE function will not operate.

data file format

The  SAVE  command creates a file with  the current settings of

memory channels, and various scan parameters. The file created is

an  ASCII  readable  file.  Each record is terminated  by a CR/LF

sequence.  One record represents one frequency channel, one  scan

bank channel, one search bank channel, one command, or a comment.

Which  type  if record  is  determined  by  the  first  non-blank

character on the line.  The following table shows the  identifier

for the various types of records:

��������������������������������������������Ŀ

� identifier   �  type of record             �

��������������������������������������������Ĵ

� number       � frequency channel info      �

��������������������������������������������Ĵ

� @            � scan bank information       �

��������������������������������������������Ĵ

� #            � search bank information     �

��������������������������������������������Ĵ

� keyword      � command to program          �

��������������������������������������������Ĵ

� *            � comment                     �

����������������������������������������������

Each record contains one or more fields delimited by blanks.  The

file can  be altered (or created) using any standard text editor.

The  maximum  record length that  the program can handle  is  200

characters.  If   the  comment  record  flag   is  detected,  all

information following the '*' is ignored  on that line.  When the

program reads the file, any missing required parameter, parameter

range error, or unrecognized keyword will terminate the file read

at that line.

The format for a frequency channel entry is as follows:

DATA FILE FORMATS  37

RADIO REMOTE CONTROL PROGRAM FOR OS/2

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � CHANNEL   � 0-9999         �channel number to set       �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   2 � FREQUENCY � 0-999.9999     �frequency in Mhz            �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   3 � MODE      � 3 chars        �valid modulation for radio  �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   4*� COUNT     � 0-999999       �count of squelch breaks     �

���������������������������������������������������������������Ĵ

�   5*� DURATION  � 0-999999       �total duration of squelch   �

���������������������������������������������������������������Ĵ

�   6*� SIGNAL    � Sxx            �Signal strength value (xx)  �

�     �           � xx=00 to 15    � optional                   �

���������������������������������������������������������������Ĵ

�   7*� VALID     � VALID/NOTVALID �indicator of channel valid  �

���������������������������������������������������������������Ĵ

�   8*� ATTEN     � ANTEN/NOANTEN  �indicator of attenuation    �

���������������������������������������������������������������Ĵ

�   9*� RESUME    � RESUME/NORESUME�indicator of resume         �

���������������������������������������������������������������Ĵ

�  10*� TSTAMP    � TSTAMP value   �squelch break time stamp    �

�     �           � TSTAMP 0       �time value is UNIX time     �

�     �           �                �(number of seconds since    �

�     �           �                � 1970) 0 = no stamp.        �

���������������������������������������������������������������Ĵ

�  11*� TONE      � PL 0 0 0       �future enhancement.         �

�     �           �                �If included, all three      �

�     �           �                �numbers must be specified.  �

�     �           �                �                            �

���������������������������������������������������������������Ĵ

�  12*� DELAY     � DELAY/NODELAY  �indicator of channel delay  �

���������������������������������������������������������������Ĵ

�  13*� SERVICE   � 80 chars       �service for channel         �

�����������������������������������������������������������������

*optional fields

The first three fields  MUST be specified and be in  the proper

order. Optional fields do NOT have to be specified.  However, the

following positional restrictions apply:,

*   If COUNT is specified, it MUST follow the MODE field.  If not

specified, the COUNT value for the channel is set to 0.

*   If DURATION is specified,  COUNT must  also be specified, and

DURATION  must  follow  the  COUNT.  If  not  specified,  the

DURATION value for the channel is set to 0.

*   Duration  value MUST be  specified  if any of  the  following

fields are specified:

DATA FILE FORMATS  38

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SIGNAL, VALID, ANTEN, RESUME, TSTAMP, TONE, DELAY, SERVICE.

*   If SIGNAL value is not  specified,  then SIGNAL value for the

channel is set to 0.

*   If VALID  is not specified, the channel will be marked VALID.

*   If ANTEN, RESUME  or  DELAY is  not specified, the respective

flags will be reset for the channel.

*   If DELAY is specified, it MUST precede the service field.

*   If  SERVICE  is specified, DELAY must also be specified.  All

characters (up to 80) following the DELAY value and up to the

end  of  the line  (CR/LF)  are taken  as  a SERVICE  string.

Leading blanks and any special ASCII characters are removed.

DATA FILE FORMATS  39

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a scan bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � @              �@ identifies a scan bank    �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- MAX         �starting channel for bank   �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- MAX         �ending   channel for bank   �

���������������������������������������������������������������Ĵ

�   5*� SCAN      � SCAN/NOSCAN    �valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSCAN is not specified, the bank is marked as valid.

DATA FILE FORMATS  40

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a search bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � #              �@ identifies a search bank  �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- 9999.99999  �starting frequency for bank �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- 9999.99999  �ending   frequency for bank �

���������������������������������������������������������������Ĵ

�   5 � INC       � 0- 9999.99999  �frequency increment         �

���������������������������������������������������������������Ĵ

�   5 � MODE      �AM,FM,FMN,etc.  �modulation for bank         �

���������������������������������������������������������������Ĵ

�   5*� SEARCH    � SEARCH/NOSEARCH�valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSEARCH is not specified, the bank is marked as valid.

DATA FILE FORMATS  41

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Certain commands can  be entered  in a text file for execution.

These  commands provide the ability to execute certain operations

automatically,  without  having  to select them from the  various

panel controls.

COMMANDS RECOGNIZED

��������������������������������������������������������������Ŀ

�command         � option(s)      � description                �

��������������������������������������������������������������Ĵ

�ACK             �    -           �turn ON acknowledgement     �

��������������������������������������������������������������Ĵ

�BANK            � ON|OFF         �BANK mode on or off.        �

�                �                �Equivalent to pressing BANK �

�                �                �button.                     �

��������������������������������������������������������������Ĵ

�CHAIN           � filespec       �start reading another text  �

�                �                �file. Maximum CHAIN depth   �

�                �                �is 3 files.                 �

��������������������������������������������������������������Ĵ

�CHANNEL         � 0 to 9999      �set channel number.         �

�                �                �Equivalent to setting       �

�                �                �CHANNEL display value.      �

��������������������������������������������������������������Ĵ

�CLEAR           �    -           �Clear all memory channels   �

�                �    ALL         �Clear all memory channels   �

�                �    COUNTS      �Clear all squelch counts    �

�                �    SERVICE     �Clear all service strings   �

�                �    SIGNAL      �Clear all strength readings �

�                �    TIME        �Clear all squelch times     �

��������������������������������������������������������������Ĵ

�DBNAME          � filespec       �change DBASE filename for   �

�                �                �service lookups.            �

��������������������������������������������������������������Ĵ

�DELAY           �    xxxx        �Set DELAY value to xxxx.    �

��������������������������������������������������������������Ĵ

�F               �  frequency     �Set VFO frequency           �

��������������������������������������������������������������Ĵ

�HALT            �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�LOAD            �     -          �load memory from radio      �

��������������������������������������������������������������Ĵ

�M               �   modulation   �set VFO modulation          �

����������������������������������������������������������������

DATA FILE FORMATS  42

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMMANDS RECOGNIZED (cont)

��������������������������������������������������������������Ŀ

�NOACK           �     -          �turn OFF acknowledgement    �

��������������������������������������������������������������Ĵ

�MANUAL          �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�MEMORY          �     -          �use radio's memory          �

��������������������������������������������������������������Ĵ

�PDELAY          �   0 to 9999    �set priority sample delay   �

��������������������������������������������������������������Ĵ

�QUIT            �     -          �exit RADIOCTL               �

��������������������������������������������������������������Ĵ

�PRIORITY        �     ON         �set priority switch on      �

�                �     OFF        �set priority switch off     �

�                �   0 to 99999   �set priority channel        �

��������������������������������������������������������������Ĵ

�RESUME          �     ON         �set resume   switch on      �

�                �     OFF        �set resume   switch off     �

�                �   0 to 9999    �set resume value            �

��������������������������������������������������������������Ĵ

�SCAN            �     -          �Start SCAN mode             �

��������������������������������������������������������������Ĵ

�SEARCH          �     -          �Start SEARCH mode           �

��������������������������������������������������������������Ĵ

�SORT            �     -          �Sort memory channels        �

��������������������������������������������������������������Ĵ

�VFO             �     -          �use computer's memory       �

����������������������������������������������������������������

For  compatibility  with  the  DOS  version  of  Radioctl,  the

following  command  keywords  are  recognized,  but ignored  when

reading:

*   BANK {bank_number}

*   BAUD

*   CD

*   COM1

*   COM2

*   DBASE

*   DIR

*   DMAX

*   DMIN

DATA FILE FORMATS  43

RADIO REMOTE CONTROL PROGRAM FOR OS/2

*   FILENAME

*   FKn

*   GOTO

*   HELP

*   INC

*   LOG

*   LOGNAME

*   LPT1

*   LPT2

*   MAX

*   MAXMEM

*   MODE

*   NOSIGNAL

*   PRINT

*   PRTLENGTH

*   PRTWIDTH

*   QFK

*   QSET

*   RECALL

*   RESET

*   SAVE

*   SET

*   SHELL

*   SIGNAL

*   SPEED

*   START

*   STOP

*   TXADR

DATA FILE FORMATS  44

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Any record that is not recognized as  a  Frequency,  Scan  bank,

Search  bank,  command or  comment  will be  flagged as a line in

error.

DATA FILE FORMATS  45

RADIO REMOTE CONTROL PROGRAM FOR OS/2

RS232 INTERFACE

This program  is designed to use the serial communications port

(RS232) of the  IBM computer. In order to  interface a  radio  to

this  port, an  interface  adapter  is needed  designed  for  the

specific radio.  Each  radio manufacturer makes such  an adapter.

However,  none  of  the  adapters supplied from  the manufacturer

provide for squelch detection. Without this detection, the signal

sensing functions of this program will not work.

For  each radio  supported,  a  circuit  and  instructions  are

available.  This circuit provides the squelch detection needed by

this  program.  The  interface  circuits  are  easy to build, and

require few parts.  All  of the parts can be  obtained cheaply at

any electronics supply store. The schematics and instructions are

provided to all registered users.

RS232 INTERFACE  46

RADIO REMOTE CONTROL PROGRAM FOR OS/2

PROGRAM LIMITS AND RANGES

MEMORY REQUIRED:       4 meg (OS/2 minimum). 8 Meg+ suggested

PROCESSOR REQUIRED:    80386 or Higher

OS/2 LEVEL REQUIRED:   2.1  or higher

MAXIMUM MEMORY CHANNELS    10000

MAXIMUM SCAN BANKS          100

MAXIMUM SEARCH BANKS        100

FREQUENCY RANGE           .01khz to 9999.99999mhz*

MINIMUM FREQUENCY STEP     10hz*

MAXIMUM FREQUENCY STEP     9999.99999mhz

*may be limited by radio model.

PROGRAM LIMITS AND RANGES  47

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INDEX

A                                  H

Ack 18                             Hot Keys

AOR3000 3                             Alt PgDn 25

attenuation 20                        Alt PgUp 25

Attenuator 11                         PgDn 20

PgUp 20

Shift PgDn 23

Shift PgUp 23

B

Banks On 18

I

ICOM 3

C                            interface 5

Channel # 9

channel RESUME 21

COMPUTER/RADIO Button 17                 K

COUNT 21

KENWOOD 3

D

L

DBASE 3

delay 20                           Load ALL 15

Digit Scrolling 11                 Load/Set Buttons 15

Logging 19

E

M

ENTER key 34

ESC key 34                         Manual Mode 12

Mem Sync 17

Mem->VFO 16

Memory Channel

F                               CHANNEL number 20

Memory Channels 19

Freq/Memory Scroll Buttons 11      Menu 27

frequency 21                          Database 28

FREQUENCY field 21                    File 28

Parms 29

MODE field 21

modulation 21

Index  48

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Scan Mode

O                               BANKS 13

COMPUTER 13

Operation Modification Buttons 16     DELAY 13

MEM SYNC 13

PRIORITY 13

RADIO 13

P                               RESUME 13

SPEED 13

Parms                                 VALID 13

Baud Rate 31                    Search Bank 24

Com Port 32                        Channel number 25

Data Bits 32                       Incr-freq 26

Delay 29                           Mode 26

Flow Control 32                    Service 26

Max Channel 31                     Start-freq 25

Max Frequency 31                   Stop-freq 26

Min Frequency 31                   Valid 25

Origin 31                       Search Mode 14

Priority 29                        BANKS 15

Radio Name 30                      DELAY 14

Radio Type 30                      MEM SYNC 15

Resume 29                          RESUME 14

Send Delay 33                      SPEED 14

Signal Detect 32                   VALID 14

Spectrum Background 29          SERVICE 22

Stop Bits 32                    Set ALL 16

Tone Decode Enable 30           shareware 3

TXADR 30                        SIGNAL 21

PgDn key 34                        Signal Meter 11

Pgup key 34                        Spectrum Display 27

Priority 17

Priority indicator 10

T

R                            tab key 34

TIME  21

Radio Selection 19                 TIME STAMP 21

Resume 18

RS232 5

V

S                            valid 20

VFO Frequency 9

Scan Bank                          VFO Modulation 9

Channel number 23               VFO Select Button 17

Service 24                      VFO step value 10

Start 23                        VFO->Mem 16

Stop 23

Valid 23

Scan Mode 12

Scan Mode Buttons 12

MANUAL 12

SCAN 12

SEARCH 14

Index  49