Software for GS4500 hand scanner

for OS/2

This package allows the use of the GS4500 scanner from an OS/2 full

screen session.

The quality of the program and device driver is "pretty rough", but I

imagine there a number of you out there who would like something like

this even if it is a bit clunky for now.

It comes in 3 parts: Genius.sys, sc.exe, and errmsg.dll.

Genius.sys is just entered into the config.sys file with a

DEVICE=GENIUS.SYS

It is the heart of the system. It uses only the default DMA channel and

the default address, and makes no use of the IRQ. I hope to fix these

lacks in the future.

SC.EXE

This is the user interface (what little there is of it). It asks how

long of a scan you want to make and wants a 2 digit number so that a 3

inch long scan MUST be entered as    03   not   3    .

It will output a standard uncompressed TIFF file that many programs can

read.

ERRMSG.DLL

This is just a standard library of error message text. Must be put

somewhere in the LIBPATH.

For those of you who might be interested in writing a better user

interface here is the info on the device driver.

device name is IMGSCAN$

device catagory is 128

and the structure that is passed backa nd forth is as follows:

ret_hdr         struc

db      0

db      0

db      7 dup (0)

db      1, 1

num_lines       dw      1

db      1, 0, 0, 0

db      1, 0

db      0, 0, 1, 0, 1, 0

bytes_to_xfer   dw      1

data_44         dw      1             ;filled in when bytes to xfer is set

dots_per_inch   dw      0C8h

data_46         dw      0C8h

toss_bits       dw      0

bits_per_line   dw      0

data_50         dw      0

data_51         dw      0

db      0, 0, 0, 0

DMA_channel     dw      0

resolution      dw      0

scan_stat       db      0

ret_hdr ends

subfunctions serviced are:

(of DOSDEVIOCTL 10h)

itest   equ     0       general test (reads back the data block)

ion     equ     1       turn scanner on

ioff    equ     2       turn scanner off

iinfo   equ     3       should update the info block (unused in sc.exe)

ireport equ     4

ireso   equ     5       receive modified data block

ido     equ     6       do the scan

iend    equ     7       turn off the device

I'm not including the full source code as it's a real mess, but I expect

to at a later time.

Patrick O'Riva

BBS 408-259-2223

Fidonet 143/37

If this program does what you need it to do a contribution of $10-20

would be appreciated and might stimulate further development.

I retain full rights to the programs. They may NOT be sold or included in

any comercial project, nor may any charge be made for their

distribution.

No warantee of any kind is made. Try it and use it at your own risk.