NOVELL NETWARE 386 v3.1 SERVER DRIVER FOR USE WITH

8003/8013 FAMILY LAN ADAPTERS

RELEASE INFORMATION

Version 2.05         		24 April 1991

The purpose of this document (file RELEASE.DOC) is to explain

the version history, bug fixes and any known problems with

the NetWare 386 8003/8013 family server driver.

**Version 2.05    24 April 1991 (Current Release) **

1. Fixed the bug in the LLC type field when transmitting

Ethernet 802.2 packet.

2. Supported 8013EP/A and 8013WP/A adapters.

**Version 2.04    11 April 1991 **

Added version number and date to CONFIG command.

No code change was made from the previous version.

**Version 2.03   31 October 1990 **

This version contains modifications for the WD83C690 Network

Interface Controller. This version or later versions must be

used with adapters which contain the WD83C690.

** NOTE **

Use of adapters which contain the WD83C690 with previous

versions of this driver may lead to data integrity problems.

This version has been tested by Western Digital.

**Version 2.02    1 September 1990 **

This version corrects a problem when using 8013 adapters in 16

bit mode with 16 bit VGA adapters in 16 bit mode. This version

also corrects a problem which caused the Operating System to

Abend (abnormal end) when IRQ, RAM Base, or RAM Size parameters

where entered  on the command line when using any of the following

software configurable adapters: 8003EB, 8003W, 8003EP, 8013EP, 8013W,

or 8013EW.

**Version 2.00    4 July 1990 **

This is the first offical release of the NetWare 386 v3.1

driver. It conforms to the the ODI driver specification

for NetWare 386 version 3.1. This driver supports the

multiple MAC protocol and network management functions of

NetWare 386 v3.1. This driver does NOT function with NetWare

386 version 3.00.  If a driver for Netware 386 version 3.00

is required, refer to version 1.00 of this driver.

This driver has one known problem, it will not function

with the 8013EBT adapter when used with other 16 bit adapters

(ie. 16-bit VGA) in 16 bit mode.  This problem will be fixed

in the next release.

Licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation. Other trademarks mentioned herein belong to

their respective companies.

Copyright (c) 1990 Western Digital Corporation

All Rights Reserved.