# Concorde SST Inertial Navigation System (INS) User Manual

# IMPORTANT: This INS upgrade is supplied and used on condition of there being no technical support available from Interactive Associates for it. Please do not ring our technical support helpline they will be unable to assist.

# Overview

The inertial Navigation System (INS) is a reliable positioning system used on many civil and military aircraft both before and after the advent of GPS Satellite navigation.    The system modelled here is a representation of the instrument fitted in the Concorde aircraft. It has 25 pre-programmed waypoints for “Round Trip” Transatlantic crossing and can also be user programmed for different flight plans.

# Specifications

The instrument used here does not have the full functionality of the real Concorde instrument but it is does have the following features:

#1 	25 built in waypoints for London to New York and return to London.

#2	25 User defined waypoints.

#3	Save and load of a single custom flightplan.

#4	Auto waypoint increment.

#5 	Automatic selection of ILS frequency and approach Hold.

#6	4 display modes.

#7	Waypoint entry error checking.

#8	Audio Cues.

You can manually enter your own 25 Waypoints and save these for future use by entering the information via the INS keypad just like in the real thing. There is a more detailed section on waypoint entry later in the Manual

Use of the INS with the pre-programmed London-New York-New York-London route

It is recommended that you read all through this section on the INS before using this first flight.

To view the INS firstly reveal the centre console (shift+6 key) then left mouse click any where on any of the three INS keypad entry areas. This will show the full size INS instrument. (To minimise the instrument back down to the centre console, left mouse click on the bottom left minimise area of the maximised INS gauge. To remove the centre console press Shift+6 key again). You do not have to have the INS maximised for this pre-programmed flight to select the hold button, but you have to have the centre console showing. Please note this is not a SID (Standard Instrument Departure) or STAR (Standard Terminal Arrival) INS plan.

Take off from Heathrow on runway 27 left and as soon as the gear is up, activate the INS hold feature (press the INS “hold” button, maximised or minimised INS instrument). You will then fly the waypoints automatically selected by the INS in sequence until you reach New York JFK ILS intercept point (waypoint 13).    Here the INS will select the correct ILS frequency for a 13L approach, and then select the autopilot approach hold as it disconnects.    For the return flight simply select INS HOLD button immediately after takeoff for waypoint 14 and you will follow the track back to London Automatically. At the London Heathrow ILS Intercept point (waypoint 25), the INS will again select the correct frequency for the 09L ILS and switch the autopilot to approach hold before disengaging. Audible cues will be heard as the ILS switches between waypoints and on disengaging. You must fly the altitudes and speeds as instructed in this manual. The INS will only navigate the route. It does not control the altitude or speed.

### A look at the instrument

On the centre console (press the shift key plus the 6 key together to view) will see 3 INS control Panels. These are the normal locations for the Captains INS, Co-pilot INS and Flight Engineers INS. In Normal operation these 3 systems cross check each other for triple redundancy. In this normal view (Figure 1), the readouts are active and are displayed normally in “non maximised view mode” but the only active control is the HOLD button. This allows you to turn the INS hold operation on or off without having to maximise the INS.

## Figure 1

### A 	Concorde centre console and throttle quadrants

B	Captains INS panel

C	Co-pilots INS panel

#### D	Engineers INS panel

Please note these instruments are NOT fully functional when viewed in this non-maximised view mode. You will need to MAXIMISE the INS panel to program or use the different modes. Just click on the centre of the INS entry keypad to maximise.

The section following contains a diagram of the MAXIMISED Instrument showing all it’s features and functions in detail.

### The Maximised INS Panel

# A	Latitude readout

# B	Longitude readout

C	Autopilot hold button

D	Waypoint set mode Button

E	Current waypoint information button

- F	Clear button

G	Distance to current waypoint button

H	Bearing to current waypoint button

I	INS programming keypad

J	Enter key

K	Waypoint increment button

L	(Inoperative in this version)

M	Mouse click area to minimise INS

N	(Inoperative in this version)
O	Autopilot hold indicator lamp
P	Current waypoint number indicator. This is the black rectangle in the middle of the INS display picture with the digits 01 in it.

# A	Latitude readout

# B	Longitude readout

The LED readouts shows different information depending on the MODE set by the user. When additional modes are set the display changes to that mode and holds for 6 seconds before returning to Standard Mode. These modes are as follows:

Normal mode- Actual current latitude and longitude shown. (Default display-no buttons pressed)

Current mode-Active waypoint latitude and longitude shown. Display this mode by pressing the CURRENT button (E)

Bearing mode -Current waypoint information shown by the bearing in latitude (left) display and estimated time of arrival (ETA) in longitude (right) display. Display this mode by pressing the BNG button (H)

Distance mode-Current waypoint information shown as distance in Nautical miles in the Latitude display and ground speed (in MPH) in the longitude Display. Display this mode by pressing the DIST button (G)

# C	Autopilot hold button

By pressing the HOLD Button the INS will command the aircraft autopilot into waypoint tracking mode. It uses the autopilot heading hold with bearing information fed to the autopilot via the INS System. On disengaging the HOLD feature the INS will sound a disconnect chime and the autopilot will remain in HDG HOLD mode until you deselect it.

D	Waypoint set mode button

Pressing the SET Button puts the INS into PROGRAM MODE. (See Programming guide later in the manual for full description.)

E	Current waypoint information button

See A+B Above

F	Clear button

Used to “Clear” an error in programming waypoints. (See programming guide later in the manual for full description.)

# G	Distance to current waypoint button

See A+B Above

H	Bearing to current waypoint button

See A+B Above

I	INS programming keypad

Enter waypoint data via this keypad for your custom flight plans (See programming guide later in the manual for a full description.)

J	Enter key

Used to “Enter” the waypoint data for your custom flight plans (See programming guide later in the manual for a full description.)

K	Waypoint increment button

Pressing the INC Button steps the INS through the waypoint memory bank. If the INS is in hold mode the INS will send the “selected” waypoint data to the autopilot so you can skip waypoints. You will need to step through from waypoint #1 to waypoint#14 for example if Flying from JFK to London using the built in route information, as waypoints #1 - #13 are the London – JFK leg and #14 – 25 are for the return leg.

- L	Inoperative

M	Minimise Area

When you have finished with the maximised Panel you can switch it back to the normal, non-maximised size by left mouse clicking in this area. The main INS panel will then minimise back to the centre console where you will still see the readouts and can activate the HOLD Button only.    To maximise the panel again, simply click anywhere on a “keypad area” on any of the 3 minimised INS panels

N	Inoperative

# O	Autopilot hold indicator lamp

This small green LED will illuminate when you have INS HOLD selected and the system is operating via the autopilot.

Note: In MAXIMISED MODE The INS Panel is fully featured as above but when MINIMISED only the CURRENT LAT LONG display is available as well as the HOLD BUTTON. No other features are available in Minimised mode.

###### INS Programming guide

If you wish to program your own flight plan routes the following section will explain the programming procedure of the INS System.

### To programme a waypoint

Press the SET BUTTON (D), this puts the INS in programme mode to allow you to enter the data.

Decide which waypoint you wish to re-program by selecting it using the INC BUTTON (K). You can programme any or all of the waypoints at any time. The LAT LON Display will blink with the currently programmed waypoint information.

You are now ready to enter the Waypoint co-ordinates.

### KEYPRESS SEQUENCE

It is important to enter the data in the correct sequence. For example to enter a waypoint at the location 51'17.28 N      3'09.50 W you must use the following sequence

[N] [0] [5] [1] [1] [7] [2] [8] [W] [0] [0] [3] [0] [9] [5] [0] [ENTER]

The INS numeric programming keypad (I) has the Letters N-S-E-W on the keys 2, 8, 6 and 4. Use these keys to enter the correct hemisphere information as part of the sequence

Please note the leading ZEROS as the INS expects a 3 figure “degrees” entry, so a figure of    22N    (22 degrees north) has to be entered as N022

If you make an error in the data entry you can press the CLR button (F) to “backspace” the entry. You can “backspace” as many times as required.    On pressing the ENTER key (J) The waypoint will be read into memory and become available for use. Repeat the above for up to a maximum of 25 waypoints.

SAVING AND LOADING YOUR CUSTOM FLIGHTPLAN

The INS doesn't keep your programmed waypoints between sessions unless you have saved it. (You can only save ONE flight plan at any one time above the included pre programmed London New York plan) However, if you programme your custom flightplan, you can save it and load back next time. Note: Saving another flight plan over writes the previously saved item.

To save your custom flightplan, press [SET][8][ENTER]. To load it back, press [SET][2][ENTER].

### RESETTING THE DEFAULT ROUTE

You can reset the default London Heathrow– John F Kennedy – London Heathrow flightplan at any time by pressing    [SET]    [ENTER]. This will load all original waypoints and reset the INS to waypoint #1

Important INS information

1) All manually programmed waypoint and the active HOLD waypoint are LOST when Flight Simulator is switched to full screen mode or back while in flight. The same happens when you select a different Concorde. If you have to switch, save your custom flightplan (if you programmed it) first, then change screen mode or aircraft then if it is a custom flightplan load it and use the WPT INC button to select the correct next waypoint and then engage the INS HOLD button. If you are using the built in London-New York-London flightplan, simply use the WPT INC button to go to the correct next waypoint and select the HOLD button. It is best to decide which mode to stay in, either normal or full screen mode and not to change during the flight. Select your mode choice before starting your flight and to use it until the end of your flight.

2) When you switch to another active application, and switch back to Flight Simulator, you may hear some of sounds the INS produces.

3) You can only save one extra flightplan above the pre-programmed item and it may only have maximum if 25 waypoints in it.

4) Do not use CTRL+F for the FMS system when using the INS. Either use the FMS of the INS do not mix them on the same flight.

5) You must have the Microsoft FS98 patch (fs98pat1.exe) loaded for the INS to function properly. If you suffer a lack of INS heading control then you do not have the patch loaded. To load the patch, uninstall Concorde as described in the manual and reinstall Concorde and select the Flight Shop converter and Patch install option this time around.