8003/8013 FAMILY LAN ADAPTER DRIVER

FOR USE WITH SUN MICROSYSTEMS' PC-NFS

INSTALLATION GUIDE

CONTENTS

The following files are needed for installing PC-NFS:

1. WDPLUS.SYS - PC-NFS Link Level Driver for 8003/8013 family

Ethernet LAN adapters.

HARDWARE AND SOFTWARE REQUIREMENTS

1. One 8003/8013 family Ethernet LAN adapter. This driver does

not support StarLAN adapters.

2. An IBM PC, PC XT, PC AT, PS/2 Micro Channel or 100%

compatible personal computer.

3. At least 512 Kbytes of usable RAM.

4. PC-DOS 3.0 or greater, or a compatible version based on

MS-DOS.

5. One or more NFS servers.

For additional information, refer to the PC-NFS User Manual,

available from Sun Microsystems.

CONFIGURING AND INSTALLING THE 8003/8013 FAMILY LAN ADAPTER

1.  Configure the adapter according to your system configuration.

The WDPLUS.SYS device driver is installed at PC boot time

according to the command line specified in the DOS CONFIG.SYS

file (described below).  You must verify that the LAN adapter

configuration corresponds to the software settings given on

the device driver command line.

Jumper settings and configuration information for some adapters

are described in the JUMPERS.DOC file.  For more information,

refer to the "User Installation Guide" that comes with each

adapter.

2.  Insert the 8003/8013 family LAN adapter in your PC.

3.  Power up your PC.

DRIVER INSTALLATION PROCEDURE

For PC-NFS Version 3.0 and above, the WDPLUS driver can be

configured and installed via the PC-NFS NFSCONF Utility.

For PC-NFS Version 2.0, the WDPLUS.SYS driver must be included

in the DOS CONFIG.SYS file, as described below.

To install the WDPLUS driver, follow the procedures outlined

below:

1.  Configure the adapter according to your system configuration,

as described above.

2.  Include the following statement in your CONFIG.SYS file:

DEVICE=\NFS\WDPLUS.SYS /i# /pxxx /myyyy

where

#     is the PC interrupt value selected during

configuration of the adapter,

xxx   is the base I/O port address in hexadecimal,

selected during configuration of the adapter,

and

yyyy  is the 4-digit memory segment base address in

hex you want the driver to use for the adapter

on-board shared buffer memory.

For example, the following statement corresponds to the factory

adapter configuration and a buffer memory base address of

D0000:

DEVICE=\NFS\WDPLUS.SYS /i3 /p280 /mD000

Check that the configuration of the adapter matches the settings

identified in the command line paramenters of the WDPLUS.SYS

driver.

Your # options for the /i interrupt parameter are: 2 through 7

for 8-bit adapters, 2 through 15 for 16-bit and Micro Channel

adapters.

Your xxx options for the /p I/O port parameter are: 3E0, 3C0,

3A0, 380, 360, 340, 320, 300, 2E0, 2C0, 2A0, 280, 260, 240, 220,

or 200.

Your yyyy options for the /m Shared Buffer parameter are: 8000 -

F000, on boundaries equal to the size of the adapter RAM

(8KB=200h, 16K=400h).  The amount of shared memory used by the

adapter is determined automatically by the driver.

3. If your CONFIG.SYS file does not specify at least 20 buffers

and files, you should modify CONFIG.SYS to include the

following:

BUFFERS=20

FILES=20

4. You must have a copy of Sun's PC-NFS (rev. 2.0 or greater).

Refer to that documentation for additional information on

PC-NFS.

Licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation.  All other marks mentioned herein belong to

their respective companies.

Copyright (c) 1990 Western Digital Corporation

All Rights Reserved.