EASY FACTOR ANALYSIS - VERSION 2.5

October 1, 1988

(C) 1987, 1988 Darren Fuerst

All Rights Reserved

+------------------------+

| EFA VERSION 2.5 UPDATE |

+------------------------+

This version of EFA replaces EFA 2.0 (originally released in June,

1987). Version 2.4 was an interim release that corrected a number of minor

bugs in 2.0, and sported a slightly modified user interface. EFA 2.4 was

never released to the public, just to beta-testers. EFA 2.5 is the first

"official" update to 2.0. It includes all of the modifications contained in

2.4, plus a few new things. Some of the changes in version 2.5 include:

1. EFA 2.5 is now compiled with Turbo Pascal 4.0. As a result of

converting EFA to the new version of Turbo, 2.5 runs

significantly faster than 2.0. The new Turbo run-time routines

also autodetect and adapt to the type of video installed in your

system. Thus, there is now only a single version of EFA that will

run on either color or mono machines. EFA 2.5 also produces

somewhat different (hopefully more informative) fatal run time

error messages as a result of the new error handling procedures

in Turbo 4.0.

2. EFA 2.5 now uses LU decomposition to invert the correlation matrix.

This method is faster and more stable than the Gauss-Jordan

method with partial pivoting used in 2.0.

3. In version 2.0, if you used SET PARAMETERS to tell EFA to look for

data on a floppy disk, but didn't have a disk in the drive at

that time, the DOS critical error handler would take control

('Abort, retry...'). This bug has been corrected in version 2.5.

EFA will now report a 'RAW DATA FILE NOT FOUND' nonfatal error,

and return you to the main menu.

4. When setting the data disk path parameter, EFA 2.0 only allowed

lower case letters to be used. You can now use upper or lower

case letters, or a mixture of the two.

5. In EFA version 2.0, when changing the factor conditions (number of

factors or minimum eigenvalue) the default values were not

printed after the input prompts, although they were recognized by

the program. Version 2.5 now prints the default values after the

prompts.

6. Some minor bugs in the printer routines have been squashed.

7. The user interface has been cleaned up slightly. Some users had

complained that tracking the program as it moved from window to

window was sometimes difficult, and that the titles tended to

look a bit cluttered. In version 2.5, the active window always

has a double-line border drawn around it, while inactive windows

have a single-line border. Window titling has been cleaned up.

Some screen updates are also a bit faster.

8. I'd like to thank users that have taken the time to send in bug

reports and registration forms. It's your support that has made

continued work on EFA possible.

+----------------------+

| TERMS AND CONDITIONS |

+----------------------+

EASY FACTOR ANALYSIS is not a public domain program. You are licensed

to use the program subject to the following conditions:

1. With the exceptions noted below, you may freely copy and distribute this

package, provided that you include all files in the original

distribution package, and all files are in their original, unmodified

state. You must not remove or modify either the copyright notice on

the package or this license agreement.

2. With the exceptions noted below, you do not have to register your copy

of EFA to use or distribute it.

3. With the exceptions noted below, registration of EFA is voluntary. If

you choose to register your copy, the fee is $10.00. Registered users

will be notified of upgrades to the package. Registered users may

upgrade to a later version at any time by sending in a blank disk and

$5.00.

4. Registration of this package is NOT voluntary if:

A) This software is used in an educational setting by more than

one person (e.g. classroom, lab, tutorial). In this case the

instructor MUST pay the $10.00 registration fee for the

master copy, plus $1.00 for each additional copy

distributed.

B) This software is placed on a local area network (note that

this does NOT included BBS's or mainframe networks such as

BITNET or ARPA). In this case the network administrator MUST

pay the $10.00 registration fee for the master copy placed

on the network server, plus $1.00 for each machine with

access to that server.

5. In cases where a fee is charged for copying and distributing this

package, that fee may not exceed $10.00 per copy.

6. This package has been carefully tested and to the best of my knowledge

functions as described in this manual. However, I disclaim all

warranties, expressed or implied, including without limitation the

warranties of merchantability and of fitness for any purpose. You are

responsible for determining the fitness or usability of this package.

I will not be liable for damages of any kind arising from the use of,

or inability to use, any of the programs contained in this package.

7. Address all correspondence to:

Darren Fuerst

2255 University Ave. W., #805

Windsor, Ontario

Canada  N9B 1E6

+-----------------------+

| HARDWARE REQUIREMENTS |

+-----------------------+

EASY FACTOR ANALYSIS requires:

* An IBM PC/XT/AT/PS2 or close compatible running DOS 3.x. DOS

version 2.x should work but has not been tested.

* 256k RAM memory.

* 1 floppy disk drive. A 360k DSDD drive is the minimum required;

however, a hard disk or HD floppy disk drive is VERY

strongly recommended. Depending on the size of the data set

you are working with, you may not be able to run all

analyses with just a 360k drive.

* A printer connected to LPT1 if you want hardcopy.

+-----------+

| EFA FILES |

+-----------+

You should have received the following files in this package:

* EFA25.EXE  - EFA program

* EFA25.DOC  - this file.

* TEST.DAT   - a test data file.

* CREATE.EXE - program to create test data sets

+--------------+

| INTRODUCTION |

+--------------+

In the behavioral sciences, factor analysis is one of the more

commonly used techniques in the statistician's bag of tricks. Researchers

working in applied settings are often faced with a very large number of

variables measuring diverse aspects of behavior that may or may not be

related to varying degrees, and may or may not be relevant to a particular

problem at hand. Although people in the physical sciences may chuckle at

this admission, in the behavioral sciences one rarely, if ever, truly knows

what variables are really measuring.

One way of getting a handle on this problem is to examine the

interrelationships between variables (their correlations) in an attempt to

find patterns or groupings of variables. If certain variables tend to group

together (correlate highly with one another), it is reasonable to assume

that those variables are measuring, to some extent at least, some common

process or phenomenon, and by considering what we know about those

variables we may be able to determine what that common process or

phenomenon is. Thus, factor analysis is a statistical technique for

'dimension reduction': it takes the dimensions measured by a set of

variables, calculates the relationships between those original dimensions,

and attempts to reduce them to a smaller set that 'spans' the space defined

by the variables.

This is not as strange or complex as it may sound. For example, the

variables total annual income, number of cars owned, value of residence,

number of shoes owned, occupation, and miles travelled to work may all be

measuring a single major dimension: socioeconomic status. Or, heart rate,

blood pressure, galvanic skin response, skin temperature, respiration rate,

and voice pitch may be measuring the dimension of state anxiety. Factor

analysis may let us determine if our variables are measuring one or more

things (dimensions), and what that thing or things may be.

In addition, once we decide what the important dimensions in our data

are, factor analysis allows us to transform a subject's scores on the

original variables into scores on those dimensions, and reduce the number

of variables that we need to use in subsequent analyses without losing any

important information. Instead of trying to draw conclusions from the

results of statistical tests on 20, 50, or 100 or more variables, we may be

able work with 3 or 4 - a much more manageable task - and come to a better

understanding of our results.

Although the most commonly used forms of factor analysis are fairly

easy to understand, and relatively easy to interpret, the calculations

involved are quite complex. Indeed, it was only with the proliferation of

mainframe computers in the 1960's that factor analysis became practical;

analyses that took months to do with a calculator could now be done in

minutes. On modern mainframes, complex factoring techniques applied to very

large data sets that were previously impossible to do by hand can now be

done in seconds. Analyses that, during the pre-mainframe era could only be

performed by people with large grants and lots of money to spend (on human

"computers"), are now routinely performed by researchers every day.

Unfortunately, many students (particularly undergraduates) never get

the opportunity to learn and apply factor analytic techniques. Although the

math involved in understanding factor analysis at a basic level and the

'mechanics' of the procedure are not beyond most students, statistical

packages that include factor analysis routines are usually complex

mainframe programs (such as SAS or SPSS), or equally complex micro versions

adapted from mainframe packages. Most professors and undergraduate students

simply do not have the time and/or the inclination to teach or learn to

program and debug analyses using mainframe packages (that's why God

invented the graduate student). Having worked for a number of years as a

statistics and computer consultant (both officially and unofficially), I

can attest to the anguish that many students (and professors) go through

when learning to do statistics on a mainframe.

EASY FACTOR ANALYSIS was written to allow even computer neophytes to

perform two common types of factor analysis: principal components analysis

(purists may choke over my calling this factor analysis), and image

covariance factor analysis. The program is menu driven, and extremely easy

to use, even without instructions. The structure of raw data files used by

the program is very simple, and data files can be created by virtually any

text editor, spreadsheet, or database. The program has a good selection of

features, such as variable criteria to limit factoring, varimax rotation,

factor scores for both rotated and unrotated solutions, and flexible

output. The program can also handle up to 100 variables, and an extremely

large number of cases, making it useful for 'real world' problems that even

some expensive micro packages choke on.

Some of the routines used in EFA are based on algorithms published by

Donald Veldman in 1967 (FORTRAN Programming for the Behavioral Sciences :

Holt, Rinehart, & Winston). His book is an excellent source of ideas for

people interested in statistics programming on micros. The original version

of EFA was written in early 1986, using KMMM Pascal on a Commodore computer

(this version is no longer available). In the fall of 1986, this early

version was ported to an MSDOS system running Turbo Pascal (version 3.0).

The program was substantially rewritten to take advantage of the enhanced

features and quirks of IBM micros and Turbo Pascal. I hope that you find it

both educational and useful.

+--------------+

| INSTALLATION |

+--------------+

Installation of EFA simply involves copying the file EFA25.EXE to

either a 'work' floppy disk, or a 'work' hard disk directory.

* Floppy Disk - If you want to run the program on a single floppy disk

drive machine, you should create a work disk that contains only

the EFA program and the raw data file that you want to analyze.

Note that the size of problems that you can work on will be

limited by the storage capacity of your drive.

* Hard Disk - It is very strongly recommended that you use a hard disk

with EFA, at least to hold your data files (the program itself

can be loaded from a floppy). As EFA can create a fairly large

number of files in the course of an analysis, you should create a

separate subdirectory to hold them (e.g. 'C:\EFA', 'C:\FACTOR',

etc.). Copy the program to your working directory.

* Note on RAM Disks - A substantial improvement in execution speed can

be obtained by using a RAM disk to hold data files. The

improvement is most noticeable when calculating a large

correlation matrix or factor scores. However, the size of

problems that you can work on will be limited by the size of your

RAM disk (a minimum size of 360k is recommended). Of course, you

will also lose the results of analyses when you reset your

computer, unless you move all of the files back to a real disk.

* Note on disk caching - If you have EMS or extended memory you can

use it as a disk cache provided that you have the appropriate

software. Using extra memory as a cache is preferable to using it

as a RAM disk as it will speed up all EFA disk operations. On the

author's machine a 384k extended memory cache significantly

improves EFA's performance.

+-------------------+

| USING THIS MANUAL |

+-------------------+

The remainder of this manual covers everything you need to know about

using EFA (and maybe just a little bit more). You don't really need to

study all of it, but you MUST read the following section on EFA data files.

Unless you understand how to set up raw data files to use as input to the

program, you won't be able to run any of the analyses. Beyond that,

however, the program is largely self-explanatory. If you hate reading

manuals and prefer to plunge right into the program, be my guest and do so.

If you don't understand something, you can always come back to the manual.

+----------------+

| EFA DATA FILES |

+----------------+

The EFA program does not contain a data editor of any sort. There are

two reasons for this: 1. an editor would gobble up memory that can be put

to better use; 2. everyone has their own favorite text editor, and pity the

programmer who writes something incompatible with it. All data files read

or written by EFA contain plain ASCII text. EFA data files can be created

or edited with any text editor that can save files as plain ASCII, with no

fancy control codes or strange proprietary file formats (e.g. EDLIN). EFA

files can also be created by most spreadsheet programs that can either save

the contents of a chunk of cells as ASCII text, or print unformatted text

to a disk file. You may be able to create EFA files with the reporting

features of your database package, provided that, once again, it can

produce standard ASCII files. When attempting to create EFA files with a

spreadsheet or database program you may have to experiment a bit to get

output in the proper format.

All data files read or written by EFA have the same general format.

All entries in a data file are separated by at least one space or a

carriage return. The first two entries in any file are two integers

specifying the number of data rows (typically observations), and the number

of data columns (typically variables) in the matrix. These are followed by

a list of column labels, one for each column. Column labels may be up to

ten characters in length. They may contain any combination of printable

alphanumeric characters (except, of course, a space). Remember, there must

be as many labels as there are columns in the data. Following the column

labels are the data rows. A row of data begins with a row label. Once

again, a row label can be up to ten characters long, and may contain any

combination of printable alphanumeric characters. Following the row label

are the actual data values for that row, one value for each column in the

data matrix. There can be no missing values. There must be as many data

rows as the number of rows specified at the beginning of the file.

For example, this is a very small EFA raw data matrix:

5 3

VAR1 VAR2 VAR3

SUBJ1 7 4 3

SUBJ2 8 4 6

SUBJ3 8 6 3

SUBJ4 4 1 9

SUBJ5 2 7 4

This data matrix contains '5' rows (subjects, observations) of data, and

'3' variables (columns) per row, labelled VAR1, VAR2, and VAR3. There is

one row of data per subject. Each row begins with a label that identifies

the subject ('SUBJ1, SUBJ2, ...') followed by that subject's scores on the

three variables. It's that simple.

Note that in data files, the concepts of 'rows' and 'columns' are

LOGICAL rather than PHYSICAL. That is, the program doesn't care where rows

and columns actually begin or end in the file, only that things are in the

correct order. For example, the small file that we just created could

actually look something like this:

5 3

VAR1

VAR2 VAR3

SUBJ1 7

4 3 SUBJ2 8 4 6

SUBJ3 8   6

3

SUBJ4 4 1 9 SUBJ5 2 7 4

This file would still be read properly by the program, since everything is

in the proper order. The reason for this flexibility is that your text

editor may not be able to handle lines longer than 200 characters or so, so

if you have a lot of variables you are free to insert carriage returns

wherever you need them (provided, of course, that you don't break up

individual values).

The only other thing you need to know about data files is how to name

them. All DOS file names consist of two parts: a filename and an extension.

EFA data files can have any valid DOS filename. However, raw data files

used as input to the program must have the extension '.DAT'. Since EFA

creates quite a few files in the course of an analysis, the program uses

file name extensions to keep track of the results of various stages of an

analysis. Forcing you to use the '.DAT' extension for raw data files

ensures that you won't confuse the program by using extensions that EFA

creates itself.

In summary, there are 6 rules for creating EFA data files:

1. Raw data files must have the extension '.DAT'.

2. Raw data files contain plain ASCII characters.

3. All entries in a file are separated by at least one space or a

carriage return.

4. The first two entries in a file specify the number of data

rows and the number of data columns.

5. The next set of entries contain the column labels, one for

each data column.

6. Rows of data contain a row label, followed by the numeric data

values - one value for each column in the matrix.

+-----------+

| USING EFA |

+-----------+

GENERAL OPERATION

When you first start EFA, a copyright notice will briefly appear,

followed by the main screen. There are two parts to the main screen. The

first part is a 'parameters' window that appears across the top of the

screen. The parameters window tells you the name of the 'Active Data File',

which the is the data file that the program is currently working with. It

also tells you the 'Data Path', which is where all data and work files are

read from or written to. It also tells you the current analysis 'Method'

(in the current version, either Principal Components or Image Factor

Analysis). The second part of the screen is the 'main menu' window, which

runs down the left side. The available options are numbered 1-6 and 9. '9'

is the Quit option: the one you select when you want to leave the program.

To select an option from the main menu, you simply press the number

corresponding to the option that you want. The option which you select will

be highlighted. The selected option does not become active until you press

the return key (ENTER), so you can change your mind or correct a typing

mistake should you need to. If you select options 2-6, a new 'analysis'

window will open up on the right side of the screen. The analysis window

might present you with another menu of options, print messages to keep you

informed about an analysis that is in progress, or tell you about errors

that have occurred in the course of an analysis.

Apart from selecting menu options, the program sometimes wants you to

type in additional information that it needs in order to perform an

analysis. You can always tell when EFA needs you to do this, because it

will print a 'prompt' (a message that gives you idea about what it wants),

and turn on a blinking cursor. Whenever you see a blinking cursor it means

that the program is waiting for you to type something in. Usually, the

program has pre-set values for these items which you can select by simply

hitting the return key, without typing anything else. This can save you

typing, or if you accidently select an option that results in a prompt, or

you don't understand what the program wants, you can rescue yourself with

the return key.

Usually, when you sit down to analyze some data, you will work through

all of the menu options in sequence. Generally speaking, you cannot select

the options out of order. If you try to, the program will print an error

message and remind you of this. The reason for this is that each analysis

option relies on information written to disk files by previous options.

Thus, you cannot extract eigenroots and vectors from a correlation matrix

until you have calculated correlations, and you can't rotate vectors before

you've extracted them, and so on. The exception to this rule is option 6,

'Print Results'. You can print the results from any completed stage of an

analysis at any time since you may want to examine the results of one stage

while another is running, or make decisions about what you want to do next

based on the initial analyses. Of course, you can also Quit (option 9) at

any time. As long as you don't erase any of the files created to that

point, you can pick up where you left off when you restart the program.

OPTION 1: Set Analysis Parameters

Before you can start running analyses, you must give EFA some general

information about what you want to do. When you select Option 1, the 'SET

PARAMETERS' window at the top of the screen will become active. EFA will

first prompt you for the 'File Name', and print the name of the currently

active file if there is one. This is the name of the data file that you

wish to analyze (e.g. TEST.DAT). You do not have to include the '.DAT'

extension - since all raw data files must have a '.DAT' extension it is

assumed. If you have already set the proper data file name, you can just

hit the return key. Otherwise, type in the file name and press the return

key.

Next, EFA will prompt you for the 'Data Disk Path'. This tells EFA the

disk drive and directory it is to use for reading and writing files. If you

have already set a data path, it will be printed after the prompt. If you

don't want to change the path, just hit the return key. Otherwise, enter

the full path to be used for data files (e.g. A:, C:\EFA, C:\FACTOR\DATA\,

etc.). If you forget the final slash (\) EFA will append it for you.

Currently, EFA supports drives A to G.

Finally, EFA will ask you for the type of analysis that you want to

perform. If you want to do a principal components analysis, enter a 'P' and

hit return. If you want to perform image factor analysis, enter an 'I'. If

you want to accept the current default, just hit return.

After you've given EFA all of this information, it checks to make sure

that the raw data file exists, and that the file doesn't contain more than

100 variables (the current maximum). If either of these two errors occur,

EFA prints the appropriate message, and resets the file name to null

(nothing). You'll have to give the program the name of a valid file before

you can run any analyses.

EFA saves these parameters in a file called 'HSEKEEP.DAT' on the

default drive (the one you loaded the program from). Whenever you start

EFA, it looks for this file, and if possible reads the parameters from it.

Unless you enjoy typing in the parameters every time you run EFA, don't

erase this file.

OPTION 2: Calculate Correlations

After you've set the parameters, you can start analyzing your raw

data. The first thing that you will want to do is calculate a correlation

matrix from it. When you select Option 2, the 'CORRELATION ANALYSIS' window

will open on the right side of the screen. The actual calculation process

is entirely automatic. EFA will keep you informed about what is going on

with messages like 'Starting Calculations', 'Reading subject #x', 'Writing

Matrix To Disk', and so on.

If you're trying to do an image covariance analysis, the correlations

module also calculates the image covariance matrix. The calculation of this

matrix involves, among other things, computing the inverse of the

correlation matrix. You should be aware of two things. First, calculating

the inverse of a large matrix takes time, so be patient! Second, if your

correlation matrix is singular, it doesn't have an inverse, and you can't

perform an image covariance analysis. The program will print an error

message if this is the case. It will also reset the analysis type parameter

to principal components, so you don't have to go back to Option 1 and do it

yourself.

OPTION 3: Extract Roots and Vectors

After you've calculated the correlation matrix (or the image

covariance matrix), you can proceed to extract the characteristic roots and

vectors from it. When you select Option 4, the 'SET FACTOR CONDITIONS'

window will open on the right side of the screen. This is where you specify

the conditions to limit the number of factors extracted. Option 1 allows

you to specify the absolute maximum number of factors that you want to

extract. The default value is the number of variables in your data. Option

2 is the minimum acceptable eigenvalue. Extraction will cease as soon as a

factor with an eigenvalue less than this is extracted. The default value is

1.0.

Note that these two conditions are evaluated in an 'either/or' fashion

by the program. That is, if any one condition is met, extraction stops. If

you want to extract a specific number of factors, you may or may not have

to change the eigenvalue criterion. If all the factors that you want to

extract have eigenvalues greater than 1.0, you just have to change the

number of factors criterion to the value that you want. On the other hand,

if some of the factors that you want to extract have eigenvalues less than

1.0 (say, 0.95), then you must lower the eigenvalue criterion (e.g. 0.90),

as well as setting the number of factors to the value that you want

(provided, of course, that you know how many you want). If for some strange

reason you want to extract as many factors as there are variables, you can

set the eigenvalue criterion to some ridiculously low value (e.g.

0.0000000001). Although this sounds complicated, it's not, and you'll get

the idea once you play with the software.

When you have set the two stopping criteria (or accepted the

defaults), select Option 3 to begin extraction. EFA will keep you informed

as to what root it's trying to extract, and the value of the previously

calculated root. When extraction is complete, you will be informed of the

value of the trace of the matrix, and the percentage of trace accounted for

by the roots extracted. The program pauses at this point because you'll

probably want to know this information. Press the return key to get back to

the main menu.

OPTION 4: Rotate Vectors

After you've extracted the roots and vectors, you'll probably want to

rotate them in an attempt to find 'simple structure'. If you select Option

4, the 'VARIMAX ROTATION' window opens up on the right side of the screen.

The rotation process is automatic and requires no input from you. The

program keeps you informed of the last angle of rotation that it

calculated. This information is provided just to let you know that the

module is indeed working and hasn't disappeared into never-never land.

Rotation can be time consuming if you have many variables and factors, so

be patient.

OPTION 5: Calculate Factor Scores

Once you have extracted and rotated some factors, you might want to

calculate each subject's scores on those factors. If you select Option 5,

the scoring window will open on the right side of the screen and tell you

'SCORING IN PROGRESS'. Scoring can be a time consuming process if you have

many subjects and variables. The program will let you know when it's about

half done with the message 'Unrotated Scores Complete'. The rotated scores

take a little longer since the program has to do a number of intermediate

calculations before it can actually compute the rotated factor scores, and

it has to swap some information back and forth from the disk to do these

calculations (another good reason to use a hard disk!).

OPTION 6: Print Results

There's not much point in doing all these analyses if you can't look

at the results. Selecting Option 6 will open up a print control menu on the

right side of the screen. You can print your raw data file (to check for

typing errors), and the results of any of the analyses that you have

performed. Choose the analysis results that you want to look at and press

return. You will then be asked if you want output printed on the screen or

the printer. Enter 'S' or 'P' as the case may be. Screen output is the

default.

Some analyses create multiple result files that are printed in a fixed

order. The files printed by each option are:

Option                             Output

---------------------------        ------------------------------

1. Raw Data                        raw data matrix

2. Correlation Results             variable means

unadjusted standard deviations

correlation matrix

image covariance matrix

3. Unrotated Factor Results        eigenvalues

percentages of trace

unrotated factor pattern

4. Varimax Rotation Results        percentage communalities

percentages of trace

rotated factor pattern

5. Factor Scores                   unrotated scoring weights

unrotated factor scores

rotated scoring weights

rotated factor scores

If you select screen output, the screen will clear, and the program

will start printing the first file. When a full screen of information has

been printed, the program will pause and prompt you with 'Press RETURN to

continue, S to skip'. If you press the return key, the next screenful of

data will be printed, and so on until you reach the end of the file. If

there is another result file to be printed, the process continues until all

the results have been displayed. At that point, the print control menu will

appear again, and you can look at the results of other analyses or quit to

the main menu. If you press S while a file is being displayed, output of

that file stops, and printing restarts with the next file if there is one.

If you select printer output, the relevant files will be sent to the

printer instead of the screen. EFA doesn't do any fancy formatting, so it

should work with virtually all printers. It expects to talk to an 80 column

printer with 66 line pages.

There are a few things you should be aware of when printing results.

First, you might want to use a software print spooler to speed things up.

This program has been tested with MSPOOL2, an excellent public domain

spooler, and has been found to get along just fine with it. However, other

spoolers, or redirection utilities, may not work. Since Turbo Pascal is

distinctly user hostile when it comes to printer errors (it may dump you

out to DOS), EFA tries to avoid problems by checking the printer status via

interrupt 17H before sending output to the printer. Any software you

install that grabs interrupt 17H must be able to return the proper status

to EFA, or the program might hang. Second, you cannot cancel printing once

started, so be sure that you really do want printed output before hitting

the return key.

Finally, make sure that you have enough paper on hand to print what

you want - analyses on large data sets can run to quite a few pages of

output. EFA does handle paper out errors gracefully, but you must have

paper to add to your printer if it runs out, since you can't cancel

printing once started. EFA tries to not waste too much paper - before it

prints a file it checks to see if it will fit on the current page. If it

can, great! If not, it advances to the next page before it starts printing

the file.  Unfortunately, most files won't fit on a single page (vector

files, like means and standard deviations, usually do). The print routines

also try to avoid "widowing" at the bottom of a page, which may increase

paper use a bit, depending on the number of rows in a matrix.

+----------------+

| ERROR MESSAGES |

+----------------+

There are two general types of errors that can occur when running EFA:

nonfatal errors, which aren't serious enough to cause the program to abort,

and fatal errors, which are bad enough to bring the program to a screeching

halt. Below is a list of errors which you might encounter, and some

suggestions for dealing with them. Note that EFA recognizes and reports

many more fatal errors than are listed below. Check your DOS manual for an

explanation of any errors that are not explained here.

NONFATAL ERRORS

Error Message                 Action

------------------------      ----------------------------------------

TOO MANY VARIABLES            Your raw data file contains too many

variables for the program to analyze. The

maximum number of variables allowed in the

current version of EFA is 100. You must edit

some of the variables out of your data set.

This error might also be caused by specifying

the wrong number of variables in your data

(e.g. 111 instead of 11). You may also be

trying to analyze a data file that doesn't

conform to EFA format. Make sure you've

entered the correct file name.

RAW DATA FILE NOT FOUND       EFA can't find the data file that you want to

analyze. Make sure that you've entered both

the proper file name and the correct data

path. Remember that EFA raw data files must

end with the extension '.DAT'. Make sure your

raw data is indeed where you think it is.

REQUIRED FILES NOT FOUND      EFA can't find the prerequisite files for a

particular analysis. You may be trying to run

the analyses out of order, or print the

results of an analysis that you haven't run

yet. It is also possible that you have

changed the data path since you did the

analyses, and EFA can no longer find the

files that it needs. It is also possible that

you have changed the type of analysis in the

middle of working through the options.

Principal components and image analysis

produce, and require, slightly different

files, so work through the options again, in

order. If you've quit, and restarted the

program at a later time, it is possible that

you've erased some of the results files.

Error Message                 Action

------------------------      ----------------------------------------

THE CORRELATION MATRIX IS     You are attempting to perform image

SINGULAR                      covariance analysis on a singular correlation

matrix. This cannot be done, period. You can

either settle for principal components

analysis, or try to modify your data so as to

produce a nonsingular correlation matrix. Try

deleting variables that have extremely high

correlations with other variables.

PRINTER NOT READY             Your printer may be not connected, not turned

on, not online, or out of paper. Fix the

problem and press return to continue

printing.

FATAL ERRORS

Error Message                 Action

------------------------      ----------------------------------------

USER BREAK / INTERRUPTION     You have pressed <Ctrl>C, which aborts the

program. EFA does not disable ^C checking so

you can break out of the program if you need

to. You should be careful about doing this,

particularly when the program is accessing

the disk.

I/O ERROR                     This error should only be possible during

disk activity. It usually occurs when there

are mistakes in your raw data file. The most

common ones are specifying the wrong number

of variables, and/or putting in too many or

too few labels. Check your data file

carefully, following the six rules outlined

above. Make sure that you specify the correct

number of subjects and variables. This error

may also happen if you're using floppies, and

don't put a disk in the drive, or if a disk

gets full or otherwise unusable. If this

happens with a hard disk, the best thing to

do is cry.

HEAP AND STACK COLLISION      This error indicates the program has run out

of memory. If you have less than 640k in your

machine, now might be a good time to upgrade.

If you have 640k, try removing all of your

memory resident software.

FLOATING POINT OVERFLOW       Some of your variables may have very

large values. Try dividing such variables

by a constant amount (e.g. 100, 1000).

UNKNOWN RUNTIME ERROR         Something very bizarre has happened to the

program. You may have received a corrupted

copy of the package.

+-------------+

| BUG REPORTS |

+-------------+

If you encounter any error messages not listed above, or if, despite

your best efforts, you can't solve one of the above errors, I'd like to

hear about it. If you find that the program ever gives incorrect results,

I'd DEFINITELY like you to tell me. When reporting bugs, it would be very

helpful if you would include a printout of your data file, as well as a

complete description of the problem. If the data file is large, and you are

a registered user, please include a copy on floppy disk. The disk will be

returned to you, at my expense. If the bug is successfully squashed, your

disk will include the updated version of EFA. All registered users will

be informed of updates and bug fixes as they become available.

+-------------+

| OTHER STUFF |

+-------------+

There are two more files in the distribution archive that you might be

interested in. TEST.DAT is a test data file that contains the scores of 132

subjects on the 16 subscales of the Personality Inventory for Children

(note that the scores have been modified - they are based on real data

however). You can use this data file to test EFA on your system, and make

sure that the software is happy with it. The second thing you may want to

try is CREATE.EXE. This program creates EFA format data files that contain

semi-random data (there is enough structure to the data that it produces to

allow you to extract a lot of factors, do image analysis, and so on). You

can create files of any size with it. CREATE lets you test EFA on a data

file that is comparable in size to a real data set that you may be thinking

of analyzing. It will allow you to determine if you have enough storage to

make using EFA practical with your data BEFORE you go to the trouble of

actually creating an EFA data file.

EFA REGISTRATION FORM

Name       _____________________________________________

Address    _____________________________________________

_____________________________________________

Prov/State __________________  Postal Code _____________

If there are features you'd like to see added, or things you'd

like to see changed in EFA, please tell me about them below.

Comments   ___________________________________________________________

___________________________________________________________

___________________________________________________________

___________________________________________________________

___________________________________________________________

___________________________________________________________

___________________________________________________________

___________________________________________________________

---------------------------------------------------------------------------

EFA BUG REPORT

Name       _____________________________________________

Are you a registered user?     YES ___   NO ___

Please describe the problem in the space below, or attach a separate

sheet if necessary. Remember to include a copy of your data. If the

program is giving you an error message make sure you tell me what it

is.

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

---------------------------------------------------------------------------

Send this form to:

Darren Fuerst

2255 University Ave. W., #805

Windsor, Ontario

Canada  N9B 1E6