.Y 1 100 110 2 0 1 0 75 8 12 1 0 25 10

1. CP/M-80 KERMIT

Program:    Bill Catchings, Columbia University, with contributions from

Charles  Carvalho  (ACC),  Bernie  Eiben  (DEC),  Nick  Bush

(Stevens), John Bray (University of Tennessee), Bruce Tanner

(Cerritos College), Greg Small (University of California  at

Berkeley),	 Kimmo	 Laaksonen   (Helskini	 University   of

Technology), and many others.

Language:   8080 Assembler or MAC80

Version:    4.03

Date:	    December 6, 1984

Documentation:

Charles Carvalho, ACC; Frank da Cruz, Columbia

KERMIT-80 Capabilities At A Glance:

Local operation:		     Yes

Remote operation:		     No

Transfers text files:		     Yes

Transfers binary files:	     Yes

Wildcard send:		     Yes

Control X/Z interruption:	     Yes

Filename collision avoidance:	     Yes

Can time out:			     Yes

8th-bit prefixing:		     Yes

Repeat count prefixing:	     No

Alternate block checks:	     Yes

Terminal emulation:		     Yes, VT52 and others

Communication settings:	     Yes; duplex, parity

Transmit BREAK:		     Yes; some versions

IBM communication:		     Yes

Transaction logging:		     No

Session logging (raw download):    Yes

Raw upload:			     Yes

Act as server:		     No

Talk to server:		     Yes; SEND, GET, FIN, BYE

Advanced commands for servers:     No

Local file management:	     Yes; DIR, ERA, SET DEFAULT disk

Handle file attributes:	     No

Command/init files:		     No

Printer control:		     Yes, limited

1.1. Summary of CP/M

CP/M-80 (version 2.2) has only five built-in commands, and they all deal

with files; other functions are done by invoking programs.

CP/M file specifications are of the form DEV:XXXXXXXX.YYY, where

DEV:		is a device name, normally  the	 A: or	B: floppy.    If

omitted, the device name defaults to your connected dis-

kette.

.e

XXXXXXXX	is a filename of up to 8 characters.

YYY		is the file type, up to 3 characters.

File names and file types may contain letters, digits, and some	 special

characters, including dash, dollar sign, and underscore, but no imbedded

spaces.	 Upper and lower case letters are equivalent.

"Wildcard" file-group specifications are permitted  in	file  names  and

file types (but not device names) within certain contexts; a "*" matches

a whole field, a "?"  matches a single character, including space.   Ex-

amples:	 "*.F??" specifies all files whose types start with F and are 1,

2, or 3 characters long; "F?.*" specifies all files  whose  names  start

with  F	 and  are  no more than two characters long (before the trailing

spaces).

The five CP/M commands are:

DIR file	Lists the the names of the specified files.  The default

file specification is "*.*".  Example: "DIR B:*.FOR".

ERA file	Erases	(deletes)  the	specified file(s); wildcards al-

lowed.

REN new old	Changes the name of a file from old to new, e.g.

"REN NEW.FOR=OLD.FOR".

SAVE		Saves the specified number of memory blocks into a file.

TYPE file	Types  the  specified	file   on   the	  screen,   e.g.

"TYPE FOO.TXT".

The most important programs are:

STAT		Gives  statistics  on  disk  usage;  sets  and	displays

IOBYTE.

PIP		Peripheral Interchange	Program.    Copies  files.    In

response to the "*" prompt, give a command of the form

disk:outfile=disk:infile

Wildcards  ("*"	 for a whole field or "?"  for a letter)

can be used.  Examples: "A:=B:*.*" to copy a whole disk,

"A:=B:*.FOR"  to copy all the Fortran programs from disk

B to disk A. If the disk specification is omitted,  your

"connected" disk is assumed.  Command line arguments are

also accepted, e.g. "PIP A:=B:*.*".

For further information on CP/M, consult your microcomputer manual or  a

CP/M handbook.

.e

1.2. Kermit-80 Description

Since  Kermit-80  runs on a standalone micro, it is always in control of

the screen -- it is always local.  Thus, it always keeps the screen  up-

dated  with  the  file	name  and  the packet number, whether sending or

receiving.  Kermit-80 is capable of an imprecise or "fuzzy"  timeout  on

an input request, and can break deadlocks automatically.  In most cases,

this is not important, because the KERMIT on  the  other  side	is  most

likely	able to handle the timeouts.  The timeouts done by Kermit-80 are

fuzzy because they depend on the speed of the processor and  other  fac-

tors that can vary from system to system.

If  despite the timeout capability, the transmission appears to be stuck

(and you can tell that this has happened if the screen fails  to  change

for  a	while) you can type carriage return to have the micro do what it

would have done on a  timeout, namely NAK the expected packet  to  cause

to  foreign  host  to  send  it	 again	(or, if the micro is sending, to

retransmit the last packet).  Micro/micro or micro/IBM-mainframe  trans-

fers could require this kind of manual intervention.

File transfers may be interrupted in several ways.

Control-C	This  will  return  you	 to  Kermit-80 command level im-

mediately, so that you can connect back	 to  the  remote

system, or take any other desired action.

Control-X	When  sending a file, this will terminate the sending of

the current file with a signal	to  the	 KERMIT	 on  the

other  side to discard what it got so far.  If there are

more files to be sent, KERMIT-80 will go on to the  next

one.   When receiving a file, KERMIT-80 will send a sig-

nal to the remote KERMIT to stop sending this file.   If

the  remote  KERMIT understands this signal (not all im-

plementations of KERMIT do), it will  comply,  otherwise

the file will keep coming.  In any case, the remote KER-

MIT will go on to the next file in the group, if any.

Control-Z	Like Control-X, except if a file group is  being  trans-

mitted,	 this  will  stop the transmission of the entire

group.	If only a single file is being	transmitted,  it

works exactly like Control-X.

Carriage Return If  you type a carriage return Kermit-80 will resend the

current packet.	 You may do this repeatedly, up	 to  the

packet	retry  limit  (somewhere between 5 and 16 times)

for a particular packet.

.e

KERMIT-80 COMMANDS

KERMIT-80 uses the DECSYSTEM-20 keyword style command  language.    Each

keyword	 may  be  abbreviated  to its minumum unique length.  "?" may be

typed to request a menu of the available options for the  current  field

at  any	 point in a command.  ESC may be typed at any point in a command

to fill out the current keyword or filename;  if  sufficient  characters

have  not  been	 typed to identify the current field uniquely, KERMIT-80

will sound a beep and allow you to continue from that point.

CONNECT Establish a "virtual terminal" connection to any host  that  may

be  connected  to  the	serial port, i. pass all typein to the

serial port and display all input from the serial  port	 on  the

screen.	   Also,  emulate  a  DEC  VT52 to allow cursor control,

screen clearing, etc., if VT52-EMULATION is ON (see  below),  in

which  case you should also set your terminal type on the remote

host to VT52.  (Some versions emulate other terminals.)	 The es-

cape  character	 differs from micro to micro; when you issue the

CONNECT command, the micro will print a message telling you  how

to  get	 back.	 The escape sequence is generally an uncommonly-

used	control	   character,	  like	   CTRL-backslash     or

CTRL-rightbracket, followed by a single letter "command".

C   Close Connection, return to Kermit-80> command level.

S   Display  Status  of	 connection, but maintain remote connec-

tion.

?   List available single-character commands.

0   (zero) Send a null (0) character.

B   Send a BREAK signal.  Only some systems provide  this  func-

tion.

^]  (or	 whatever  - a second copy of the escape character) Send

the escape character itself to the remote host.

SEND filespec

Send  file(s)  specified  by filespec to the remote Kermit.  The

filespec may contain CP/M wildcards.

RECEIVE Receive file(s) from the remote Kermit.	 Store	them  under  the

names  provided in the file headers supplied by the remote host.

If the names aren't legal, use as many legal characters from the

name  as  possible  (see  the  description  of	SET FILE-WARNING

below).	 If there's a conflict, and FILE-WARNING is ON, warn the

user  and  try to build a unique name for the file by adding "&"

characters to the name.

GET filespec

When  Kermit-80	 is  talking to a Kermit Server on the host, you

should use the GET command to request the server to  send  files

to you, for example:

get hlp:k*.hlp

.e

Limitation:  If	 you request an alternate block check type using

the SET BLOCK command, the GET command will not	 communicate  it

to  the	 remote	 server.   If you want to have type 2 or 3 block

checks done when getting files from the server, you have to  is-

sue  the  appropriate SET BLOCK command to the remote KERMIT be-

fore putting it in server mode.

LOG filespec

When CONNECTed to a foreign host as a terminal, log the terminal

session to the specified  diskette  file.    This  functionality

depends	 to  some  extent  on  the  remote  host's ability to do

XON/XOFF  flow	control,  and  does  not  guarantee  a	complete

transcript  (after all, that's what the KERMIT protocol is for).

The log file is closed when the connection is closed  by  typing

the  escape  character	followed by the single-character command

"C".

TRANSMIT filespec

Send  the  specified  file to the system on the other end of the

connection as though it were being typed at  the  terminal,  one

line  at  a  time.    No  KERMIT protocol is involved.	You must

manually confirm each line.  This is useful for sending files to

systems	 that don't have a KERMIT program.  During transmission,

you may type the escape	 character  followed  by  one  of  these

single-character commands:

C   Cease transmission

R   Re-transmit the previous line

BYE	When  talking to a remote Kermit Server, this command shuts down

the server and logs it out, and also  exits  from  Kermit-80  to

CP/M command level.

LOGOUT	Like BYE, but leaves you at Kermit-80 command level.

FINISH	Like LOGOUT, but shuts down the remote server without logging it

out.  Leaves you at Kermit-80 command level; subsequent	 CONNECT

commands will put you back at host system command level.

VERSION Show  the  name,  edit	number,	 and edit date of several of the

modules that make up Kermit-80.

SET parameter [value]

Set  the  specified  parameter to the specified value.	Possible

settings:

WARNING ON (or OFF)

Warn  user  of	filename  conflicts when receiving files

from remote host, and attempt to generate a unique  name

by  adding  "&"	 characters  to	 the  given name.  ON by

default.

.e

VT52-EMULATION ON (or OFF)

When connected as a terminal to a foreign host, controls

whether the micro emulates a VT52  or  runs  in	 "native

mode".    VT52  emulation  is  ON  by default, except on

micros that already have  terminal  functionality  built

in,  such  as  the  DEC	 VT180 and DECmate (these act as

VT100-series terminals).   Some	 systems  emulate  other

terminals, like the ADM3A; see table 1-3.

LOCAL-ECHO ON (or OFF)

When you CONNECT to a remote host, you must  set  LOCAL-

ECHO  ON if the host is half duplex, OFF if full duplex.

OFF by default.

ESCAPE	Change the escape character for virtual terminal connec-

tions.	  Kermit-80  will  prompt you for the new escape

character, which you enter literally.

BAUD-RATE

Change	the  baud rate of the communications port.  This

command only works  on	some  systems.	  value	 is  the

numeric	 baud  rate (300, 9600, etc.) desired.	Type SET

BAUD followed by a question mark for a list of supported

baud  rates.	On systems that do not support this com-

mand, you must set the port baud rate from CP/M or other

setup mechanism outside of KERMIT-80.

PARITY	Sets  parity  for outgoing characters to one of the fol-

lowing: NONE, SPACE, MARK, EVEN, or ODD.  On  input,  if

parity	is  NONE,  then	 the  8th bit is kept (as data),

otherwise it is stripped and ignored.  The  parity  set-

ting applies to both terminal connection and file trans-

fer.  If you set parity to  anything  other  than  none,

KERMIT-80  will	 attempt  to  use "8th bit prefixing" to

transfer binary files.	If  the	 other	KERMIT	is  also

capable	 of  8th bit prefixing, then binary files can be

transferred successfully; if not, the 8th  bit	of  each

data  byte  will be lost (you will see a warning on your

screen if this happens).

TIMER ON (or OFF)

Enable	or  disable the "fuzzy timer".	The timer is off

by default, because in the normal case KERMIT-80 is com-

municating  with  a  mainframe	KERMIT	that has its own

timer.	Mainframe KERMIT timers tend to be more	 precise

or  adaptable  to  changing  conditions.  You should SET

TIMER ON if you are communicating  with	 a  KERMIT  that

does  not have a timer.	 You should SET TIMER OFF if you

are communicating over a network with long delays.

IBM ON (or OFF)

Allow the transfer of files to and from an IBM mainframe

computer.  This makes Kermit-80 wait for  the  IBM  tur-

naround character (XON), ignore parity on input, add ap-

propriate parity to output, and use local echoing during

CONNECT.  As distributed, KERMIT-80 uses MARK parity for

IBM communication.  If you don't give this command,  IBM

mode  is  OFF.	  Since	 IBM VM/CMS KERMIT does not have

timeout capability, SET IBM ON also turns on the  "fuzzy

timer" automatically.

BLOCK-CHECK-TYPE

The options are:

1-CHARACTER-CHECKSUM

Normal, default, standard 6-bit checksum.

2-CHARACTER-CHECKSUM

A 12-bit checksum encoded as two characters.

3-CHARACTER-CRC-CCITT

A  16-bit  CCITT-format Cyclic Redundancy Check,

encoded as 3 characters.

The 2 and 3 character options should only be used  under

conditions  of extreme line noise.  Many implementations

of KERMIT only support the single character checksum.

FILE-MODE

Tells KERMIT-80 what kind of file it is sending, so that

KERMIT can correctly determine the end of the file.  SET

FILE BINARY means to send all the 128-byte blocks of the

file, including the last block in its entirety; SET FILE

ASCII  is  used	 for  text files, and transmission stops

when the first Control-Z is encountered anywhere in  the

file (this is the CP/M convention for marking the end of

a text file).  SET FILE DEFAULT tells Kermit to	 attempt

to  determine  the file type by examining the file being

transmitted.  If a Control-Z  appears  before  the  last

block  of the file, it is assumed to be BINARY; if, when

the first Control-Z is encountered, the remainder of the

file  contains	only  control-Z's, it is assumed to be a

text file.  Unfortunately, not	all  programs  fill  the

remainder  of  the  last  record  of  a	 text  file with

Control-Z's, so this algorithm is not always successful.

If  binary transmission is used on a text file, some ex-

traneous characters (up to 127 of them)	 may  appear  at

the  end  of  the  file	 on the target system.	If ASCII

transmission is used on a binary file, the  entire  file

will not be sent if it happens to contain any data bytes

that correspond to Control-Z.

DEFAULT-DISK

This  allows  you  to set the default disk as source and

destination of file transfers.	  In  addition,	 issuing

this  command causes you to switch to the specified disk

and log it in, write-enabled.  The  colon  must	 be  in-

cluded in the disk name (A:).  The selected disk appears

in your KERMIT-80 prompt, for instance

Kermit-80 A:>

PORT	Allows you to  switch  between	different  communication

ports.	  This	command is not available on all systems.

Type SET PORT ? for a list of  valid  options  for  your

system.

PRINTER ON  or OFF.  Turns copying of CONNECT session to printer

on and off.  No attempt is made to do buffering or  flow

control; Kermit assumes the printer can keep up.

DEBUG	ON  or	OFF.   Enables/disables displaying of packets on

the screen during file transfer.

DIRECTORY

This provides a directory listing of the specified files.  If no

files are specified, all files on the default disk  are	 listed.

File  sizes,  in K, are included.  You may interrupt the listing

at any time by typing any  character.	 The  listing  (even  if

interrupted)  concludes	 with  a  display  of the amount of free

storage left on the disk.

ERASE	This executes the CP/M ERA command  on	the  specified	file(s).

The names of the files being erased are not displayed.

1.3. Kermit-80 Flavors

Many  of  the  systems supported use an external terminal, rather than a

built-in console.  Kermit may be further customized for these systems by

defining  (at  assembly time) the terminal type to be used.  If the ter-

minal type is unknown or does not match any of the existing terminal op-

tions,	the  generic "CRT" option may be selected.  In this case, Kermit

cannot do fancy screen control during file transfer; it simply types the

file names, packet numbers, and messages in sequence across and down the

screen.	 This works best if you	 can  put  your	 micro	or  terminal  in

"autowrap"  mode;  otherwise  the  packet  numbers  will  pile up in the

rightmost column; the filenames and messages will always appear on a new

line, however.	If no specific terminal has been selected, Kermit cannot

do VT52 emulation; it can act as a "dumb terminal" (sometimes  called  a

"glass TTY"), or else its own built in terminal firmware provides cursor

control functions independent of the Kermit program.

1.3.1. Generic Kermit-80

"Generic Kermit-80" is an implementation of Kermit that	 should	 run  on

any 8080-compatible CP/M 2.2 system with no modification at all, or per-

haps only a minor one.	Unlike other Kermit-80 implementations, it  con-

tains  no  system-dependent manipulation of the serial port.  All I/O is

done with standard CP/M BIOS calls, and I/O redirection	 is  done  using

the  CP/M IOBYTE function, which, according to the Digital Research CP/M

Operating System Manual, is an optional feature of any	particular  CP/M

implementation.	   If  your system does not provide the IOBYTE function,

Generic Kermit-80 will not work; furthermore, not all systems  that  im-

plement	 IOBYTE do so in the same way.	The SET PORT command may be used

to select the devices to be used for input and output.	Table 1-1  lists

the options to the SET PORT command and their effects.

.e

------------------------------------------------------------------

SET PORT xxx	input from	output to

CRT	    CRT:	    CRT:

PTR	    PTR:	    PTP:

TTY	    TTY:	    TTY:

UC1	    UC1:	    UC1:

UR1	    UR1:	    UP1:

UR2	    UR2:	    UP2:

Table 1-1:   Kermit-80 SET PORT Options

------------------------------------------------------------------

The  default is SET PORT PTR.  In all cases, the console (CON:) and list

(LST:) devices used are those selected when Kermit is started.

The reason all Kermit-80 implementations aren't generic is that  a  good

deal  of  speed is sacrificed by getting all services from the operating

system.	 While a specific implementation of Kermit-80  may  be	able  to

operate	 at  4800, 9600, or even 19200 baud, Generic Kermit will fail to

work on some systems at speeds in excess of 1200  baud.	   In  addition,

many  features of Kermit require more specific knowledge of the hardware

involved -- Generic Kermit cannot send a BREAK	signal,	 or  change  the

baud rate.

1.3.2. CP/M 3 Kermit

CP/M  3	 Kermit	 should run on most CP/M 3 (CP/M-Plus) systems.	 It uses

the auxilliary port (AUX:) to communicate to the remote Kermit.	 The SET

BAUD  and  SET PORT commands are not supported; nor can a BREAK be sent.

Like Generic Kermit, a terminal may be selected at assembly time.

1.3.3. System-Specific Versions

There are also many versions of Kermit-80 tailored to specific	systems.

Most  of  these	 operate  uniformly, but some of them take advantage (or

suffer limitations) of the specific system.  Here are some of  the  spe-

cial features for particular systems:

Apple II -- two variations:

APMMDM:

Apple with Z80 Softcard and Micromodem	II  in	slot  2	 Dialout

capability  provided  in  connect  command; user is prompted for

phone number if carrier is not present.	  During  connect  mode,

^]D  drops  carrier.	BYE  command  also  causes carrier to be

dropped.

.e

AP6551:

Apple  with  Z80  Softcard,  and  one of several 6551-based com-

munication cards; the slot number is  a	 compile-time  parameter

(default  is  slot  2).	   SET	BAUD-RATE  supported; speeds are

110-19200 baud.

BigBoard II:

Uses serial port A. To use port B, change mnport, mnprts, and baudrt

and reassemble.  Can  generate  BREAK.    SET  BAUD-RATE  supported;

speeds are 300-38400 baud.

Digicomp Delphi 100:

SET BAUD-RATE supported; speeds are 50-19200 baud.

CPT-85xx word processors:

Can	 generate  BREAK.    SET BAUD-RATE supported; speeds are 50-9600

baud.

DEC DECmate II word processor (with Z80 card):

Can generate BREAK.

DEC VT180 (Robin):

Three output ports, referred  to  as  COMMUNICATIONS,  GENERAL,  and

PRINTER.  Can generate BREAK.

Intertec SuperBrain:  SET BAUD-RATE supported; speeds are 50-19200 baud.

Kaypro:

Should work on most Kaypro models, as well as some	related	 systems

(Ferguson  BigBoard	 I, Xerox 820).	 For the newer Kaypros with mul-

tiple ports, Kermit uses the one labeled "serial  data";  it  cannot

use	 the  serial  printer  or internal modem ports (but it should be

possible to modify the values for mnport, mnprts, and baudrt  to  do

this).    Can  generate  BREAK.  SET BAUD-RATE supported; speeds are

50-19200 baud.

Morrow Decision I:

Uses the Multi-I/O board.  Port 1 is the console, port 3 is the com-

munications line.  SET  BAUD-RATE  supported;  speeds  are	75-56000

baud.

Nokia MicroMikko:

Will not echo control-O (which locks keyboard).  SET BAUD-RATE  sup-

ported; speeds are 75-9600 baud.

Ohio Scientific:

Doesn't have screen control.

Osborne	 1:Uses	 serial	 line,	not  internal  modem.	 Left-arrow  key

generates  <DEL>  ("delete"	 or  "rubout"  character) during connect

mode.  SET BAUD-RATE supported; speeds are 300 and 1200 baud.

TRS-80: Two versions, one for Lifeboat CP/M, one  for  Pickles	&  Trout

CP/M.

.e

1.4. Installation of Kermit-80

Kermit-80  was written originally for the Intertec SuperBrain in lowest-

common-denominator  8080 code with the standard assembler,  ASM	 (single

source module, no macros, no advanced instructions), so that it could be

assembled on any CP/M-80 system (the 8080 assembler is distributed as  a

standard  part	of  CP/M-80, whereas the fancier Z80 or macro assemblers

are normally commercial products).  It has since been modified to run on

many  other  systems  as  well.	  Kermit-80 should be able to run on any

8080-, 8085- or Z80-based microcomputer under	 CP/M  with  appropriate

minor  changes to reflect the port i/o and screen control for the system

(see below).

The proliferation of new systems supported by Kermit-80 made the program

grow  so  large and complicated that it had to be broken up into system-

independent and system-dependent modules, as of version 4 (this was done

by Charles Carvalho of ACC).  Each module is composed of multiple files.

This has reduced  the  time  and  disk	space  necessary  for  assembly;

Kermit-80  may	once  again  be	 assembled on a CP/M system with roughly

150Kbytes of space.  The majority of the code does not need to be  reas-

sembled to support a new system.  Unfortunately, it can no longer be as-

sembled with ASM, since ASM does not support multiple input files.    To

allow it to be assembled on any CP/M system, the public-domain assembler

LASM is included in the distribution kit;  Kermit-80  may  also	 be  as-

sembled	 with  Microsoft's  M80  (not supplied), or cross-assembled on a

DEC-10 or DEC-20 with MAC80 (also supplied in the distribution kit).  In

theory,	 any  8080  assembler  supporting the INCLUDE directive ought to

work, as well.

All versions of Kermit-80 are assembled from the same  set  of	sources,

with  system  dependencies  taken  care of by assembly-time conditionals

within the system-dependent  module  (eventually,  the	system-dependent

module	will  itself  be  broken  up  into  multiple files, one for each

system).  The most important system dependencies are terminal  emulation

(when  CONNECTed  to the remote host) and screen handling, which are de-

pendent on the individual micro's escape codes (these features are table

driven and easily modified for other CP/M systems), and the lowest level

i/o routines for the serial communications port.  The port routines  are

best  done  only  with	BDOS  calls, but some systems do not allow this,

primarily because the BDOS routines strip the  parity  bit  during  port

i/o, and the parity bit is used for data when transmitting binary files.

Kermit-80's  I/O routines must check the port status and go elsewhere if

no input is available; this  allows  for  virtual  terminal  connection,

keyboard  interruption	of  stuck  transmissions,  etc.	 On systems that

fully implement I/O redirection via the optional CP/M  IOBYTE  facility,

this  may  be  done  by	 switching  the	 IOBYTE	 definition.  On others,

however, IN/OUT instructions  explicitly  referencing  the  port  device

registers must be used.

CP/M-80	 KERMIT	 versions 3.8 and later include a "fuzzy timer" that al-

lows a timeout to occur after an interval ranging from 5 to  20	 seconds

(depending  upon  the  speed  of  the processor and the operating system

routines) during which expected input does not appear at the port.    In

this  case,  retransmission  occurs automatically.  In any case, you may

type a carriage return during transmission to simulate	a  timeout  when

the transfer appears to be stuck.

.e

1.4.1. Organization of Kermit-80

Kermit-80  consists of two modules, each of which is generated from mul-

tiple source files.  The first module  contains	 the  system-independent

code; the second module is configured for a particular system and merged

with the system-independent module to produce a customized Kermit-80.

The distribution kit contains:

- the system-independent module, CP4KER.HEX;

- the system-dependent modules, CP4*.HEX (see table 1-2);

- the source files, CP4*.ASM,

- the DEC-10/DEC-20	cross-assembler	 and  linker,  MAC80.*	and

LINK80.*,

- the public-domain CP/M assembler, LASM.*,

- the public-domain CP/M load/patch utility, MLOAD.*

------------------------------------------------------------------------

Symbol	Filename System

AP6551	CP4APL	Apple II, Z80 Softcard, 6551 ACIA in serial interface

APMMDM	CP4APM	Apple II, Z80 Softcard, Micromodem II in slot 2

BBII	CP4BB2	BigBoard II (terminal required)

BRAIN	CP4BRN	Intertec SuperBrain.

CPM3	CP4CP3	"generic": CP/M 3.0 (CP/M Plus) systems (terminal req'd)

DELPHI	CP4DEL	Digicomp Delphi 100 (terminal required)

DMII	CP4DM2	DECmate II with CP/M option

GENER	CP4GEN	"Generic": CPM 2.2 systems with IOBYTE (terminal req'd)

HEATH	CP4H89	Heath/Zenith H89.

KPII	CP4KPR	Kaypro-II (and 4; probably supports all Kaypro systems)

MDI	CP4MDI	Morrow Decision I (terminal required)

MIKKO	CP4MIK	MikroMikko

MMDI	CP4UDI	Morrow Micro Decision I (terminal required)

OSBRN1	CP4OSB	Osborne 1

OSI	CP4OSI	Ohio Scientific

ROBIN	CP4ROB	DEC VT180

TELCON	CP4TEL	TELCON Zorba portable

TRS80LB CP4TLB	TRS-80 model II with Lifeboat 2.25C CP/M Display

TRS80PT CP4TPT	TRS-80 model II with Pickles + Trout CP/M Display

VECTOR	CP4VEC	Vector Graphics.

Z100	CP4Z00	Z-100 under CP/M-85

"symbol"  is the symbol used to select the target system, in CP4TYP.ASM;

"filename" is the name	under  which  the  module  is  supplied	 in  the

distribution.

Table 1-2:   Systems supported by Kermit-80

------------------------------------------------------------------------

1.4.2. Downloading Kermit-80

You'll  need either a pre-configured .COM file or the system-independent

module, CP4KER, in binary (.COM) or hex (.HEX) format and the system-de-

pendent overlay for your system (from table 1-2).  If your system is not

listed in the table, get the generic CP/M 2.2 Kermit or the generic CP/M

3 Kermit.  If you already have a version of Kermit on your micro and you

want to install a new version, simply use your present	version	 to  get

the  new  files.    Transfer  the files to your system and skip ahead to

"merging the modules".

If you do not have a copy of Kermit on your micro, and you cannot borrow

a  Kermit  floppy  but you do have access to a mainframe computer with a

copy of the Kermit-80 distribution, you should read this section.

There are several ways to get Kermit from a host system to  your  micro.

The easiest is to "download" the necessary "hex" files into your micro's

memory and then save it on the disk.  If you have  a  terminal	emulator

program on your micro which can save a copy of the session to disk, con-

nect to your host,  and	 type  the  necessary  files.	 Exit  from  the

emulator,  saving  the	session log, and edit the session log to extract

the hex files.	Skip ahead to "merging the files".

The following is a procedure which, though far	from  foolproof,  should

allow  you  to	get  a	version	 of Kermit to your CP/M based micro.  It

depends upon the host prompt, or at least the  first  character	 of  the

host  prompt, being some character that cannot appear in a hex file (the

valid characters for hex files are the digits 0-9, the upper  case  let-

ters A-F, the colon ``:'', carriage return, and line feed).  As soon the

prompt character is encountered, the transfer will terminate.	If  your

host  does not issue a prompt that will accommodate this scheme, you can

achieve the same effect by adding an atsign ``@'' to the very end of the

hex  file  before sending it from the host.  The program below looks for

an atsign (the normal DEC-20 prompt, hex 40).  DECSYSTEM-10 users  would

look for a dot, hex 2E.

1. On  your	micro, connect to a floppy disk with plenty of free

space.  Run DDT and  type	 in  the  following  (the  comments

should  not  be  typed  in;  they	 are there just to tell you

what's happening):

-a100			;Begin assembling code at 100.

0100 lxi h,ffe		;Where to put HEX file.

0103 shld 300		;Save the address.

0106 mvi e,d		;Get a CR.

0108 mvi c,4		;Output function.

010A call 5

010D mvi c,3		;Input function.

010F call 5

0112 ani 7f		;Turn off the parity.

0114 cpi 40		;Our DEC-20 prompt atsign?

0116 jz 124		;Yes, we have whole file.

0119 lhld 300		;Get the pointer.

011C mov m,a		;Else, store the char.

011D inx h		;Increment the pointer.

011E shld 300		;Save the pointer.

011F jmp 10d		;Go around again.

0124 mvi a,1a		;Get a control-Z.

0126 lhld 300		;Get the pointer.

0129 mov m,a		;Store the char.

012A shld 300		;Save the pointer.

012D lxi h,1000		;Pointer to file.

0130 shld 310		;Save the pointer.

0133 mvi c,16		;Make file.

0135 lxi d,5c

0138 call 5

013B lhld 310		;Get the file pointer.

013E xchg		;Put it in DE.

013F mvi c,1a		;Set DMA.

0141 call 5

0144 mvi c,15		;Write DMA to file.

0146 lxi d,5c

0149 call 5

014C lhld 310		;Get the file pointer.

014F lxi d,80		;Get the DMA size.

0152 dad d		;Adjust file pointer.

0153 shld 310		;Save it.

0156 lda 301		;Get high order byte.

0159 cmp h		;Have we passed the end?

015A jm 170		;Yes.

015D jz 163		;Maybe.

0160 jmp 13b		;No.

0163 lda 300		;Get low order byte.

0166 cmp l		;Passed the end?

0167 jm 170		;Yes.

016A jz 170		;Yes.

016D jmp 13b		;Not quite.

0170 mvi c,10		;Close file.

0172 lxi d,5c

0175 call 5

0178 ret

0179

-^C			;(control-C):Return to CP/M

A>SAVE 1 FETCH.COM	;Save this program;

; we need to run it twice.

2. Connect to your host using a terminal or a terminal emulation

facility.	  Ensure that your host does not have your terminal

in "page mode".  E.g. on the DEC-20, give	 the  Exec  command

TERMINAL NO PAUSE END-OF-PAGE.

3. Tell   the   host	  to   display	the  first  hex	 file  (the

system-independent module) at your terminal.    E.g.  on	the

DEC-20, give the Exec command TYPE CP4KER.HEX, without a ter-

minating carriage return.

4. Return to your micro.  Make sure your IOBYTE is set  so  that

RDR: and PUN: correspond to the I/O port that is connected to

the DEC-20 (this would normally be the case unless  you  have

done  something  special to change things).  Load the program

you entered in the first step with DDT, and use it to capture

the first hex file:

DDT FETCH.COM

-icp4ker.hex		;Setup FCB for file CP4KER.HEX.

-g100,179		;Execute the program.

Now there should be a file CP4KER.HEX on your connected disk.

5. Return  to  the  host,  and tell it to display the second hex

file (the system-dependent module	 for  your  configuration).

Again, do not type the terminating carraige return.

.e

6. Return to your micro, and run the capture program again:

DDT FETCH.COM

-icp4ovl.hex		;Setup FCB to create CP4OVL.HEX

-g100,179		;Execute the program.

Now there should be a file CP4OVL.HEX on your connected disk.

Merging the files:

1. For  purposes  of illustration, we will assume the system-de-

pendent overlay is called "cp4ovl.hex".  The  two	 hex  files

may  be  combined	 with  MLOAD or DDT.  If you already have a

running Kermit, you can transfer MLOAD.HEX to your system and

create  MLOAD.COM	 by  running LOAD.  If you're bootstrapping

Kermit, you could transfer MLOAD.HEX to your system the  same

way  you	got  the  other	 two  .HEX files, but it's probably

simpler to use DDT to get Kermit running, and get MLOAD later

if you need it.

2. Using MLOAD, the two pieces may be easily merged:

A>mload kermit40=cp4ker,cp4ovl

[some messages about program size, etc.]

A>

3. If you don't have MLOAD running, it's a bit more complex:

A>ddt cp4ker.hex

NEXT  PC

3500 0100

-icp4ovl.hex

-r

NEXT  PC

xxxx 0000

-^C

A>save dd kermit40.com

The  page count ("dd") used in the SAVE command is calculated

from the last address ("xxxx") given by DDT  in  response	 to

the  R  command:	drop  the last two digits and add 1 if they

were not zero, then convert from	hexadecimal  (base  16)	 to

decimal  (base  10): 384F becomes 39 hex, which is 57 decimal

(3 times 16 plus 9) -- but 3700 becomes 37 hex, or 55 decimal

(consult	an  introductory computing book if you don't under-

stand number base conversion).

4. Note that CP/M hex files have checksums on  each	line.	 If

there  were  any	transmission  errors during the downloading

process, MLOAD or DDT will notice a  bad	checksum  and  will

report  an  error	 (something like "Illegal Format").  If you

get any errors during loading, either fix the  hex  file	lo-

cally with an editor, or repeat the transfer.

You now should have a running version of Kermit-80, called KERMIT40.COM.

Test  your new Kermit by running it.  If it gives you a prompt, it might

be OK. (don't delete your old one yet...).  Instead  of  a  prompt,  you

could  get  one of two messages indicating that the configuration infor-

mation is invalid:

?Kermit has not been configured for a target system

or

?Consistency check on configuration failed

Of course, neither of these messages should appear  if	you're  building

Kermit	from the distribution kit.  The first message indicates that the

overlay was not found where the system-independent  module  expected  to

find  it,  probably because the overlay address is incorrect; the second

indicates that the version of CP4LNK used in the system-dependent module

is incompatible with the system-independent module.

Once  you are satisfied that KERMIT40 works correctly, you should rename

your old KERMIT.COM to something  else,	 like  OKERMIT.COM,  and  rename

KERMIT40.COM to KERMIT.COM.

1.4.3. Assembling Kermit-80 from the sources

Kermit-80 is built in two pieces from the following 12 files:

The system-independent files:

CP4KER.ASM	header file

CP4DEF.ASM	definitions for both KERMIT and KERSYS

CP4MIT.ASM	initialization,	 main loop, miscellaneous commands (BYE,

EXIT, LOG, SET, SHOW, STATUS, and VERSION)

CP4PKT.ASM	the KERMIT protocol handler (SEND, RECEIVE, LOGOUT,  and

FINISH commands)

CP4TT.ASM	 the transparent commands (TRANSMIT, CONNECT)

CP4CPM.ASM	CP/M commands (DIR, ERA)

CP4WLD.ASM	the wildcard handler

CP4CMD.ASM	the command parser

CP4UTL.ASM	utility routines and data

CP4LNK.ASM	linkage area description

The system-dependent files:

CP4TYP.ASM	system selection

CP4SYS.ASM	system-specific code

The  system-independent	 module	 contains  all of the system-independent

files except for CP4LNK.ASM, which is assembled into the  system-depend-

ent  module to provide the structures needed to connect the two modules.

As distributed, the system-independent module is named CP4KER.HEX.    If

you have a copy of CP4KER.HEX, you do not need to reassemble the system-

independent module to configure Kermit for your system.

The  system-dependent  module  consists	  of   CP4TYP.ASM,   CP4DEF.ASM,

CP4LNK.ASM,  and CP4SYS.ASM.  One copy of the system-dependent module is

supplied already assembled for each supported system; the  filename  may

be obtained from table 1-2.

After  assembling  the two pieces separately, they are combined with DDT

or MLOAD into a system-specific Kermit.

If you want to rebuild the system-independent module,  the  only  change

you  may  need	to  make  is  to  select  the  assembler  to be used, in

CP4KER.ASM.  Define one of MAC80, M80, or LASM to TRUE to select  it  as

the assembler; the others should be defined FALSE.

Assuming  you  have  the  Microsoft  Macro  Assembler package (M80/L80),

you'll need to do the following:

A>m80 cp4ker=cp4ker.asm

A>l80 /p:100,cp4ker,cp4ker/n/e

This will produce CP4KER.COM.

If you are using LASM instead, do this:

A>lasm cp4ker

LASM will generate CP4KER.HEX and CP4KER.PRN.  LASM allows options to be

specified in the same way as the standard assembler, ASM, so the command

A>lasm cp4ker.abz

will read the source files from drive A, send the .HEX file to drive  B,

and suppress the listing file.

If   you   have	  access  to  a	 TOPS-10  or  TOPS-20  system,	you  can

cross-assemble Kermit there with MAC80, producing CP4KER.HEX:

.run mac80

*=cp4ker.asm

If you want to generate a system-dependent overlay for a particular sys-

tem,  or  want	to  change  the terminal supported, you'll need to check

three areas in CP4TYP.ASM:

First, the overlay start ADDRESS.  The symbol "ovladr" is EQUated to the

address	 of  "LNKFLG"  in the system-independent module, as the starting

address of the overlay (3400H for version 4.03).  You'll  need  to  know

this  value if you're building the overlay with M80/L80.  You won't nor-

mally need to change this value.

Second, the assembler being used.  Again, define one of MAC80, M80,  and

LASM  to  be  TRUE to select it, and define the others to be FALSE.  The

two modules (system-independent and system-dependent) do not need to  be

built with the same assembler.

Third,	the system configuration.  Locate your system in table 1-2, then

define the appropriate symbol TRUE, and the rest FALSE.	 If  the  system

comes  with a builtin console terminal, define all the terminal switches

FALSE.	If the system uses an external terminal as the	console,  locate

the  terminal  in  table 1-3 and define the appropriate symbol TRUE, and

the remainder FALSE.  If the terminal is not listed in	table  1-3,  use

the CRT switch; in this case, VT52 emulation is not supported.

In  addition,  there are a few general and system-specific symbols which

may be altered to fit your system:

APSLOT		For Apple with 6551 ACIA, defines the slot number of the

serial card

CPUSPD		Processor  speed in units of 100KHz (currently used only

for bbII and kpII for timing loops)

.e

TAC		For users connecting through ARPAnet TACs: set	to  TRUE

if  you	 wish the default TACTRAP status to be ON. (This

may be overridden with the SET	TACTRAP	 command).    If

you're  not  connecting  through a TAC, set tac to FALSE

and ignore tacval.

TACVAL		For ARPANET TAC users: defines the default TAC intercept

character  (may	 be  overridden	 with  the  SET	 TACTRAP

command)

If you are just assembling an existing	configuration,	you'll  need  to

edit  CP4TYP.ASM  only.	 If you are adding support for a new system, you

should not modify CP4DEF.ASM or CP4LNK.ASM; if you do,	you'll  have  to

change the system- independent module also.  Eventually, CP4SYS.ASM will

be split into separate files, each of which will generate  one	or  more

related systems.  When this happens, you'll want to pick the one closest

to your system to use as a starting point.

After editing CP4TYP.ASM as necessary, assemble and link the overlay  as

follows:

- With  M80	(where	"xxxx"	is  the	 hex  value  of ovladr from

CP4LNK.ASM):

A>m80 cp4typ=cp4typ.asm

A>l80 /p:xxxx,cp4typ,cp4typ/n/x/e

- With LASM:

A>lasm cp4typ

- With MAC80 on TOPS-10:

.run mac80

*=cp4typ.asm

- With MAC80 on TOPS-20:

@run mac80

*=cp4typ.asm

The overlay (cp4typ.hex) may then be merged with the  system-independent

module as described above (creating a runnable Kermit from the distribu-

tion kit).

If you have a TOPS-10 or TOPS-20  system  and  already	have  a	 running

Kermit-80  v3.9	 or  later, you can merge the two .HEX files into a .COM

file with LINK80, and transfer the new .COM file to your micro with Ker-

mit:

- Tops-10:

.copy kernew.hex=cp4ker.hex,cp4typ.hex

.link80 kernew

- Tops-20:

@append cp4ker.hex,cp4typ.hex (to) kernew.hex

@link80 kernew

producing  KERNEW.COM.	If LINK80 says "?Data overlaid", you have an old

version of LINK80, and will have to transfer the .HEX files to the micro

and merge them there.

------------------------------------------------------------------------

Symbol	       Terminal description

crt	       Basic CRT, no cursor positioning

adm3a	       ADM3A Display or lookalike

smrtvd	       Netronics Smartvid-80

tvi925	       TVI925, Freedom 100

vt52	       VT52 or VT52 emulator such as Heath H19, H29, etc.

vt100	       VT100 or emulator (most ANSI terminals should work)

Table 1-3:   Terminals known to Kermit-80

------------------------------------------------------------------------

1.5. Adding Support For A New System

Kermit-80 is built from a common set of source files; the system-depend-

ent module makes heavy use of conditional  assembly  (this  complication

will be removed in future releases).  The system dependencies arise from

attempts to answer some questions:

1. What kind of terminal is to be supported?

For many micros, the console is an integral part of the  sys-

tem,  but	 others	 can  use  an external terminal.  In either

case, the commands to manipulate	the  screen  (position	the

curser, erase the screen, etc) must be defined.

2. How is the serial line accessed?

For   systems   supporting   the	IOBYTE	function,  this	 is

straightforward; the symbol "IOBYT" is defined TRUE.  If	the

serial  line is accessed with IN and OUT instructions, it may

be possible to use the simple I/O routines provided.  In this

case,  the  symbol  "INOUT"  is  defined TRUE, the MNPORT and

MNPRTS are defined to be	the  data  and	control	 addresses,

respectively,  and  bit  masks  for  testing  for "input data

available" and "output buffer empty" must be defined.  If the

interface is strange, leave IOBYT and INOUT set to FALSE, and

provide the I/O routines.

3. What initialization is necessary?

You may wish to set the baud rate	 or  configure	the  serial

line  at	startup.    Examples  for  a  number of devices are

present.

4. What special features are to be supported?

You may want to provide  the  capability	to  select  one	 of

several  serial lines with the SET PORT command, or to change

the speed of the serial line with the SET BAUD-RATE  command.

To  do  this, you'll need to build a command table, using the

systems already supported as examples.  The ability to send a

BREAK  signal is desirable.  Again, examples for several dif-

ferent interfaces (ACIA, SIO, etc) are present.

1.6. Notes on New Features in Kermit-80 Version 4

Debugging aids:	 SET DEBUG ON will add two fields  to  the  SEND/RECEIVE

display,  labelled  "Spack"  and "Rpack".  These display the last packet

sent and received.  Of course, this slows down the transfer,  especially

if  the	 console  is  an external terminal.  SET DEBUG OFF removes these

fields.	 The VERSION command displays the name, edit  number,  and  edit

date of several of the modules that make up Kermit.

TAC  support: ARPAnet TACs (and many other communication devices such as

terminal concentrators, modems, port  contention  units,  network  PADs,

etc)  use a printing character (normally "@") as an intercept character,

to allow commands to be issued to the  TAC.    In  order  to  send  this

character  to  the  host,  it must be typed twice.  The command "SET TAC

CHARACTER" to Kermit enables the TACtrap and asks the  user  to  specify

the  TAC  intercept  character.	   This	 character will be automatically

doubled when it appears in Kermit protocol messages (sent by the SEND or

RECEIVE	 commands)  or	when  it  appears  in a file being sent with the

TRANSMIT command.  It is not automatically doubled  when  typed	 by  the

user  in  CONNECT  mode.   "SET TAC ON" enables the TACtrap but does not

change the TAC intercept character, which is initially "@".    "SET  TAC

OFF"  disables  the  TACtrap.  (These comments apply equally to any com-

munication device that uses a printable	 attention  character  which  it

will pass through if doubled.)

File buffering:	 Previous versions of Kermit-80 buffered only one sector

(128 bytes) at a time during file transfer  operations.	   This	 version

buffers	 16Kbytes  at  a  time,	 reducing the number of times the floppy

drive must be spun up and down, and increasing the effective  throughput

of  the link.  If the disk transfer rate is too slow, howver, the remote

Kermit may time out and retransmit packets.  This will show  up	 on  the

screen	in the "Retries:" field; if this occurs after disk activity, you

may want to increase the timeout value on the remote  Kermit,  or  reas-

semble	Kermit	with  a	 smaller value for MAXSEC (in CP4SYS.ASM).  This

buffer is also used by the TRANSMIT command; the log file enabled by the

LOG command is still written a sector at a time.

1.7. Future Work

Work that needs to be done in future releases includes:

- Merge  in support for additional CP/M-80 systems, particularly

those for which support was recently added to  the	 monolithic

v3.x source.

.e

- Break  up CP4SYS into discrete source files, one for each sys-

tem.  These source files should serve as simple models for ad-

ding  support  for	 new  systems to Kermit-80 -- only the very

basic screen definitions, flags, i/o  primitives,	initializa-

tions,  and  so  forth  should appear in each system-dependent

file.

- Make the file-stepping mechanism faster (buffer the  FCB's  in

chunks  of 16 or 32 or 64).  Currently, to access the nth file

in a directory requires n(n+1)/2 lookups...

- Addition  of  missing  features  --  compression  of  repeated

characters	 during	 packet	 transmission, transmission of file

attributes (particularly size, so that "percent done"  can	 be

displayed for both incoming and outbound files), advanced com-

mands for servers (REMOTE DIRECTORY, etc), command macros	and

initialization  files,  login  scripts,  remote  operation and

server mode, etc etc.

.e

Index

8080	 8, 10				XON/XOFF   4

ARPAnet   20				Z80   10

Attention Character	20

Baud	 6

BIOS	 8

Block Check	6

Bootstrap   12

BYE	5

Connect   4

CP/M	 10

Downloading	12

Eighth-Bit Prefix   6

Escape Character   4, 5

File-Warning	 4

FINISH   5

Generic Kermit-80   8

GET	4

IBM	6

Intercept Character	20

Local	  3

Local-Echo   5

LOG	4

LOGOUT   5

NAK	3

Parity   6

RECEIVE   4

SEND	 4

SET	5

TAC	20

Timeout   3, 11

TIMER	  6

TRANSMIT   5

VERSION   5

Virtual Terminal   4

VT100	  5

VT52	 5

Warning   5

.e

Table of Contents

1. CP/M-80 KERMIT						       1

1.1. Summary of CP/M						       1

1.2. Kermit-80 Description					       3

1.3. Kermit-80 Flavors					       8

1.3.1. Generic Kermit-80					       8

1.3.2. CP/M 3 Kermit					       9

1.3.3. System-Specific Versions				       9

1.4. Installation of Kermit-80				      10

1.4.1. Organization of Kermit-80				      11

1.4.2. Downloading Kermit-80				      12

1.4.3. Assembling Kermit-80 from the sources		      16

1.5. Adding Support For A New System				      19

1.6. Notes on New Features in Kermit-80 Version 4		      20

1.7. Future Work						      21

List of Tables

Table 1-1:	Kermit-80 SET PORT Options			       8

Table 1-2:	Systems supported by Kermit-80			      12

Table 1-3:	Terminals known to Kermit-80			      19

###################################################################