SCSI-ID.COM

-----------

Version 4

Description

-----------

The AMPRO SCSI-ID.COM utility reports the system's SCSI Initiator ID to the

console, and also sets the DOS ERRORLEVEL so it may be tested in a batch file.

One use of the SCSI-ID.COM utility is that it allows you to easily verify the

SCSI ID setting of your Little Board.  A more interesting use of this program

is to allow multiple PC-DOS Little Boards to boot from a single SCSI device,

yet automatically begin execution of unique applications based on each board's

SCSI ID.

Command Line Operation

----------------------

To use the program, simply enter the program's name on the DOS command line or

in a batch file.  For example, if your system's SCSI ID is 7 and you use the

following command line:

C>SCSI-ID<Enter>

SCSI-ID.COM will display:

My SCSI ID is 7

Batch File Operation

--------------------

In addition, SCSI-ID sets the DOS ERRORLEVEL to a number corresponding to the

Little Board's SCSI-ID jumpering, for testing within batch files.  Using the

program in this manner, it is possible to boot multiple AMPRO Little Boards

from a single SCSI drive under PC-DOS, yet have each board come up running a

different application.

To accomplish this, the AUTOEXEC.BAT file on the boot must test for

"ERRORLEVEL" and branch to a unique batch file appropriate for the specific

Little Board.  Then, each Little Board is jumpered for a different SCSI ID,

between 1 and 7, and the boot device is jumpered for ID 0.

The following ampro application note discusses the use of SCSI-ID.COM in a

PC-DOS system composed of multiple AMPRO Little Boards.

=============================================================================

--------------------------------------

AMPRO COMPUTERS, INC. APPLICATION NOTE

--------------------------------------

Number:      #AAN-8803      Date:  Mar 24, 1988          Author:  R. Lehrbaum

Title:       Using multiple Little Boards on a single SCSI bus under PC-DOS.

Product(s):  Little Board/PC, Little Board/186

Abstract:    This Ap Note describes a simple scheme for sharing a SCSI boot

drive, including a primitive means for inter-processor

communications via files on the shared SCSI drive.

-------------------------------------------------------------------------------

Introduction

------------

The Small Computer System Interface (SCSI) is a "peer-to-peer" I/O bus which

allows multiple devices to share multiple resources.  Inherent in SCSI's

design is a mechanism for bus arbitration.  Consequently, SCSI can actually be

used as a "network" in which resources and information are shared.

If you simply daisy-chain several Little Boards together on one SCSI bus with

a shared boot drive, you will find a basic problem.  Since all boards will

read the same AUTOEXEC.BAT file, they will all execute precisely the same

commands, and perform exactly the same operations.  Unfortunately, this

situation has limited usefulness, since in any useful application you will

probably want the boards to work as a team, with each board performing unique

functions.  In addition, to make this "team" work effectively, some means will

be needed to allow the boards to communicate with each other.

Booting from a Shared Drive

---------------------------

The AMPRO SCSI-ID.COM utility reports the system's SCSI Initiator ID to the

console, and also sets the DOS ERRORLEVEL so it may be tested in a batch file.

One use of the SCSI-ID.COM utility is that it allows you to easily verify the

SCSI ID setting of your Little Board.  A more interesting use of this program

is to allow multiple PC-DOS Little Boards to boot from a single SCSI device,

yet automatically begin execution of unique applications based on each board's

SCSI ID.

To use the program, you simply enter the program's name on the DOS command

line or in a batch file.  For example, if your system's SCSI ID is 7 and you

use the following command line:

C>SCSI-ID<Enter>

SCSI-ID.COM would display:

My SCSI ID is 7

In addition, SCSI-ID sets the DOS "ERRORLEVEL" to a number corresponding to

the Little Board's SCSI-ID jumpering, for testing within batch files.  Using

the program in this manner, it is possible to boot multiple AMPRO Little

Boards from a single SCSI drive under PC-DOS, yet have each board come up

running a different application.

To accomplish this, the AUTOEXEC.BAT file on the boot must test for

"ERRORLEVEL" and branch to a unique batch file appropriate for the specific

Little Board.  Then, each Little Board is jumpered for a different SCSI ID,

between 1 and 7, and the boot device is jumpered for ID 0.

Here is a sample AUTOEXEC.BAT file that can be used for this application:

echo on

myscsiid

if errorlevel 8 goto ERROR

if errorlevel 7 START7

if errorlevel 6 START6

if errorlevel 5 START5

if errorlevel 4 START4

if errorlevel 3 START3

if errorlevel 2 START2

if errorlevel 1 START1

:ERROR

echo ERROR: The SCSI/BIOS is not installed.

In addition to this AUTOEXEC.BAT file, the boot drive must also include a

batch file for each SCSI ID that is included in the overall multi-board

system.  The names of the batch files for each of the seven possible drives

based on the above example would be:  START1.BAT, START2.BAT, START3.BAT, ...,

START7.BAT.

(Alternatively, a single application program could be passed a unique ID-

related parameter by the AUTOEXEC.BAT command lines, for example instead of

lines of the form "if errorlevel 7 START7", you could use lines of the form

"if errorlevel 7 MYPROG ID7".  This would result in all boards loading and

runing the same program, MYPROG.COM, but each board would pass it a unique

parameter based on its SCSI ID.)

One restriction of booting multiple boards from a single SCSI drive is that

PC-DOS may generate "timeout" errors due to delays in gaining access to the

SCSI bus.  With all boards attempting to boot simultaneously, the drive's head

will have a tendency to "thrash".  Sequencing the boards' resets, limiting the

number of boards, or using a fast drive should eliminate this limitation.

Once the boards are booted, it may be desireable to load a DOS driver that

intercepts the disk function and performs automatic retries on device

timeouts, to assure proper operation on a shared SCSI drive.

>>> WARNING <<<

Do not attempt to have multiple Little Boards WRITE to a single

DOS drive partition, as data will be corrupted due to

uncoordinated use by multiple CPU's of a single DOS directory.

The AMPRO HEDIT.COM utility can be used to partition a single SCSI drive into

up to four DOS partition, allowing four Little Boards to share a single drive

without difficulties.  In this manner, four Little Boards might work together

using a single SCSI bus interconnect and sharing a single SCSI disk drive.

Each of the four boards would be assigned one of the four partitions on the

drive as a writable partition, and could read from the remaining three

partitions.

If all partitions are accessed by all boards via Version 4.03 (or later) of

the AMPRO HARD.SYS driver (with removable drive support), then DOS will re-log

the directory information of the other boards' partitions on each access.

This will assure that accesses to files written by the other boards are based

on correct directory data.  Be sure to also re-install the boot partition via

HARD.SYS and only access it via its HARD.SYS device letter (not "C"), or the

automatic re-logging will not occur.

Inter-Processor Communications

------------------------------

The four boards can send messages to each other by using file "mailboxes".

Each board writes to mailboxes in its own partition, and reads from mailboxes

in the other boards' partitions.  For example, the board with ID 7 might be

assigned the first partition, ID 6 the second, ID 5 the third, and ID 4 the

fourth.  ID 7 could leave messages for each of the other three boards in files

called ID6.MSG, ID5.MSG, ID4.MSG, within the FIRST partition.  Similarly, the

board with ID 6 would write to mailboxes in the SECOND partition for the other

boards, having names ID7.MSG, ID5.MSG, and ID4.MSG, and so on.

Be sure that your software does not require constant polling of the disk-based

mailboxes, as the constant drive head movement would bring the system to its

knees.  A SCSI RAM disk would provide a perfect solution to this problem!

Fortunately, one is available in the form of the AMPRO SCSI/IOP, which can be

used as an inexpensive SCSI RAM disk, with up to 48K bytes of onboard memory.

Using the SCSI/IOP's read and write memory commands, you can implement an

efficient multi-ported high speed mailbox.

It is theoretically possible to implement direct board-to-board communication

over SCSI.  However, this would require an extension to the AMPRO SCSI/BIOS so

that selection of one board (as a Target) by another (as an Initiator) does

not interfere with the normal use of SCSI for drive access.

#####################################################################