If you are interested in making your computer talk,

you should consider the SP0256-AL2.   Radio Shack sells the

chip for $12.95 and the total project should be less than

$25 and 2 hours of construction time.   The SP0256-AL2 gener-

ates allophones, basic word fragments.   Using combinations

of these 59 allophones, an unlimited, if somewhat mechanical

sounding, speech can be created.   Allophones are very frugal

with the memory, only 100 bits per second.  You could hold an

hour of speech in 50k bytes!

Requirements:

7 output lines   6 data, ald* (active low write strobe)

1 input line     lrq* (active low busy signal)

gnd

+5v		 power supply

parts list

sp0256-al2		(Radio Shack #276-1784)

lm386			audio amp, RS#276-1731

3.12 mhz crystal 	(I used a 3 mhz crystal)

speaker

10k petentiometer	volume control

misc. capacitors,

resistors, sockets,

protoboard

The 18 pages of documentation are supplied by Radio

Shack.   Their circuit contains a number of errors.

1) unlabeled audio amp - lm386

2) '-5Y' should be '+5V'

3) A7, A8 should/can be grounded

4) PIC1665A could read 'any ttl compatible parallel computer

port'

5) sby reset can be tied to reset

6) the discrete components between the sp0256 and the pic1665a

can be eliminated except for the 100k resistor connected

between the reset pin and +5v ('-5Y') and the .01uf

cap between the reset pin and ground.   This eliminates

2 transistors, 2 diodes, 4 resistors and 1 capacitor.

good luck

thought for the day:  I expect speech synthesis to become very

important.   The two most cost effective devices for presenting

information are the crt and the speech processors.   One device

was developed before the transistor and the other is the stuff

of science fiction.######################################################################################################################