Addendum for 86-DOS 1.1

This disk contains 86-DOS version 1.1 which is necessary to run the Microsoft

high-level language compiler or MACRO-86 assembler which you have purchased.

It also contains the newer versions of the system utilities which go with

86-DOS 1.1.

Differences between 86-DOS version 1.0 and 86-DOS version 1.1 (included on this

disk),  which the user must know about in order to use the new software:

* 86-DOS no longer fits on the space reserved at the beginning of the

disk.  86-DOS resides in an invisible file which is always the first

file on the disk.  The boot sequence is now as follows:

1> the boot ROM loads the first sector from the disk and jumps to the

code loaded from that sector.

2> the code from the first sector loads in the I/O system which

follows the first sector in the reserved area and is typically

1024 bytes in length.  A jump is made to the initialization

program in the I/O system.

3> the initialization program in the I/O system uses the disk read

function of the I/O system to load 86-DOS from the invisible file.

4> the initialization code then uses 86-DOS to load in the command

interpreter, COMMAND.COM, from the disk and then jumps to it.

* The SYS utility which copys 86-DOS from one disk to another also

copies the invisible file, 86DOS.SYS, which contains 86-DOS.  Because

this file must be the first file on the disk (so the I/O system can

find it when 86-DOS is booted up), SYS can only transfer the

operating system to a disk which is freshly CLEARed or already has a

file (invisible or not) called 86DOS.SYS which is large enough to

hold 86-DOS.

* The CHKDSK utility prints the total number of files on the disk

including invisible files.  This allows the user to check for the

presence of invisible files such as 86DOS.SYS since the total number

of files reported by CHKDSK will be greater that that reported by the

DIR command.  Note:  be sure to use this new version of CHKDSK with

86-DOS version 1.1 because old versions of CHKDSK run under 86-DOS

version 1.1 can't "see" the invisible files and will free the disk

space allocated to these files thus destroying them.

* The COMMAND interpreter (COMMAND.COM) can now load executable machine

language files with the extension .EXE (for EXEcute).  These are

similar to .COM files but have a 512-byte preface which contains

information on how to patch inter-segment jumps and calls within the

file allowing multiple-segment files to be loaded.

* DEBUG also can load .EXE files so that they can be debugged.

* The assembler has had some rather significant features added to it.

The expression analyser can now perform multiplication and division

as well as addition and subtraction.  Use "*" to multiply and "/" to

divide.  Multiplications and divisions are performed before additions

and subtractions.  Parentheses may be used to change this normal

order of precedence.

Errors messages are sent directly to the console as well as put in

the .PRN print file so that it is not necessary to make a print file

to find out what errors are in the source code.  Two more options

have been added to the third letter of the drive assignment

extention which tells the assembler what to do with the print file.

A-O send print file to disk drive A-O

P   send print file directly to printer (tabs are NOT expanded)

X   send print file to console

Y   force a second pass through the source file so that error

messages sent to the console include the text of the source file

line on which the error occured

Z   no print file.  Error messages sent to the console will indicate

what error occured and on which line, but will not include the

text of the line in which the error occured

The Z option is normally only used if you are assembling a file which

probably has no errors in it (perhaps you've assembled it before and

know it has no errors), so that incomplete error messages are of no

consequence.  Using the Z option results in faster assembly since the

assembler doesn't have to make a second pass through source file.

Line numbers have been added to the print file to make it easier to

find particular lines using the editor.

The assembler now handles 8087 opcodes.