Installing your new bios in your old CP/M

In order to run the new double density roms on your old single density

system, you will first need to modify your bios. The new CP/M will

provide you with a larger transient program area because most of the

bios routines are actually in ROM. The maximum size system (63K) maps out

like this:

FA80h-FFFFh	Reserved for ROM variables and disk deblocking buffers

FA00h-FA7Fh	Interrupt vectors

F600h-F9FFh	Bios Space

There are a number of files on this disk which are used to create and

maintain your new system.

SYSGEN8D.COM 	sysgen program for 8 inch double density

SYSGEN8S.COM	sysgen program for 8 inch single density

SYSGEN5.COM	sysgen program for 5 1/4 inch double density

FORMATX7.COM	formatting program to create any of the supported disk

formats

GEN8S.SUB	submit program for creating 8 inch single density system

GEN8D.SUB	submit program for creating 8 inch double density system

GEN5.SUB	submit program for creating 5 1/4 inch, 48 TPI, system

GEN5Q.SUB	submit program for creating 5 1/4 inch, 96 TPI, system

GEN560.SUB	submit program for creating 5 1/4 inch, 48 TPI, 60K system

GEN8S60.SUB	submit program fro creating 8 inch SD, 60K system

EX14.COM	a public domain replacement for SUBMIT

The source for the BIOS is in XBIOSx.AZM which may be assembled with M80

or the Z80MR macro assembler from Micro Cornucopia (user disk K25- $12.00

503-382-8048). The file BOOT.ASM is an image for the first sector of the

system tracks and must be edited to reflect the drive arrangement on your

system. It can be assembled with ASM.COM.

To decide which programs you need to use to create your system disk you

must choose a format suitable for your A drive. If you are presently

running 8 inch single density you would want to create a single density

system disk initially then install the daughter board. Once you boot this

single density system you will be able to format a double density disk

and create a double density system. You will be able to read and write

double density on your other drives even if you boot a single density

disk.

You should be able to generate an 8 inch single density system disk on

any system running single density 8 inch CP/M, but remember that the CP/M

that is made will be constructed from the CP/M that is running on the

system at the time.  A system track can also be made using one of the

DISK UTILITIES, but this is something that requires some experience using

the disk utility.

If you are presently running a Kaypro, you will be able to create a 5 1/4

inch system disk and boot that in the Xerox on a 5 1/4 inch drive.

The first thing you must do is assemble BOOT.ASM (after editing to describe

your system) by entering ASM BOOT<carriage return>. The assembler will generate a BOOT.HEX and BOOT.PRN file, which you will use to make your new system.

Then put all of the files on this disk, the BOOT.HEX file you got from the

proceding step, MOVCPM.COM, and DDT.COM from your CPM system master on a

blank formatted disk in drive B. Log on to drive B by entering B:<carriage

return>. Now enter:

EX14 GEN8S	to create an 8 inch system disk or

EX14 GEN5       to create a 5 1/4 inch system disk

(If you've already booted a single density 8 inch XPRO system and want to

make an 8 inch double density system disk, first format a disk in b: with

FORMATX, then put all the afore mentioned files on the disk in b (with pip)

log onto b as above and enter EX14 GEN8D )

The disk in b: will be sysgened with the new XPRO system. A couple of

'gotcha's' need to be mentioned here. If you have the MOVCPM from Digital

Research of Texas (the big board people) you will need to enter:

EX14 DRMOVCPM.SUB

**********************************************************************

************************   Only do this once!   **********************

*** And only if you have a 60K CP/M from DIGITAL RESEARCH of Texas ***

**********************************************************************

This will patch your MOVCPM program to org at the correct address. (Not

512 bytes lower). Also if you have reassembled the bios and want to use

these .SUB files to load it onto a system disk, You will have to change

the name of the hex file to reflect the type of disk it is going to.

Use:

XBIOS5.HEX	for 5 1/4 inch disks

XBIOS8S.HEX	for 8 inch single density

XBIOS8D.HEX	for 8 inch double density

Now do the modifications to your board and boot up the XPRO system.

Good luck, and happy computing,

DmC & BG

last update 11/25/84  BG

#