TERMINAL ATTRIBUTES FOR THE

XEROX 820-1 WITH THE XPRO ROM SET

The Xerox 820-1 with the XPRO rom set from Micro Cornucopia -

Emerald Microware, has all of the screen (terminal) control

commands of both the Xerox 820-1 and the older ('83) KAYPRO

models (also ADM-3a).  This means that you can 'install' such

programs as Wordstar and Dbase II for either of these computers

and run them directly on your modified 820-1.  The commands are

also compatible with the Xerox 820-II to a limited extent, with

the exception of the additional graphics and inverse video on

to 'II'.

The following is a list of these commands:

Cursor Controls (note that ^ means that you press the control key

at the same time as the ASCII character indicated)

command:			hex:	decimal:	ASCII:

cursor left (back space).......	08h	08		^H

cursor right (non-destruct)....	0Ch	12		^L

cursor down (linefeed)........	0Ah	10		^J

cursor up .................... 	0Bh	11		^K

home cursor ..................	1Eh	30

clear screen & home cursor ...	1Ah	26		^Z

carriage return ..............	0Dh	13		^M

Cursor Positioning -

The cursor may be positioned (primarily used in programs such as

DbaseII to create screens) using an "escape sequence".  This

consists of a series of four characters: escape,"=", row position,

and column position.  Row position and column position are determined

by the actual character position in the column and row, each added to

20h (32 decimal).  The possible range is listed below.  To place the

cursor in the 20th character of line 7, you would enter:

"^[" + "=" + 26h + 33h   or 1Bh + 3Dh + 26h +33h

Remember that you must subtract 1 from each row and column amount since

the numbering starts at zero. Note that ESC+"B" means that you would press

the ESCAPE key and then the "B" key (not at the same time).

Escape Sequence	(ESC+"=") .... 	1Bh,3Dh

Cursor Rows ..................  0-23

Cursor Columns ...............	0-79

Positioning Sequence:

In MBASIC ...

PRINT chr$(27)+"="+chr$(32-1+row)+chr$(32-1+col);

Try it from your keyboard - "ESC"=00  should put a "?" at character 10

of line 11.  (or something like that)

Line Insert/Delete -

Line Insert (ESC+"E") ........	1Bh,45h

Line Delete (ESC+"R") ........  1Bh,52h

Clear to End of Screen/Line -

Clear EOL (^X) ..................  18h

Clear EOS (^W or ^Q) ............  17h or 11h

Set ASCII or alternate character set -

Set ASCII (ESC+"A") ..............  1Bh,41h

Set alternate (ESC+"G") ..........  1Bh,47h

After setting the alternate character set, lower case

letters will print as the Greek, or semi graphic

characters depending on which character rom that you have.

Try it from your keyboard again, see what your alternate

character set looks like.

Display control codes -

The alternate character set may be display in another manner

call the "display control code" command.  This is a control

character itself, control underscore (^_, 1Fh, or 31 decimal).

This character must immediately preceed each control character

that you want to display.  This is useful if you want to display

an occational control code such as a cent sign, etc. Most of

these codes can be displayed from the keyboard, except where

CP/M intercepts them as command characters.

Have fun, BG

#############################################