README.DOC FILE FOR MC5250PC ADAPTER HANDLER

Please note that the adapter handler is designed to provide access for one

AS/400 PC Support session.  This one session must be run from the PC or PS/2

containing the PC5250 adapter card.  The adapter handler does not currently

provide support for gateway or cluster sessions.

In order for AS/400 PC Support to function using the adapter handler:

The AS/400 must either be set to auto-config OR

it must define a data passthrough device as the terminal which will run PC

Support.  When a data passthrough device is configured, its station address

must match that in the CONFIG.PCS file.

NOTES ON RUNNING AS/400 PC SUPPORT WITH MC5250PC ADAPTER HANDLER

INSTALLATION

------------

1.  Run PC Support installation by typing INSTALL.  This batch file creates a

PCS subdirectory, copies files and creates the file CONFIG.PCS.

At the first menu select either   #1 - Work Station Emulation  OR

#2 - Enhanced 5250 Emulation

The second menu contains the following fields:

Drive - specify the drive from which you will run PC Support

Location - any name will work here

System Name - use the reference name for your AS/400

Workstation Address - specify a unique station address.  This address

MUST BE different from any logical unit address already in existence!

You may select any or all of the PC Support functions as they are all

supported by the adapter handler.       Organizer

Work Station Function

Message Function

Virtual Printer

At this point in the installation all necessary files will be copied onto the

specified drive.  You must reboot when installation has completed because

three new lines were added to the config.sys file.

2.  Edit the file STARTPCS.BAT as instructed in the MC5250PC Adapter Handler

manual.  Edit the first line to replace E5250AH.COM with L5250AH.COM.

3.  Update the Adapter Handler's settings if necessary.  The default settings

of 368 and D000 will work in most instances.  To update the settings follow

the directions in the Adapter Handler's doc.  A sample line using default

settings is:    L5250AH -AD000 -I368 -U

OPERATION

---------

1.  Type STARTPCS, the name of the batch file which will begin PC Support.

The message "Opening Adapter" will appear.  If auto-configure is being used,

it is at this point where the AS/400 detects the device type and station

address of the Adapter Handler and defines them on this line.  If auto-

configure is off, the AS/400 will attempt to establish a link to the device

type and station address which is already defined for this line.  It is my

experience that most reported problems happen here as a result of address

conflicts.  See the section on troubleshooting for more information.

2.  Enter User ID and User Password to log onto the host.  The first time PC

Support is run you will also have to set up object authority.  The AS/400 may

also download some files if there have been any recent updates.

3.  Whichever functions were selected during installation will be initialized.

The workstation screen will appear first then will hot-key out to the PCO

screen if that also was selected.

4.  Workstation function - You can access dos by hot-keying from the

workstation (press Alt-Escape) or by running PCO. Up to FIVE display or

printer sessions can be accessed at a time even though only one station

address is used on the AS/400.  To enable multiple sessions, enter the command

CFGWSF onto the dos screen.  You can configure for a standard or graphics

display, or for a 5224, 5219 or 5256 printer.  You must reboot after

configuration to enable the additional sessions.  To start up more than one

session, enter the command STARTWSF.  Alt-Esc will toggle you between multiple

sessions.  A unique station address will display on each status line.  To run

the graphics program, type WFGSHOW at dos, then do a test request from the

workstation screen.   To assign shared folders, type FSPC at dos.

5.  Virtual printer - Type GO PRINTER to bring up the AS400 printer menu.

Select #2 Work with printers to see a list of system printers.  Modify

your profile to show the correct printer ID.

TROUBLESHOOTING

---------------

The major source of customer problems so far has been address conflicts.

Symptoms of this problem are the following two PC Support error messages:

5118 Adapter cannot be opened

5119 Link connection failed

These messages appear after the startrtr command as the link between the AS400

and the PC is attempted.

To verify the address which is being used on the PC side, check the CONFIG.PCS

file.  A sample CONFIG.PCS file is shown below.

RTYP 5250

RTLN APPN.MYPC

EMLI MYAS400,5

The first line shows how you're connected (local twinax here).  The second

line shows the remote location name (MYPC). This name is unique for each user.

The third line shows the system name (MYAS400) and the station address (5).

This station address MUST BE different from any logical unit address being

used by Twinax.

To verify the address being used on the AS400 side, enter the command:

GO CFGLCL

The Configure Local Hardware menu will appear.  Check options #2 for printers

and #3 for displays to determine which station addresses are in use.  It is

helpful to modify the text description for each device so that you can see at a

glance which port and switch setting are being used.  The port number is the

line being used and the switch setting is the station address.  The adapter

handler will be shown as a 5150 device.  Verify that no two devices are trying

to use the same station address.  Compare the address in the config.pcs file

to that shown for the 5150 device on the AS400.  You can use this menu to vary

on or off devices.  You may also need to re-IPL to recover from some address

conflicts.

Note that PC Support does not have any configurable keyboard support.  It is

possible that problems may be seen on non-standard keyboards.