This is the 1.14 release of CPL   's 10/100VG-AnyLAN NDIS 2.01 Windows for

Workgroups driver.

To install:

1) Select "Network" from the Program Manager window

2) Select "Network Setup" from the Network window

3) The Network Setup will display the current installed network drivers and

the protocols bound to that adapter.

4) Click on "Add Adapter"

5) Select "Unlisted or Updated Network Adapter"

6) Click on "OK"

7) Enter the driver volume and directory, typically A:\WFW, and click on "OK"

8) Select "CPL    10/100 VG-AnyLAN"

9) Click on "OK"

10) The driver files are copied to the Windows directories.

11) The PROTOCOL.INI and SYSTEM.INI are modified, click "OK"

12) To start the adapter the computer must be restarted.

NOTE:  The IOADDRESS parameter in the PROTOCOL.INI file is REQUIRED on most

CPUs; if not specified, a default ISA address of 0x300 will be used!

Please refer to the IOADDRESS section of the keywords data (below).

Keywords supported:

DRIVERNAME

Identifies the network device driver.  The DRIVERNAME is R100VG$ for

the first driver, R100VG$2 for the second, etc.    This parameter is

required.

IOADDRESS

Identifies the I/O address of the adapter.  For EISA adapters the I/O

address is 0x?C38 where ? is the slot that the adapter is located

(0x1C38 for slot 1, 0x2C38 for slot 2, etc.).   For ISA adapters the

I/O address is set with the VGCFG.EXE utility; and for PCI adapters,

the I/O address must be determined by examining the system's PCI BIOS.

If not specified, a default ISA address of 0x300 will be used.   This

parameter is required.

BUFFERSIZE

Specifies the driver's input buffer size.    A value of 1 allocates a

34K input buffer, and a value of 2 allocates a 60K buffer (for "peer"

workstations or other high-use DOS systems).

NETADDRESS

This entry allows the driver to override the node address located in

the 100VG EEPROM.  The entry must be a 12-digit string of hexadecimal

characters, and must be enclosed in quotes.  Also, the low-order bit

of byte 0 must be zero.

To use a keyword edit the protocol.ini in the windows directory.  This is

usually \windows.  In the appropriate NIF section, add the keywords. Below

is an example using each of the keywords above.  For most situations you

will not need any keywords.

The following is a sample PROTOCOL.INI file for an ISA adapter:

[RVGNDIS_NIF]

DRIVERNAME = R100VG$

IOADDRESS = 0x300

BUFFERSIZE = 2

NETADDRESS = "000000000312"