S.T.B. Systems Compatibility Report

1651 North Glenville

Richardson ,TX  75085             Product Tested: Powergraph ET 4000

MONITOR MAKE & SIZE:NEC XV15	DATE OF REPORT: 7/21/95

C.P.U. MAKE & SPEED:GW P5-60	VIDEO  BIOS  REV: Powergraph VGA 2.3

SYSTEM  &  BIOS   VER:GW Award	WIN & UTIL DRV. REV: Win 95 Upgrade

MONITOR MAKE & SIZE:	 	DATE OF REPORT:

C.P.U. MAKE & SPEED:	 	VIDEO  BIOS  REV:

SYSTEM  &  BIOS   VER:		WIN & UTIL DRV. REV:

TECHNICIANS NAME: Gene Pippenger

-----SPECIFIC HARDWARE OR APPLICATION ERRORS -----

Refresh rates established by Win 95 for the NEC XV15:

640 x 480: 60 Hz, 800 x 600: 72 Hz, 1024 x 768: 60Hz NI,

1280 x 1024 or 1600 x 1200: not offered.

Applications/Accessories Run without incident: Those that come with Win 95:

Videos, Paint, Diagnostics, DOS prompt, games, all on the same screen with no lockup.

Drivers run without problems:

640 x 480 x 16		640 x 480 x 256	640 x 480 x 16 bit

800 x 600 x 16		800 x 600 x 256

1024 x 768 x 256

1024 x 768 x 16 was not offered.

800 x 600 x 16 bit was offered but had severe flicker.

640 x 480 x true color was offered but gave no video.