| Field Note: 149 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Installed lines in SYSTEM.INI

Date Created: 	July 30, 2026

Product: 	Captivator

The Captivator Setup adds lines to SYSTEM.INI and these are described in the README.TXT file for Captivator v1.0.3.  If there is an install problem, the lines in the section [vlcap.drv] will not be added.  In this case, it is best to change the I/O base address link on the card to a value not used by other cards in the system, and then run the Setup again.

If this fails as well, then try to add the lines manually as shown below.  Explanations are given to the right of each line.

[vlcap.drv]

Interrupt=9		; Set this interrupt request to a value not used by

other cards in the system.  Choose from 3, 5, 7, 9,

10, 15.

PortBase=310		; Set this I/O base address to the same value as the link

on the card

autoisrscan=0		; Auto interupt scan; 0 is off.  This line is optional.

videostandard=0	; 0 is NTSC, 1 is PAL

videocable=1		; 1 is S-video DIN, 0 is compostite phono(RCA)

width=160			; The video capture size

height=120

constrained=1		; Aspect ratio constrained to 4:3; 1 is on

videoformat=1		; DIB capture format; 0 is 8-bit, 1 is 16-bit,

2 is 24-bit

autodetect=1		; Video standard auto-detect; 1 is on

## Hints

Interrupt 3 is normally used by the serial port COM2.

Interrupt 5 is the default for SoundBlaster.

Interrupts 7 or 9 are often used by network cards.

Interrupt 9 is used by some SVGA cards.

I/O address 300 is used by some SCSI and network cards.

Steve Everitt

July, 26