***************************************

*                                     *

*    MIC System I SuperVGA support    *

*        for ATI graphics cards       *

*             Version 1.0             *

*                                     *

***************************************

*************************  I M P O R T A N T  *************************

1.  This enhancement is supplied to allow the DVA-4000 to operate in

supported SuperVGA modes with selected ATI graphics cards and with

the DELL 466ME on-board S3-based VGA.

2.  The enhancement has been tested on an ATI Integra with 512Kb RAM

(supporting 640 x 480 256 colours and 1024 x 768 16 colours), and

with an ATI VGAWONDER XL24 with 1Mb RAM (supporting 640 x 480 256

colours, 1024 x 768 16 colours and 1024 x 768 256 colours).

- You must have MIC System I v4.0 or above to use this enhancement

with MIC System I

- For the ATI graphics cards to correctly sense that the monitor

connected is capable of 1024 x 768 interlaced modes, you must

have a VGA terminator attached to the ATI graphics output connector.

If the terminator is not present, the boards assume that only

640 x 480 modes are supported, and this will cause confusion if

any attempt is made to enter modes S3 or S4 (1024 x 768).

- Certain ATI graphics cards are not supported by MIC. The ATI

Graphics Ultra has no feature connector, and the ATI Graphics

Ultra+ displays palette corruption.

3.  The S3 enhancement is listed as VESA compatible VGAs within

MICSETUP.EXE. However there is no guarantee that any particular

VESA compatible VGA will work as the standard does not appear to be

strictly adhered to.

4.  MIC System I Installation procedure

- This software is installed for MIC System I from within AMS in the

usual way from the "Install an AMS Application" option on the main

menu.

The procedure renames the file

DVA-VGA.HLP ---> DVA-VGA.OLD

DVA-SMOD.EXE --> DVA-SMOD.OLD

and then copies the DVA-VGA.HLP and DVA-SMOD.EXE files from this

diskette into the MICFILES directory. For that reason the destination

drive specified must be the one on which MIC is installed.