| System Unit | Dimensions (HxWxD) |  |  | 20.25 x 8.38 x 18.60 in/51.44 x 21.29 x 47.24 cm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weight (approximate) |  |  | 40 lbs/18.1 kg |  |  |  |
|  | Weight Supported |  |  | 100 lbs/45.5 kg |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Power Supply |  |  |  |  |  |  |
|  | Operating Voltage Range |  |  | 90 - 132 VAC/180 - 264 VAC |  |  |  |
|  | Rated Voltage Range |  |  | 100 - 120 VAC/220 - 240 VAC |  |  |  |
|  | Rated Line Frequency |  |  | 47 - 63 Hz |  |  |  |
|  | Rated Input Current |  |  | 5.5A |  |  |  |
|  | Maximum Rated Power |  |  | 200 W |  |  |  |
|  | Temperature |  |  |  |  |  |  |
|  | Operating |  |  | 50° to 95°F/10° to 35°C |  |  |  |
|  | Nonoperating |  |  | -4° to 140°F/-20° to 60°C |  |  |  |
|  | Humidity (noncondensing) |  |  |  |  |  |  |
|  | Operating |  |  | 8% to 90% |  |  |  |
|  | Nonoperating |  |  | 5% to 95% |  |  |  |
|  | Maximum Altitude (unpressurized) |  |  |  |  |  |  |
|  | Operating |  |  | 10,000 ft/3048 m |  |  |  |
|  | Nonoperating |  |  | 30,000 ft/9144 m |  |  |  |
| Enhanced Keyboard | Dimensions (HxWxD) Weight |  |  | 1.5 x 18.0 x 6.5 in/3.81 x 45.72 x 16.51 cm 3.15 lbs/1.59 kg |  |  |  |
| 1.44-MB Diskette Drive | Diskette Size |  |  | 3.5-in |  |  |  |
|  | LED Indicators |  |  | Green |  |  |  |
|  | Read/Write Capacity/Diskette (high/low density) |  |  | 1.44-MB/720 KB |  |  |  |
|  | Drive Height |  |  | 1 (in) |  |  |  |
|  | Drive Rotation |  |  | 300 rpm |  |  |  |
|  | Transfer Rate (high/low density) |  |  | 500/250 Kb/s |  |  |  |
|  | Bytes/Sector |  |  | 512 |  |  |  |
|  | Sectors/Track (high/low density) |  |  | 18/9 |  |  |  |
|  | Tracks/Side (high/low density) |  |  | 80/80 |  |  |  |
|  | Seek Time |  |  |  |  |  |  |
|  | Track-to-Track(high/low density) |  |  | 3/6 ms |  |  |  |
|  | Average (high/low) |  |  | 94/173 ms |  |  |  |
|  | Settling Time |  |  | 15 ms |  |  |  |
|  | Latency Average |  |  | 100 ms |  |  |  |
|  | Cylinders (high/low) |  |  | 80/80 |  |  |  |
|  | Read/Write Heads |  |  | 2 |  |  |  |
| SMART II Ultra ATA Hard Drives1 | Formatted Capacity |  | 3.2 GB2 3200 MB |  | 6.4 GB2 6400 MB |  | 10.0 GB2 10000 MB |
|  | Drive Type |  | 65 (soft) |  | 65 (soft) |  | 65 (soft) |
|  | Interface |  | Ultra ATA |  | Ultra ATA |  | Ultra ATA |
| SMART II Ultra ATA Hard Drives1 (cont’d) |  |  | 3.2 GB2 |  | 6.4 GB2 |  | 10.0 GB2 |
|  | Transfer Rate |  |  |  |  |  |  |
|  | Heads |  | 68-131 Mb/s |  | 85-136 Mb/s |  | 18.6 Mb/s |
|  | Interface |  | 33.3 MB/s |  | 33.3 MB/s |  | 33.3 MB/s |
|  | Seek Time (typical including settling) |  |  |  |  |  |  |
|  | Single Track |  | 2.0 ms |  | 2.0 ms |  | < 1 ms |
|  | Average |  | 9.7 ms |  | 9.7 ms |  | 9.5 ms |
|  | Full Stroke |  | 19 ms |  | 20 ms |  | <20 ms |
|  | Disk Rotational Speed |  | 5400 rpm |  | 5400 rpm |  | 5400 rpm |
|  | Cylinders |  |  |  |  |  |  |
|  | Physical |  | 5691-8765 |  | 7913 |  | 11658 |
|  | Logical |  | 6296 |  | 13328 |  | 16383 |
|  | Data Heads |  |  |  |  |  |  |
|  | Physical |  | 6 |  | 6 |  | 6 |
|  | Logical |  | 16 |  | 15 |  | 16 |
|  | Sectors per Track |  |  |  |  |  |  |
|  | Physical |  | 114-312 |  | 168-312 |  | 216-320 |
|  | Logical |  | 63 |  | 63 |  | 63 |
|  |  | Buffer Size | 128 KB |  | 256 KB |  | 256 KB |
| SMART II Wide UltraSCSI Hard Drives1 | Formatted Capacity |  |  | 4.3 GB2 4.29 GB2 |  |  | 9.1 GB2 9.1 GB2 |
|  | Transfer Rate |  |  |  |  |  |  |
|  | Media |  |  | 109-171 Mb/s |  |  | 109-171 Mb/s |
|  | Interface |  |  | 40 MB/s |  |  | 40 MB/s |
|  | Disk Rotational Speed |  |  | 7200 rpm |  |  | 7200 rpm |
|  | Seek Time (typical including settling) |  |  |  |  |  |  |
|  | Single Track |  |  | .75 ms |  |  | 1.5 ms |
|  | Average |  |  | 7.5 ms |  |  | 7.6 ms |
|  | Full Stroke |  |  | 15 ms |  |  | 15 ms |
|  | Cylinders |  |  |  |  |  |  |
|  | Physical |  |  | 8,419 |  |  |  |
|  | Logical |  |  | — |  |  | — |
|  | Data Heads |  |  | 5 |  |  | 10 |
|  | Sectors per Track |  |  |  |  |  |  |
|  | Physical |  |  | 165-264 |  |  | 165-265 |
|  | Logical |  |  | — |  |  | — |
|  | Cache Buffer Size |  |  | 384 KB |  |  | 384 KB |

| 32X Max3 trayload CD-ROM Drive IDE . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Disk Applicable Disk |  | CD-ROM (Mode 1 and 2), CD-DA, CD-XA Ready, |  |
| --- | --- | --- | --- | --- |
|  |  |  | Photo CD (single and multi session), Mixed Mode (audio and data combined), CDI Ready |  |
|  | Capacity |  | 650 MB |  |
|  | Block Size |  | 2048 bytes (Mode 1) 2340, 2336 bytes (Mode 2) |  |
|  |  |  | 2352 bytes (CD-DA) |  |
|  |  |  | 2328 bytes (CD-XA) |  |
|  | Rotational Speed |  | 5200 rpm |  |
|  | Diameter |  | 12 cm, 8 cm |  |
|  | Center Hole |  | 15 mm |  |
|  | Thickness |  | 1.2 mm |  |
|  | Track Pitch |  | 1.6 µm |  |
|  | Interface |  | IDE |  |
|  | Access Time (typical) Random |  | < 100 ms |  |
|  | Full Stroke |  | < 150 ms |  |
|  | Audio Output Level |  |  |  |
|  | Line Out |  | .7 V @ 47 K ohm |  |
|  | Headphone |  | 0.6 V @ 32 ohm |  |
|  | Data Transfer Rate |  |  |  |
|  | Sustained |  | 150 KB/s |  |
|  | Variable |  | 1800-4800 KB/s |  |
|  | Cache Buffer |  | 128 KB |  |
|  | Startup Time |  | < 7 seconds |  |
|  | Stop Time |  | < 4 seconds |  |
|  | Laser Parameters |  |  |  |
|  | Type |  | Semiconductor Laser ConA1 As |  |
|  | Wave Length |  | 780 +/- 25 nm |  |
|  | Divergence Angle |  | 53.5° +/- 1.5° |  |
|  | Output Power |  | 0.14 mW |  |
|  |  | Operating Conditions Temperature | 41° to 113° F/5° to 45°C |  |
|  |  | Humidity | 10 to 80% |  |
|  |  | Dimensions (HxWxD, maximum) | 1.69 x 5.91 x 8.2 in/4.29 x 15.01 x 20.8 cm |  |
|  |  | Weight | 2.65 lbs/1.2 kg |  |
| Compaq 10/100 TX PCI Intel WOL UTP Controller4 |  | Network Interface Expansion Bus Architecture Network Transfer Rate | 10/100 Base TX 32-Bit PCI bus-master |  |
|  |  | 10Base-T (Half-Duplex) | 10 Mb/s |  |
|  |  | 10Base-T (Full-Duplex) | 20 Mb/s |  |
|  |  | 100Base-TX (Half-Duplex) | 100 Mb/s |  |
|  |  | 100Base-TX (Full-Duplex) | 200 Mb/s |  |
|  |  | Connectors | RJ-45 |  |
|  |  | Standards Compliance |  |  |
|  |  | 10 Mb/s Ethernet | IEEE 802.3 |  |
|  |  | 100 Mb/s Ethernet | IEEE 802.3u |  |
| Compaq 10/100 TX PCI Intel WOL UTP Controller4 (cont.) |  | I/O Address | Auto-configured |  |
|  |  | Remote Wakeup | Yes |  |
|  |  | Environmental Requirements |  |  |
|  |  | Operating | 10° to 35°C; 10% to 90% humidity |  |
|  |  |  | (noncondensing); 3048 m altitude |  |
|  |  | Nonoperating | 10° to 50°C; 10% to 90% humidity |  |
|  |  |  | (noncondensing); 9144 altitude |  |
|  |  | Shipping | 30° to 50°C; 20% to 80% humidity |  |
|  |  |  | (noncondensing); 3084 m altitude |  |
|  |  | Communication Processor | Intel 82558 |  |
|  |  | OS Driver Support MS Windows NT Server and Workstation v. 3.51 MS Windows NT Workstation 4.0 MS Windows 95 (OSR2) | NDIS 3.0 Compliant NDIS 4.0 Compliant NDIS 2.0, 3.0/4.0 Compliant |  |
|  | WFW v.3.11 |  | NDIS 3.0 Compliant |  |
|  | MS LAN Manager |  | NDIS 2.0 Compliant |  |
|  | Novell NetWare Server v. 3.x, 4.x |  | ODI Compliant |  |
|  | Novell NetWare DOS |  | ODI Compliant |  |
|  | IBM OS/2 v. 2.x, 3.x, 4.x, WARP5, SMP |  | OS/2 NDIS 2.0 Compliant |  |
|  | IBM OS/2 NetWare Requestor v.2.x, 3.x |  | ODI Compliant |  |
| ATI RAGE PRO TURBO AGP Graphics Controller | Resolution (maximum) 1600 x 1200 |  | 4-MB SGRAM Colors Supported 65K | Refresh Rate 85Hz |
|  | Acceleration Memory |  | 3 ROP BitBLT, Line Draw, Text, Color Expansion, Basic Video Color, Conversion and Scaling and Triangle Setup Engine 4-MB SGRAM, not upgradeable |  |
| ATI RAGE PRO TURBO AGP 2X Graphics Controller | Resolution (maximum) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. 1280 x 1024 | 8-MB SGRAM Colors Supported 16.7M | Refresh Rate 75Hz |
|  | Acceleration Memory |  | 3 ROP BitBLT, Line Draw, Text, Color Expansion, Basic Video Color, Conversion and Scaling and Triangle Setup Engine 4-MB SGRAM, upgradeable to 8-MB |  |
|  | 1 Hard drive specifications will vary depending on vendor. 2 For Hard Drives, GB = Billion Bytes. 3 Data transfer rates will vary from 150 KB/s to 4800 KB/s. 4 Also known as the Compaq NC3121 PCI 10/100 WOL (Wake-on-LAN) Fast Ethernet NICD. 5 Not all product features are supported in this OS. Compaq, registered United States Patent and Trademark Office. Armada, Armada Station, Ministation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation. Intel, the Intel Inside Logo, and Pentium are registered trademarks of Intel Corporation. Microsoft, MS, Windows and Windows NT are registered trademarks of the Microsoft Corporation. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |  |  |