| System Unit | Dimensions (HxWxD) | 5.0 x 17.71 x 16.72 in/12.70 x 44.98 x 42.47 cm |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Weight (approximate) | 23.7 lb/10.74 kg |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |
| Power Supply |  |  |  |  |  |
|  | Operating Voltage Range | 90-132 VAC/180-264 VAC |  |  |  |
|  | Rated Voltage Range | 100-120 VAC/220-240 VAC |  |  |  |
|  | Rated Line Frequency | 50-60 Hz |  |  |  |
|  | Rated Input Current | 5A/3A |  |  |  |
|  | Maximum Rated Power | 200 W |  |  |  |
|  | Temperature |  |  |  |  |
|  | Operating | 50° to 95°F/10° to 35°C |  |  |  |
|  | Nonoperating | 50° to 122°F/10° to 50°C |  |  |  |
|  | Humidity (noncondensing) |  |  |  |  |
|  | Operating | 8% to 90% |  |  |  |
|  | Nonoperating | 5% to 95% |  |  |  |
|  | Maximum Altitude (unpressurized) |  |  |  |  |
|  | Operating | 10,000 ft/3048 m |  |  |  |
|  | Nonoperating | 30,000 ft/9144 m |  |  |  |
| Enhanced Keyboard | Dimensions (HxWxD) Weight | 1.5 x 18.0 x 6.5 in/3.81 x 45.72 x 16.51 cm 2.58 lb/1.17 kg |  |  |  |
| 1.44-MB Diskette Drive | Diskette Size | 3.5-in |  |  |  |
|  | LED Indicators | Green |  |  |  |
|  | Read/Write Capacity/Diskette (high/low density) | 1.44 MB/720 KB |  |  |  |
|  | Drive Height | One-third (in) |  |  |  |
|  | Drive Rotation | 300 rpm |  |  |  |
|  | Transfer Rate (high/low density) | 500/250 Kb/s |  |  |  |
|  | Bytes/Sector | 512 |  |  |  |
|  | Sectors/Track (high/low density) | 18/9 |  |  |  |
|  | Tracks/Side (high/low density) | 80/80 |  |  |  |
|  | Seek Time |  |  |  |  |
|  | Track-to-Track (high/low density) | 3/6 ms |  |  |  |
|  | Average (high/low) | 94/173 ms |  |  |  |
|  | Settling Time | 15 ms |  |  |  |
|  | Latency Average | 100 ms |  |  |  |
|  | Cylinders (high/low) | 80/80 |  |  |  |
|  | Read/Write Heads | 2 |  |  |  |
| Ultra ATA Hard Drives1 |  | 2.1-GB Ultra ATA |  | 3.2-GB Ultra ATA |  |
|  | Formatted Capacity | 2.1 GB |  | 3.2 GB |  |
|  | Transfer Rate |  |  |  |  |
|  | Media | 27-55 MB/s |  | 87.8 MB/s |  |

Interface33 MB/s33 MB/s

| Ultra ATA Hard Drives1 (cont’d) |  | 2.1-GB Ultra ATA | 3.2-GB Ultra ATA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Seek Time (typical including settling) |  |  |  |  |  |
|  | Single Track | 2.0 ms | 2.0 ms |  |  |  |
|  | Average | 12.0 ms | 12.0 ms |  |  |  |
|  | Full Stroke | 20 ms | 20 ms |  |  |  |
|  | Disk Rotational Speed | 4500 rpm | 4500 rpm |  |  |  |
|  | Cylinders |  |  |  |  |  |
|  | Physical | 5691 | 5691 |  |  |  |
|  | Logical | 4092 | 6218 |  |  |  |
|  | Data Heads | 16 | 16 |  |  |  |
|  | Sectors per Track |  |  |  |  |  |
|  | Physical | 114-234 | 114-234 |  |  |  |
|  | Logical | 63 | 63 |  |  |  |
|  | Buffer Size | 128 KB | 128 KB |  |  |  |
| 16X Max2 CD-ROM Drive IDE (slot-load) | Disk Applicable Disk | CD-ROM (Mode 1 and 2), CD-DA, CD-XA |  |  |  |  |
|  |  | (Mode 2, Form 1 and 2), Photo CD (single and multi session), Mixed Mode (audio and data combined) |  |  |  |  |
|  | Capacity | 550 MB (Mode 1), 640 MB (Mode 2), 180 MB (8 cm) |  |  |  |  |
|  | Block Size | 2048, 1024 bytes (Mode 1) |  |  |  |  |
|  |  | 2340, 2336, 1024 bytes (Mode 2) |  |  |  |  |
|  |  | 2352 bytes (CD-DA) |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |
| Rotational Speed | 1840-4240 rpm |  |  |  |  |  |
|  | Diameter | 4.7 in, 3.15 in/12 cm, 8 cm |  |  |  |  |
|  | Center Hole | .6 in/15 mm diameter |  |  |  |  |
|  | Thickness | .05 in/1.2 mm |  |  |  |  |
|  | Track Pitch | 1.6 µm |  |  |  |  |
|  | Interface | IDE |  |  |  |  |
|  | Access Time (typical) |  |  |  |  |  |
|  | Random | 100 ms |  |  |  |  |
|  | Full Stroke | 200 ms |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |
|  | Line Out | 0.7 VRMS at 47 kOhms |  |  |  |  |
|  | Headphone | 0.6 VRMS at 32 Ohms (maximum volume) |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |
|  | Sustained | 3000 KB/s |  |  |  |  |
|  | Burst | 4 MB/s |  |  |  |  |
|  | Cache Buffer | 128 KB |  |  |  |  |
|  | Startup Time | Less than 7 seconds |  |  |  |  |
|  | Stop Time | Less than 4 seconds |  |  |  |  |
|  | Laser Parameters |  |  |  |  |  |
|  | Type | Semiconductor Laser GaAIA |  |  |  |  |
|  | Wave Length | 790 +/- 25 nm |  |  |  |  |
|  | Divergence Angle | 53.5° +/- 1.5° |  |  |  |  |
|  | Output Power | 0.14 mW |  |  |  |  |
| 16X Max2 CD-ROM Drive | Operating Conditions |  |  |  |  |  |
| IDE (slot-load) (cont’d) | Temperature | 41-113° F/5-45° C |  |  |  |  |
|  | Humidity | 10-80% |  |  |  |  |
|  | Dimensions (HxWxD, maximum) | 1.69 x 5.90 x 8.19 in/4.29 x 15.0 x 20.8 cm |  |  |  |  |
|  | Weight | 2.09 lbs/.95 kg |  |  |  |  |
| 24X Max3 CD-ROM Drive IDE (slot-load) | Disk Applicable Disk | CD-ROM (Mode 1 and 2), CD-DA, CD-XA |  |  |  |  |
|  |  | (Mode 2, Form 1 and 2), Photo CD (single and multi session), Mixed Mode (audio and data combined) |  |  |  |  |
|  | Capacity | 650 MB |  |  |  |  |
|  | Block Size | 2048, bytes (Mode 1) |  |  |  |  |
|  |  | 2040, 2336 bytes (Mode 2) |  |  |  |  |
|  |  | 2352 bytes (CD-DA) |  |  |  |  |
|  |  | 2328 bytes (CD-XA) |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |
| Rotational Speed | 5200 rpm |  |  |  |  |  |
|  | Diameter | 12 cm, 8 cm |  |  |  |  |
|  | Center Hole | N/A |  |  |  |  |
|  | Thickness | 1.2 mm |  |  |  |  |
|  | Track Pitch | 1.6 µm |  |  |  |  |
|  | Interface | IDE |  |  |  |  |
|  | Access Time (typical) |  |  |  |  |  |
|  | Random | < 100 ms |  |  |  |  |
|  | Full Stroke | < 200 ms |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |
|  | Line Out | 700 MV (.7 volts RMS) |  |  |  |  |
|  | Headphone | 0.6 VRMS |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |
|  | Sustained |  |  |  |  |  |
|  | Burst | 3.6 MB/s (variable) |  |  |  |  |
|  | Cache Buffer | 128 KB |  |  |  |  |
|  | Startup Time | Less than 7 seconds |  |  |  |  |
|  | Stop Time | Less than 7 seconds |  |  |  |  |
|  | Laser Parameters |  |  |  |  |  |
|  | Type | Semiconductor Laser GaAIAs |  |  |  |  |
|  | Wave Length | 780 +/- 25 nm |  |  |  |  |
|  | Divergence Angle | 53.5° +/- 1.5° |  |  |  |  |
|  | Output Power | 0.14 mW |  |  |  |  |
|  | Operating Conditions |  |  |  |  |  |
|  | Temperature | 41-113° F/5-45° C |  |  |  |  |
|  | Humidity | 10-80% |  |  |  |  |

Dimensions (HxWxD) 1.69 x 5.90 x 8.19 in/4.29 x 15.0 x 20.8 cmWeight2.65 lbs/1.20 kg

| S3 Trio 64V2/GX Integrated PCI | Resolution | 1-MB SGRAM Colors Supported | 2-MB SGRAM Colors Supported |  | Refresh Rate |  |
| --- | --- | --- | --- | --- | --- | --- |
| Grap . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . hics Controller |  |  |  |  |  |  |
| 640 x 480 | 16.7M (32 bpp) | 16.7M (32 bpp) | 85 Hz |  |  |  |
|  | 800 x 600 | 64K (16 bpp) | 16.7M (32 bpp) |  | 85 Hz |  |
|  | 1024 x 768 | 256 | 64 K (16 bpp) |  | 85 Hz |  |
|  | 1280 x 1024 |  |  |  |  |  |
| 256 (8 bpp)75 Hz | Acceleration Memory | 2D GUI, 170 MHz RAMDAC 1-MB 60 ns SGRAM with upgrade to 2 MB available |  |  |  |  |
| Matrox MGA 1064/SG Performance Graphics | Resolution | 2-MB SGRAM Colors Supported |  | 4-MB SGRAM Colors Supported |  | Refresh Rate |
|  | 640 x 480 800 x 600 1024 x 768 1280 x 1024 1600 x 1200 | 256, 65K, 16.7M 256, 65K 256, 65K 256 256 |  |  |  |  |
| 16.7M 16.7M 16.7M 16.7M 65K85 Hz 85 Hz 85 Hz 85 Hz 75 Hz | Acceleration Memory |  |  |  |  |  |
| 2D GUI, 220 MHz RAMDAC 2-MB SGRAM standard with 2-MB SGRAM option available (2 + 2 MB total) |  |  |  |  |  |  |
|  | 1 Pentium Processor with MMXÔ technology models manufactured prior to February 15, 1998 may be configured with EIDE hard drives. 2 Data transfer rate will vary from 150 KB/s to 2400 KB/s. 3 Data transfer rate will vary from 150 KB/s to 3600 KB/s. |  |  |  |  |  |

Compaq, Compaq Insight Manager, Deskpro, registered United States Patent and Trademark Office. Netelligent, SmartQ,

MultiLock, QuickBlank, QuickLock, are trademarks and /or service marks of Compaq Computer Corporation.

Novell and NetWare are registered trademarks of Novell, Inc. Microsoft, Windows, Windows NT and SQL Server are trademarks

of Microsoft Corporation. IBM and OS/2 are registered trademarks of IBM Corporation. SCO is a registered trademark of the Santa

Cruz Operation, Inc. UNIX is a registered trademark of the Open Group in the US and other countries, Inc. ORACLE is a registered

trademark and ORACLE7 is a trademark of Oracle Corporation. Cheyenne and ARCserve are registered trademarks of Cheyenne

Software, Inc. JETserve is a trademark of Cheyenne Software, Inc.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.