APR 1988                FAT-VIDEO 1.21  SETVM           Page V-1

SETVM(MODE)-Set the Video Mode

Usage:

call setvm(mode)

integer*2 mode

inputs: mode

returns: none

MS-Fortran 4.0 Large Model

Description:

This program sets the video mode. The available modes are

defined below.

See also getmod().

mode  Display

----   --------

00   = 40*25 text B&W,cga

01   = 40*25 color text,cga

02   = 80*25 B&W text

03   = 80*25 color text

04   = 320*200 4-color,cga

05   = "   "    "         ,color burst off

06   = 640*200 2-color,cga

07   = Mono & EGA display

08   = 160*200 pc jr

09   = 320*200 pc jr

0A   = 640*200 pc jr

0D   = 320*200 EGA

0E   = 640*200 16-color EGA

0F   = 640*350 mono graphics,EGA

10   = 640*350 4/16 color graphics,EGA

(4/16-depends on ram)

Int 10h, fx-00h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  SETCP           Page V-2

SETCP(ROW,COL,PAGE) - Set Cursor Position

Usage:

call setcp(row,col,page)

integer*2 row,col,page

inputs:   page,row,col

returns:  none

MS-Fortran 4.0 Large Model

This subroutine sets the cursor position to

the values given in row, column and page.  The row

values are 0-23, the column values are 0-39 or 0-79

depending on the text mode in use.

The cursor may be located in graphics modes using

this function the same as in text modes. See the

setdp() function regarding pages.

See also getcp().

Int 10h, fx-02h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  GETCP           Page V-3

GETCP(ROW,COL,PAGE) - Get Cursor Position

Usage:

call getcp(row,col,page)

integer*2 row,col,page

inputs:   page

returns:  row,col

MS-Fortran 4.0 Large Model

This subroutine gets the cursor position for

the specified page, and returns row, and column.

The row values are 0-23, the column values are

0-39 or 0-79 depending on the text mode in use.

See the setdp() function regarding pages in the

various adapters.

See also setcp().

Int 10h, fx-03h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  SETDP           Page V-4

SETDP(PAGE) - SET the Display Page

Usage:

call setdp(page)

integer*2 page

inputs: page

returns: none

MS-Fortran 4.0 Large Model

Description:

This subroutine sets the active display page.

The Mono Adapter only has 1 display page. The CGA

adapter has 8 display pages in modes 0 and 1, and

4 text display pages in modes 2 and 3. See the chart

below for other modes.

See also getmod().

Page    Modes(hex)

-----   ----------

0-7     00-01 CGA

0-3     02-03 CGA

0-7     02-03 EGA

0-7     0D    EGA

0-3     0E    EGA

0-1     0F    EGA

0-1     10h   EGA

Int 10h, fx-05h

CGA, EGA

APR 1988                FAT-VIDEO 1.21  SCWUP           Page V-5

SCWUP(LINES,ATTR,ULR,ULC,LRR,LRC) - Scroll Window Up

Usage:

call scwup(lines,attr,ulr,ulc,lrr,lrc)

integer*2 lines,attr,ulr,ulc,lrr,lrc

inputs:  lines,attr,ulr,ulc,lr,lrc

returns: none

MS-Fortran 4.0 Large Model

Description:

This subroutine scrolls up the rectangular area defined

by ulr,ulc and lrr,lrc the specified number of lines,

filling the blank lines with the background color defined in

the attribute byte.

The attribute byte may be set by setab(), or from a

call to getac().

See  also scwdn(), cls(), getmod(), setvm().

lines = 0-25 no. of lines to scroll(0 -> clears window)

attr  = attribute byte to use, for background color

ulr   = 0-24 upper left y-coord(row) corner of scroll region

ulc   = 0-79 "     x "     " (col)              "

lrr   = 0-24 lower right y-coord(row)

lrc   = 0-79  "    "     x-coord(col)

Int 10h, fx-06h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  SCWDN           Page V-6

SCWDN(LINES,ATTR,ULR,ULC,LRR,LRC) - Scroll Window Down

Usage:

call scwdn(lines,attr,ulr,ulc,lrr,lrc)

integer*2 lines,attr,ulr,ulc,lrr,lrc

inputs:   lines,attr,ulr,ulc,lrr,lrc

returns:  none

MS-Fortran 4.0 Large Model

Description:

This subroutine scrolls down the rectangular area defined

by ulr,ulc and lrr,lrc the specified number of lines,

filling the blank lines with the background color defined in

the attribute byte.

The attribute byte may be set by setab(), or returned from a

call to getac().

See also scwup(), cls(), getmod(), setvm().

lines = 0-25 no. of lines to scroll(0 -> clears window)

attr  = attribute byte to use, for background color

ulr   = 0-24 upper left y-coord(row) corner of scroll region

ulc   = 0-79 "     x "     " (col)              "

lrr   = 0-24 lower right y-coord(row)

lrc   = 0-79  "    "     x-coord(col)

Int 10h, fx-07h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  GETAC           Page V-7

GETAC(ATTR,KAR,PAGE) - Get the Attribute and Character at the

cursor.

Usage:

call getac(attr,kar,page)

integer*2 attr,kar,page

inputs:   page

returns:  attr,kar

MS-Fortran 4.0 Large Model

Description:

This subroutine gets the attribute and ascii character code,

or extended character code, at the current cursor position. The

display page is not needed in graphics modes. In graphics modes

if the character is not recognized, a 0 is returned.

This function will return the ascii or extended code in either

text or graphics modes.

See also getab(), wrtac(), wrtacp(), wrtas(), wrtasp().

wprint(), wprnas(), setab()

attr = attribute byte, as output by setab()

kar  = character ascii code

Page = legal Page for adapter in use

Int 10h, fx-08h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  WRTAC           Page V-8

WRTAC(ATTR,KAR,PAGE) - Write Attribute and Character at the cursor.

Usage:

call wrtac(attr,kar,page)

integer*2 attr,kar,page

inputs:   attr,kar,page

returns:  none

MS-Fortran 4.0 Large Model

Description:

This subroutine writes the attribute and character at the

current cursor position. The attribute is the output of getac,

or of setab. This function works in text and graphics modes.

The cursor does not advance automatically. The attribute

controls the color of the foreground, background, intensity,

and blinking.

The special codes backspace, carriage return, linefeed

and bell are not recognized and do cause characters to be

printed.

Text Mode:

The attribute byte can be defined by setab, and both fore

and back should be specified. The values for the foreground

color may be 0-15, background 0-7, blink 0 or 1, and inten

0 or 1.

Valid attributes in Mono are:

1) Normal     7-foreground, 0-background

2) Underlined 1-foreground, 0-background

3) Reverse    0-foreground, 7-background

4) Invisible  0-foreground, 0-backgound

Blinking and Intensity work as well, and may be combined

with the above foreground/backgound combinations.

APR 1988                FAT-VIDEO 1.21  WRTAC           Page V-9

Graphics Mode:

The color attribute byte should be the foreground color

0-15. If bit 7 is set the color is xor'ed with the background,

thus assuring visibility. The attribute byte may be formed

using setab, by inputting the fore color(0-15), 0 for the back

color, 0 for inten, and a 1 for blink to xor the color, or 0

in blink not to.

Page need not be specified in graphics mode.

See also getab(), getac(), wrtacp(), wrtas(), wrtasp().

wprint(), wprnas(), setab()

Note: To set the background and border color in graphics

see setpal(),setbc().

attr = attribute byte

kar  = character ascii code

Page = legal Page for adapter in use

Int 10h, fx-09h

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  WRTACP          Page V-10

WRTACP(ATTR,KAR,ROW,COL,PAGE) - Write Attribute and Character at

the position given.

Usage:

call wrtacp(attr,kar,row,col,page)

integer*2 attr,kar,row,col,page

inputs:   attr,kar,row,col,page

returns:  none

MS-Fortran 4.0 Large Model

Description:

This subroutine writes the attribute and character at the

defined cursor position. The attribute is the output of getac,

or of setab. This function works in text and graphics modes.

The cursor does not advance automatically. The attribute

controls the color of the foreground, background, intensity,

and blinking.

The special codes--backspace, carriage return, linefeed

and bell--are not recognized and do cause characters to be

printed.

Text Mode:

The attribute byte can be defined by setab, and both fore

and back should be specified. The values for the foreground

color may be 0-15, background 0-7, blink 0 or 1, and inten

0 or 1.

Note: To set the border color in text see setpal().

Graphics Mode:

The color attribute byte should be the foreground color

0-15. If bit 7 is set, the color is xor'ed with the

background thus assuring visibility. The attribute byte

may be formed using setab by inputting the fore color (0-15),

0 for the back color, 0 for inten, and 1 for blink to xor the

color, or 0 in blink not to.

Page need not be specified in graphics mode.

See also getab(), wrtac(), getac(), wrtas(), wrtasp().

wprint(), wprnas(), setab()

APR 1988                FAT-VIDEO 1.21  WRTACP          Page V-11

Note: To set the background and border color in graphics

see setpal().

attr = attribute byte

kar  = character ascii code

row  = row(0-24)

col  = col(0-79)

Page = legal page for the adapter in use.

Note: This function is a composite of setcp(), and wrtac().

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  WRTCH           Page V-12

WRTCH(KAR,COLOR,PAGE) - Write a character at the curren cursor

position.

Usage:

call wrtch(kar,color,page)

integer*2 kar,color,page

inputs:   kar,color,page

returns:  none

MS-Fortran 4.0 Large Model

Description:

This subroutine writes the character at the current

cursor position. In text mode color is not used.

In graphics mode the color of the foreground does need

to be specified.

The special codes backspace, carriage return, linefeed

and bell are not recognized and do cause characters

to be printed.

kar   = character ascii code

color = foreground color in graphics

Page = legal page for adapter in use

Int 10h, fx-0Ah

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21  SETPAL          Page V-13

SETPAL(ID,COLOR) - Set Palette, Border, Background

Usage:

call setpal(id,color)

integer*2 id,color

inputs:   id,color

returns: none

MS-Fortran 4.0 Large Model

Description:

This subroutine sets the border color if in text mode, or the

border and background color if in graphics mode. In either mode,

it may also be used to select the active color palette for the

4-color graphics mode.

This function only works in the medium resolution graphics

display (mode 4), on the CGA adapter. The Pcjr or a EGA adapter

is required in text modes. To set the CGA border color in text

modes use setbc().

Text Modes:

ID    = 0  -> sets border color - EGA only

1  -> sets palette to use

COLOR = 0-15 for ID = 0  set border color

0-1  for ID = 1  set palette

Graphics Modes:

ID    = 0 -> sets color of background and border CGA,EGA

1 -> sets palette to use

COLOR = 0-3 for ID = 0, in mode4

0-1 for ID = 1

palette 0:

color 0 = current background

1 = green

2 = red

3 = brown

palette 1:

color 0 = current background

1 = cyan

2 = magenta

3 = white

Int 10h, fx-0Bh

CGA

APR 1988                FAT-VIDEO 1.21  WRTGP           Page V-14

WRTGP(COLOR,ROW,COL) - Write Graphics Pixel.

Usage:

call wrtgp(color,row,col)

integer*2 color,row,col

inputs:  color,row,col

returns: none

MS-Fortran 4.0 Large Model

Description:

This subroutine writes the graphic pixel at the specified row

and column with the given color.

color = legal pixel values: in mode 4,5 med res(0-3)

in mode 6 high res(0-1)

other modes, see your EGA manual.

row = row(y) coord

modes: 4,6,0Dh,0Eh(0-199)

modes: 0Fh-10h(0-349)

col   = column(x) coord:

modes: 4,5,0Dh (0-319)

modes: 6,0Eh,10h(0-639)

See also getgp().

Int 10h, fx-0Ch

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   GETGP           Page V-15

GETGP(COLOR,ROW,COL) - Get Graphics Pixel Color.

Usage:

call getgp(color,row,col)

integer*2 color,row,col

inputs:   row,col

returns:  color

MS-Fortran 4.0 Large Model

Description:

This subroutine reads the graphic pixel at the specified row

and column with the given color.

color = legal pixel values: in mode 4,5 med res(0-3)

in mode 6 high res(0-1)

EGA - see your manual.

row = row(y) coord

modes: 4,6,0Dh,0Eh(0-199)

modes: 0Fh-10h(0-349)

col   = column(x) coord:

modes: 4,5,0Dh (0-319)

modes: 6,0Eh,10h(0-639)

Int 10h, fx-0Dh

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   CHTEL           Page V-16

CHTEL(CHAR,PAGE,COLOR) - Write a Character in Teletype mode.

Usage:

call chtel(char,page,color)

integer*2 char,page,color

inputs:  char,page,color

returns: none

MS-Fortran 4.0 Large Model

Description:

This subroutine writes a character to the specified

page, in the specified foreground color, in graphics mode.

This function advances the cursor after each write.

Automatic line wrapping and scrolling are also provided.

The special codes--backspace, carriage return, linefeed

and bell--are recognized and do not cause characters

to be printed.

CHAR  = character to write.

PAGE  = Page to write to, not needed in graphics modes.

COLOR = color of foreground of character, in graphics only.

See also wrtch().

Int 10h, fx-0Eh

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   GETMOD          Page V-17

GETMOD(MODE,NCOL,PAGE) - Get Mode, Number of Columns, and active page.

Usage:

call getmod(mode,ncol,page)

integer*2 mode,ncol,page

inputs:  none

returns: mode,ncol,page

MS-Fortran 4.0 Large Model

Description:

This subroutine gets the current screen mode, number of columns

on screen, and display page.

See also setvm().

mode  Display

----   --------

00   = 40*25 text B&W,cga

01   = 40*25 color text,cga

02   = 80*25 B&W text

03   = 80*25 color text

04   = 320*200 4-color,cga

05   = "   "    "         ,color burst off

06   = 640*200 2-color,cga

07   = Mono & EGA display

08   = 160*200 pc jr

09   = 320*200 pc jr

0A   = 640*200 pc jr

0D   = 320*200 EGA

0E   = 640*200 16-color EGA

0F   = 640*350 mono graphics,EGA

10   = 640*350 4/16 color graphics,EGA

(4/16-depends on ram)

EGA,VGA: See your manual.

mode = active mode

ncol = 40 or 80 columns

Page = active page

Int 10h, fx-0Fh

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   SETAB           Page V-18

SETAB(ATTR,FOR,BACK,INT,BLINK) - Set the Attribute Byte.

Usage:

call setab(attr,fore,back,int,blink)

integer*2 attr,fore,back,int,blink

inputs:   fore,back,int,blink

returns:  attr

MS-Fortran 4.0 Large Model

Description:

This subroutine forms an attribute byte from the input

foreground, background, intensity, and blink bytes.

See also getab().

ATTR  = attribute byte

FORE  = foreground color: (0-15), B&W: (0,1,7)

BACK  = background color: (0-7),  B&W: (0,7)

INT   = intensity  0-normal, 1-high

BLINK = blinking   0-off, 1-on

Colors

------

0 Black         8  Gray

1 Blue          9  Light Blue

2 Green         10 Light Green

3 Cyan          11 Light Cyan

4 Red           12 Pink

5 Magenta       13 Light Magenta

6 Brown         14 Yellow

7 White         15 Bright White

Int none

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   GETAB           Page V-19

GETAB(ATTR,FORE,BACK,INT,BLINK) - Get the Attribute Byte Values.

Usage:

call getab(attr,fore,back,int,blink)

integer*2 attr,fore,back,int,blink

inputs:  attr

returns: fore,back,int,blink

MS-Fortran 4.0 Large Model

Description:

This subroutine determines from the attribute byte the

input foreground, background, intensity, and blink bytes.

See also setab().

ATTR  = attribute byte

FORE  = foreground color: (0-15), B&W: (0,1,7)

BACK  = background color: (0-7),  B&W: (0,7)

INT   = intensity  0-normal, 1-high

BLINK = blinking   0-off, 1-on

Colors

------

0 Black         8  Gray

1 Blue          9  Light Blue

2 Green         10 Light Green

3 Cyan          11 Light Cyan

4 Red           12 Pink

5 Magenta       13 Light Magenta

6 Brown         14 Yellow

7 White         15 Bright White

Int none

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   WRTAS           Page V-20

WRTAS(ATTR,STRING) - Write Attribute and String at the cursor.

Usage:

call wrtas(attr,string)

integer*2    attr

character*80 string

inputs:  attr,string

returns: none

MS-Fortran 4.0 Large Model

Description:

This routine writes a string to the screen using the

attribute byte given.  Special characters are not recognized,

and are all printed. No carriage return, linefeeds or

backspace functions work with this function. The string is

written at the current cursor position.

In graphics mode, if bit 7 is set in the attribute byte,

that is if blink is set in a call to setab() and you are in

graphics mode, then the foreground color is xor'ed with the

background screen to ensure the character is visible. In text

mode, the blink byte simply causes the character to blink. Any

input string must end with a character(0), the null character,

as shown below.

See also wrtasp(), wrtac(), wrtacp().

character*80 string

string = 'text'//char(0)  Most compilers allow this.

'text`'          All compilers allow this.

'text'C          Microsoft only.

ATTR   = attribute byte define by setab()

STRING = N-byte string terminated with null character

Int none

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   WRTASP          Page V-21

WRTASP(ATTR,STRING,ROW,COL,PAGE) - Write the Attribute

and String at Position.

Usage:

call wrtasp(attr,string,row,col,page)

integer*2   attr,row,col,page

character*n string

inputs:  attr,string,row,col,page

returns: none

MS-Fortran 4.0 Large Model

Description:

This routine writes a string to the screen using the attribute

byte given.  Special characters are not recognized, and are all

printed. No carriage return, linefeeds or backspace functions

work with this function. The string is written at the row, column

and page position specified on input. This function is otherwise

identical to wrtas().

In graphics mode, if bit 7 is set in the attribute byte, that

is if blink is set in a call to setab() and were in graphics mode,

then the foreground color is xor'ed with the background screen to

ensure the character is visible. In text mode, the blink byte

simply causes the character to blink. Any input string must end

with a character(0), the null character, as shown below.

character*80 string

string = 'text'//char(0)        Most compilers allow this.

'text`'                All compilers allow this.

'text'C                Microsoft Only.

See also wrtas(), wrtac(), wrtacp().

ATTR   = attribute byte define by setab()

STRING = N-byte string terminated with null character

Int none

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   PEEKB           Page V-22

PEEKB(OFF,SEG,VAL) - Get BYTE at SEG:OFF, anywhere in memory.

Usage:

call peekb(off,seg,val)

integer*2    off,seg or integer*4 off,seg

integer*2    val

inputs:  off,seg

returns: val

MS-Fortran 4.0 Large Model

Description:

This routine peeks at the byte located at seg:off and places it

in the lower byte of VAL.

This function must pass an integer*2 for VAL, and an integer*2 or

integer*4 for off and seg.  If off and/or seg is passed

as an integer*4, only the lower word is used. Seg and off must be

declared as integer*4 if they exceed +32k or -32k, as this is the

largest integer*2 value Fortran can assign. That is if you

explicitly declare seg= #B800 (hexadecimal cga screen address),

then seg must be declared as integer*4 or else the compiler will

assign a negative value to seg. In general, just declare any seg

definitions as integer*4. Note:  If a video function returns a

segment address for later use it may be declared integer*2 in all

cases, as the video functions treat integer*2 values as 0 through

+64k (unsigned integers).

See also peekw(), pokeb(),pokew(), allmem(), fremem().

CGA, Mono, EGA

APR 1988                FAT-VIDEO 1.21   PEEKW           Page V-23

PEEKW(OFF,SEG,VAL) - Get WORD at SEG:OFF, anywhere in memory.

Usage:

call peekw(off,seg,val)

integer*2    off,seg or integer*4 off,seg

integer*2    val

inputs:   off,seg

returns:  val

MS-Fortran 4.0 Large Model

Description:

This routine peeks at the word located at seg:off and places it

in the variable VAL. This function must be passed an integer for

VAL, and an integer*2 or integer*4 for off and seg.  If off and/or

seg is passed as an integer*4, only the lower VAL is altered. Seg

and off must be declared as integer*4 if they exceed +32k or -32k,

as this is the largest integer*2 value Fortran can assign. That is

if you explicitly declare seg= #B800 (hexadecimal cga screen

address), then seg must be declared as integer*4, or else the

compiler will assign a negative value to seg. In general, just

declare any seg definitions as integer*4. Note:  If a video

function returns a segment address for later use, it may be

declared integer*2 in all cases as the video functions treat

integer*2 values as 0 through +64k (unsigned integers).

See also peekb(), pokeb(),pokew(),allemm(),fremem().

Off     = offset

Seg     = segment paragraph

Val     = returned

Int none

CGA, Mono, EGA