Using DecTalk with JAWS

If your synthesizer is already connected and ready to go you can

proceed to the instructions for loading the Jaws software below,

titled "Loading the Jaws DecTalk version".  If you need help

connecting the synthesizer go to one of the sections further on

down about the Dectalk, the Kurzweil Personal Reader, the PC-KPR,

the Portable DecTalk, etc.

Loading Jaws software for DecTalk

The default communication parameters for Jaws and most DecTalk's

are Com 1 and 9600 Baud, no parity, 8 data bits, 1 stop bit.  Jaws

will default to these values and set the serial port by itself,

without the "Mode" command.  Simply enter:

J_DECTLK

If you want to use COM2 enter:

J_DECTLK COM2:

It should put up the JAWS copyright screen, then say the user's

name (or the demo message) and "Jaws Ready".  If you do not get

speech, or your computer locks, check the cable and its

connection, and be sure you are entering the proper parameters.

For more details on troubleshooting refer to the section titled

"Serial synthesizers and Jaws".

If you like to operate your DECTALK with different communication

parameters refer to the examples below to see how to run JAWS from

the DOS prompt and change the

parameters.  The syntax is similar to the "mode" command, but no

spaces are allowed following the ":".

J_DECTLK COM2:1200,O,7,1

J_DECTLK COM1:2400,N,8,1

If you have problems with the installation, if Jaws is not speaking

or acting strange, refer to the section titled "Serial synthesizers

and Jaws".

DecTalk DTC01 from Digital Equipment

This is the "original" Dectalk, the large box with built in speaker

and two serial ports.  Connect the serial cable to the "COM" port

on the back of the Dectalk. This is the outside port, the one

nearest the corner.

A null-modem cable is required, with female connectors at each end.

The standard DecTalk cable from DEC has the model #BC22D-10, also

has #47746 printed on the label.  This cable works fine if you

still have it around.

Many different configurations of Null modem cables work, if you

have a "null modem" try it.  If not you can buy one from us or use

the following cable specs:

#1: 2-3 crossed, 6-20 crossed, 7 straight thru, 4&5 crossed to 8.

These cables are symmetrical, it does not matter which end goes

where.

#2: 2-3, 3-2, 4&5 jumpered, 7-7, 6&8 jumpered.

#3: 2-3, 3-2, 4-5, 5-4, 6&8-20, 7-7, 20-6&8;

You could also use a straight thru modem cable with a null modem

adaptor available from most computer stores, or from the Braille 'n

Speak interface kit.

Most DecTalks that we encounter are set for 9600 BAUD, No parity,

8 data bits, 1 stop bit.  If you know that yours is different than

you can adjust Jaws to suit the DecTalk, see "Loading Jaws software

for DecTalk" for details.  If you want to change your DecTalk refer

to your owners manual.  If you are not sure what yours is set to

and you do not want to change it just give it a try as is.  For

more help you can refer to "Serial synthesizers and Jaws" for some

trial-and-error ideas.

Once you have it all cabled up and turned on you can proceed to

"Loading Jaws software for DecTalk".

The Portable DecTalk from Children's Hospital

This is a newer DecTalk, smaller, lighter, and faster.  It requires

an external speaker or headphones to be plugged in.  It uses the

same cable as the KPR: a straight-through modem cable, no wires

crossed, male on one end and female on the other.

It will announce a message when it is turned on, if you do not get

this check for obvious reasons and/or refer to "Serial synthesizers

and Jaws" for tips.

If it is connected and ready to go refer to "Loading Jaws software

for DecTalk" to run Jaws.

The Kurzweil Personal Reader

Setting up the KPR communications parameters

1.  Change key pad to the Communications Setup key pad:  On the KPR

key pad, press the key above the nominator three times (forth row

from top far left key).  It should say "communications set up key

pad".

2.  Press the key on the third row far left eight times, it should

say "restore factory communications settings".

3.  Go back to the key pad key (forth row far left) and press that

three times, it should say "reading key pad".

4.  Press the key to the right of that (row four, second key from

the left) seven times, or until it says "DecTalk Input".

At this point the communications parameters are: 9600 baud, no

parity, eight data bits, one stop bit, Xon/Xoff hand shake and

block transfers.

Using the Proper Cable:

You need a RS-232 cable with a female connector at one end and a

male connector at the other end.  This should be a "modem" cable

preferably with at least eight wires or pins inside.  The wires or

pins should not be crossed or jumpered, they should be

straight through.  Do not use a "null modem" cable.

1.  Connect the male end to the back of the KPR.  The male

connectors have a rough surface, the female connectors have a

smooth surface.  There is a female connector on the back of the

KPR.

2.  Connect the female end of the cable to a com port or serial

port on your PC.  The PC's com ports are usually male connectors,

therefore have a rough surface.  The PC may have a twenty-five pin

connector, like the KPR or it could have a nine pin

connector.  Naturally if the PC has a nine pin port you must have

a nine pin female connector on your cable.  Note whether you are

connected to COM1 or COM2.  If you are not sure refer to the

section titled "Serial Synthesizers and Jaws".

Now that you are all cabled up and configured for the correct

communication parameters refer to the section "Loading Jaws

software for Dectlk" to load the Jaws software.

PC KPR internal DecTalk

This DecTalk is an internal card, a circuit board that is to be

installed inside your PC.  It is to be connected by a short serial

cable to one of the serial ports.  The default communications

parameters are: 9600 baud, no parity, eight data bits, one stop

bit, Xon/Xoff hand shake.  You should know which COM port it is

connected to, for more information refer to "Serial synthesizers

and Jaws".  If the synthesizer is installed refer to "Loading Jaws

software for Dectalk" to run Jaws.