==========================================================================

README file for EPSON Stylus(R) High Quality ESC/P 2 printer driver

for Microsoft (R) Windows NT (R) Version 4.0

==========================================================================

Thank you for purchasing an EPSON printer.  This document contains notes regarding your

new printer.

This README file has the following sections:

[Contents]

1. Introduction

2. Important legal information

3. Driver Installation Notes

4. Using EPSON Printer Port

5. Using EPSON Status Monitor 3

6. Q & A

7. Application Notes

8. Known printer driver problems

--------------------------------------------------------------------------------------------------------------------------

1. Introduction

==========================================================================

Before calling technical support, be sure to read this README file, which includes

solutions to printer driver and application-specific problems.

2. Important legal information

==========================================================================

Software called the EPSON Stylus High Quality ESC/P 2 driver for Windows NT 4.0

(the "Software") is provided "AS IS."  NO IMPLIED WARRANTIES OF

MERCHANTABILITY AND/OR FITNESS FOR A PARTICULAR PURPOSE ARE

GIVEN.

The Software may be copied and distributed without charge only to users of EPSON

brand printers. All files on the disk must be copied without modification, including all

proprietary marks and notices. All components of the Software must be distributed

together. The Software may not be distributed for profit.

Copyright (c) 1999 SEIKO EPSON Corporation

3. Driver Installation Notes

==========================================================================

First time installation:

Insert the EPSON printer software CD in your CD-ROM drive. Click the Start button

and select RUN. Type in the appropriate driver letter that identifies your CD ROM drive.

Click Browse and select the SETUP.EXE file and press Enter.  Select the EPSON printer

driver then click OK.

Removing driver:

When you're reinstalling the printer driver or installing a printer driver for a second

port, make sure to do the following:

* Check that all print jobs have finished printing before you begin installation.

* Close all applications

* Run the "EPSON Printer Utility Uninstall" program by the following steps:

- Click on "Start"

- Click on "Settings"

- Select "Control Panel"

- Double click the "Add/Remove Programs" icon

- Select the "EPSON Printer Software" from the list and click

"Add/Remove..."

4. Using EPSON Printer Port

==========================================================================

* Using EPSON Printer Port for increased speed

To use EPSON Printer Port for increased speed, change the parallel port setting to ECP

using your computer's BIOS Setup program.

To learn how to change the BIOS settings, refer to your PC manual.

* Viewing the EPSON Printer Port setting dialog

To view the EPSON Printer Port setting dialog box, double-click [My Computer],

double-click [Printer], select your printer icon, and select [File]-[Properties].

In the printer driver window, click the [Port] tab, click the appropriate port tab ([LPTx]),

then click the [Port Configuration] button.

* Uninstalling EPSON Printer Port

To uninstall EPSON Printer Port, double-click "EPSON Printer Port Uninstall" in the

"EPSON Printers" folder. To make the changes take effect, restart Windows.

Note: Uninstalling the printer driver will not uninstall EPSON Printer Port.

* Using EPSON Printer Port with scanner

If your printer is connected to your computer through your scanner's parallel port, do

not attempt to import a scanned image while printing, as the printer may not complete

printing successfully.

* For DELL Xpi series PC users

If you are using a DELL Xpi series PC, you may have problems printing data or using

the EPSON Status Monitor 3 utilities. In this case, change the ECP setting to OFF using

your computer's BIOS settings, or uninstall EPSON Printer Port.

* COMPAQ  LTE series PC

If you are using a COMPAQ  LTE series PC and have set the DMA Transfer setting to

ON in the EPSON Printer Port settings, you may have difficulties to print any data or

use the EPSON Status Monitor 3 utilities. If you do, set the DMA Transfer setting to

OFF. To change the DMA Transfer setting, open the EPSON Printer Port setting dialog

box as described in Viewing the EPSON Printer Port setting dialog, above.

* DELL Dimension XPS series PC

If you are using a DELL Dimension XPS series PC, you may have problems printing

data or using the EPSON Status Monitor 3 utilities. In this case, change the Parallel

Port setting to [Enabled] using your computer's BIOS Advanced - Peripheral

Configuration settings.

5. Using EPSON Status Monitor 3

==========================================================================

5-1. General troubleshooting solutions

5-1-1. Notes for using EPSON Status Monitor 2 and EPSON Status Monitor 3

together

EPSON Status Monitor3 turns on monitoring of the following types of

connections, even if these settings are disabled by EPSON Status

Monitor 2.

* local printers

* LPR printers

* NetWare printers

* Windows shared printers

These settings should work most in cases.

If you install EPSON Status Monitor 2 and change the monitoring settings,

it will also change EPSON Status Monitor 3's settings. This

can cause problems printing and monitoring.

(Note)   *If a particular communication protocol is not installed, the

monitor settings will not be enabled.

*To setup EPSON Status Monitor 3, please re-install your printer

driver.

5-1-2. If a Windows shared printer cannot be monitored, check that the following

settings have been made:

* Make sure that the printer driver is installed on the computer that is

hosting the Windows shared printer, that it is set to be shared, and

"Allow monitoring of shared printers" is selected in the

"Monitoring Preferences" of the printer driver.

* (Windows98/95 only) Open the "Network" icon in the control panel on the

computer that is sharing the printer, and check that File and Printer

Sharing for Microsoft Networks component is installed.

* (Windows98/95 only) As above, check that IPX/SPX-compatible Protocol is

NOT installed on either the client or server computers.

5-2. Solutions for Windows NT environment

5-2-1. When un-installing EPSON Status Monitor 3 from a multi-user Windows NT environment,

turn off the Shortcut Icon on all clients before un-installing. You can turn off

the Shortcut Icon in Monitoring Preferences.

5-2-2. When using a Windows NT 4.0 shared printer in a non-updated Windows NT environment,

if you check "Allow monitoring of shared printer", and then start EPSON Status Monitor

3, the Application Error Notice Window appears and you cannot monitor NetWare shared

printers. To avoid this problem, make sure you have installed Windows NT 4.0 Service

Pack 2 or later.

5-2-3. When using a Windows NT 4.0 shared printer, if you check "Allow monitoring of shared printer",

and then start EPSON Status Monitor 3, you cannot monitor a NetWare shared printer . To avoid

this problem open "Services" from the control panel, stop the EPSON Printer Status Agent

service, and start EPSON Status Monitor 3. After EPSON Status Monitor 3 has started, restart

the printer status service. You must do this each time you log into Windows NT 4.0.

5-2-4. In Windows NT, when connecting a shared printer on the server, be sure to do the

following to all client PCs:

*Clear the "Allow monitoring of shared printer" checkbox in the driver's Monitoring

Preferences window.

*Stop EPSON Net WebManager.

You need Administrator's privileges to change these settings.

See the EPSON Net WebManager User's Guide for more details.

6. Q & A

==========================================================================

Q: The error message "Insufficient memory to print. Please consult the 'ReadMe' file

for more information." is displayed, printing cannot be carried out. Why?

A: There is insufficient memory for printing. To resolve this problem try the following:

* Expand the installed Memory

* Expand the size of the Virtual Memory

For details on this method please consult the Windows NT Help page

titled "To change the size of the virtual memory paging file"

Q: The error message "Insufficient disk space to print. Please consult the 'ReadMe' file

for more information." is displayed, printing cannot be carried out. Why?

A: There is insufficient free hard disk space to carry out the print job.

To resolve this problem try the following:

* Expand the amount of free space on your Hard Disk

Q: The error message "A Fatal Error has occurred. Unable to print. Please consult the

'ReadMe' file for more information." is displayed, printing cannot be carried out.

Why?

A: The printer driver is not installed correctly. As per the instructions listed in Section 3

(Driver Installation Notes) of this readme, please re-install the printer driver.

Q: Cannot print. There is no error message displayed.

A: If you  print a document with image data, like Tiff file, after specifying the resolution

of either 1440dpi or 720dpi where "Finest Detail" has been selected in the driver

settings, some Drawing software creates a large quantity of interim print data.

To print with software like this, a large quantity of both Virtual Memory and free HD

space is necessary.

Without enough Virtual Memory and free HD space, the software can't allocate the

required Memory and may possibly lose the print data during spooling and/or greatly

slow down the print job.

In such a case, to avoid the problem:

* Expand the installed Memory

* Expand the size of the Virtual Memory

For details on this method please consult the Windows NT Help page

titled "To change the size of the virtual memory paging file"

* Expand the amount of free space on your Hard Disk

Q: How can I increase the printing speed?

A: If you're printing with black ink only,  turn off MicroWeave to increase print speed.

Only select MicroWeave for presentation quality printing (MicroWeave does not

affect color print speed significantly).

Other ways to speed up your printing is by turning on High Speed mode, turning off

Halftoning,printing in black ink instead of color, or printing at a lower resolution.

Your CPU speed, amount of system memory, application type, document type, and

degree of hard disk fragmentation also significantly affect printing speed. Regularly

defragmenting your hard drive, and increasing or maximizing physical RAM will also

help speed up printing.

Verify that your system meets the requirements listed in your printer User's Guide

Introduction.

Q: I can't seem to print correctly using the "Reduce/Enlarge" or "Print Layout" options.

Why?

A: Try setting print properties from the application's Print dialog box before printing;

or setting print properties from control panel, then restart your application.

For example, to print from Microsoft Word 7.0 or Word 97 do the following:

1) From the File menu, select Print.

2) Click Properties.

3) Select the Paper menu.

4) Set the desired paper size.

5) Select the Layout menu.

6) For Reduce/Enlarge setting, select Fit to Page, then select the desired

paper size. After clicking OK, the Properties dialog box closes and the

Print dialog box displays again.

7) Make any other selections in the Print dialog box, and click OK.

Q: The output on my paper never appears to the settings I use in my application software.

A: Settings that are changed in the printer driver override the settings made in your

application software. For example, when the Print Quality is set to Fine-720 dpi with

Plain paper selected as the Media Type, your documents are printed at Fine-720 dpi

even though the resolution in your application software is set to another resolution.

Q: After I changed print settings, the print output is not what I expected. Paragraph

positioning is off, and text or graphics are missing.  Why?

A: This problem can occur when settings for Print Quality, Reduce/Enlarge, or Print

Layout have changed since the document was created.

Try adjusting margins and font sizes. When you change the printing size by the

Reduce/Enlarge printing item, sometimes the edges of your document are not printed.

Make sure to place enough spaces to the edges of your document when you use

theReduce/Enlarge printing. If your application has a print preview feature, use it

before printing.

Q: Color Gradations (Gradual shift in color) are not printed correctly. Can this be

corrected?

A: This problem occurs as result of a difference between the Application color

handling method and the Driver Automatic Color Correction method.

Try following the steps below to correct the problem.

1) From the application's File menu, select Print.

2) Click Properties.

3) Select Custom from the Main menu.

4) Click Advanced.

5) Select Photo-realistic or Vivid from  Color Adjustment - Mode selection.

7. Application notes

==========================================================================

* Lotus WordPro 96

1. Very rarely, an imported bitmap image is not printed correctly.

2. The frame line is printed incorrectly. (All or some parts of the frame line exceed the document

dimensions)

* Works 4.0

If the "Cancel" button is clicked while printing, a "memory low" message is sometimes be

displayed.

* Excel 97

Color of pattern occasionally looks different from print preview.

* WordPad

When printing a document just after system start-up the print position is sometimes incorrect

(skipped page/skipped column).  This problem has been noted only on the 1st print job.

* MS-Graph(MS-Word)

The name of a graph in a document is sometimes printing incorrectly.  The position of the graph

name is different from the screen.

* Corel Draw 6.0

Very rarely: An imported photographic image is not printed correctly.

* MS-Word 7.0

Sometimes the frame line (dot line) color is different from the screen.

* Lotus1-2-3  5.0

Sometimes the line feed position is different from the preview.

* PowerPoint 7.0

The background color of a document is not printed correctly when printing using gray.

* Microsoft Word 6.0, Microsoft Works 4.0, CorelDRAW 7.0, PowerPoint 7.0

Changing the copy count in the "copies" field on the paper dialog box does not change the

# of copies printed.

* Others applications

Very rarely: when there are multiple spool files the print driver stops printing the spool file.

When one file is finished, then screen displays a message - "deleting..."

8. Known printer driver problems

==========================================================================

There are no known bugs within the printer driver itself.

--------------------------------------------------------------------------------------------------------------------------

For the latest information regarding your EPSON printers, visit the EPSON Web

Site at http://www.epson.com. Go to the EPSON Connection page and select FAQs,

Inkjet Printers. Or you can access one of the following EPSON Electronic

Support Services:

Fax-On-Demand:

Dial (800) 442-2110 or (310) 782-4214 select option 1, then 1 again

(to order a document).

Service:

File Area: Inkjet Printers

CompuServe:

Forum: EPSON

Section: Inkjet Printers

--------------------------------------------------------------------------------------------------------------------------

Microsoft is a registered trademark and Windows is a trademark of Microsoft

Corporation in the United States and other countries.  Product and Company

names listed in this document are registered trademarks of said Company.