#E#(10U

Oak Expanded Memory Manager (OakEmm) Version 4.XX

(c) CopyRight Oak Technology Incorporation 1987, 1988, 1989, 1990

������������������������������������������������������������������

The OakEmm conforms fully to the LIM EMS 4.0,

the Major Features are the followings:

Small Frame and Large Frame EMS model.

Multiple Alternate Hardware Maps.

DMA between a selected channel and an Alternate Map.

Device = OakEmm.sys [HANDLE=hhhh] [ALTMAP=aaaa] [EXPMEM=mmmm]

[STACK=ssss] [WIN=wwww] [PAGE=pppp] [SMALL]

All parameters are optional. If a parameter is detected,

the value it specified will override the default value.

The order of presence of the parameters are not important.

Parameters should appear in the same line after the 'OakEmm.sys',

HANDLE	   Specify the number of EMM Handles to be supported.

Range:    1 to 254

Default:  65

Note:     1 handle will automatically be added as the

system handle.

ALTMAP	   Specify the number of simulated Alternate Map.

Range:    2 to 20

Default:  6

Note:     The OTI-30 implements 2 Hardware Maps.

Emm performs map contexts swapping to

support more alternate maps.

EXPMEM	   Specify the amount of Expanded Memory in Kbytes.

Range:    16 to 8192

Default:  Automatic Searching

Note:     Bad Emm Pages are mapped out automatically.

EXPMEM	   Specifies the amount of Expanded Memory in 512K blocks

Range:    0 to 14

Default:  Occupy all available Extended Memory automatically.

Note:     A) 384K is automatically added to the Expmem paramenter,

so, Expmem=0 will get 384K

Expmem=1 will get 512K+384K

B) If Expmem > Available, will get all available Extended Memory.

C) Int 15h is trapped to report the adjust Extended memory size.

STACK	   Specify the number of register stacks.

Range:    20 to 256

Default:  20

Note:     The register stack saves the 4 page

registers located in the Window Frame.

WIN	   Specify the Start Segment of the 64K Window Frame.

Range:    C000, C400 to E000 at 400 interval

Default:  Automatic Searching starting at Segment C000.

Note:     Emm will not load if fail to find a 64K

contiguous free space.

PAGE	   Specify the number of Mappable EMS Page.

Range:    4 to 12

Default:  Automatic Searching for all contiguous

mappable EMS page.

Note:     This parameter can be used to control

the Ems mappable region to avoid conflicts

with other devices.

SMALL	   Set the EMM to install in Small model.

Default:  Large

Note:     If small is specified, memory region

between 0K to 640K will not be mappable.

Example:

In the config.sys file, add one of the following statement:

Device = Oakemm.Sys

Device = Oakemm.Sys Small

Device = Oakemm.Sys Win=C800