|                                                                  Version 2

|          UPDATE TO IBM 16-bit AT FAST SCSI ADAPTER DRIVER

|  Although IBM has never announced support for IWPM/2 scanning in other than

|  a PS/2 MicroChannel environment, some customers may wish to use ISA-bus

|  (AT-bus) machines such as the ValuePoint. Each such configuration should be

|  tested before bidding to customers, as they have not all been tested by

|  ImagePlus development and cannot be remedied if they fail.

This directory contains an update to the Future Domain device driver for

the IBM 16-bit AT Fast SCSI Adapter.  This driver was shipped with OS/2

2.1 in April 1993, but was subsequently modified to function with Scanners

|  (OS/2 APAR PJ09293).  This APAR fix is included in OS/2 2.1 CSD XR06200.

|

|  An additional change is required to the OS/2 CSD XR06200 to operate with

|  the Kodak ImageLink 900 (see below).

For installation, select the "IBM 16-bit AT Fast SCSI adapter" from the

OS/2 INSTALL program. It will install the FD16-700.ADD and OS2SCSI.DMD

files in the \OS2 directory on the boot drive and add a BASEDEV= statement

to CONFIG.SYS. Then replace the FD16-700.ADD file ONLY IF IT IS OLDER than

|  the one provided in this directory, and only for the Kodak ImageLink 900.

|  Ensure that the following statements appear in your CONFIG.SYS in order:

|

|        BASEDEV=FD16-700.ADD

|        BASEDEV=OS2SCSI.DMD

Then re-boot.

Notes:

------

1) Future Domain and other manufacturers support several other ISA SCSI

adapters under OS/2;  the IBM 16-bit AT Fast SCSI (built by Future

Domain) is marketed by IBM. It has been successfully installed and

operated with ImagePlus Workstation Program/2.

|     Note that some versions of this adapter exhibit some system boot

|     difficulties.  For example, with the 2456 scanner the system may not

|     boot while the 2456 scanner is powered on.  The 2456 power has to be

|     shut off until the OS/2 Boot Manager menu appears, then powered on.

2) The .ADD code is provided "as is".  ImagePlus cannot provide support

for it since it belongs to OS/2.  It was obtained from Future Domain.

|     From time to time, Future Domain updates its drivers on its electronic

|     Bulletin Board system; the access number (USA) is 714 253 0432.

3) The FDSDA code is provided for convenience. If you have the support

diskette which was provided with your SCSI adapter, use FDSDA from that.

|  4) The Kodak ImageLink 900 series scanners have a very long Diagnostic

|     sequence after a SCSI bus Reset.  The 16-bit AT Fast SCSI adapter

|     issues a SCSI bus Reset both when the PC boots, and when the .ADD

|     driver is loaded. The .ADD Device poll must be delayed for the Kodak

|     device to be found by the .ADD driver. Specify

|

|         BASEDEV=FD16-700.ADD /RD:30

|

|     This will add 30 seconds to the system boot time.

|

|  5) Note that the TMC-850IBM adapter (also built by Future Domain and

|     marketed by IBM) exhibits problems in several scanning configurations

|     and is specifically NOT recommended.

File          Description

------------------------------------------------------------------------------

FD16-700.ADD  Support for "IBM 16-bit AT Fast SCSI adapter" manufactured

by Future Domain for IBM (IBM P/N 32G4088).  This requires

cable 32G4089.

|                Install this driver ONLY for the Kodak ImageLink 900.

FDSDA.EXE     Test\setup program to verify adapter and device installation.

Runs under DOS or in an OS/2 DOS session.

|

|