Pseudo ECA   2647 505, and 2662 505 (Diablo/Cascade)

Abstract:  ECA 2647-505, 2662-505 ThinkPad HDD failure

PURPOSE:

The purpose of this ECA is to replace the failed HDD due to Dragon chip problem in HDD.

This ECA is to be implemented in EMEA only.

The Repair Policy for this ECA is "Fix on Fail".

Fix on Fail means:  Repair only if the system fails meets the required prerequisites in

the "Physical Check" section of this document.

The implementation date for this ECA is　Oct22, 2004.

The expiration date for this ECA is Mar. 31, 2005.

FEATURES:

Type,                   Machines Affected

Model,   With          and/or Feature/Device     		B/M to be   Service  System

Stage    B/M           Description                 	 Installed    Hours   Hours

2366     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2628     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2629     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2647     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2648     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2652     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2653     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2655     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2656     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2658     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2660     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2662     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2675     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

2676     B/M9999999    SEE PHYSICAL SECTION       B/M9999999   0.5     0.5

PHYSICAL CHECK:

In order for a system to be repaired under this ECA, it must meet ALL of

the following criteria:

1) Must be a ThinkPad machine type 2647 or 2662

2) The machine must have one of the following HDD.

05K9286	07N7639	08K9508	08K9509	08K9585	08K9590	08K9599	08K9653	08K9654	08K9655	08K9656	08K9661	08K9663	08K9665	08K9683	08K9700	08K9721	08K9723	08K9729	08K9733	08K9742	08K9761	08K9796	22L0088	22L0089	27L4037	27L4075	27L4123	27L4286	27L4291	27L4305	27L4384

3) Must receive a request for service from Chevron Texaco US.  The system may show one of the following symptoms:

- Unusual or loud clicking and scraping noises coming from the hard disk drive.

- Slow or unresponsive performance.

- Blue Screen (BSOD) on boot or during operation.

- "No Hard Disk Drive Found" on boot up.

- "No OS Found" on boot up.

This World Wide Tip below explains special problem determination designed to detect these failing

Hard Drives.

http://w3.pc.ibm.com/helpcenter/infotips/techinfo/MIGR-53254.html

PREREQUISITES: NONE

COMPANION: NONE

CONCURRENT: NONE

DETAIL:

Replace old FRU with new one.

FRU No.

Capacity	 OLD		NEW

4200 10.0G	05K9286	92P6317

4200　7.5G	07N7639	08K1093

4200 10.0G	08K9508	92P6320

5400 32.0G	08K9509	92P6335

4200 15.0G	08K9585	92P6323

4200 30.0G	08K9590	92P6330

5400 48.0G	08K9599	92P6338

4200 10GB	08K9653	92P6356

4200 15GB	08K9654	92P6360

4200 20GB	08K9655	92P6364

4200 30GB	08K9656	92P6368

4200 10GB	08K9661	92P6384

4200 15GB	08K9663	92P6392

4200 30GB	08K9665	92P6404

4200 40.0G	08K9683	92P6332

5400 60.0G	08K9700	92P6340

4200 30GB	08K9721	92P6352

4200 20GB	08K9723	92P6348

4200 20GB	08K9729	92P6396

4200 30GB	08K9733	92P6408

4200 40GB	08K9742	92P6412

4200 20GB	08K9761	92P6400

4200 10GB	08K9796	92P6388

4200 10.0G	22L0088	08K1094

4200 20.0G	22L0089	08K1095

4200 8.1G	27L4037	92P6310

4200 20.0G	27L4075	92P6327

4200 20.0G	27L4123	92P6314

4200 20.0G	27L4286	92P6327

4200 30.0G	27L4291	92P6330

4200 7.5G	27L4305	92P6303

5400 40.0G	27L4384	92P6342

NOTES:

This ECA is non-billable as long as it is active.

The replaced HDD will be sent to Teleplan for the repair.