The Star Commander, Version 0.80

written by Joe Forster/STA

This is still not the final  release  of  The Star Commander.  Look  out  for

further releases. Please, report bugs and ideas to me because  I  want  Version

1.0 to be the final release. Since I finish my university studies  in  mid-1998

and will probably lose my Internet access, I'm  going  to  release  Version 1.0

then but after that I'm not sure if I can still be online. Sorry for all  kinds

of bugs in the Commander but I'm in a hurry, as always.

INTRODUCTION:

This program is designed to handle the disk, tape  and  file  images  of  the

C64 Software Emulator (C64S, (C) by Miha Peternel and  Seattle Lab, 1994-1997),

CCS64 emulator ((C) by Per H�kan Sundell, 1996-1997)  and  Personal C64  (PC64,

(C) by Wolfgang Lorenz, 1994-1997),  to  copy  binary  files  and  whole  disks

between the PC and a Commodore drive and to convert several  Commodore  archive

formats. It is very similar to The Norton Commander so it will surely  be  easy

to use but before you start using it, please, read this documentation  and  the

online help carefully for features, differences from the other Commanders,  the

description of the X1541 interfaces and other details.

COPYRIGHT AND LEGAL ISSUES:

The Commander is giftware. You may use the unregistered version  as  long  as

you wish and you may spread it as you like provided that it is in the  original

unmodified archive. Don't distribute single files only the  whole  package  and

don't ask for money above the normal fee of the distribution media itself.  The

Commander may not be included in any compilation or  sold  on  disk  or  CD-ROM

without the prior permission of the author.

The unregistered version is in no way crippled, there are no nag  screens  or

delays in it. However, if you are frequently using the Commander  and  you  are

satisfied with it then you are encouraged to register. Please,  read  the  file

REGISTER.TXT for more details. You must not distribute the personal keyfile you

receive when you register.

The Commander is copyrighted software.  You  must  not  disassemble,  reverse

engineer, hack the program and the related files or modify them in any way. The

Commander is provided "as is", without a warranty of any kind. You are using it

at your own risk, the author is not liable for any damage or data  loss  caused

by the program (especially now, that the Commander is still under development).

SYSTEM REQUIREMENTS:

You must have an IBM or compatible PC with a 80286 processor or  higher  with

MS-DOS 3.20 or above installed on it and about 500 KBytes of free  conventional

memory. You may try running the Commander under the DOS emulator of OS/2  after

enabling the HW_TIMER option in the DOS settings - it has been reported to work

although I could not make it access an external Commodore drive.  I  have  also

tried to run the Commander under the DOS  emulator  of  Linux  but  the  kernel

steals too much time making the data transfer from and to the  Commodore  drive

impossible. Remember that, although the Commander may run  under  multi-tasking

systems, it does not officially support them. However, if you make use  of  the

parallel cable then you have a much higher chance under  multi-tasking  systems

since the parallel data transfer routines are asynchronous.

INSTALLATION:

You can install the  Commander  simply   by  decompressing  the  distribution

package using PKZIP 2.xx. You don't need all the files to  run  the  Commander,

most of them are only for your comfort:

- for an absolute minimum installation you need SCMAIN.EXE (required)

- if you want to have much more memory in the DOS shell, keep SC.EXE and launch

that instead of the main executable (highly recommended)

- you can enable the online help by extracting SC.HLP (recommended)

- with SCSETUP.EXE you can change all settings of the Commander (recommended)

- SCVIEW.EXE displays and SCEDIT.EXE edits DOS files and Commodore files inside

image and uncompressed archive files (recommended)

- if you want  to  use  the  Commander  under  OS/2,  Windows,  Windows '95  or

Windows NT then SC.ICO and SC.PIF are just for you (optional)

- there are some sample menu and extension files to help you with  handling  PC

and Commodore archives and formatting PC disks: *.EXT, *.MNU (optional)

- the archive PALETTES.ZIP contains some predefined color palettes (optional)

- the archive SC_ARC.ZIP contains  the  following  external  archive  utilities

(optional):

- Star Arkive creates  Arkive  archives  out  of  disk  images,  lists  their

contents and extracts them into disk images

- Star LHA creates Commodore LHA archives out of  disk  images,  lists  their

contents and extracts them into disk images

- Star Library creates Library archives  out  of  disk  images,  lists  their

contents and extracts them into disk images

- Star List lists the contents of image and archive files into a text file or

onto the screen

- Star Lynx creates Lynx archives out of multiple DOS files or  the  contents

of disk images, lists the contents of Lynx archives and extracts them  into

multiple DOS files or into disk images

- Star Tape collects multiple PC64 file images into tape images

- Star Zip creates diskpacked or filepacked  ZipCode  archives  out  of  disk

images, lists filepacked archives, tests diskpacked archives  and  extracts

both types into disk images

ADVANTAGES OF THE COMMANDER:

- it is comfortable to use the well known environment of The Norton  Commander,

isn't it? You don't have to press a sequence of weird key combinations only a

familiar one and you can always clearly see what is happening on the screen

- hopefully you remember Disk-Demon, the great  C64  disk  editor  (written  by

G. Brandt and A. Wellie in 1986/87) because a similar disk  editor  is  built

into the Commander so that you can change the data of disk images  and  disks

in an external Commodore drive directly

- many configuration options make the use of the Commander easy and comfortable

- the Commander can optionally display everything with the  C64  character  set

(only on EGA/VGA video cards)

- most of the image file handling routines are faster than those of  the  other

similar utilities

- the Commander has built-in read support for several Commodore archive formats

(Lynx, filepacked ZipCode, LHA, Arkive and TAR  archives)  and  the  external

utilities also help you with mass-converting them

- here is a benchmark of the Commander using an original 1541-II  drive  and  a

normal 35 track disk containing a single 210 block file  stored  below  track

#18 with the normal 10 sector interleave (transfer modes marked with (*)  are

equipped with manual retry on disk errors, (+) means optional verify):

��������������������������������������������������������������������ͻ

�   Whole disk copy   �  Read from the 1541  �   Write to the 1541   �

��������������������������������������������������������������������Ķ

�     Normal mode     �         7:50 (*)     �         9:55 (*)      �

��������������������������������������������������������������������Ķ

�     Turbo mode      �         2:25 (*)     �         2:25 (*)      �

��������������������������������������������������������������������Ķ

�      Warp mode      �         1:16 (*)     �         1:19 (*)      �

��������������������������������������������������������������������Ķ

� Parallel turbo mode �         0:58 (*)     �         0:58 (*)      �

��������������������������������������������������������������������Ķ

� Parallel warp mode  �         0:25 (*)     �         0:30 (*+)     �

��������������������������������������������������������������������͹

� 210 block file copy �  Read from the 1541  �   Write to the 1541   �

��������������������������������������������������������������������Ķ

�     Normal mode     �         2:20         �         2:25          �

��������������������������������������������������������������������Ķ

�     Turbo mode      �         1:00         �         1:01          �

��������������������������������������������������������������������Ķ

�      Warp mode      �         0:26 (*)     �         0:26 (*)      �

��������������������������������������������������������������������Ķ

� Parallel turbo mode �         0:23         �         0:23          �

��������������������������������������������������������������������Ķ

� Parallel warp mode  �         0:10 (*)     �         0:11 (*+)     �

��������������������������������������������������������������������ͼ

CONNECTING A COMMODORE DRIVE TO YOUR PC:

The serial connection is done using the well-known X1541  interface.  If  you

want to achieve a much higher transfer speed and you're willing to modify  your

1541 drive and to buy a secondary LPT port then you can make use of the  XP1541

parallel interface. You can  find  both  interface  descriptions  in  the  next

section.

Unless you're using the XP1541 parallel cable, don't try to  access  external

drives under a multi-tasking system and remove memory managers, device  drivers

and other resident programs, if possible. Don't forget to switch the  LPT  port

of the X1541 cable to SPP mode and the LPT port of the XP1541 parallel cable to

EPP or ECP mode.

The Commander has a machine independent synchronization method that uses  the

hardware system timers and therefore no automatic  calibration  is  inside  the

Commander. If your PC has a 80386 or 80486 CPU, the default delay value  of  48

will probably be fine and you will be able to  access  the  external  Commodore

drive without adjusting it. However, a 80286 CPU might be too slow and you  may

have to lower the delay value in  the  'Advanced options...'  menu.  Similarly,

users having a Pentium or above should raise it, to about 80. I admit  it  that

this is a bit annoying but don't forget that you have to go through this tiring

procedure only once with every new release, if at all.

The Commander is equipped with optional fast transfer modes: in turbo mode it

transfers data from and to the external Commodore drive about 2-3 times  faster

and in warp mode 5-6 times faster. If you also  connect  your  Commodore  drive

with the XP1541 parallel cable to the PC, using turbo mode  the  data  transfer

will be 6-10, with warp mode  10-18  times  faster.  The  Commander  has  turbo

command routines, as well, which speed up deleting files and  validating  disks

to 2-10 times the original speed (it depends on the number and  length  of  the

files on the disk) and disk format takes only about 12 seconds.

I had the opportunity to try the Commander with the 1571 drive of  a  friend.

Although I haven't changed anything in the setup of  the  Commander,  it  could

execute every 1541 function properly for the first try, even in  warp  transfer

mode. However, native 1571 mode  (double  sided  or  MFM-coded  disks)  is  not

supported. I looked into a 1581 ROM dump, and it made  clear  to  me  that  the

Commander won't work with it in turbo and warp mode, if at all since  it's  not

software compatible with the 1541 drive. I would have to get a  1581  drive  to

implement the necessary transfer routines.

THE X1541 INTERFACES:

Here is the description of the X1541 interfaces, with which you can connect a

Commodore drive to your PC to use with the Commander. All diagrams are pictured

as viewed from the solder or plug end.

The PC parallel port:

Data 7 - Data 0       Strobe

���������������������������Ŀ   �

�                           �   V

������������������������������������������������������ͻ

� 13  12  11  10   9   8   7   6   5   4   3   2   1   �

�  o   o   o   o   o   o   o   o   o   o   o   o   o   �

�ͻ                                          ���     �ͼ

�  o   o   o   o   o   o   o   o   o   o   o   o   �

� 25  24  23  22  21  20  19  18  17  16  15  14   �

��������������������������������������������������ͼ

�                           �   ^   ^       ^

�����������������������������   �   �       �

Ground        SelectIn   Init   AutoFeed

The Commodore drive serial bus port:

Reset

�

V

�������ͻ �������ͻ

�ͼ       �ͼ       �ͻ

�     5         1     �

Data ��> �     o    6    o     � <�� SrqIn

�          o          �

�     4         2     �

Clk ��> �     o    3    o     � <�� Gnd

�          o          �

�ͻ                 �ͼ

�����������������ͼ

^

�

Atn

The Commodore 1541 drive VIA#1 periphery chip:

PA0 - PA7

�������������Ŀ

�             �

� � � � � � � � � � � � � � � � � � � �

�����������������������������������������ͻ

� 1 2 3 4 5 6 7 8 9                    20 �

�                                         �

� 21                                   40 �

�����������������������������������������ͼ

� � � � � � � � � � � � � � � � � � � �

The X1541 interface connects the following pins:

CBM drive serial port   PC parallel port

2    Gnd           18-25 Ground

3    Atn            1    Strobe

4    Clk           14    AutoFeed

5    Data          17    SelectIn

6    Reset         16    Init

Although the Commander doesn't make use of it, the original specification  of

the X1541 interface requires the short connection of  pins  2  and  15  on  the

parallel port plug, since X1541 uses it for auto detection. If  you  intend  to

use other transfer programs with the interface or want to access your Commodore

drive under the emulators, you might want to do this alteration, as well.

You have to connect the X1541 interface to an SPP parallel  port.  Older  I/O

cards and parallel port cards only have this mode, with Pentium and  newer  486

motherboards you can set the mode of the integrated parallel port in  the  BIOS

setup.

The XP1541 parallel interface connects the following pins:

CBM 1541 VIA#1      PC parallel port

2    PA0            2    Data 0

...                 ...

9    PA7            9    Data 7

Please, note that the XP1541 interface is not a substitute for  the  original

X1541 interface, you have to connect both cables to the Commodore drive and the

PC at the same time to acquire the parallel transfer capabilities.

You have to connect the XP1541 interface to an  EPP  or  ECP  parallel  port,

therefore you cannot have both interfaces connected to the same port, you  need

two separate cables and a second parallel port. If you have a Pentium or  newer

486 motherboard with an integrated parallel port  then  the  best  solution  is

buying a cheap old parallel port card for the X1541 interface and switching the

integrated parallel port to EPP or ECP mode for the XP1541 interface.

In 1541 drives nothing is defined to any of  the  bits  of  Port A,  you  can

implement the XP1541 interface without any problem. In 1541C  drives  bit 0  of

Port A is used for the detection of the R/W head being at the  bump,  that  is,

over track 1. After stripping this connection off of the chip, you will have to

change the DOS ROM to that of the 1541 or the  1541-II,  otherwise  you'll  get

strange results when the drive is seeking because the DOS tries to rely on  the

detector line which doesn't exist anymore. In 1541-II drives bit 0 of Port A is

grounded, strip this connection off of the  chip.  In  1571  drives  there  are

several lines connected to the bits of Port A  therefore  you  cannot  use  the

XP1541 interface with them.

If you already have a floppy speeder like  Speed DOS  or  Jiffy DOS  in  your

1541 drive then you probably have a parallel plug at the rear of the drive.  In

this case you have many options of implementing the XP1541 interface.  You  can

create another cable to connect the drive to the PC with. You  can  also  split

the cable between the drive and the C64 into a Y-shaped cable, one end plugging

into the drive, another into the C64 and the third one into the  PC,  in  which

case remember not to plug the cable into the C64 and the PC at the  same  time.

However, your best choice is creating a small converter imitating the C64  user

port on one side and plugging into the PC parallel port on the other side.

If your 1541 drive has no parallel capabilities then you might still want  to

create a plug at its rear. This way there will be no cable always  hanging  out

of the drive and with another cable you'll be able  to  use  parallel  transfer

with the C64, too. Please, note that the parallel copy  programs  for  the  C64

require some additional connections. Read  their  cable  specifications  before

soldering so that you can connect the additional pins to the plug.

Always connect the XP1541 interface to your 1541 and your PC before switching

either of them on and switch both machines off before you pull out  the  cable,

otherwise you risk damaging the periphery chips.

TROUBLESHOOTING:

If you encounter problems in the part of the program that does not access  an

external Commodore drive then you should contact me at  once  with  a  detailed

description of the bug. However, if you can't  access  the  external  Commodore

drive properly, here are some ideas for you to try first.

First of all, bare boot your computer, disable  all  the  resident  programs,

memory managers and device drivers and exit multi-tasking systems such as OS/2,

Linux, Windows, Windows '95 or Windows NT since these circumstances may  affect

the data transfer. Boot your machine with plain DOS and  strip  everything  you

don't need off  your  CONFIG.SYS  and  AUTOEXEC.BAT.  You  may  create  a  boot

configuration menu or a bootdisk, too.

You should also switch the Commander into normal transfer mode since this  is

the most stable mode and it is not as sensible to the change of the delay value

as the turbo modes are. Furthermore, don't plug anything other than  the  X1541

interface into your PC and your Commodore drive. Connecting a printer  to  your

PC or other Commodore machines or drives to your Commodore drive will  probably

mess up the data transfer, lock up  the  connection  or  may  even  damage  the

periphery chips in your Commodore machine. This also means that C128D and  SX64

owners can't use the Commander with the built-in drive.

If the connection with the Commodore drive  locks  up  then  pull  the  X1541

interface out of the drive, wait for the "Device not present" error,  plug  the

interface back, reset your drive (e.g. with Control-Alt-Backspace) and try  the

following:

- raise or lower the delay value

- make sure that the X1541 interface is  assembled  well  (try  it  with  other

machines and/or other transfer programs), it is correctly shielded and it  is

not too long

- plug your Commodore drive to a Commodore machine to see if it  works  at  all

after all those years or borrow a drive from a friend

- if you have a Pentium or a newer 486 motherboard and only use the X1541 cable

then enter the BIOS setup and change the mode of the LPT port to SPP; in case

you also use the XP1541 parallel cable then you must set its LPT port to  EPP

or ECP

- you might be the unlucky owner of an (integrated) I/O  card  that  makes  the

data transfer via the LPT printer port impossible - try swapping it  with  an

I/O card of a friend or buy an old LPT card

- as a last resort you can try switching the turbo button of your machine on or

off or disable/enable the shadow RAM's or  the  cache  RAM's - these  changes

might slow down or speed up your machine so that data transfer  then  becomes

possible on a too slow or too fast computer

BUG REPORTING:

As I do not have the possibility to test the program on many kinds of PC's  I

ask you to contact me if you

- found a bug in the program (you will possibly find some as it is still  under

development)

- have an idea of what improvements I should do in the future

- saw a grammatical error, misspelling, wrong punctuation, wrong  justification

or something misunderstandable in the online help or the documentation

- know good FTP sites or WWW pages with C64 areas to which I should upload this

program

- are just interested in this program or you simply like it and  want  to  have

the final version as soon as possible

Please, E-Mail your detailed bug reports along with the version number of the

Commander you use, the configuration of your PC (CPU type and  speed)  and  the

configuration of the Commander when the  bug  occurred  (especially  the  delay

value and the transfer mode if you used an external Commodore  drive).  If  the

bug occurred only with a certain file you can send it to me uuencoded, as well.

BUGS FIXED SINCE VERSION 0.73:

- now the Commander does run with only 500 KBytes of free  conventional  memory

just as the documentation states (I forgot  to  change  the  check  for  free

memory at startup)

- the Commander can create backup files under Windows'95, too

- the Format menu item in the user menu for tape images now works OK

- the automatic filename conversion works correctly if you use joker characters

or change the file extension in the Copy, Rename or move and Delete  filename

input dialog boxes

- the device number of both panels is remembered correctly in the setup

- the keys PgDn, PgUp, Home and End don't  change  the  active  button  in  the

online help index of the viewer and the external setup

- the "Next" and "Previous" items in the online help  don't  skip  the  section

"Memory allocation"

- the 'Write block' item at the bottom of the disk editor menu is not hidden

- when 'Full screen' is off, the speed search dialog box is  still  sticked  to

the bottom of the panel

- if you save the setup when one of the panels is an external panel and restart

the Commander then the "Change drive" dialog box will show  you  the  correct

default drive number for that panel

- the 'Color' palette color of the numbers in  the  function  key  bar  at  the

bottom is light grey on black instead of cyan on black

- the sample Commander screens in the palette setup of the external setup  have

been changed according to the new  features  (I  should  have  done  that  in

Version 0.73)

- when viewing or editing files inside images, always the  internal  viewer  or

editor is called

- the viewer  scrolls  upwards  in  a  smarter  way  in  files  containing  Tab

characters

- the viewer displays Commodore filenames with graphical  characters  correctly

in the upper status line

- the viewer works properly if you specify the name of the file  to  be  viewed

and the active panel is an image panel

- Star LHA now omits the path stored in LHA archives and only uses the filename

itself when displaying or extracting the contents

- Star Lynx doesn't create corrupt archives out of more than some 130 files

- Star LHA now doesn't display the option 'V' for the command 'L' in its  usage

info because such an option doesn't exist at all

OTHER CHANGES SINCE VERSION 0.73:

- introducing a file editor for DOS files and Commodore  files  inside  images:

handles files up to amount of free  conventional  memory,  ability  to  mark,

copy, move and delete text and  hexadecimal  blocks,  adjustable  line  feed,

optional PETSCII and screen code display, built-in C64 character set

- introducing built-in support for Lynx, filepacked ZipCode,  LHA,  Arkive  and

TAR archives - you can enter them, copy files into and out  of  them,  rename

and delete files inside them, even File Info and the Info  panel  works  with

them (note that the Commander needs LHA 2.14 or newer in order to be able  to

extract files properly)

- introducing the XP1541  parallel  cable  that  speeds  up  transferring  data

between the PC and the external Commodore drive  even  more  (see  the  cable

specifications in 'THE X1541 INTERFACES' section above)

- introducing the warp file copy PC->CBM direction

- introducing Star Library, yet another  external  utility  to  handle  Library

archives

- the Commander, the external setup, the viewer and the editor  all  check  the

version number of the resident Commander/loader

- the description of the X1541 interfaces has been moved to the docs  from  the

online help

- you are warned if you try to access an external drive under  a  multi-tasking

system via the X1541 cable, you try to use the X1541 cable on an EPP  or  ECP

parallel port or try to use the XP1541 parallel cable on an SPP parallel port

- the delay value defaults to 80 on Pentiums and to 48 on earlier CPU's

- you can sort files in CBM panels, too

- you can press the right mouse button on the key bar  at  the  bottom  of  the

screen to access the Alt functions

- the maximum number of entries in tape images has been raised to 500,  it  was

unnecessary to decrease it to as low as 99

- the Commander and all the external programs give up their  time  slice  under

DESQview and in the DOS shells of OS/2, Linux and any  Windows  when  waiting

for input

- the Commander takes the "COUNTRY=" settings in your CONFIG.SYS into  account,

it displays numbers and displays and inputs dates and times accordingly - you

can view these settings in the menu item "Country info" of the external setup

- by checking a new option, 'Verify write', the Commander verifies data written

onto disks in the external Commodore drive during warp disk copy

- the process of copying files from an external panel in  warp  mode  has  been

sped up a bit by reading 256 blocks of data in one go  instead  of  track  by

track

- "22,READ ERROR" and "23,READ ERROR" errors are also retried as many times  as

the option "Number of retries" specifies during warp disk and file copy

- you can format the destination disks during disk copy

- when you press Enter or double-click on a DOS file, the Commander first reads

through the extension file to find a command specified for  that  extension -

this way you can override the normal action of  entering  image  and  archive

files

- it is now possible to specify filename patterns  besides  extensions  in  the

extension files

- you can specify the same commands for a sequence  of  name  patterns  in  the

extension files by listing the patterns in subsequent lines or by  separating

the patterns with semicolons

- now you can use  the  "<imagetype>:<path>\<imagename>\<filename>"  file  name

format (used in the Copy dialog box) in the View and  Edit  dialog  boxes  to

view or edit files inside images

- the PETSCII to DOS filename conversion now includes stripping off leading and

trailing underscores of the filename and the extension

- it is now possible to change the file type of files being copied by appending

",<file-type>" to the filename pattern

- now you can set the Filter to display only archive files

- the Commander doesn't read the error channel of the external drive  if  there

were no errors while reading the directory, which speeds up the process

- if you delete the image file displayed in one of the panels then  that  panel

turns back to DOS mode and no error message is written onto the screen

- the disk image cleaner is much faster than before

- the confirmation option 'Write block' has transformed into the  option  'Disk

editor' and now it controls all confirmations in the disk editor

- a new confirmation option, "Abort data transfer" tells whether the  Commander

should ask for confirmation when aborting data transfer between  the  PC  and

the external Commodore drive or not

- with two other new confirmation options you can control  the  warnings  about

possible transfer and file size problems

- another new  option, "Auto unselect files"  allows  you  to  control  whether

processed files are automatically unselected or not

- the option 'Wipe scratched files' allows you to clean up the sectors of files

being scratched in disk images

- another new option, 'Orig format pattern' controls whether empty sectors  are

filled up with zeros or the original 1541 format pattern

- you can copy frozen files out of tape images as programs and  File Attributes

can turn files in tape images into frozen files

- there is no label displayed in file images

- if you allocate or free a whole track in the BAM editor, the cursor will jump

a column to the right, helping you with allocating or freeing the whole disk

- now you can also use the clipboard of the disk editor to  copy  data  between

different disks and disk images

- if the screen height allows, the lower part of the disk editor  screen  shows

the current contents of the clipboard

- pressing Escape doesn't stop reading the directory in any panel mode

- if you press Space on the selected item of a  radio  button  group  then  the

selection and the cursor will jump onto the next item

- the Commander informs you if an invalid sector link is found  in  the  source

disk image during file copy

- you can select multiple disk images and copy them one by one onto disks in an

external drive

- the "Configuration..." menu of the external setup shows you the current  mode

of the LPT ports

- the viewer and the editor are now capable of reading and writing uncompressed

(Lynx, Arkive) archives

- since you can use the viewer and the editor with  other  programs,  as  well,

neither of them display registration info in the online help

- when in text mode, the Goto function in the viewer and the editor ask for the

number of the line you want to jump to, jumping to  an  offset  is  available

only in hexadecimal mode

- a new line feed mode in the viewer and the editor allow you to have  a  fixed

line length

- an empty line within the selected block is marked with  a  phantom  space  by

both the viewer and the editor so that you see the block has not ended yet

- the viewer now scrolls much faster when you press PgDn or PgUp

- the viewer is a bit more tolerant, it won't lock up when viewing files inside

damaged disk images

- the viewer and the editor write no messages onto the DOS screen if everything

is OK - this way you can use them under other programs without that  annoying

screen jump

- the viewer and the editor now scan the first 4 kilobytes of  the  input  file

to autodetect the line feed and whether to switch Tab translation on or off

- you can switch the display of special characters (Space, Tab  and  line  feed

marks) both in the viewer and the editor

- the viewer and the editor put a "$" sign before the hexadecimal number  input

line of the Goto and User Line Feed dialog boxes

- Star Arkive, Star LHA, Star Library, Star Lynx and Star Zip display the names

of files being skipped during extraction

- when renaming a file during  extraction,  Star Arkive,  Star LHA,  Star Lynx,

Star Library and Star Zip offer you the original filename and you can use the

cursor keys, Home and End to move around the filename, Backspace  and  Delete

to delete characters and Control+Backspace to delete the whole filename

- Star LHA lets you extract files even  if  another  one  with  the  same  name

already exists in the destination disk image

- Star LHA recognizes filenames longer than 16 characters in LHA archives

- with a new command of Star LHA you can prepend the SFX header to  an  already

existing LHA archive or strip it off of an SFX archive

- Star List is now capable of listing file images,  Lynx,  filepacked  ZipCode,

LHA, LHA SFX, Arkive and TAR archives

- Star List now displays the total number and size of files at the end  of  the

file list of the current image or archive file

- some of the archive checks were changed in Star Lynx  to  make  it  recognize

more Lynx archives

- Star Tape can handle list files created by the '!@' option of the Commander

KNOWN BUGS AND LIMITATIONS:

- you must not try to access an external Commodore drive if the X1541 interface

is not plugged into the appropriate LPT port or all  the  external  Commodore

drives are switched off

- timeouts are not handled - as all PC interrupts are disabled while  accessing

the Commodore drive, errors during data transfer might lock up your PC

- you must not try to delete any file from a write protected DOS floppy or hard

disk otherwise you will  get  very  strange  results  (possibly  an  internal

Borland Pascal bug)

- instead of the usual extract command of LHA the print command is used by  the

Commander which only works correctly in LHA 2.14 and newer

- it is possible to create such file images with the Commander that will not be

recognized by Personal C64 because of some invalid characters in the original

C64 filename - if you encounter such files, please, send some samples to me

- timezone settings are not standardized in DOS therefore  there  may  be  some

unwanted changes in the date stamps of  files  inside  TAR  archives  if  you

exchange the archives between the Commander and a real Unix system

COMING SOON:

I'm planning the following changes for the next version. Please, tell me if I

should start working on the items marked with (?), as well.  However,  unmarked

items are of higher priority than marked ones:

- bug fixes, of course...

- an external directory editor for disk images - for renaming, moving, deleting

and killing files, inserting phantom files  and  separators  etc.,  maybe,  a

built-in disk editor, too

- built-in support for diskpacked ZipCode archives (?)

- support for directory structures in LHA and TAR archives (?)

- recursive directory listing capability for Star List (?)

- a much faster validate routine for external disks that does not track through

files one by one but first reads all sector chains on the disk and only  then

starts working on the BAM (?)

There are many ideas that I will not put inside the Commander since  I  don't

think that they are closely related to a DOS shell like the Commander but to  a

multi-purpose utility instead. If you  still  insist  on  these  ideas,  I  may

implement them, but only in external programs.

THANKS TO:

I would like to thank my beta testers for their valuable help:

Bacchus/Fairlight

Clarence/Graffity

Darrin Smith

Edhellon/Resource

Gustavo Ayala

Hal�sz Csaba

J�rgen Bullinger

Lion/Chromance

Mathias Beilstein

Matthias Hartung

Michael J. Darschewski

Sorex/WOW

Stephen Lloyd

Suba P�ter

Sven Goldt

Szigetv�ri J�zsef

Tam�si Gy�rgy

Todd A. Aiken

Vic/COMA

Wojtek Wasilewski

Special thanks go to:

Bernhard Schwall              the author of Trans64

Borland International         for Turbo Pascal and Borland Pascal

Marko M�kel�                  for the original ZipCode and UnLynx for DOS

Miha Peternel                 for the C64 Software Emulator

Per H�kan Sundell             for the CCS64 emulator

Peter Norton                  for The Norton Commander

Peter Schepers                the author of 64COPY

Vsevolod V. Volkov            for The Volkov Commander

Wolfgang Lorenz               for Personal C64

Chris Link                    for many good ideas concerning the development

Frank Kontros                 for help with parallel cables and disk verify

H�rsfalvi Levente             for the idea of supporting TAR archives

WHERE TO FIND THE COMMANDER:

I always send the newest releases of the Commander to these platforms:

http://ludens.elte.hu/~sta/sc.html

http://hem.passagen.se/bacchus/tools/pc.html

ftp://frodo.hiof.no/pub/c64/utils

ftp://ftp.armory.com/pub/user/spectre/EMUL-UTIL

ftp://ftp.cs.tu-berlin.de/pub/c64/tools/transfer/pc

ftp://ftp.df.lth.se/pub/c64/PC_Tools

ftp://ftp.funet.fi/pub/cbm/c64/emulation/c64s

ftp://ftp.giga.or.at/pub/c64/transfer/1541-pc

ftp://ftp.inf.bme.hu/pub/c64/Tools/MSDOS

ftp://ftp.kiae.su/msdos/emulator/c64

ftp://ftp.seattlelab.com/UTILS

ftp://ftp.vekoll.vein.hu/pub/c64/tools

ftp://rsls6.sprachlit.uni-regensburg.de/pub/c64/transfer/sc

ftp://utopia.hacktic.nl/pub/c64/Tools/Convert

Wildfire BBS, Hungary         (36-)1-156-3770 (22:30-07:00), SysOp: Dino

(FidoNet: 2:371/41)           USR Sportster 14.4, FREQ Magic Name: SC

You can also E-Mail me at any time to ask for a uuencoded copy. Finger me  at

the address "sta@ludens.elte.hu" to find out the version number of  the  newest

release.

The first address in the above list is the  World Wide Web  homepage  of  the

Commander. There you can always download the latest public release, have a look

at the documentation and see some facts (bug  fixes,  new  features  and  other

enhancements) about the internal beta versions being developed and tested.

If you want to have the newest versions of the Commander in the  future,  you

can subscribe to a mailing list by sending an E-Mail to me. When a new  version

is out, you will get it on the release day in a uuencoded E-Mail. If  you  have

troubles with large E-Mails (about 500-600 KBytes) then, please,  indicate  the

maximum mail size your mail program can handle.

THE AUTHOR:

Please, don't hesitate but send an E-Mail to the address "sta@ludens.elte.hu"

if you have any questions, problems, ideas or wishes concerning the  Commander.

Use the address "sta@dtalk.inf.elte.hu" for and only for sending  E-Mails  that

contain large files. You can call the phone number (36-)1-285-3881  to  contact

me and you can also send snail-mails to me at this address:

Kov�cs Bal�zs

Budapest, XX.,

Orsolya utca 5. IV/12.

1204, Hungary

If you wish to send some files to me, either by E-Mail or  snail-mail,  then,

please, ask me before you do it - I don't like being flooded with large E-Mails

or lots of disks without being warned. Note that all the  Internet  accounts  I

have will probably cease to exist after mid-1998 when I  finish  my  university

studies.

26th May, 1997                                Joe Forster/STA