A SUMMARY OF THE SMALL C LANGUAGE (for Version 1.7 Oct. 1985)

Introduction

Small C is a subset of the standard C language.  It was written by Ron

Cain and originally published in Dr.Dobbs Journal of Computer Calisthenics and

Orthodontia,  No.45.   The version described has been modified to generate Z80

mnemonics instead of the original 8080 ones.  A number of bug fixes,  most  of

them  sent  in  by readers of DDJ, have also been incorporated.  Several extra

features have also been  added,  many  based  on  other  enhancements  of  the

original.  This version has been developed under CP/M 2.2.  Unlike the earlier

version  in Vol 15, no attempt has been made to keep the code 8080 compatible.

The libraries have been modified in various ways since the earlier release but

since most of them will only run in RAM, there has been no  particular  effort

to optimise their length by using shorter (but often slower) Z80 instructions.

The  compiler  inputs  a  program  written  in Small C from a file and

produces a version of the program in Z80 assembler  language  mnemonics  which

can  be  assembled and run on a Z80 system.  The original run time library has

been supplemented by some additional routines which use CP/M  I/O  facilities.

The  library  is divided into modules so that routines that are not needed for

particular applications can be omitted.

SMALL  C  SYNTAX  SUMMARY

Variable and Function names

All  variables must be declared.  If they are declared outside a function

they are global.  Variables and function names should consist of not more than

8 alpha-numeric characters.  Global variables and  function  names  appear  as

labels  in  the assembler language file produced by the compiler, so they must

satisfy any special requirements of the assembler that is to be used.

Permitted Data  types

char - 8 bit signed integer with values between -128 and +127

eg.  char cin,cout; declares two character variables called cin and cout

int -  16  bit signed  integer  with  values  between  -32768  and  +32767

eg.  int num,val;  declares two integer variables called num and val

Pointers  to  char  or  int data.

Pointers  contain  the  addresses  of  data elements.

eg.  int *size; declares a pointer to an integer

char *pch; declares a pointer to a  character

Single  dimension  arrays  of  character  or  integer  variables.

eg.  char line[80]; declares a character array of elements line[0] to

line[79].

Operators

The operators listed below can be applied to expressions rather than just

to single variables where appropriate.

1. Unary  (These operators group right to left)

-	Minus.   Forms 2's complement.

*	Pointer. Refers to data whose address is given by an expression.

&	Gives the address of the expression if any.

++   Increment the expression. ++expression will increment the expression

before using it. Expression++ will use the expression then increment it.

--   Decrement. This works like ++.

2. Binary   (These operators group left to right except for assignment)

* /  Multiply, Divide

%	Modulo. Gives remainder after division.

+ -  Add, Subtract

>> <<  Right and left shift.   eg. x>>2 will give x shifted right by 2 bits.

(**NB** These are arithmetic shifts: for right shift the highest order

bit is propagated into the vacant bit positions)

|	Bitwise logical Inclusive OR

&	Bitwise logical AND

^	Bitwise logical Exclusive OR

=	Assignment of value on the right to the left. a=b=c=5 is a legal

expression with a final value of 5.

3. Relational   (These also group left to right)

Pointers are unsigned.

Expressions are considered 'true' if their value is non-zero.

==   Equal  (Unlike the "=" above, this does not cause assignment when used)

!=   Not equal

<	Less than

>	Greater than

<=   Less than or equal

>=   Greater than or equal

The && and || logical operators of standard C are missing. The bitwise & and |

can be used with care between proper logical  expressions in brackets  because

these are always given a value of 1 or 0 depending on whether they are true or

false.

Types of Statement

N.B. All reserved words such as if,else,while,return etc.MUST be in lower case.

Expression;

if(expression) statement; Statement executed if expression is non-zero

if(expression) statement; Statement executed if expression is non-zero

else statement;	     Statement executed if expression is zero

while(expression)statement; Statement executed while expression is

non-zero.     (It is possible for the statement not to be executed at all)

do statement while(expression); Do statement until expression is false.

The test is at the end of the loop.

for(expr1;expr2;expr3)statement; expr1 initialises a variable. expr2 tests

a condition involving the variable. expr3 increments or otherwise alters

the variable. The statement is executed until expr2 becomes false.

switch(expression)

{ case value1:statement;  Different case statements are executed depending

case value2:statement; on the value of the switch expression. Most

etc.		       case statements end with a break; to avoid the

default:statement;	     other statements below.

}

break;    Control transferred from the innermost while loop

continue; Return control to the top of the while loop

return; return from function

return expression; return from function with value given by expression

; null statement

{  statement;statement;statement;..statement;}  compound statement

which may be used anywhere instead of a simple statement.

Constant values

These can be expressed as:

Decimal numbers within the permitted range

Hexadecimal numbers in upper or lower case preceded by '0x' or '0X'

One or two characters in single inverted commas.  eg.  'A' is equivalent to 65

(decimal).   Within  the  program  '\b','\t','\n','\f' and '\r' have values of

8,9,10,12 and 13 respectively.   In  the  output functions '\n'  outputs  '\r'

followed by '\n'.

A  string  in  double  inverted  commas.  eg.  "Fred Jones" this has a 'value'

consisting of a pointer to the string, terminated by a null byte, somewhere in

memory.

Pseudo-preprocessor  Statements

#define  name  string  Replace name by string throughout the program

#include filename Insert named file at  this  point  in the  program

eg.   #include CRUN2.LIB

Included files may not contain #include statements.

#asm ...  #endasm This allows assembly language to be included in a program.

Anything between #asm and #endasm is passed straight  to  the  O/P file.

Comments  are  written:  /*  Comment */

The Run Time Libraries

Small  C programs should start with a statement: "#include CRUN2.LIB".The

run time routines in CRUN2.LIB include an initial section which  will  set  up

the  stack  and  then  call  the  function  "main()" which must exist in all C

programs.  The group of routines in CRUN2.LIB are essential  in  all  Small  C

programs,  except  for  the  "getp"  and  "putp"  functions.   The other three

libraries can be #included if required.  CONIO2.LIB  contains  functions  that

will  normally  be  needed for CP/M character and string I/O from the console,

FILE2.LIB (which assumes CONIO2.LIB is also present) can be #included if files

are going to be used.  NUMIO2.LIB contains some routines in  C  for  inputting

and outputting decimal and hexadecimal numbers.

The  introduction  of the printf and scanf functions have complicated the

library situation because they use a large  number  of  the  routines  in  the

libraries described above.  If you want to use the formatted I/O you will have

to include all the libraries listed above, except NUMIO2.LIB which is embedded

in  FORMAT.LIB  anyway,  even  if you don't want to use any files.  Even quite

small programs will finish up with a length of 8k or more because of this.  It

is often possible to get along only with the simpler functions such as getdec,

putdec, gets and puts.

To  keep  the  management of file channels simple, only one input and one

output file may be open at a time.  CON: and LST: are  acceptable  O/P  names.

After  compilation  the stack pointer setting and the ORG pseudo-op can be set

to suit a particular application if required.

The  library  routines  are  quite  heavily  commented.   To speed up the

processing of #included files, it may be worth preparing uncommented versions.

If  software  for  a  stand  alone  system is being written, the run time

routines can be reduced to the group of essential routines in  CRUN2.LIB  plus

the "getp" and "putp" functions. These are supplied in CRUNLONE.LIB.

Functions in CRUN2.LIB

getp(n) will get an 8bit value from port n.

putp(ch,n) will output an 8 bit value to port n.

(The rather  more  obvious  function names "in" and "out" don't work

with Z80ASM as they clash with Z80 mnemonics.)

The  file  starts  with  default settings of: ORG 100H and a setting of SP

below the BDOS.  There are initialising routines which move any  command  line

arguments  to  a  buffer.   The rest of the file consists of essential utility

routines from the original library published by Ron Cain and  a  few  by  Mike

Bernson  for  handling  the switch/case structure.  CRUNLONE.LIB is a cut-down

version of CRUN2.LIB which omits the command line handling and  other  RAM  or

CP/M dependent features.

Functions in CONIO2.LIB

cpm(DE_value,C_value) makes a CP/M system call. Returns with value of A.

*	getchar()    Get a character from the keyboard. (Echoed to screen)

*	putchar(c)   Put a character to the VDU

*	gets(buff)   Get a null terminated string into a buffer pointed to by

buff

*	puts(buff)   Output a null terminated string pointed to by buff to VDU

\n,\b and \t give CR/LF,backspace and tab.

getkbd()   Scan keyboard. Returns 0 if nothing there (CP/M 2.2 only)

crlf()	 Output carriage return and line feed to VDU

lston()	 Turn on LST: device for CON: output

lstoff()   Turn off LST:

Functions in FILE2.LIB

*	putc(char,iochannel)  Output character to opened output file. Returns

character value or -1 if error.

*	getc(iochannel)  Get character from opened input file. Returns a value

or -1 if error.

*	fopen(filename,mode) eg.io= fopen("FRED.DAT","r"); use "r" for read

and "w" for write files.The file name is passed as a

pointer. LST: and CON: are acceptable output names.

The function returns the value of the iochannel

assigned, or zero if the open fails. Only 1 "r" and

1 "w" file may be open at a time. The channels in

this version are always 1 for "w" and 2 for "r".

*	fclose(iochannel)  Close a file. (fclose(0) causes the closing of the

only write file without a terminating ^Z.)

eof()	 	This returns a value of 0 until an attempt has been

made to read a sector beyond the end of a file. ASCII

files will normally have returned ^Z before eof()

becomes 'true'.

*	exit()	 Exit C program, close write files, and do a warm start.

Functions in NUMIO2.LIB

getdec()    Get a decimal number from the keyboard with a '?' prompt

putdec(n)   Outputs a signed decimal number to the VDU

gethex()    Gets a hex number from the keyboard with '$' prompt

putbyte(ch) Output a character as a 2 digit hex number

puthex(n)   Output an integer as a 4 digit hex number

*	atoi(ptr)   Convert an ASCII string no. pointed to by ptr to integer

hextoi(ptr) Convert ........ hex....................................

Functions in FORMAT.LIB (CRUN2.LIB,CONIO2.LIB and FILE2.LIB needed)

This library contains the NUMIO2.LIB routines plus the following:

*	 printf(format_pointer,arg1,arg2....) Formatted output. %d, %x, %c

and %s supported with optional field widths. Also \ chars

supported though they will mess up fields if included in

formatted strings. If a field is too small the stuff is

printed anyway. Variable no. of arguments. Hex numbers are

always printed as 4 digits.

*	sprintf(mem_pointer,format_pointer, args..) Like printf but it sends

output to memory.

*	fprintf(channel,format_pointer,args..) Like printf but sends O/P to

opened write file.

*	scanf(format_pointer,&arg1,&arg2 etc) Only %x and %d are supported in

free format. Returns the number of args found. Args not

found are unaltered. Intervening non-numeric or non-

hexadecimal characters are ignored. 0x & 0X ignored in %x.

ADDRESSES of arg1 etc must be passed to the function.

*	fscanf(channel,format_pointer,&arg1 etc) Like scanf but gets input

from an opened read file.

*	sscanf(pointer,format_pointer, &arg1 etc) Like scanf but gets input

from memory pointed to by first pointer.

*	fputs(pointer,channel) Sends a string to a file

sputs(source_pointer,target_pointer) Moves a NULL terminated string.

*	fgets(pointer,channel) Not very standard. Gets a string from a file

until a NULL,EOF or CR is found. If CR it reads the next char

and assumes it is a LF. Terminates input with NULL.

Returns last char. CR,EOF or NULL.

fputdec(value,channel) Sends a decimal number to a file

fputhex(value,channel) Sends a 4 digit hex number to a file

Functions in MORELIB.LIB (Z80 translation from Mike Bernson's STDLIB.ASM)

*	 toupper(ch) Convert to upper case

*	 tolower(ch) Convert to lower case

*	 isupper(ch) Check for upper case     Return 1/0 for true/false

*	 islower(ch) Check for lower case     ...................

*	 isdigit(ch) Check for numeric decimal digit ............

*	 isalpha(ch) Check for alphabetic digit .................

*	 isspace(ch) Check for ' ' or tab     ...................

*	 strlen(ptr) Return length of string pointed to by ptr

*	 strcpy(ptr1,ptr2) Copy string from ptr2 to ptr1

*	 strcat(ptr1,ptr2) Copy string ptr2 to end of ptr1 string

*	 strpos(ptr1,ptr2) Look for string ptr2 in string ptr1. Return position

AMS.LIB is an experimental library that includes the basic run-time routines

and has some simple I/O and graphics functions that run under the Amsdos

operating system. In conjunction with AMLOAD.COM they allow programs for

Amstrad CPC464 computers (and probably other CPCs) to be developed under

CP/M and then coverted from HEX files to Amsdos BIN files.

See the separate DOC file for more details.

[* indicates a function which is similar (but not always identical) to one

in 'standard' C.]

Using command line arguments

The function 'main' can start with:

main(argc,argv)

int argc,argv[]

{ etc.

argc  is  an integer equal to the number of command line arguments and

will be equal to 1 if the command line only consisted of a .COM filename, argv

is an array of pointers to the beginings of the arguments that are stored in a

buffer by the initialising process.  Small C doesn't support pointer arrays so

the pointers have to be stored in an integer array.  They should be moved to a

character pointer before use.  argv[0] is junk.  arg[1] contains a pointer  to

the first extra argument etc.

Using  the  Small C Compiler.

Use  a  normal  editor  to  write your program, .C is the usual file type.

There are two ways of running the compiler: Either (a) the compiler  can  just

be  run and a set of questions answered about options, file names etc.  or (b)

the file names can be entered on the command line with  default  options.   If

you  want  to use non-standard file name types you should use method (a) since

in method (b) the file types are assumed to be .C and .ZSM.  If you don't like

the type for the output file the letters of "ZSM" are at addresses 2CAH,  2E0H

and 2F6H.

Method  (a): It is usually a good idea to ask for your C program to be

included in the assembler file as comments.  Global  variables  will  normally

need to be defined, unless a very large program is being compiled into several

separate  assembler  files.   Choose a small number for the start of the label

numbering.  It is probably best to start by asking for  a  pause  after  error

messages.  You can turn the pause off by entering '.'  after an error message.

Any  other  character  will  cause  a  continuation of compilation to the next

error.  Once one error has been encountered the compiler often gets a bit lost

and generates further messages which will disappear when the original  problem

has  been  cleared  up.   The  program  will  abort  after  40 errors.  During

compilation the names of the functions are listed as they are compiled.   This

should  help  you to find errors.  The names of the functions are displayed by

printing the lines that the compiler thinks are function headers, if you  make

an  error  you may find that the compiler will get mixed up and start printing

other lines where it thinks a function ought to start.  Give the names of  the

output  and input files when they are asked for (OUTPUT FIRST).  Names must be

given in full and may include a drive name.  If you try to use a file of type

.C as an output file you will get a message to say that a file of type .O will

be  used.   LST:  and  CON:  are  acceptable  output  names if you just want a

compiler output listing of a correct program,  but  you  will  get  a  certain

amount  of  character doubling on stuff such as error messages that is usually

sent to both the screen and the file.  When compilation is complete  you  will

be  asked  for  another input file name which can be added to the one compiled

previously.  Usually you just enter a <RETURN> to exit from the compiler.   It

is  possible  to  enter  <RETURN> as the answer to the first four questions on

entering the compiler, in which case the program will default to including the

C program as comments, definition of globals and labels  starting  at  0,  and

pauses after errors.

Method (b):  You can give file names in the command line:

i.e.  ZSC FILE [OFILE]    (Default file types .C and .ZSM )

If  the  second  file name is omitted it will be assumed to be the same as the

first but of type .ZSM.  The first should be of type .C.  Don't enter the file

types in the command line.  There will be assumptions that C source is  to  be

included  as comments, globals are to be defined, labels start at 0 and pauses

are to occur after errors.  It is assumed that only a single source program is

to be compiled (though several can be  linked  together  with  a  sequence  of

#include statements).

If  you  are  trying to produce ROMable code you must confine yourself to

CRUNLONE.C.  In any case you will probably have to edit CRUNLONE.LIB and  some

of  the  final  assembler  language file.  You will find that global variables

have been given storage at the end of the listing.  If the program is intended

for a stand-alone application with its final code in EPROM you  will  have  to

put  in  another ORG pseudo-op before the variable addresses to locate them in

RAM.  The other LIB files, except the extra routines in MORELIB.LIB,  all  use

buffers,flags  etc  which  have to be in RAM or rely on routines that do.  For

normal CP/M programs editing shouldn't be needed.   When  the  file  has  been

compiled, assemble the output file in the usual way and run it.

Linking  Small  C  output  to  machine  code

If a function is called: eg.  jim(arg1,arg2,arg3), the compiler pushes the

values  of  arg1,arg2  and arg3 on the stack in the order that they are listed

and then generates a call to a label "jim".  If an argument is a  string  such

as "Fred Jones", the argument value will be a pointer to the string which will

be  somewhere  in memory terminated by a null byte.  Values should be returned

in HL.  8 bit values should be sign extended to 16 bits and the  simplest  way

to  do  this  is  to  put  the value into A and jump to a routine in CRUN2.LIB

called "ccsxt" which will do this for you.  The stack must be restored to  its

previous state before returning.  The last argument will still be in HL when a

function  is  entered, so if a function has only one argument there is no need

to get it off the stack.There are examples  of  this  in  the  library  files.

Machine  code  functions  can be put between #asm and #endasm directives among

the Small C functions or they can be #included  from  a  separate  file  (also

between #asm and #endasm).  The functions printf,sprintf,ssprintf,scanf,fscanf

and  fscanf  are recognised by name by the compiler and have an extra argument

added at the end of the others which gives the  number  of  arguments  in  the

function call.  This allows variable numbers of arguments to be used for these

functions.   Other  functions with these names or which start with the special

names should be avoided.

Some other details of the compiler

The  original Small C program published by Ron Cain was itself written in

C.  The version described here was  originally  typed  into  an  Onyx  in  the

Electrical  Engineering Department at Brighton Polytechnic and compiled on the

Onyx's C compiler to produce  a  Small  C  compiler  running  under  UNIX.   A

slightly  modified  version of the Small C source was then fed into the Onyx's

Small C compiler to produce a Z80 assembler file  which  was  assembled.   The

final  Z80/8080  code  was then moved to a microcomputer and hence to a floppy

disc.  This version has been further modified under CP/M.  As far as is  known

the  slight  eccentricities  of Z80ASMUK have been accomodated.  The output is

also  compatible  with  other  normal  Z80 assemblers including ZSM. The ASZ80

assembler  in  Vol  517 of the Netherlands Library is also very nice. It gives

.COM files directly and allows inclusion of assembler versions of libraries at

assemble  time (the libraries need some minor modifications).      Quite a lot

of  additional  code  has  been  included  in  this  version  to  support  the

switch/case, for loop and command line arguments.  This has all  been  heavily

based  on a version of Small C in Vol 9 of the C User's group by Mike Bernson.

I have not been able to compile a Z80 version of his compiler  on  my  C80  or

earlier  Small  C  compilers  successfully  so far.  The do-while routines are

taken from the 8086 Small C adapted by Glen Fisher.  One of my  colleagues  at

Brighton  Poly,  Peter  Mercer,  added  hex constants starting with '$' to the

version of Small C that we have been running under UNIX.  I've put  these  in,

but  modified  them  to  use  '0X'  instead  of  '$'.  In the compiler support

routines, 1k buffers are used for the read and #include input streams and  the

normal  buffer  at  $80 for output.  The #include routine in the compiler also

passes an argument "i" to the fopen routine so as to distinguish the call from

the read fopen call which passes the standard "r". The routines are similar to

the ones in CONIO2.LIB and FILE2.LIB except for the larger i/p buffers and the

possibility of "i" mode.  Included file names may optionally  be  enclosed  in

"<" and ">" symbols or quotes.

The  compiler  is  about  28K  long  and  when  compiled (with C comments

included) the assembler code takes up about 200K of disk space so that it will

need  a  system  with  at  least  48k  of  memory  to  run in.  I've  used  an

Amstrad CPC 464 for smaller programs quite successfully.

The  compiler  source  is  divided  into two sections ZSC-1.C and ZSC-2.C.

They can be combined into a single file if desired, but I find them easier  to

edit  in  their present form.  If it is desired to recompile the compiler just

run  ZSC.COM and enter first ZSC-1.C as the input file and then  ZSC-2.C  when

the  name  of  the  next  input  file  is  asked  for.   The  support  library

ZSC-C.LIB can be pulled in after ZSC-2.C in a similar manner.

References.

Dr. Dobbs Journal of Computer Calisthenics and Orthodontia, Box E, Menlo Park,

CA 94025

No. 45  (May 1980)   "A Small C Compiler for the 8080's"  by Ron Cain

A listing in C of the 8080 compiler + notes on implementation.

This issue also contains other articles about C.

No. 48  (Sept. 1980) "A Runtime Library for the Small  C  Compiler"

by  Ron Cain -   Run time library + useful notes.

(See also Nos. 52, 57 and 62 for bug notes by P.L.Woods, K.Bailey,

M.Gore & B.Roehl, J.E.Hendrix)

Nos. 81 and 82 (July, Aug. 1983) "RED - A better Screen Editor" by E.K.Ream

A screen editor written in Small C with a number of library

routines.

There is  a later version of Small C by J.E.Hendrix in DDJ Nos.74 and 75

but this is copyright and so presumably not in the public domain.

"The Small C Handbook" J.E.Hendrix  This is mainly devoted to an update

of Hendrix's development of Small C.  No support routines given, though

they are given in the DDJ version.

C Users Group Vol 9. This contains Mike Bernson's Small C. This is an

8080 version and uses a special assembler which is provided. The

original was got going via BDS C.

SIG/M Vol 149  This includes C86 adapted by Glen Fisher

SIG/M Vol 224  This contains a floating point, Z80 version of Small C.

There is also a Z80 assembler and linker. There are some formatted

I/O facilities. The programming constructs are those in the original

compiler. Modifications by J.R.Van Zandt.

"A book on C" by R.E.Berry and B.A.E.Meekings. (Macmillan 1984)

This includes a listing of 'Rat.C' which is very similar to Ron Cain's

version but has useful cross-referrence  listings  etc.  This  reference is

also worth examining if you want to adapt the compiler to other CPU's. It's

also quite a good introductory low priced text on C in general.

"The C Programming Language" by B.W.Kernighan and D.M.Ritchie (Prentice-Hall)

The standard text on C.

"The  UNIX System" by S.R Bourne. (Addison-Wesley). A superb book on UNIX with

a chapter on C  and an appendix which defines most of the important standard

C functions.

The C/80 compiler from Software Toolworks is an 8080 code compiler which seems

to be based on Small C but is considerably enhanced.

John  Hill  (Nov.1983/Mar.1984/Oct 1985)

#hi