XPAPER.DOC

----------------------------------

documentation for xpaper utility $

----------------------------------

David Harvey    November 1986

XPAPER is an enhanced version of the Amstrad CP/M PAPER utility. It

allows the setting of:

Page length:	PIn	sets to n inches

PLn	sets to n lines

Line spacing:	L6	sets 6 lines per inch

L8	     8

L10	    10

Margins: 	M(n	sets left margin n columns from left

M)n	sets right margin n columns from left

Continuous:	C	sets continuous paper

S	sets single sheet

Gap		Gn	sets page end gap n lines

Paper end:	PN	paper end overridden (for single sheet paper)

PF	paper end detection on (for continuous paper)

High/draft:	QH	sets high quality print

QD	sets draft quality

Zero style:	ZO	(letter O) sets zero without slash

Z0	(zero) sets zero with slash

Italics: 	IN	italics on

IF	italics off

Typestyle:	TP	sets pica (10 cpi)

TL	sets elite (12 cpi)

TC	sets condensed (17 cpi)

TE	sets enlarged (doubles width of current typestyle's

characters

TB	sets bold type

TD	sets double type

Language:	Vn	n=0..8, respectively - US, French, German, UK, Danish,

Swedish, Italian, Spanish, Japanese

Defaults:	D	makes current settings default

R	resets the printer to the

last default set.

Pre-set values:	A4	sets for A4 single sheet

A5	sets for A5 single sheet

n	(any number 1-22) sets continous

paper form length n

-----------------------------------------------------------

Example of command syntax: at CP/M prompt -

A>xpaper C PI6 L6 G5 PF TL TB IN Z0 V0 m(10 m)80 D

==> sets continuous paper, form length 6 inches, 6 lines per inch, paper end

detected, elite typestile (12 cpi), printed bold, in italics, zero with slash,

language version U.S. (# really comes out as hash rather than pound), margins

at column 10 and 80, these settings made the default (to which printer will

return if reset). N.B. after setting C or S set page length (in lines or

inches): there appears to be a bug which makes the line-setting of the printer

behave erratically after C or S, and setting the page length automatically

sets top of form. Unrecognised commands are thrown back at you!

The control codes (#defined in the header file xpaperh.c) should be compatible

withmost Epson-standard printers. If they don't fit yours, and if you have

access to the escape control sequences for the printer, it should be a

straightforward matter to convert the codes in the header file, and recompile

for your particular printer. The compiled (xpaper.com) program supplied is, in

fact, a version compiled under HiSoft C, which because of that compiler's more

restricted implementation of the language, is smaller than that produced by the

MIX C compiler. It will run independently, not requiring the presence of

separate runtime files.

# than that produced by the