Source      :  Amstrad Action

Issue       :  No.19

Date        :  April 1987

Section     :  Programming

Page        :  27

Author      :  AA

There is a simple way of using any standard 5 1/4 inch drive on the

Amstrad (well, practically any - the author claimed that he haven't

yet found a drive for which this won't work). The cable from the Amstrad

to the drive will need disk-ready line grounded - this can easily be

done by shorting the last two lines together. The simple program

below should be run before using the drive under Amsdos. It slows the

disc access rates down so that the drive can be used without the ready

line. The drive can also be used under CPM 2.2 by altering the stepping

rates in the SETUP.COM program.

10 PRINT"SETTING DISK RATES FOR 5.25-INCH DRIVE"

20 MEM=HIMEM

30 MEMORY &3000

40 GOSUB 110

50 FOR A=&4000 TO &401F

60 READ N:POKE A,N:NEXT

70 DATA 50,0,250,0,175,30,30,1,1

80 DATA 130,&21,0,&40,&CD,&D4,&BC,&79,&32,&1D,&40,&22,&1B,

&40,&21,0,&60,&1E,&20,&16,&20,&E,&20,&DF,&1B,&40,&C9,0,

0,0,0,0,0

90 CALL &4001

100 MEMORY MEM:NEW

110 FOR X=&6000 TO &6008

120 READ D:POKE X,D:NEXT

130 RETURN

There is some disk drive which was recommended by the Amstrad according

to David Robinson in the Amstrad CPC464 User magazine May 1985. One of

the characteristic that 5 1/4 drive must have is:

1) The DRIVE must have a READY signal on pin 34

2) Have suitable step rate, motor on & off timeout.

3) Have a shuggart interface.

The author recommended The CHINNON F051-MD 5/14 disc drive

The next purchase is the power supply providing 12V 5V and ground via a

four pin connector arranged 12V/G/G/5V.

Finally we need a way of connecting the ribbon cable from the disc

controller to your new disc drive. You need 2 items here:

1. A 34 way card edge connector

2. A 40 pin PCB header which you convert to 34 ways by cutting off

three pairs of pins from one end.

Remove the ribbon cable clamp from the card edge connector then carefully

solder the SHORTER of the pins to the terminal on the back.

Where to get this CHINON F 051-MD 5 1/4 inch disk drive here is it:

ALTEK MICROCOMPUTERS LTD.

22 Market Place,

Wokingham,

Berks.

Where to get the power supply:

MICROWORLD.

12 York Place,

Brighton.

Where to get the Connector and cable:

Our local TANDY store.       1) 40 position header - Cat. No. 2761540

2) 34 way card edge connector-Cat No. 2761564A

I like to advice you that I am not liable to any damage arouse after

this program as I am just provide and information to you. If you like

it keep it if not throw it away. And also there is no guarantee that

this program will work as I haven't test it myself. Thanks.

###########################################e is no guarantee that

this program will work as I haven't test it myself. Thanks.

##########################################