' This program was created by James Gerrard Delaney, Sheffield, UK. - DESC v1.0

' Copyright JD Digital Electronics 1997,98. All Rights Reserved. Patent Pending.

begin:

CLS

COLOR 13, 0: PRINT TAB(13); "  Z80 DESCATOC Card Control Centre Version 1.0"

PRINT

COLOR 7: PRINT TAB(11); " Copyright JD Digital Electronics Ltd, Sheffield UK  1997,98"

COLOR 15, 0

OPEN "DESC1.DAT" FOR INPUT AS #1   'get directory info

INPUT #1, ZX1$

CLOSE #1

OPEN ZX1$ + "\IO.DAT" FOR INPUT AS #1

INPUT #1, Z2$

CLOSE #1                'Z2 string now holds 'fresh' or 'stale'

IF Z2$ <> "" THEN GOTO displaydetail  'already got address

PRINT: PRINT: PRINT

PRINT TAB (4);"Searching for a DESCATOC Card..."

FOR P = &H250 TO &H2E0 STEP &H08

DELAY 1

LOCATE 12, 28, 0: PRINT "Looking at ISA address ";: COLOR 14,0: PRINT               HEX$(P);:COLOR 15, 0

PRINT "h"

A = INP(P + &H08)       'port for reading status of HALT/ARDY

X2 = (A AND &H05)       'look only for ARDY/HALT status

IF X2 <> &H01 OR X2 <> &H03 THEN GOTO nexxtn1

A = INP(P)	        'ARDY 1 HALT 0, ok, now read byte in

IF A <> &H44 THEN GOTO nexxtn1 ' not Char D, so next port

RESTORE dayter

FOR E = 1 TO 7          ' got D, so test further

OUT (P + &H08), &H02    ' get Z80 to send another Char(Z80 int)

FOR DEL = 1 TO 10: NEXT DEL  'short delay

OUT (P + &H08), &H03    ' release ASTB from being active low

CO = 100000

leopy1:

A = INP(P + &H08)       'prepare to read status for HALT/ARDY

X2 = (A AND &H05)       'look for only ARDY/HALT status

IF C0 = 0 THEN GOTO nexxtn1   'status never change, not DESC so jump

IF X2 <> &H01 OR X2 <> &H03 THEN CO = CO - 1: GOTO leopy1

A =INP(P)               ' ARDY 1, HALT 0 so read byte in

READ X1(E)              ' fetch comparison byte

IF X1 <> A THEN GOTO nexxtn1  ' not a match, so not Card jump!

NEXT E                  ' passed all tests so found Card.

GOTO foundit1

dayter:

DATA &H45, &H53, &H43, &H41, &H54, &H4F, &H43 'ESCATOC

nexxtn1:

NEXT P                  ' Card not found so get another address

PRINT: PRINT

PRINT " This Computer has not found a DESCATOC Card."

PRINT

PRINT " You have done one of three things: "

PRINT: PRINT

PRINT " a) You have changed the Binary I/O Address outside the normal"

PRINT "    address range of 250h to 2E0h."

PRINT

PRINT " b) You have not yet installed the Card into the PC.

PRINT

PRINT " c) You have made the Binary address of the Control Ports "

PRINT "    not 8 addresses above the Data Ports. for example :

PRINT "    If Data Ports = 250h you would now make the Control ports 258h."

PRINT

PRINT

PRINT " Please do not install the Card until you have read the following...."

DELAY 30: GOTO program

foundit1:

OPEN ZX1$ + "\IO.DAT" FOR OUTPUT AS #1

WRITE #1, HEX$(P) + "h/" + HEX$(P + &H08) + "h"  'save card addresses

CLOSE #1

LOCATE 14, 28, 0: PRINT " Found a DESCATOC Card at ";HEX$(P);"h and ";HEX$(P+&H08);"h."

PRINT: PRINT

PRINT TAB(28);" Setting up to test Card RAM Memory...."

FOR T = 1 TO 10000: NEXT T: GOTO testmem1

displaydetail:

OPEN ZX1$ + "\IO.DAT" FOR INPUT AS #1

INPUT #1, AD$: CLOSE #1

PRINT TAB(4);" The DESCATOC Card has the Binary address of ";AD$

PRINT: PRINT

PRINT TAB(4);" Continuing this program......"

DELAY 20: GOTO program

testmem1:

A = INP(P + &H08)

X2 = (A AND &H05)         'test only HALT/ARDY

IF X2 <> &H01 OR X2 <> &H03 THEN GOTO testmem1 'wait for Z80

A = INP(P)                'port ready, get byte

A2 = A                    ' temp storage in A2

OUT(P + &H08), &H02       ' signal active ASTB Z80 gets next byte

FOR T = 1 TO 10: NEXT T   ' small delay

OUT(P + &H08), &H03       ' signal inactive ASTB

testmem2:

A = INP(P + &H08)

X2 = (A AND &H05)         'test only HALT/ARDY

IF X2 <> &H01 OR X2 <> &H03 THEN GOTO testmem2 'wait for Z80

A = INP(P)

IF A2 = &HCF AND A = &H00 THEN GOTO ramFAIL

PRINT: PRINT

FOR A3 = 1 TO 1024

LOCATE 18, 28, 0: PRINT "Testing address ";HEX$(A3);" OK."

FOR DEL = 1 TO 30: NEXT DEL

NEXT A3

LOCATE 20, 28, 0: PRINT " All of the Cards RAM has passed.."

LOCATE 22, 28, 0: PRINT " Continuing with this program..."

DELAY 30: GOTO program

ramFAIL:

CLS: COLOR 15, 0

PRINT "                      WARNING !!              "

PRINT

PRINT " For some reason the Card's RAM Memory has failed. Please send"

PRINT " this Card back in the original packaging well rapped "

PRINT " to the following address:                  "

PRINT

PRINT

PRINT "                      Jim Delaney              "

PRINT "                      4 Hyacinth Road          "

PRINT "                      High Wincobank           "

PRINT "                      Sheffield                "

PRINT "                      S5 6BE                   "

PRINT

PRINT " if you have filled in the registration form beforehand"

PRINT " this card will recieve immediate attention. If you have not"

PRINT " filled in the form yet, please do so, then send both the Card"

PRINT " and the form together to the above address. all faulty Cards"

PRINT " returned within the warranty period are free of service charges."

PRINT " your Card will be returned to you within a few working days."

PRINT

PRINT              Continuing Batch job..........": END

program:

CLS : DEF SEG = 0: S = PEEK(&H417)  'Read keyboard status.

IF S >= 128 THEN S = S - 128

IF S = 96 THEN : POKE &H417, (S XOR &H60)

IF S = 64 THEN : POKE &H417, (S XOR &H40)

IF S = 32 THEN : POKE &H417, (S XOR &H20)

IF S = 16 THEN : POKE &H417, (S XOR &H10)

SCREEN 0

start:

' test for the mouse

REG 1, 0

CALL INTERRUPT &H33

AX = REG(1)

BX = REG(2)

IF AX <> -1 AND BX <> 2 THEN MOUSEOFF = 1: GOTO skipmouse

GOTO skipmouse

mouse1:

MOUSEOFF = 0

'define graphics cursor block

REG 1, &HA

REG 2, &H0

REG 3, &H6EFF

REG 4, &HB200

CALL INTERRUPT &H33

'finally switch cursor on

REG 1, &H1

CALL INTERRUPT &H33

RETURN

skipmouse:

' 1ST LINE

CLS

COLOR 15: L = 240: M = 177: GOSUB partof: L = 3: GOSUB partof: COLOR 11, 14

PRINT CHR$(201); CHR$(205); : L = 69: M = 205: GOSUB partof: PRINT CHR$(187);

COLOR 15, 0: L = 5: M = 177: GOSUB partof

' 2ND LINE

L = 3: GOSUB partof: COLOR 11, 14: PRINT CHR$(186); : L = 70: M = 32: GOSUB partof

PRINT CHR$(186); : COLOR 8, 8: PRINT CHR$(178); CHR$(178); : COLOR 15, 0:L=3

M = 177: GOSUB partof

' 3RD LINE

L = 3: GOSUB partof: COLOR 11, 14: PRINT CHR$(186); "    "; : COLOR 14, 14

PRINT "  Welcome to Jim Delaney's PC DESCATOC Card Programming Suite.";

PRINT "    "; : COLOR 11, 14: PRINT CHR$(186); : COLOR 8, 8

PRINT CHR$(178); CHR$(178); : COLOR 15, 0: L = 3: M = 177: GOSUB partof

' MIDDLE OF PAGE

FOR T = 1 TO 15: GOSUB blankline: NEXT T

' LAST PAGE LINE

COLOR 15: L = 3: GOSUB partof: COLOR 11, 14: PRINT CHR$(200); CHR$(205);

L = 69: M = 205: GOSUB partof: PRINT CHR$(188); : COLOR 8, 8: PRINT CHR$(178);

PRINT CHR$(178); : COLOR 15, 0: L = 8: M = 177: GOSUB partof: COLOR 8, 8: L = 72: M = 178

GOSUB partof: COLOR 15, 0: L = 3: M = 177: GOSUB partof: COLOR 15, 0: L = 160

GOSUB partof: R = 0: SOU = 1

page1:

LOCATE 7, 8, 0: COLOR 8, 14

PRINT "   There now follows instructions on how to set this Card up."

LOCATE 8, 10, 0

PRINT "The initial procedure requires the operator to time the PC's"

LOCATE 9, 10, 0

PRINT "Boot sequence to either the prompt for DOS or Desktop for W95."

LOCATE 11, 10, 0

PRINT "You will need to familiarise yourself with the Programming"

LOCATE 12, 7, 0

PRINT " Commands first of all, and then assign each distinctive software"

LOCATE 13, 7, 0

PRINT " Channel these commands. These commands can be entered into each"

LOCATE 14, 6, 0

PRINT " Channel in any order, thus providing a total programming solution."

LOCATE 15, 6, 0

PRINT " You will next see a list of all the programming commands you can"

LOCATE 16, 6, 0

PRINT " use followed by a brief demonstration. The demo gets the PC to run "

LOCATE 17, 6, 0

PRINT " a very nice DOS PC comms package called Terminate 5.00. Here, the "

LOCATE 18, 11, 0

PRINT " the Host section of the program is loaded by the example."

LOCATE 20, 11, 0

COLOR 15, 0: PRINT "S"; : COLOR 11, 15: PRINT "kip info": LOCATE 20, 26, 0

COLOR 15, 0: PRINT "C"; : COLOR 11, 15: PRINT "ontinue": LOCATE 20, 63, 0

COLOR 14, 14: PRINT " "; : COLOR 14, 14: PRINT "   ": LOCATE 20, 41, 0

COLOR 15, 0: PRINT "P"; : COLOR 11, 15: PRINT "rint": LOCATE 20, 53, 0

COLOR 15, 0: PRINT "E"; : COLOR 11, 15: PRINT "xit"

L = 80: M = 177: LOCATE 25, 1, 0: COLOR 15, 0: GOSUB partof

IF SOU = 1 THEN SHELL "F:\DESCATOC\PLANY F:\DESCATOC\WELDESC.WAV -r 44500"

IF SOU  =1 AND MOUSEOFF = 0 THEN SHELL "F:\DESCATOC\PLANY F:\DESCATOC\MOUSE1.WAV"

'after WAV, switch the mouse on, and set up its cursor

IF SOU=1 AND MOUSEOFF = 0 THEN GOSUB mouse1: GOSUB mousepoint

IF MOUSEOFF = 0 THEN GOSUB fillit2

IF P = 3 AND MOUSEOFF = 0 THEN

'Remove char left by mouse cursor

DO

REG 1, &H3

CALL INTERRUPT &H33

CX = REG(3)     'CX equals the current horizontal cursor position

DX = REG(4)     'DX equals the current vertical position

IF CX <> HOR OR DX <> VER THEN GOTO ridchar

LOOP

END IF

therest:

P = 2: IF R = 1 THEN RETURN

DIM A%(10)

R = 1

KEY 15, CHR$(&H0) + CHR$(&H2E)                'Set up C as KEY 15.

KEY 16, CHR$(&H0) + CHR$(&H30)                'Set up B as KEY 16.

KEY 17, CHR$(&H0) + CHR$(&H1F)                'Set up S as KEY 17.

KEY 18, CHR$(&H0) + CHR$(&H19)                'Set up P as KEY 18.

KEY 19, CHR$(&H0) + CHR$(&H12)                'Set up E as KEY 19.

KEY(15) ON: KEY(16) ON: KEY(17) ON: KEY(18) ON: KEY(19) ON

GOTO circ2

mousepoint:

'we now need to reset position of mouse to 3, 1

'due to page swap.

REG 1, &H04

REG 3, &H01

REG 4, &H10

CALL INTERRUPT &H33

RETURN

fillit2:

'refill 1, 1 with character 177

DO

REG 1, &H03

CALL INTERRUPT &H33

CX = REG(3)

DX = REG(4)

IF CX<>&H01 OR DX<>&H01 THEN

COLOR 15, 0

LOCATE 3, 1, 0

PRINT CHR$(177)

LOCATE 20,1,0

PRINT CHR$(177)

LOCATE 21,1,0

PRINT CHR$(177)

RETURN

END IF

LOOP

circ2:

'were depressing the keyboard keys!

DO

MOU=0

IF MOUSEOFF = 1 THEN GOTO skipmousey

GOSUB fillit2

'get button press info

REG 1, &H5

REG 2, &H0

CALL INTERRUPT &H33

AX = (REG(1) AND &H1)

CX = REG(3)

DX = REG(4)

HOR = CX: VER = DX

'calculate cursor position verses but. press

'for skip undo

IF CX >= &H50 AND CX <= &H90 AND DX = &H98 AND AX = &H1 THEN MOU=1: GOSUB click: GOSUB pequals5

'for coniue

IF CX >= &HC8 AND CX <= &H100 AND DX = &H98 AND AX = &H1 THEN MOU=1: GOSUB click: GOSUB goforward

'for print

IF CX >= &H140 AND CX <= &H160 AND DX = &H98 AND AX = &H1 THEN MOU=1: GOSUB click: GOSUB printscreen

'for exit

IF CX >= &H1A0 AND CX <= &H1B8 AND DX = &H98 AND AX = &H1 THEN MOU=1: GOSUB click: GOSUB leaveprog

'for backwards

IF CX >= &H1F0 AND CX <= &H208 AND DX = &H98 AND AX = &H1 AND P <> 1 THEN MOU=1: CX = HOR: DX = VER: GOSUB click: GOSUB goback

skipmousey:

ON KEY(15) GOSUB goforward

ON KEY(16) GOSUB goback

ON KEY(17) GOSUB pequals5

ON KEY(18) GOSUB printscreen

ON KEY(19) GOSUB leaveprog

LOOP

click:

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\STOP.WAV": RETURN

pequals5:

'using keyboard, reset mouse x,y if using the mouse

P = 5

goforward:

'using keyboard reset mouse x,y if using the keyboard

IF MOUSEOFF=0 AND MOU=0 THEN GOSUB mousepoint

IF P = 5 AND MOUSEOFF = 0 AND MOU = 1 THEN GOSUB mousepoint

IF MOU=1 THEN FOR del = 1 TO 250000: NEXT del

IF P = 2 THEN GOTO page2

IF P = 3 THEN GOTO page3

IF P = 4 THEN GOTO page4

IF P = 5 THEN GOTO page5

goback:

'using keyboard, reset mouse x,y if using the keyboard

SOU = 0

IF MOUSEOFF=0 AND MOU=0 THEN GOSUB mousepoint

IF MOU=1 THEN FOR del = 1 TO 250000: NEXT del

IF P = 3 THEN

LOCATE 10, 1, 0

GOSUB blankline

LOCATE 9, 8, 0

COLOR 11, 14, 0

PRINT "  ";

LOCATE 9, 72, 0

PRINT " "

LOCATE 18, 8, 0

PRINT "   "

GOTO page1

END IF

IF P = 4 THEN GOTO page2

IF P = 5 THEN GOTO page3

ridchar:

COLOR 11, 14: LOCATE 20, 63, 0: PRINT "    ": GOTO therest

leaveprog:

DEF SEG = 0

S = PEEK(&H417)

IF S >= 128 THEN S = S - 128

POKE &H417, (S OR &H60): END                 'Caps and Num Lock to on

page2:

LOCATE 7, 8, 0: COLOR 8, 14

PRINT "     Here are a list of commands that you will need to input; "

GOSUB blankline: COLOR 15, 8: LOCATE 9, 8, 0: PRINT " WAIT-[mins] , [secs] ";

COLOR 8, 14: PRINT " syntax : WAIT-1,34 - wait 1 minute 34 secs"

LOCATE 10, 8, 0: COLOR 15, 8: PRINT " SEND$ ";

COLOR 8, 14

PRINT "syntax : SEND$(,E:,CD\,TERMINAT,) seperate DOS commnds by a"

LOCATE 11, 8, 0

PRINT "        comma, and enclose the statement in brackets.        "

LOCATE 12, 8, 0: COLOR 15, 8: PRINT " ALT-[x] ";

COLOR 8, 14

PRINT " syntax : ALT-whatever key.                              "

LOCATE 13, 8, 0: COLOR 15, 8: PRINT " SHIFT-[x] ";

COLOR 8, 14

PRINT " syntax : SHIFT-whatever key.                        "

LOCATE 14, 7, 0: COLOR 8, 14: PRINT " ";

COLOR 15, 8: PRINT " CNTRL-[x] ";

COLOR 8, 14: PRINT " syntax : CNTRL-whatever key.                         "

LOCATE 15, 7, 0: COLOR 8, 14: PRINT " ";: COLOR 15, 8

PRINT " RESET ";

COLOR 8, 14

PRINT " syntax : RESET  sends CTRL,ALT DELETE in that order.    "

LOCATE 16, 7, 0: COLOR 8, 14: PRINT " ";

COLOR 15, 8: PRINT " DWNAR-[x] ";

COLOR 8, 14

PRINT " syntax : DWNAR-no. of times you want arrow to go down. "

LOCATE 17, 7, 0: COLOR 8, 14: PRINT" ";

COLOR 15, 8: PRINT " UPAR-[x] ";

COLOR 8, 14

PRINT " syntax : UPAR-no. of times you want arrow to go up.    "

LOCATE 18, 7, 0: COLOR 8, 14: PRINT " ";

COLOR 15, 8: PRINT" ENTER ";

COLOR 8, 14: PRINT" syntax : ENTER  sends a carriage return.                  "

LOCATE 20, 63, 0: COLOR 15, 0: PRINT "B";

COLOR 11, 15: PRINT "ack"

P=3

RETURN

page3:

LOCATE 7, 8, 0: COLOR 15, 8: PRINT " POW0 "; : COLOR 8, 14

PRINT " syntax : POW0  cut mains power supply to controlled        "

LOCATE 8, 15, 0: PRINT "devices and reset, pending another call."

LOCATE 9, 8, 0: COLOR 15, 8: PRINT " ICCR "; : COLOR 8, 14

PRINT " syntax : ICCR  make this the very first command of your    "

LOCATE 10, 8, 0

PRINT "       channel, and Intelligent Close Call Recognition will be     "

LOCATE 11, 15, 0

PRINT "enabled for that channel.                                 "

LOCATE 12, 8, 0: COLOR 15, 8: PRINT " HOLD-[x$] , [y secs] "; : COLOR 8, 14

PRINT " syntax : HOLD-(DEL),4   hold the DEL key  "

LOCATE 13, 8, 0

PRINT "                       for 4 seconds.                        "

LOCATE 14, 8, 0: COLOR 15, 8: PRINT " PGDWN-[x] "; : COLOR 8, 14

PRINT " syntax : PGDWN-4  send Pg Dn four times.              "

LOCATE 15, 8, 0: COLOR 15, 8: PRINT " PGUP-[x] "; : COLOR 8, 14

PRINT " syntax : PGUP-2  send Pg Up two times.            "

LOCATE 16, 8, 0: COLOR 15, 8: PRINT " NUM-[x] "; : COLOR 8, 14

PRINT " syntax : NUM-0 or NUM-1  toggle the NumLock switch on   "

LOCATE 17, 8, 0

PRINT "          the internal keyboard 0 = off, and 1 = on.              "

LOCATE 18, 1, 0: GOSUB blankline: LOCATE 19, 1, 0

GOSUB blankline: P = 4

RETURN

page4:

LOCATE 7, 8, 0: COLOR 15, 8: PRINT " CAPS-[x] "; : COLOR 8, 14

PRINT " syntax : CAPS-1 or CAPS-0  toggle the CapsLock switch   "

LOCATE 8, 15, 0

PRINT "    on or off on the internal keyboard.                   "

LOCATE 9, 8, 0: COLOR 15, 8: PRINT " TAB-[x] "; : COLOR 8, 14

PRINT " syntax : TAB-5  send 5 tabs.                            "

LOCATE 10, 8, 0: COLOR 15, 8: PRINT " ESC-[x] "; : COLOR 8, 14

PRINT " syntax : ESC-6  send escape six times.                 "

LOCATE 11, 8, 0: COLOR 15, 8: PRINT " EXIT "; : COLOR 8, 14

PRINT " syntax : EXIT  leave reading these instructions for now,  "

LOCATE 12, 8, 0

PRINT "       and wait for a count down to power off signal from the PC. "

LOCATE 13, 1, 0: GOSUB blankline: LOCATE 14, 7, 0

COLOR 8, 14

PRINT " All these commands may be entered in any random order for each   "

LOCATE 15, 7, 0

PRINT " channel. You will be asked to enter two sets of instructions "

LOCATE 16, 7, 0

PRINT " per channel, because the DESCATOC Card needs one set for booting"

LOCATE 17, 7, 0

PRINT " and one set for powering down. This means that 8 sets are needed."

LOCATE 18, 7, 0

PRINT " If you now need to time your PC and write down all the necessary"

LOCATE 19, 7, 0

PRINT " command sequences, please press E now, else press C to program.."

P = 5

RETURN

page5:

aft = 0: eofentry = 0: command = 1

L = 80: M = 219: COLOR 7, 0: LOCATE 1, 1, 0: GOSUB partof

GOSUB nextring

IF eofentry = 1 THEN GOSUB entry

IF command = 1 THEN GOSUB cmnd

IF aft = 1 THEN GOSUB afterpages: GOTO continue1 ELSE GOTO continue1

nextring:

LOCATE 1, 3, 0: COLOR 7, 7: PRINT "                                                                  "

LOCATE 1, 3, 0: COLOR 0, 7: PRINT "                           Command ? ";

RETURN

cmnd:

LOCATE 1, 29, 0: COLOR 0, 7: PRINT "    Command ?": RETURN

entry:

LOCATE 1, 57, 0: COLOR 12, 7: PRINT "F3";

COLOR 0, 7: PRINT " = Program Card": RETURN

afterpages:

LOCATE 1, 30, 0: COLOR 12, 7: PRINT "F2"; : COLOR 0, 7

PRINT " = Previous 40 entries": RETURN

continue1:

GOSUB clearpage

X = 3: Y = 201: Z = 187: M = 205: GOSUB blankline2: P = 6

FOR K = 17 TO 77 STEP 20

LOCATE 3, K, 0: COLOR 15, 8: PRINT CHR$(177); CHR$(177): NEXT K

A = 0: K = 10

DO WHILE A < K

X = X + 1: Y = 186: Z = Y: M = 219: GOSUB blankline2

X = X + 1: M = 32: GOSUB blankline2

A = A + 1

LOOP

Y = 200: Z = 188: M = 205: GOSUB blankline2

X = X + 1

COLOR 8, 8

FOR A = 5 TO 77 STEP 20      'bottom shadow

FOR B = 0 TO 13

LOCATE X, B + A, 0: PRINT CHR$(178);

NEXT B: NEXT A

Y = 3: GOSUB line3

X = 5: GOSUB numbers1t9

B = 49                       ' bottom row decades

FOR A = 4 TO 65 STEP 20: LOCATE 22, A, 0: PRINT CHR$(B); "0"

B = B + 1: NEXT A

D = 49

FOR N = 24 TO 80 STEP 20     ' prints remaining decades in columbs

FOR A = 1 TO 9

LOCATE A * 2 + 2, N, 0: PRINT CHR$(D): NEXT A: D = D + 1: NEXT N

' end of page 5, the programming page.

' now start the keyboard routines for this page

KEY(15) OFF: KEY(16) OFF: KEY(17) OFF: KEY(18) OFF: KEY(19) OFF

POKE &H417, &H40: K = 0

LOCATE 1, 5, 0: COLOR 0, 7

GOSUB mouse1

PRINT "    how many distinctive ring channels do you want ?,";

PRINT " ";: COLOR 9, 3: PRINT"1";: COLOR 0, 7: PRINT ", ";

COLOR 9, 3: PRINT"2";: COLOR 0, 7: PRINT ", ";

COLOR 9, 3: PRINT "3";: COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "4";: COLOR 0, 7: PRINT " ?"

arounditt:

X$ = INKEY$

IF X$ = CHR$(49) THEN CH = 1: GOSUB obliterate: GOTO nextthing

IF X$ = CHR$(50) THEN CH = 2: GOSUB obliterate: GOTO nextthing

IF X$ = CHR$(51) THEN CH = 3: GOSUB obliterate: GOTO nextthing

IF X$ = CHR$(52) THEN CH = 4: GOSUB obliterate: GOTO nextthing

MENU = 1

IF MOUSEOFF=1 THEN GOTO arounditt

mousepos:

REG 1, &H05

REG 2, &H00

CALL INTERRUPT &H33

AX = (REG(1) AND &H01)              'strip bits 1 to 15. keep bit 0.

CX = REG(3)       'horizontal pos

DX = REG(4)       'vertical pos

IF MENU = 1 AND CX = &H01D0 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: CH = 1

GOSUB obliterate: GOTO nextthing

END IF

IF MENU = 1 AND CX = &H01E8 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: CH = 2

GOSUB obliterate: GOTO nextthing

END IF

IF MENU = 1 AND CX = &H0200 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: CH = 3

GOSUB obliterate: GOTO nextthing

END IF

IF MENU = 1 AND CX = &H0228 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: CH = 4

GOSUB obliterate: GOTO nextthing

IF MENU = 1 THEN GOTO arounditt

IF MENU = 2 AND CX = &H0158 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: COLOR 7, 7: LOCATE 1, 33, 0

PRINT "                                         ": GOSUB nextring: GOTO fromscratch

END IF

IF MENU = 2 AND CX = &H0180 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: GOTO recall

IF MENU = 2 THEN GOTO arounditt2

IF MENU = 3 AND CX = &H0190 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: re = 0: GOSUB obliterate: GOTO recall2

IF MENU = 3 AND CX = &H01B8 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: re = 1: GOSUB obliterate: GOTO recall2

IF MENU = 3 AND CH = 2 THEN GOTO recall3

IF MENU = 3 AND CX = &H01E0 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: re = 2: GOSUB obliterate: GOTO recall2

IF MENU = 3 AND CH = 3 THEN GOTO recall3

IF MENU = 3 AND CX = &H0208 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: re = 3: GOSUB obliterate: GOTO recall2

IF MENU = 3 THEN GOTO arounditt3

IF MENU = 4 AND CX = &H00F8 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: GOSUB obliterate: GOTO powerup

IF MENU = 4 AND CX = &H0160 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: GOSUB obliterate: GOTO powerdown

IF MENU = 4 THEN GOTO arounditt4

IF MENU = 5 AND CX = &H0170 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: GOTO edit

IF MENU = 5 AND CX = &H0180 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: GOSUB nextring: GOTO selecty

IF MENU = 5 AND CX = &H01F0 AND DX = &H0000 AND AX = &H01 THEN GOSUB click: GOSUB mousepoint: BLO = 1: GOTO blowcard

IF MENU = 5 THEN GOTO arounditt5

nextthing:

OPEN "DRIVES.DAT" FOR INPUT AS #1       'fetch from install.exe data

INPUT #1, XX$                           'about the location of this program

CLOSE #1                                'and store in xx string.

nextnaround:

LOCATE 1, 5, 0: COLOR 0, 7

PRINT "       want to program or recall data, ";

COLOR 9, 3: PRINT "P";: COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "R";: COLOR 0, 7: PRINT " ?                    "

arounditt2:

B$ = INKEY$

IF B$ = CHR$(80) THEN COLOR 7, 7: LOCATE 1, 33, 0

PRINT "                                         ": GOSUB nextring: GOTO fromscratch      'program

END IF

IF B$ = CHR$(82) THEN GOTO recall            'recall

MENU = 2

IF MOUSEOFF=1 THEN GOTO arounditt2

GOTO mousepos

obliterate:

LOCATE 1, 1, 0: COLOR 0, 7: PRINT "                                                                             "

RETURN

recall:

LOCATE 1, 5, 0: COLOR 0, 7

IF CH = 1 THEN PRINT "               recalling data for ring channel 0.                   ": re = 0: DELAY 1: re = 0

GOSUB obliterate: GOTO recall2    'openforinput channel

END IF

IF CH = 2 THEN

PRINT "          recall data for what ring channel ? ";

COLOR 9, 3: PRINT "0";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "1";

COLOR 0, 7: PRINT " ?              "

END IF

IF CH = 3 THEN

PRINT "          recall data for what ring channel ? ";

COLOR 9, 3: PRINT "0";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "1";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "2";

COLOR 0, 7: PRINT" ?          "

END IF

IF CH = 4 THEN

PRINT "          recall data for what ring channel ? ";

COLOR 9, 3: PRINT "0";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "1";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "2";

COLOR 0, 7: PRINT " or ";

COLOR 9, 3: PRINT "3";

COLOR 0, 7: PRINT " ?      "

END IF

arounditt3:

B$ = INKEY$

IF B$ = CHR$(48) THEN re = 0: GOSUB obliterate: GOTO recall2

IF B$ = CHR$(49) THEN re = 1: GOSUB obliterate: GOTO recall2    'openforinput channel 1

IF CH = 2 THEN GOTO recall3

IF B$ = CHR$(50) THEN re = 2: GOSUB obliterate: GOTO recall2    'openforinput channel 2

IF CH = 3 THEN GOTO recall3

IF B$ = CHR$(51) THEN re = 3: GOSUB obliterate: GOTO recall2    'openforinput channel 3

recall3:

MENU = 3

IF MOUSEOFF = 1 THEN GOTO arounditt3

GOTO mousepos

recall2:

LOCATE 1, 5, 0: COLOR 0, 7

PRINT "     do you want the power ";

COLOR 9, 3: PRINT "U";: COLOR 0, 7

PRINT "p, or power ";

COLOR 9, 3: PRINT "D";: COLOR 0, 7

PRINT "own info for ch";re; "? "

arounditt4:

B$ = INKEY$

IF B$ = CHR$(85) THEN GOSUB obliterate: GOTO powerup            'power up required

IF B$ = CHR$(68) THEN GOSUB obliterate: GOTO powerdown          'power down required

MENU = 4

IF MOUSEOFF = 1 THEN GOTO arounditt4

GOTO mousepos

powerup:

IF re = 0 THEN ZX1$ = XX$ + "\DESC0A.DAT": ZX = 1: GOTO extractfrdisk

IF re = 1 THEN ZX1$ = XX$ + "\DESC1A.DAT": ZX = 3: GOTO extractfrdisk

IF re = 2 THEN ZX1$ = XX$ + "\DESC2A.DAT": ZX = 5: GOTO extractfrdisk

IF re = 3 THEN ZX1$ = XX$ + "\DESC3A.DAT": ZX = 7: GOTO extractfrdisk

powerdown:

IF re = 0 THEN ZX1$ = XX$ + "\DESC0B.DAT": ZX = 2: GOTO extractfrdisk

IF re = 1 THEN ZX1$ = XX$ + "\DESC1B.DAT": ZX = 4: GOTO extractfrdisk

IF re = 2 THEN ZX1$ = XX$ + "\DESC2B.DAT": ZX = 6: GOTO extractfrdisk

IF re = 3 THEN ZX1$ = XX$ + "\DESC3B.DAT": ZX = 8

extractfrdisk:

OPEN ZX1$ FOR INPUT AS #1  'assign open file as zz$

INPUT #1, ZZ$

SP = 0

FOR S = 1 TO LEN(ZZ$)

IF MID$(ZZ$, S, 1) = "?" THEN SP = SP + 1         'found a qmark add 1

skipit2:

NEXT S                        'all de above tested ok.

IF SP < 11 THEN NT = 6        'set up to reprint to the screen

IF SP < 21 THEN NT = 26

IF SP < 31 THEN NT = 46

IF SP < 41 THEN NT = 66

IF SP < 11 THEN NC = SP

IF SP > 10 THEN NC = 10

S0 = 1: ZNY = 1: GOSUB resethatch: GOSUB goforit

M2 = 1

FOR X = 6 TO NT STEP 20              ' 6 to 66

FOR T = 1 TO NC                     ' 1 to 10

leave3:

FOR G = M2 TO LEN(ZZ$)                 'get the string in saved version

X$ = MID$(ZZ$, G, 1)

IF X$ = "?" THEN GOTO leave2

NEXT G                                  'g points to a space

leave2:

G = G - M2

G1$ = LEFT$(MID$(ZZ$, M2, G), 10)      'mid string now equals sub string

leave4:

LOCATE T * 2 + 2, X, 0: COLOR 11, 0

M2 = M2 + G + 1: PRINT G1$

NEXT T: NEXT X: ZNY = 1: BLO = 0: GOTO yippee

fromscratch:

ZNY = 0

IF CH = 1 THEN ZX4 = 1: ZX5 = 2: GOSUB obliterate: GOSUB nextring: GOTO ruunit

IF CH = 2 THEN ZX4 = 1: ZX5 = 4: GOSUB obliterate: GOSUB nextring: GOTO ruunit

IF CH = 3 THEN ZX4 = 1: ZX5 = 6: GOSUB obliterate: GOSUB nextring: GOTO ruunit

IF CH = 4 THEN ZX4 = 1: ZX5 = 8: GOSUB obliterate: GOSUB nextring: GOTO ruunit

IF X$ = CHR$(65) THEN ZX4 = 1: ZX5 = 8: GOSUB obliterate: GOSUB nextring: GOTO ruunit

ruunit:

FOR ZX = ZX4 TO ZX5

GOSUB goforit

resethatch:

FOR X = 6 TO 80 STEP 20                             'reset display hatches

FOR Y = 1 TO 10

COLOR 0, 0: LOCATE Y * 2 + 2, X, 0: PRINT "          "

NEXT Y: NEXT X

IF ZNY = 0 THEN GOTO onwitit3

RETURN

goforit:

LOCATE 2, 15, 0: COLOR 15, 13

IF ZX = 1 THEN ZX1$ = XX$ + "\DESC0A.DAT": PRINT " P O W E R    U P    R I N G    C H A N N E L   0"; : GOTO color2

IF ZX = 2 THEN ZX1$ = XX$ + "\DESC0B.DAT": PRINT " P O W E R    D O W N    R I N G    C H A N N E L  0"

IF ZX = 3 THEN ZX1$ = XX$ + "\DESC1A.DAT": PRINT " P O W E R    U P    R I N G    C H A N N E L   1"; : GOTO color2

IF ZX = 4 THEN ZX1$ = XX$ + "\DESC1B.DAT": PRINT " P O W E R    D O W N    R I N G    C H A N N E L  1"

IF ZX = 5 THEN ZX1$ = XX$ + "\DESC2A.DAT": PRINT " P O W E R    U P    R I N G    C H A N N E L   2"; : GOTO color2

IF ZX = 6 THEN ZX1$ = XX$ + "\DESC2B.DAT": PRINT " P O W E R    D O W N    R I N G    C H A N N E L  2"

IF ZX = 7 THEN ZX1$ = XX$ + "\DESC3A.DAT": PRINT " P O W E R    UP     R I N G    C H A N N E L   3"; : GOTO color2

IF ZX = 8 THEN ZX1$ = XX$ + "\DESC3B.DAT": PRINT " P O W E R    D O W N    R I N G    C H A N N E L  3"

RETURN

color2:

COLOR 15, 0: PRINT CHR$(177); CHR$(177); CHR$(177): RETURN

onwitit3:

DIM D$(240, 2)

B$ = "": Z0$ = ""

FOR X = 6 TO 80 STEP 20

FOR T = 1 TO 10

OPEN ZX1$ FOR OUTPUT AS #1

A$ = "": COLOR 0, 7

ED = 0                         ' not coming from editor

around:

K = 0

skipthis:

B$ = INKEY$

errrr = 0

FOR Q = 1 TO 255

GOSUB cursorflash

IF A$ = "" AND B$ = CHR$(13) THEN BEEP: GOTO around   'leftmost on the line

IF B$ = CHR$(8) THEN GOSUB delete1: GOTO around       'delete the char

IF B$ = CHR$(13) THEN GOTO around2                    'newline

IF LEN(A$) = 37 THEN BEEP: A$ = LEFT$(A$, (LEN(A$) - 1)): GOTO around 'rightmost

IF B$ = CHR$(Q) THEN A$ = A$ + CHR$(Q): LOCATE 1, 44, 0: PRINT A$;    'cursor flash

NEXT Q: GOTO around

delete1:

IF A$ = "" THEN BEEP: GOTO around

A$ = LEFT$(A$, (LEN(A$) - 1))

LOCATE 1, 44, 0: PRINT A$; : COLOR 7, 7: PRINT " ": COLOR 0, 7

RETURN

cursorflash:

V = LEN(A$)

LOCATE 1, 43, 1: COLOR 0, 7: PRINT A$

LOCATE 1, 43 + V, 0: COLOR 14, 15: PRINT CHR$(219)

FOR D = 1 TO 5: NEXT D

COLOR 7, 7: LOCATE 1, 43 + V, 0: PRINT "K": RETURN

around2:

LOCATE 1, 43, 0: COLOR 7, 7: PRINT "                                    "

GOSUB syntchek                           'check syntax and if passed

fromeditor:

IF errrr = 1 AND ED = 1 THEN GOTO around

IF ED = 1 THEN GOTO bactoeditor          'back to the editor we go

LOCATE T * 2 + 2, X, 1: COLOR 11, 0      'save and load at this point

IF errrr = 1 AND ED = 0 THEN GOTO around

PRINT LEFT$(A$, 10)                      'so, print only 10 chars of Astring

Z0$ = Z0$ + A$ + "?": GOSUB loaddisk

CLOSE #1

IF A$ = "EXIT" THEN BLO = 0: GOTO yippee

NEXT T: NEXT X

yippee:

IF ZNY = 1 THEN RESET

LOCATE 1, 13, 0: COLOR 0, 7: PRINT "   want to edit any on this page? ";

COLOR 9, 3: PRINT "Y";: COLOR 0, 7

PRINT"/";: COLOR 9, 3: PRINT "N";: COLOR 0, 7

PRINT ", or program ";: COLOR 9, 3: PRINT "C";

COLOR 0, 7: PRINT "ard ?   "

arounditt5:

B$ = INKEY$

IF B$ = CHR$(67) THEN BLO = 1: GOTO blowcard                            ' program card

IF B$ = CHR$(89) THEN GOTO edit                                'yes

IF B$ = CHR$(78) THEN GOSUB nextring: GOTO selecty             'no go back to zx

MENU = 5

IF MOUSEOFF = 1 THEN GOTO arounditt5

GOTO mousepos

edit:

LOCATE 1, 2, 1: COLOR 0, 7: INPUT "  please type in the bright white number next to the command in question."; X$

OPEN ZX1$ FOR INPUT AS #1

INPUT #1, ZX5$

CLOSE #1

A = VAL(X$): B = 0                 ' get the value of your input number

FOR G = 1 TO LEN(ZX5$)             ' start to scan the string read from drive

X$ = MID$(ZX5$, G, 1)              ' scan 1 char at a time.

IF X$ = "?" THEN B = B + 1         ' b increments 1 for every ? read

IF B = A THEN GOTO edit2           ' if the number read = your input no. jump

NEXT G                             ' if it does not, go back around and pick up another

edit2:

M2 = 0

FOR M = G - 1 TO 1 STEP -1          'now go back to find the previous ? or begining

X$ = MID$(ZX5$, M, 1)

IF X$ = "?" THEN GOTO edit3         'now g = ? ahead, string wanted between m and g"

M2 = M2 + 1

NEXT M                              ' m now equals 1

edit3:

IF M = 1 THEN GOTO edit4            ' no need to correct m if m equal,to 1 as mid$ can use this

M = M + 1                           ' correct, all the above.

edit4:

X$ = MID$(ZX5$, M, M2)           ' x$ now has the string in question.

Y$ = ""

IF M = 1 THEN GOTO edit5         ' if x string the first word of zx5, then skip behind.

Y$ = LEFT$(ZX5$, M - 1)            ' now split the main string into 2 peices

' first, the bit behind X string.

edit5:

Z$ = ""

IF M + M2 + 1 = LEN(ZX5$) THEN GOTO edit6

Z$ = RIGHT$(ZX5$, ((LEN(ZX5$) - (M + M2)) + 1))

'at this point, zx5 string is split into 3 seperate strings y,x and z strings.

'x string will now be sent to the editor only to be replaced a little later.

GOSUB obliterate: GOSUB nextring: XXX$ = X$: A$ = X$: ED = 1: GOTO around

bactoeditor:

' now recreate zx5 string with the new substring and its former substrings

' a string is now the new substring and x is the old, y is all left, and

' z is all of the right.

edit6:

ZX5$ = Y$ + A$ + Z$

' put our big string back together again ZX5 is now reassembled as the new

' string now save to disk and go back and clear the existing display, and

' redisplay zx5.

OPEN ZX1$ FOR OUTPUT AS #1

WRITE #1, ZX5$

CLOSE #1

ZNY = 1: GOSUB resethatch: GOTO extractfrdisk

selecty:

IF ZNY = 0 THEN GOTO cds

IF ZNY = 1 THEN GOTO nextnaround

cds:

NEXT ZX                      'once finished programming, test and exit

blowcard:

ZNY = 0

IF CH = 1 THEN ZX4 = 1: ZX5 = 2

IF CH = 2 THEN ZX4 = 1: ZX5 = 4

IF CH = 3 THEN ZX4 = 1: ZX5 = 6

IF CH = 4 THEN ZX4 = 1: ZX5 = 8

FOR ZX = ZX4 TO ZX5                     ' set up to go around and check each channel

IC1 = 0

IF ZX = 1 THEN ZX1$ = XX$ + "\DESC0A.DAT"

IF ZX = 2 THEN ZX1$ = XX$ + "\DESC0B.DAT"

IF ZX = 3 THEN ZX1$ = XX$ + "\DESC1A.DAT"

IF ZX = 4 THEN ZX1$ = XX$ + "\DESC1B.DAT"   ' reselect new string

IF ZX = 5 THEN ZX1$ = XX$ + "\DESC2A.DAT"   ' for the following

IF ZX = 6 THEN ZX1$ = XX$ + "\DESC2B.DAT"   ' channels

IF ZX = 7 THEN ZX1$ = XX$ + "\DESC3A.DAT"

IF ZX = 8 THEN ZX1$ = XX$ + "\DESC3B.DAT"

OPEN ZX1$ FOR INPUT AS #1               'open up the file data

INPUT #1, ZZ$                           'zz string now equals the fetched file

CLOSE #1

blowcard1:

REM COLOR 15, 0: PRINT ZZ$: PRINT : PRINT

M2 = 1

blowcard3:

FOR G = M2 TO LEN(ZZ$)                 'get the string len in saved version

X$ = MID$(ZZ$, G, 1)

IF X$ = "?" THEN GOTO blowcard2        'get position of a sub string

NEXT G: GOTO testforexit               'g points to a qmark between subs

'if finshed testing either go around

'again, or finish and store on hdrive.

blowcard2:

G = G - M2

G1$ = MID$(ZZ$, M2, G)                'mid string now equals sub string

FOR XC3 = 1 TO LEN(ZX1$)

IF MID$(ZX1$, XC3, 1) = "B" AND G1$ = "ICCR" THEN GOSUB error6: GOTO blowcard4

NEXT XC3

IF M2 = 1 AND G1$ = "ICCR" THEN IC1 = IC1 + 1

IF IC1 > 1 THEN GOSUB error5: GOTO blowcard4                     'more than 1 iccr statement error

IF M2 <> 1 AND G1$ = "ICCR" THEN GOSUB error7: GOTO blowcard4    'iccr not at beginning, error

'above means that iccr included in the power down instructions. error.

REM PRINT G1$; "  "; G; " ";        'g1 string is a sub of zx1 string.

M2 = M2 + G + 1

GOTO blowcard3                      'around to test for more subs of zx1 string.

savezstring:

OPEN ZX1$ FOR OUTPUT AS #1

WRITE #1, ZZ$

CLOSE #1

RETURN

testforexit:

IF RIGHT$(ZZ$, 5) <> "EXIT?" THEN GOSUB error8: GOTO blowcard4

NEXT ZX             'now translate these instructions into

GOSUB savezstring   'machine code ready to be transferred later

'to the Z80 when it recieves a call.

biinary:

FOR ZX = ZX4 TO ZX5

IF ZX = 1 THEN ZX1$ = XX$ + "\DESC0A.DAT": XX4$ = XX$ + "\DES0ABI.DAT"

IF ZX = 2 THEN ZX1$ = XX$ + "\DESC0B.DAT": XX4$ = XX$ + "\DES0BBI.DAT"

IF ZX = 3 THEN ZX1$ = XX$ + "\DESC1A.DAT": XX4$ = XX$ + "\DES1ABI.DAT"

IF ZX = 4 THEN ZX1$ = XX$ + "\DESC1B.DAT": XX4$ = XX$ + "\DES1BBI.DAT"                                  ' reselect new string

IF ZX = 5 THEN ZX1$ = XX$ + "\DESC2A.DAT": XX4$ = XX$ + "\DES2ABI.DAT"                                  ' for the following

IF ZX = 6 THEN ZX1$ = XX$ + "\DESC2B.DAT": XX4$ = XX$ + "\DES2BBI.DAT"                                  ' channels

IF ZX = 7 THEN ZX1$ = XX$ + "\DESC3A.DAT": XX4$ = XX$ + "\DES3ABI.DAT"

IF ZX = 8 THEN ZX1$ = XX$ + "\DESC3B.DAT": XX4$ = XX$ + "\DES3BBI.DAT"

OPEN ZX1$ FOR INPUT AS #1

INPUT #1, ZZ$                              'read 1st string from disk and

CLOSE #1                                   'store in zz$.

' delete file from drive

ZZZ$ = ""                                  'fresh data to be held in a new string zzz.

FOR XC = 1 TO LEN(ZZ$)

X$ = MID$(ZZ$, XC, 3)                       'find 2 characters and compare

X8$ = MID$(ZZ$, XC, 5)

IF X$ = "WAI" THEN ZZZ$ = ZZZ$ + CHR$(ASC("W") OR &H80) + CHR$(ASC("A") OR &H80): GOTO wa

IF X$ = "SEN" THEN ZZZ$ = ZZZ$ + CHR$(ASC("S") OR &H80) + CHR$(ASC("E") OR &H80): GOTO se

IF X$ = "ALT" THEN ZZZ$ = ZZZ$ + CHR$(ASC("A") OR &H80) + CHR$(ASC("L") OR &H80): XY = 4: GOTO al

IF X$ = "SHI" THEN ZZZ$ = ZZZ$ + CHR$(ASC("S") OR &H80) + CHR$(ASC("H") OR &H80): XY = 6: GOTO al

IF X$ = "CNT" THEN ZZZ$ = ZZZ$ + CHR$(ASC("C") OR &H80) + CHR$(ASC("N") OR &H80): XY = 6: GOTO al

IF X$ = "RES" THEN ZZZ$ = ZZZ$ + CHR$(ASC("R") OR &H80) + CHR$(ASC("E") OR &H80): XY = 5: GOTO rel

IF X8$ = "DWNAR" THEN ZZZ$ = ZZZ$ + CHR$(ASC("D") OR &H80) + CHR$(ASC("W") OR &H80): XY = 6: GOTO al

IF X$ = "UPA" THEN ZZZ$ = ZZZ$ + CHR$(ASC("U") OR &H80) + CHR$(ASC("P") OR &H80): XY = 5: GOTO al

IF X$ = "ENT" THEN ZZZ$ = ZZZ$ + CHR$(ASC("E") OR &H80) + CHR$(ASC("N") OR &H80): XY = 5: GOTO rel

IF X$ = "POW" THEN ZZZ$ = ZZZ$ + CHR$(ASC("P") OR &H80) + CHR$(ASC("O") OR &H80): XY = 4: GOTO rel

IF X$ = "ICC" THEN ZZZ$ = ZZZ$ + CHR$(ASC("I") OR &H80) + CHR$(ASC("C") OR &H80): XY = 4: GOTO rel

IF X$ = "HOL" THEN ZZZ$ = ZZZ$ + CHR$(ASC("H") OR &H80) + CHR$(ASC("O") OR &H80): GOTO ho

IF X$ = "PGU" THEN ZZZ$ = ZZZ$ + CHR$(ASC("P") OR &H80) + CHR$(ASC("U") OR &H80): XY = 5: GOTO al

IF X$ = "PGD" THEN ZZZ$ = ZZZ$ + CHR$(ASC("P") OR &H80) + CHR$(ASC("D") OR &H80): XY = 6: GOTO al

IF X$ = "NUM" THEN ZZZ$ = ZZZ$ + CHR$(ASC("N") OR &H80) + CHR$(ASC("U") OR &H80): XY = 4: GOTO al

IF X$ = "CAP" THEN ZZZ$ = ZZZ$ + CHR$(ASC("C") OR &H80) + CHR$(ASC("A") OR &H80): XY = 5: GOTO al

IF X$ = "TAB" THEN ZZZ$ = ZZZ$ + CHR$(ASC("T") OR &H80) + CHR$(ASC("A") OR &H80): XY = 4: GOTO al

IF X$ = "ESC" THEN ZZZ$ = ZZZ$ + CHR$(ASC("E") OR &H80) + CHR$(ASC("S") OR &H80): XY = 4: GOTO al

IF X$ = "EXI" THEN ZZZ$ = ZZZ$ + CHR$(ASC("E") OR &H80) + CHR$(ASC("X") OR &H80): XY = 4: GOTO rel

eeexxxiiittt:

NEXT XC

REM COLOR 0, 0: COLOR 15, 0: LOCATE (ZX * 2), 2, 0: PRINT ZZZ$

OPEN XX4$ FOR OUTPUT AS #1

WRITE #1, ZZZ$

CLOSE #1: NEXT ZX: GOTO distinct             ' end of the .DAT file. get another,

' else set up distinctive ring translations

wa:

FOR DF = XC + 5 TO LEN(ZZ$)

X$ = MID$(ZZ$, DF, 1)

IF ASC(X$) = 63 THEN GOTO wa3                      'qmark, so jump

IF ASC(X$) = 44 THEN GOTO wa3                      'comma. so jump

IF ASC(X$) >= 48 OR ASC(X$) <= 57 THEN GOTO wa3    'numeric, so jump

wa2:

NEXT DF: GOTO eeexxxiiittt

wa3:

IF ASC(X$) = 63 AND DF = LEN(ZZ$) THEN X$ = CHR$(ASC("*") OR &H80)

ZZZ$ = ZZZ$ + CHR$(ASC(X$) OR &H80)

IF ASC(X$) = 63 THEN GOTO eeexxxiiittt ELSE GOTO wa2

se:

FOR DF = XC + 7 TO LEN(ZZ$)

X$ = MID$(ZZ$, DF, 1)

IF X$ = ")" AND DF = LEN(ZZ$) - 1 THEN ZZZ$ = ZZZ$ + CHR$(ASC("*") OR &H80): GOTO eeexxxiiittt

IF X$ = ")" AND DF <> LEN(ZZ$) - 1 THEN ZZZ$ = ZZZ$ + CHR$(ASC("?") OR &H80): GOTO eeexxxiiittt

ZZZ$ = ZZZ$ + CHR$(ASC(X$) OR &H80)

NEXT DF: GOTO eeexxxiiittt

al:

FOR DF = XC + XY TO LEN(ZZ$)

X$ = MID$(ZZ$, DF, 1)

IF X$ = "?" THEN GOTO al2

ZZZ$ = ZZZ$ + CHR$(ASC(X$) OR &H80)

NEXT DF: GOTO eeexxxiiittt

al2:

IF DF = LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("*") OR &H80): GOTO eeexxxiiittt

IF DF <> LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("?") OR &H80): GOTO eeexxxiiittt

rel:

IF MID$(ZZ$, XC + XY, 1) = "?" AND (XC + XY) <> LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("?") OR &H80): GOTO eeexxxiiittt

IF MID$(ZZ$, XC + XY, 1) = "?" AND (XC + XY) = LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("*") OR &H80): GOTO eeexxxiiittt

ho:

FOR DF = XC + 6 TO LEN(ZZ$)

X1$ = MID$(ZZ$, DF, 1)

IF X1$ = ")" THEN GOTO ho2

IF X1$ = "?" AND DF = LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("*") OR &H80): GOTO eeexxxiiittt

IF X1$ = "?" AND DF <> LEN(ZZ$) THEN ZZZ$ = ZZZ$ + CHR$(ASC("?") OR &H80): GOTO eeexxxiiittt

ZZZ$ = ZZZ$ + CHR$(ASC(X1$) OR &H80)

ho2:

NEXT DF: GOTO eeexxxiiittt

blowcard4:

GOSUB savezstring: GOTO extractfrdisk      'test for exit and if passed store.

syntchek:                            'all this a subroutine

RESTORE daytes

C = 1: COLOR 0, 7                  '19 commands to compare with

DO WHILE C < 20

READ e$                            ' get command C

C = C + 1

C$ = LEFT$(e$, 2)                'use only leftmost 2 chars of read string

X$ = LEFT$(A$, 2)                'use only leftmost 2 chars of aye string

IF C$ = X$ THEN GOTO testit      'there has been a match, so test further

LOOP

RESTORE daytes: GOTO error2                    'there has been no match, illegal function.

testit:

RESTORE daytes                    'restore the data pointer to the start

IF X$ = "WA" THEN GOTO wayte      'A$ equals wayte

IF X$ = "SE" THEN GOTO send       'A$ equals send$

IF X$ = "AL" THEN GOTO alt        'A$ equals alt

IF X$ = "SH" THEN GOTO shift      'A$ equals shift

IF X$ = "CN" THEN GOTO control    'A$ equals cntrl

IF X$ = "RE" THEN GOTO reeest     'A$ equals reset

IF X$ = "DW" THEN GOTO dwnar      'A$ equals downarrow

IF X$ = "UP" THEN GOTO uparrow    'A$ equals uparrow

IF X$ = "EN" THEN GOTO enter      'A$ equals enter

IF X$ = "PO" THEN GOTO power      'A$ equals pow0

IF X$ = "IC" THEN GOTO iccr       'A$ equals iccr

IF X$ = "HO" THEN GOTO hold       'A$ equals hold

IF X$ = "PG" THEN GOTO page       'A$ equals either pageup or down

IF X$ = "NU" THEN GOTO num        'A$ equals numlock

IF X$ = "CA" THEN GOTO caps       'A$ equals caps

IF X$ = "TA" THEN GOTO tabby      'A$ equals tab

IF X$ = "ES" THEN GOTO esscaype   'A$ equals eeascape

GOTO eexit                        'A$ equals eexit

wayte:

COLOR 0, 7

X$ = LEFT$(A$, 5)

IF X$ <> "WAIT-" THEN GOTO error2  'there has been a syntax error jump

FOR R = 6 TO LEN(A$)               'test for numerics

X$ = MID$(A$, R, 1)

IF X$ = "," THEN R = R - 6: GOTO nestt

NEXT R

'error occurred jump

error2:

errrr = 1: LOCATE 1, 6, 0: PRINT "     syntax error.      "

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\SYNTAX.WAV"

DELAY 1: GOSUB nextring: RETURN

error3:

errrr = 1: LOCATE 1, 6, 0: PRINT "Please insert a comma before the last bracket"

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\COMMAL.WAV"

DELAY 2: GOSUB nextring: RETURN

error4:

errrr = 1: LOCATE 1, 6, 0: PRINT "Please insert a comma after the first bracket"

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\COMMAA.WAV"

DELAY 2: GOSUB nextring: RETURN

error5:

errrr = 1: LOCATE 1, 5, 0: PRINT "only 1 ICCR command allowed per Ring Channel  "

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\ONEICR.WAV"

DELAY 4: GOSUB nextring: RETURN

error6:

errrr = 1: LOCATE 1, 5, 0: PRINT "no need for ICCR to be included in power down instructions.     "

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\NONEED.WAV"

DELAY 4: GOSUB nextring: RETURN

error7:

errrr = 1: LOCATE 1, 5, 0: PRINT "ICCR not at the begining of the power up instructions.          "

SHELL "F:\DESCATOC\PLANY F:\DESCATOC\ICCRMID.WAV"

DELAY 4: GOSUB nextring: RETURN

error8:

errrr = 1: LOCATE 1, 5, 0: PRINT "          EXIT command not at the end of this set of instructions.        "

DELAY 4: GOSUB nextring: RETURN

nestt:

G$ = ""

R = R - 1: I = R + 8         ' R now exactly the lenght of 1st No.

FOR M = 6 TO 6 + R           ' I now points to the start of 2nd No.

X$ = MID$(A$, M, 1)          ' test for numerics

IF ASC(X$) < 48 OR ASC(X$) > 57 THEN GOTO error2    'not numerical, jump

G$ = G$ + X$

NEXT M                       '1st number passed

H$ = ""

FOR M = I TO LEN(A$)

X$ = MID$(A$, M, 1)

IF ASC(X$) < 48 OR ASC(X$) > 57 THEN GOTO error2    'not numerical, jump

H$ = H$ + X$

NEXT M

IF LEFT$(H$, 1) = "0" AND LEFT$(G$, 1) = "0" THEN GOTO error2

RETURN                          ' the command has passed,save to disk and display

send:

D = 0

X$ = LEFT$(A$, 7)

IF LEFT$(A$, 6) <> "SEND$(" THEN GOTO error2

IF X$ <> "SEND$(," THEN GOTO error4

FOR M = 8 TO LEN(A$)

X$ = MID$(A$, M, 1)

IF ASC(X$) = 44 THEN GOTO comma

IF ASC(X$) = 41 THEN GOTO closedbrak

arnd:

NEXT M: GOTO closedbrak

comma:

D = D + 1                      ' 1 is the equiv of 1 comma in the statement

GOTO arnd

closedbrak:

IF RIGHT$(A$, 1) <> ")" THEN GOTO error2

IF RIGHT$(A$, 2) <> ",)" THEN GOTO error3

IF D = 0 THEN GOTO error3

RETURN                         ' more than 1 command and no errors, ret.

alt:

e = 0

X$ = LEFT$(A$, 4)

IF X$ <> "ALT-" THEN GOTO error2

C = 5: D = 0: GOSUB testx       'upon ret, it test ok, so store and display.

IF e = 1 THEN GOTO error2       ' if there was an error then ret

RETURN                          'no errors, so display and store.

shift:

e = 0: G$ = "SHIFT-"

shift2:

D = 0

X$ = LEFT$(A$, 6)

IF X$ <> G$ THEN GOTO error2

C = 7: D = 0: GOSUB testx

IF e = 1 THEN GOTO error2

RETURN

control:

e = 0: G$ = "CNTRL-": GOTO shift2

reeest:

X$ = LEFT$(A$, 5)

IF LEN(A$) > 5 THEN GOTO error2

IF X$ <> "RESET" THEN GOTO error2

RETURN                              'no errors, so store and display

dwnar:

C = 7: X$ = LEFT$(A$, 6)

IF X$ <> "DWNAR-" THEN GOTO error2

testy:

FOR M = C TO LEN(A$)                            'test for numeric values

X$ = MID$(A$, M, 1)

IF ASC(X$) = 48 AND MID$(A$, (LEN(A$) - 1), 1) = "," THEN GOTO testyyy

IF ASC(X$) = 48 AND MID$(A$, (LEN(A$) - 1), 1) <> "-" THEN GOTO testyy

testyyy:

IF ASC(X$) < 49 OR ASC(X$) > 57 THEN GOTO error2

testyy:

NEXT M: RETURN                       'no errors, so store and display

testx:                                'test there are chars after the command

e = 0

M = LEN(LEFT$(A$, (C - 1)))

IF D = 1 THEN GOTO testx3

IF M <> LEN(A$) THEN RETURN

GOTO testx2

testx3:

IF M = LEN(A$) THEN RETURN

testx2:

e = 1

RETURN                              ' there are no numeric errors, so go back

uparrow:

X$ = LEFT$(A$, 5)

IF X$ <> "UPAR-" THEN GOTO error2

C = 6: GOSUB testy: RETURN

enter:

D = 1

X$ = LEFT$(A$, 5)

IF X$ <> "ENTER" THEN GOTO error2

C = 6: GOSUB testx                 'test for no chars appearing after command

IF e = 1 THEN GOTO error2

RETURN                              'success, save and display

power:

X$ = LEFT$(A$, 4)

IF X$ <> "POW0" THEN GOTO error2

D = 1: C = 5: GOSUB testx           'check there are no chars after command

IF e = 1 THEN GOTO error2

RETURN                              'success, save and display

iccr:

X$ = LEFT$(A$, 4)

IF X$ <> "ICCR" THEN GOTO error2

D = 1: C = 5: GOSUB testx           'check there are no chars after command

IF e = 1 THEN GOTO error2

RETURN                              'success, save and display

hold:

IF LEFT$(A$, 6) <> "HOLD-(" THEN GOTO error2

FOR M = 7 TO LEN(A$)

X$ = MID$(A$, M, 2)

IF X$ = ")," THEN GOTO another1      ' ), found continue to test for numeric

NEXT M: GOTO error2                  ' ), not found error.

another1:

M = M + 2: C = M: GOSUB testy: RETURN           'no errors, so store and display

page:

X$ = LEFT$(A$, 3)                    'determine wether or pu or pd

IF X$ = "PGU" THEN GOTO pgu

IF X$ = "PGD" THEN GOTO pgd

pgu:

X$ = LEFT$(A$, 5)

IF X$ <> "PGUP-" THEN GOTO error2    'check syntax of the first expression

C = 6: GOSUB testy

RETURN

pgd:

X$ = LEFT$(A$, 6)

IF X$ <> "PGDWN-" THEN GOTO error2    'check syntax of the first expression

C = 7: GOSUB testy

RETURN

num:

X$ = LEFT$(A$, 4)

IF X$ <> "NUM-" THEN GOTO error2

L = 5

testz:

IF LEN(A$) <> L THEN GOTO error2                 'test for 0 or 1

X$ = RIGHT$(A$, 1)

IF ASC(X$) = 49 OR ASC(X$) = 48 THEN RETURN      'all tests past, display and store

GOTO error2

caps:

X$ = LEFT$(A$, 5)

IF X$ <> "CAPS-" THEN GOTO error2

L = 6: GOTO testz

tabby:

X$ = LEFT$(A$, 4)

IF X$ <> "TAB-" THEN GOTO error2

C = 5: GOTO testy

esscaype:

X$ = LEFT$(A$, 4)

IF X$ <> "ESC-" THEN GOTO error2

C = 5: GOTO testy

eexit:

X$ = LEFT$(A$, 4)

IF X$ <> "EXIT" THEN GOTO error2

D = 1: C = 5: GOSUB testx

IF e = 1 THEN GOTO error2

RETURN

loaddisk:

WRITE #1, Z0$

RETURN

enddisk:

CLOSE #1

END

numbers1t9:

FOR N = 0 TO 60 STEP 20

FOR A = 1 TO 9               ' numbers 1 to 9

COLOR 15, 6: LOCATE A * 2 + 2, X + N, 0: PRINT CHR$(48 + A)

NEXT A: NEXT N: RETURN

line3:          'Adds 2 x char 205 to top left and bottom left of columbs

F = 0

DO WHILE F < 2

COLOR 0, 6

FOR A = 4 TO 64 STEP 20

LOCATE Y, A, 0: PRINT CHR$(205); CHR$(205)

NEXT A: F = F + 1: Y = Y + 20: LOOP: RETURN

blankline:

COLOR 15, 0: L = 3: M = 177: GOSUB partof: COLOR 11, 14: PRINT CHR$(186);

L = 70: M = 32: GOSUB partof: PRINT CHR$(186); : COLOR 8, 8: PRINT CHR$(178);

PRINT CHR$(178); : COLOR 15, 0: L = 3: M = 177: GOSUB partof

RETURN

blankline2:

'creates the lines for the program screen

FOR J = 3 TO 72 STEP 20

LOCATE X, J, 0: COLOR 0, 14: PRINT CHR$(Y); : PRINT "  "; : L = 10

GOSUB partof: PRINT CHR$(Z); : COLOR 8, 8: PRINT CHR$(178); CHR$(178): NEXT J

RETURN

clearpage:

COLOR 15, 0: LOCATE 3, 3, 0: L = 1595: M = 177: GOSUB partof: RETURN

partof:

F = 0

DO WHILE F < L

F = F + 1

PRINT CHR$(M);

LOOP

RETURN

printscreen:

SHELL "F:\DESCATOC\PRINTER"

RETURN

daytes:

DATA "WAIT","SEND","ALT","SHIFT","CNTRL","RESET","DWNAR","UPAR"

DATA "ENTER","POW0","ICCR","HOLD","PGDWN","PGUP","NUM","CAPS"

DATA "TAB","ESC","EXIT"

distinct:

OPEN "DESC1.DAT" FOR INPUT AS #1

INPUT #1, ZX1$

CLOSE #1

OPEN ZX1$ + "\IO.DAT" FOR INPUT AS #1

INPUT #1,XC$

CLOSE #1

IF XC$ = "" THEN GOTO noinstall  'if the Card is not installed, endit

XC1$ = LEFT$(XC$, 3)

XC2$ = "&H" + XC1$             'signal a HEX value

P = VAL(XC2$)                  'P has the base 10 value now

XC3$ = MID$(XC$,6,3)

XC2$ = "&H" + XC3$

P2 = VAL(XC2$)                 'P2 now has the second port value

'P2 is the equiv 0f P + 8

arundy1:

A = INP(P2)                    'prepare to read HALT status

X = (A AND &H04)               'test bit 2, the HALT wire

IF X <> &H00 THEN GOTO arundy1 'wait for the Z80 to be ready

OUT (P), CH                    'send the amount of chans to Z80

arundy2:

A = INP(P2)

X = (A AND &H06)               'test for BRDY and HALT only

IF X <> &H02 THEN GOTO arundy2 'wait for Z80

OUT (P2), &H01                 'signal BSTB acticve, Z80 reads in CH

FOR R = 1 TO 20: NEXT R        ' small delay

OUT (P2), &H03                 'signal BSTB inactive

FOR T = 0 TO CH                'set up no of chans loopy

CLS

COLOR 15, 0

PRINT "Please enable the ring pattern for channel ";T

PRINT

PRINT "press any key upon the first instance of sound."

PRINT

PRINT "this is not critical as the Card synchronises before"

PRINT "doing A to D conversion..."

arundit1:

IF INKEY$ <> "" THEN GOTO arundit2

GOTO arundit1

arundit2:

A = INP(P2)

X = (A AND &H06)                   'test HALT and BRDY only

IF X <> &H02 THEN GOTO arundit2

OUT (P2), &H01                  'signal BSTB active Z80 comes out of HALT

FOR R = 1 TO 20: NEXT R

OUT (P2), &H03                 'signal BSTB inactive

arundit3:

A = INP(P2)

X = (A AND &H04)                  'bit 2 HALT tested only

IF X <> &H00 THEN GOTO arundit3   'HALT not enable PC waits.

PRINT: PRINT

PRINT "The DESCATOC Card recognises this pattern as ...."

PRINT

arundit4:

A = INP(P2)

X = (A AND &H02)                  'test BRDY

IF X <> &H02 THEN GOTO arundit4:  'wait for Z80 to be ready

OUT (P2), &H01                    'signal BSTB active, Z80 fetches pattern data

FOR R = 1 TO 20: NEXT R

OUT (P2), &H03                    'signal BSTB inactive.

arundit5:

B$ = ""

FOR L = 1 TO 4                    '4 bytes to collect per ch

A = INP(P2)

X = (A AND &H04)                  'bit 2 HALT tested only

IF X <> &H00 THEN GOTO arundit5

arundit6:

A = INP(P2)                       'HALT enabled, test for ARDY

X = (A AND &H01)                  'bit 0, ARDY testing

IF X <> &H01 THEN GOTO arundit6   'Z80 not port ready

A = INP(P)                        'ARDY 1, HALT 0, so read byte in

PRINT "      ";HEX$(A);"h ";      'display the byte read

B$ = B$ + HEX$(A) + "h "          'save for disk storage later

OUT (P2), &H02                    'signal ASTB active, Z80 fetches next byte

FOR T = 1 TO 20: NEXT T

OUT (P2), &H03                    'signal ASTB inactive

NEXT L                            'get another Byte or exit

PRINT

PRINT  "    please press any key to continue.   "

arundit7:

IF INKEY$ <> "" THEN GOTO arundit7

IF T = 0 THEN ZX2$ = ZX1$ + "\CH0DATA.COV"

IF T = 1 THEN ZX2$ = ZX1$ + "\CH1DATA.COV"

IF T = 2 THEN ZX2$ = ZX1$ + "\CH2DATA.COV"

IF T = 3 THEN ZX2$ = ZX1$ + "\CH3DATA.COV"

OPEN ZX2$ FOR OUTPUT AS #1              'save B$ the ring pattern

WRITE #1, B$                            'data

CLOSE #1

NEXT T                            'do another ring ch or exit

PRINT: PRINT

PRINT " You have successfully set up the DESCATOC Card from scratch."

PRINT " The DESCATOC Card is now ready to take automated calls !"

PRINT

PRINT "***********************************************************"

PRINT "*                                                                                                                  *"

PRINT "*                 DO YOU NOW WISH TO:                                                       *"

PRINT "*                                                                                                                  *"

PRINT "*                                                                                                                  *"

PRINT "*           a) Continue Batch Job to DOS/W95                                                *"

PRINT "*                                                                                                                  *"

PRINT "*           b) Power off and wait for Calls                                                        *"

PRINT "*              (no harm if you have seen the W95 screen)                                 *"

PRINT "*                                                                                                                  *"

PRINT "*                                                                                                                  *"

PRINT "*                  Please press a or b.                                                                   *"

PRINT "*                                                                                                                  *"

PRINT "***********************************************************"

arundit8:

A$ = INKEY$

IF A$ = "a" OR A$="A" THEN A2 = &HFE: GOTO arundit9

IF A$ = "b" OR A$="B" THEN A2 = &HFD: GOTO arundit9

IF A$ <> "b" OR A$ <> "B" THEN GOTO arundit8

IF A$ <> "a" OR A$ <> "A" THEN GOTO arundit8

GOTO arundit8

arundit9:

A = INP(P2)

X = (A AND &H06)        'test for HALT and BRDY only (bits 1 & 2)

IF X <> &H02 THEN GOTO arundit9

OUT (P), A2             ' FE = continue, FD = power off reset

OUT (P2), &H01          ' signal BSTB active, Z80 reads Byte in

FOR T = 1 TO 20: NEXT T

OUT (P2), &H03          ' signal BSTB inactive

IF A2 = &HFD THEN

PRINT: PRINT

PRINT "    Thank You and Goodbye !    "

DELAY 3

SHELL ZX1$ + "\PLANY GOODBYE.WAV"

DELAY 90             'powers off here.

END IF

PRINT: PRINT

PRINT "       Continuing Batch Job....       "

PRINT

PRINT "       Thank You and Goodbye !    "

SHELL ZX1$ + "\PLANY GOODBYE.WAV"

END         'DESC1.EXE TERMINATED!!!!

noinstall:

IF MOUSEOFF = 0 THEN

REG 1, &H02

CALL INTERRUPT &H33

END IF

COLOR 15, 15: CLS           ' Blank out screen

COLOR 6, 15

LOCATE 11, 9, 0: PRINT " This program will re-run automatically upon next boot, once"

LOCATE 12, 9, 0: PRINT " you have installed the Card, type the amount of channels when "

LOCATE 13, 9, 0: PRINT " prompted.  Now type  a 1,  then U  now  type C the program"

LOCATE 14, 9, 0: PRINT " will  store your  data in  coded form on the Hard Disk but"

LOCATE 15, 9, 0: PRINT " will now also ask you to program the ring patterns in. "

LOCATE 16, 9, 0: PRINT " you will need your telephone connected to the same line"

LOCATE 17, 9, 0: PRINT " as your Modem, and someone to ring up for you all the "

LOCATE 18, 9, 0: PRINT " phone numbers associated with your line. once the sounds"

LOCATE 19, 9, 0: PRINT " that are generated have been listened too, this program "

LOCATE 20, 9, 0: PRINT " will terminate permanently or until you have a total power"

LOCATE 21, 9, 0: PRINT " failure whereby it will re run automatically, just re-apply"

LOCATE 22, 9, 0: PRINT " what this screen is stating, and all the data will be restored."

LOCATE 24, 30, 0: PRINT " Please press any Key."

againn:

DO UNTIL INKEY$ <> ""

LOCATE 8, 30, 0

COL = INT(RND * 15) + 1

FOR G = 1 TO 5000: NEXT G

COLOR COL, 0: PRINT " Z80 Card not installed. "

LOOP

CLS : LOCATE 12, 9, 0

COLOR 15, 0

PRINT " Were out of here... continuing bootstrap...."

PRINT: PRINT

PRINT "               GOODBYE !               "

SHELL ZX1$ + "\PLANY GOODBYE.WAV": END